3GPP TSG-RAN5 Meeting # 103	R5-242320

# Fukuoka, Japan,  20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3434 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR NTN: Addition of GSO Test Model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN5#102 have finalised the technical specifications to convey NR NTN GSO testing demands. A corresponding test model definition along with test method have to be added to enable the implementation of the NR NTN test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced new clauses relevant for NR NTN testing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR NTN test cases cannot be implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.5(new), 7.5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

ASP	Abstract Service Primitive

ATS	Abstract Test Suite

AWGN	Additive White Gaussian Noise

BWP	Bandwidth Part

CA	Carrier Aggregation

CBRA	Contention Based Random Access

CC	Component Carriers

CCE	Control Channel Element

CFRA	Contention Free Random Access

CORESET	Control Resource Set

CSS	Common Search Space

DAI	Downlink Assignment Index

DCI	Downlink Control Information

DL	Downlink

DL-SCH	Downlink Shared Channel

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

EHC	Ethernet Header Compression

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

FDD	Frequency Division Duplex

FD-FDD	Full-Duplex FDD

FR1	Frequency Range 1

FR2	Frequency Range 2

GSO	Geosynchronous Orbit

HD-FDD	Half-Duplex FDD

HO	Handover

H-SFN	Hyper SFN

ICS	Implementation Conformance Statement

IUT	Implementation Under Test

IXIT	Implementation eXtra Information for Testing

LBT	Listen Before Talk

LSB	Least Significant Bit

MCG	Master Cell Group

MN	Master Node

MSB	Most Significant Bit

MUSIM	Multi-Universal Subscriber Identity Module

NCD-SSB	Non Cell Defining SSB

NE-DC	NR-E-UTRA Dual Connectivity

NGSO	Non-Geosynchronous Orbit

NR	NR Radio Access

NR-DC	NR-NR Dual Connectivity

NTN	Non-Terrestrial Network

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PEI	Paging Early Indication

PEI-O	Paging Early Indication-Occasion

PF	Paging Frame

PH	Paging Hyperframe

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PSCell	Primary SCG Cell

PSS	Primary Synchronisation Signal

PTC	Parallel Test Component

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

RA	Random Access

RACH	Random Access Channel

RAR	Random Access Response

RAT	Radio Access Technology

RMSI	Remaining Minimum SI

SCell	Secondary Cell

SCG	Secondary Cell Group

SFN	System Frame Number

SN	Secondary Node

SRS	Sounding Reference Signal

SS	System Simulator

SSB	Synchronization Signal and PBCH block

SSS	Secondary Synchronisation Signal

TC	Test Case

TDD	Time Division Duplex

UDC	Uplink Data Compression

UL	Uplink

UL-SCH	Uplink Shared Channel

USS	UE-specific Search Space

UT	Upper Tester

VNG	Virtual Noise Generator

<<SKIPPED SECTIONS>>

## 5.5	Non-Terrestrial Network

The NR/5GC test models specified in clause 5.2 are re-used for NTN testing, with additional aspects specified hereafter.

The UE under test shall be pre-configured with a geographical position that will not change throughout the execution of the test case.

The system simulator (SS) shall implement one or several simulated NR NTN cell(s). An NR NTN cell is defined in clause 7.5.

<<SKIPPED SECTIONS>>

7.5	Non-Terrestrial Network

### 7.5.1	NTN cell configuration

An NR cell to be used in a Non-Terrestrial Network (NTN) test environment is an NR cell configured with NTN-specific configuration parameters. It is called an NR NTN cell.

In particular an NR NTN cell is configured to operate on an NTN-specific NR band and is broadcasting a system information block combination, defined in TS 38.508-1 [5] clause 4.4.3.1.2, that contains SIB19.

Typically, an NTN test case can be executed under one of the following two sub-NTN test environments:

-	the Geosynchronous Orbit (GSO) scenario, or

-	the Non-Geosynchronous Orbit (NGSO) scenario.

NOTE:	The NGSO test model is FFS in the present version of this specification.

7.5.2	NTN timing considerations

#### 7.5.2.1	General

DL and UL are frame aligned at the UL time synchronization reference point (RP). To accommodate the long propagation delays in NTN, the timing relationships are defined using a Common Timing Advance (Common TA) and two offsets: Koffset and Kmac, with Koffset ≥ TTA.

The various NTN timing relationships are illustrated in Figure 7.5.2.1-1.

Figure 7.5.2.1-1: NTN timing relationship parameters

#### 7.5.2.2	NTN timing advance

The Timing Advance (TA) formula, specified in TS 38.211 [19] clause 4.3.1, can be summarized as follows:

TTA = TTA,legacy + TTA,NTN

TTA,NTN = TTA,common + TTA,UE-specific

with:

-	TTA,legacy = (NTA + NTA,offset) * Tc: the (legacy) part of the TA formula that applies also to Terrestrial Networks,

-	TTA,common = NTA,adjcommon * Tc: the Common TA between the RP and the NTN payload,

-	TTA,UE-specific = NTA,adjUE * Tc: the TA corresponding to the service link delay.

The GSO test environment in TS 38.508-1 [5] specifies that both the serving satellite (NTN payload) and the UE under test have a static position during the lifetime of a test case execution. Therefore, the corresponding TTA is also static for a given NTN cell. TTCN provides to the SS the following inputs to enable the SS to calculate TTA:

Table 7.5.2.2-1: NTN timing advance - input parameters

| TTA element | Associated parameter(s) |  | Relationship |
| --- | --- | --- | --- |
| (in sec) | Protocol parameter | ASP field | (TS 38.213 [21] clause 4.2) |
| TTA,legacy |  | InitialValue integer (0..3846) | NTA = InitialValue * 16∙642μ |
| TTA,common | NTA,adjcommon | NtnCommonTA integer (0..66485757) | See below. |
| TTA,UE-specific | NTA,adjUE | NtnUeSpecificTA integer (0..66485757) | NTA,adjUE= 8* NtnUeSpecificTA See below. |
| NOTE: The time unit TC is defined in TS 38.211 [19] clause 4.1. |  |  |  |

From the TA-Info-r17 specified in TS 38.331 [16], only the value of ta-Common-r17 is defined in TS 38.508-1 [5] SIB19 for the GSO scenario. TTCN will forward this value to the SS as NtnCommonTA for the calculation by the SS of NTA,adjcommon according to the formula given in TS 38.213 [21] clause 4.2.

NOTE:	For the GSO scenario it is assumed that the SS uses the calculation: NTA,adjcommon = 8 * NtnCommonTA.

NTA,adjUE is calculated using the GSO serving satellite static position specified in TS 38.508-1 [5] SIB19's ephemeris data and the UE static position specified in TS 38.508-1 [5] clause 4.5C1. The end result is provided to the SS by TTCN. The calculation is provided hereafter:

The two-way propagation delay in sec between the GSO serving satellite and the non-moving UE is assumed to be constant at any time and is calculated according to the formula:

t=2*d/c

where c is the speed of light (299792458 m/sec) and d is the distance in meters between the serving satellite and the UE.

For two points in Euclidian geometry d in meters is derived by:

d=∆x+∆y+∆z

with

∆x=(xsat-xUE)2

∆y= (ysat-yUE)2

∆z=(zsat-zUE)2

and

xsat, ysat, zsat being serving satellite coordinates, in m

xUE, yUE, zUE being UE coordinates, in m

To use above formulas the positioning data provided in TS 38.508-1 [3] is first converted to the coordinates accordingly.

Example of the calculation of ∆x:

given UE latitude 25.08439333 and positionX-r17 of -16976014:

xsat= -16976014* 1.3 m = -22068818.2 m

xUE= -3025296.94 m

∆x=(-22068818.2 m-(-3025296.94 m))2=(-19043521.26 m)2= 362655701980071.94 m2

Following the above calculations, the final value t results in 0.24338972709427653 sec that yields NtnUeSpecificTA to have a value of 59771543.981895022, that is rounded to 59771544.

#### 7.5.2.3	NTN scheduling offsets

The NTN timing parameters listed in Table 7.5.2.3-1 come into play when scheduling data in an NTN cell.

Table 7.5.2.3-1: NTN timing parameters in an NTN cell

| NTN timing parameter | Definition | Reference |
| --- | --- | --- |
| UE-gNB RTT | TTA + Kmac | TS 38.321 [13] clause 3.1 |
| Kmac | k-Mac or 0 | TS 38.213 [21] clause 8.2 |
| Koffset | cellSpecificKoffset - Differential Koffset | TS 38.213 [21] clause 4.2 |

TTCN configures the NTN offsets Koffset and Kmac in the SS as part of the NTN cell configuration. The SS shall use those parameters to adjust the timing & scheduling of DL & UL.

#### 7.5.2.4	RRC connection release sequence

The sequence specified in clause 7.3.5.2 applies, with the following differences: T is set by the value of UE-gNB RTT ms in advance of RRCRelease.