3GPP TSG-RAN5 Meeting #103	R5-242333

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5304 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to E-UTRA RRC test case 8.1.2.15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The test purpose does not describe accurately what is the object of testing in this test case. - The Conformance requirement clause is missing one extract from TS 36.331 core specification. - The test procedure sequence does not describe accurately the test case details. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The test purpose has been updated. - The Conformance requirement clause has been updated. - The test procedure sequence clause has been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case specification will remain inaccurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.1.2.15	RRC connection establishment / Extended value, spare fields and non critical extensions in SI

8.1.2.15.1	Test Purpose (TP)

(1)

with { UE is powered on and receives system information }

ensure that {

when { system information messages include extended value / spare field / non-critical extension that is not defined in the version of the ASN.1 transfer syntax supported by the UE }

then { UE considers the extended value / spare field / non-critical extension as not comprehended, treats the system information messages as if those fields were absent and UE establishes an RRC connection }

}

8.1.2.15.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.2.2.3, 5.7.1, 5.7.3, 5.7.5 and A3.3.

[TS 36.331 clause 5.2.2.3]

The UE shall:

1>	ensure having a valid version, as defined below, of (at least) the following system information, also referred to as the 'required' system information:

2>	if in RRC_IDLE:

3>	if the UE is a NB-IoT UE:

4>	the MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB and SystemInformationBlockType1-NB as well as SystemInformationBlockType2-NB through SystemInformationBlockType5-NB, SystemInformationBlockType22-NB;

3>	else:

4>	the MasterInformationBlock and SystemInformationBlockType1 (or SystemInformationBlockType1-BR depending on whether the UE is a BL UE or the UE in CE) as well as SystemInformationBlockType2 through SystemInformationBlockType8 and SystemInformationBlockType24 (depending on support of the concerned RATs), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking when the UE is connected to EPC), SystemInformationBlockType25 (depending on support of E-UTRA/5GC);

2>	if in RRC_INACTIVE:

3>	the MasterInformationBlock and SystemInformationBlockType1 as well as SystemInformationBlockType2 through SystemInformationBlockType8 (depending on support of the concerned RATs), SystemInformationBlockType25;

2>	if in RRC_CONNECTED; and

2>	the UE is not a BL UE; and

2>	the UE is not in CE; and

2>	the UE is not a NB-IoT UE:

3>	the MasterInformationBlock, SystemInformationBlockType1 and SystemInformationBlockType2 as well as SystemInformationBlockType8 (depending on support of CDMA2000), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking when the UE is connected to EPC), SystemInformationBlockType25 (depending on support of E-UTRA/5GC);

2>	if in RRC_CONNECTED and T311 is running; and

2>	the UE is a BL UE or the UE is in CE or the UE is a NB-IoT UE;

3>	the MasterInformationBlock (or MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT), SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) and SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT), and for NB-IoT SystemInformationBlockType22-NB;

1>	delete any stored system information after 3 hours or 24 hours from the moment it was confirmed to be valid as defined in 5.2.1.3, unless specified otherwise;

1>	consider any stored system information except SystemInformationBlockType10, SystemInformationBlockType11, systemInformationBlockType12 and systemInformationBlockType14 (systemInformationBlockType14-NB in NB-IoT) to be invalid if systemInfoValueTag included in the SystemInformationBlockType1 (MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT) is different from the one of the stored system information and in case of NB-IoT UEs, BL UEs and UEs in CE, systemInfoValueTagSI is not broadcasted. Otherwise consider system information validity as defined in 5.2.1.3;

[TS 36.331 clause 5.7.1]

The generic error handling defined in the subsequent sub-clauses applies unless explicitly specified otherwise e.g. within the procedure specific error handling.

The UE shall consider a value as not comprehended when it is set:

-	to an extended value that is not defined in the version of the transfer syntax supported by the UE.

-	to a spare or reserved value unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/ reserved value.

The UE shall consider a field as not comprehended when it is defined:

-	as spare or reserved unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/ reserved field.

[TS 36.331 clause 5.7.3]

The UE shall, when receiving an RRC message on any logical channel:

1>	if the message includes a field that has a value that the UE does not comprehend:

2>	if a default value is defined for this field:

3>	treat the message while using the default value defined for this field;

2>	else if the concerned field is optional:

3>	treat the message as if the field were absent and in accordance with the need code for absence of the concerned field;

2>	else:

3>	treat the message as if the field were absent and in accordance with sub-clause 5.7.4;

[TS 36.331 clause 5.7.5]

The UE shall, when receiving an RRC message on any logical channel:

1>	if the message includes a field that the UE does not comprehend:

2>	treat the rest of the message as if the field was absent;

NOTE:	This clause does not apply to the case of an extension to the value range of a field. Such cases are addressed instead by the requirements in clause 5.7.3.

[TS 36.331 clause A.3.3]

Each PDU (message) type is specified in an ASN.1 section similar to the one shown in the example below.

-- /example/ ASN1START

RRCConnectionReconfiguration ::=	SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcConnectionReconfiguration-r8		RRCConnectionReconfiguration-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReconfiguration-r8-IEs ::= SEQUENCE {

-- Enter the IEs here.

...

}

-- ASN1STOP

Hooks for critical and non-critical extension should normally be included in the PDU type specification. How these hooks are used is further described in clause A.4.

Critical extensions are characterised by a redefinition of the PDU contents and need to be governed by a mechanism for protocol version agreement between the encoder and the decoder of the PDU, such that the encoder is prevented from sending a critically extended version of the PDU type, which is not comprehended by the decoder.

Critical extension of a PDU type is facilitated by a two-level CHOICE structure, where the alternative PDU contents are alternatives within the inner level c1 CHOICE. Spare alternatives (i.e., spare3 down to spare1 in this case) may be included within the c1 CHOICE. The number of spare alternatives to be included in the original PDU specification should be decided case by case, based on the expected rate of critical extension in the future releases of the protocol.

Further critical extension, when the spare alternatives from the original specifications are used up, is facilitated using the criticalExtensionsFuture in the outer level CHOICE.

In PDU types where critical extension is not expected in the future releases of the protocol, the inner level c1 CHOICE and the spare alternatives may be excluded, as shown in the example below.

-- /example/ ASN1START

RRCConnectionReconfigurationComplete ::= SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

rrcConnectionReconfigurationComplete-r8

RRCConnectionReconfigurationComplete-r8-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReconfigurationComplete-r8-IEs ::= SEQUENCE {

-- Enter the IEs here. --														-- Cond condTag

...

}

-- ASN1STOP

Non-critical extensions are characterised by the addition of new information to the original specification of the PDU type. If not comprehended, a non-critical extension may be skipped by the decoder, whilst the decoder is still able to complete the decoding of the comprehended parts of the PDU contents.

Non-critical extensions at locations other than the end of the message or other than at the end of a field contained in a BIT or OCTET STRING are facilitated by use of the ASN.1 extension marker "...". The original specification of a PDU type should normally include the extension marker at the end of the sequence of information elements contained.

Non-critical extensions at the end of the message or at the end of a field that is contained in a BIT or OCTET STRING are facilitated by use of an empty sequence that is marked OPTIONAL e.g. as shown in the following example:

-- /example/ ASN1START

RRCMessage-r8-IEs ::=					SEQUENCE {

field1									InformationElement1,

field2									InformationElement2,

nonCriticalExtension					SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

The ASN.1 section specifying the contents of a PDU type may be followed by a field description table where a further description of, e.g., the semantic properties of the fields may be included. The general format of this table is shown in the example below. The field description table is absent in case there are no fields for which further description needs to be provided e.g. because the PDU does not include any fields, or because an IE is defined for each field while there is nothing specific regarding the use of this IE that needs to be specified.

%PDU-TypeIdentifier% field descriptions

%field identifier% Field description.

%field identifier% Field description.

The field description table has one column. The header row shall contain the ASN.1 type identifier of the PDU type.

The following rows are used to provide field descriptions. Each row shall include a first paragraph with a field identifier (in bold and italic font style) referring to the part of the PDU to which it applies. The following paragraphs at the same row may include (in regular font style), e.g., semantic description, references to other specifications and/ or specification of value units, which are relevant for the particular part of the PDU.

The parts of the PDU contents that do not require a field description shall be omitted from the field description table.

8.1.2.15.3	Test description

8.1.2.15.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

UE:

None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

8.1.2.15.3.2	Test procedure sequence

Table 8.1.2.15.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | System information messages, that include fields and values that the UE does not comprehend, are broadcasted on BCCH and BCCH-BR. (See NOTE 1 & 2) | <-- | RRC: SYSTEM INFORMATION (BCCH and BCCH-BR) | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 1? | --> | RRCConnectionRequest | 1 | P |
| 4 | The SS transmits an RRCConnectionSetup message on Cell 1. | <-- | RRCConnectionSetup | - | - |
| 5 | Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the Attach procedure by including the ATTACH REQUEST message? The PDN CONNECTIVITY REQUEST message is piggybacked in ATTACH REQUEST. | --> | RRCConnectionSetupComplete NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST | 1 | P |
| 6-18 | Steps 5 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. | - | - | - | - |
| NOTE 1: MIB content enables testing the handling of spare field in UE up to Rel-16. Spare field testing is not applicable for Rel-12 UE because a spare field in MIB does not exist in Rel-12. NOTE 2: SIB-Type and SIB-Type-v12j0 in SystemInformationBlockType1 and SystemInformationBlockType1-BR are set to values in Rel-17 to check spare value, extended value and nonCriticalExtension in UE up to Rel-16. |  |  |  |  |  |

8.1.2.15.3.3	Specific message contents

Table 8.1.2.15.3.3-1: Message MasterInformationBlock (step 1, Table 8.1.2.15.3.2-1)

| Derivation Path: 36.508 Table 4.4.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock ::= SEQUENCE { |  |  |  |
| schedulingInfoSIB1-BR-r13 | 10 |  | when UE under test is CAT M1 |
|  | 0 |  |  |
| partEARFCN-17 CHOICE { |  |  |  |
| spare | ‘11’B |  |  |
| } |  |  |  |
| spare | ‘1’B | From Rel-8 to Rel-16, the lowest bit is considered spare according to TS 36.331 [17]. | For Rel-8 to Rel-16 |
| } |  |  |  |

Table 8.1.2.15.3.3-2: Message SystemInformationBlockType1 (step 1, Table 8.1.2.15.3.2-1)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE { |  | 3 entries for Rel-8 to Rel-15 2 entries for Rel-16 |  |
| si-Periodicity[1] | rf16 |  |  |
| sib-MappingInfo[1] |  | SIB2, SIB3 |  |
| si-Periodicity[2] | As per TS 36.508[18] clause 4.4.3.4 |  |  |
| sib-MappingInfo[2] | sibType3 |  |  |
| si-Periodicity[3] | rf32 |  | For Rel-8 to Rel-15 |
| sib-MappingInfo[3] SEQUENCE (SIZE (0..maxSIB-1)) OF { | 1 entry |  |  |
| SIB-Type[1] | sibType29-v1610 | For Rel-8 to Rel-15 UE, the value used for sibType29-v1610 is considered as a not comprehended, non-critically extended value according to TS 36.331 [17]. |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v890-IEs | For Rel-16 |
| lateNonCriticalExtension { |  |  |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v9e0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v10j0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v10l0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v10x0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v12j0-IEs |  |
| schedulingInfoExt-r12 (SIZE (1..maxSI-Message)) OF SEQUENCE { | 1 entry |  |  |
| si-Periodicity-r12[1] | rf32 |  |  |
| sib-MappingInfo-v12j0[1] SEQUENCE (SIZE (0..maxSIB-1)) OF { | 1 entry |  |  |
| SIB-Type-v12j0[1] | sibType32-v1700 | For Rel-12 to Rel-16 UE, the field used for sibType32-v1700 is considered as a not comprehended spare value according to TS 36.331 [17]. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v920-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1130-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1250-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1310-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1320-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1350-IEs |  |
|  |  | SystemInformationBlockType1-v1360-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1430-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1450-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1530-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1540-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1610-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1700-IEs |  |
| cellAccessRelatedInfo-NTN-r17 SEQUENCE { |  |  |  |
| cellBarred-NTN-r17 | notBarred | To simulate handling of a not comprehended nonCriticalExtension of a UE version < Rel-17. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.15.3.3-3: Message SystemInformationBlockType1-BR-r13 (step 1 when UE under test is CAT M1, Table 8.1.2.15.3.2-1)

| Derivation Path: 36.508 Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE { |  | 3 entries for Rel-13 to Rel-15 2 entries for Rel-16 |  |
| si-Periodicity[1] | rf16 |  |  |
| sib-MappingInfo[1] |  | SIB2, SIB3 |  |
| si-Periodicity[2] | As per TS 36.508[18] clause 4.4.3.4. |  |  |
| sib-Mappinginfo[2] | sibType3 |  |  |
| si-Periodicity[3] | rf32 |  | For Rel-13 to Rel-15 |
| sib-MappingInfo[3] SEQUENCE (SIZE (0..maxSIB-1)) OF { | 1 entry |  | For Rel-13 to Rel-15 |
| SIB-Type[1] | sibType29-v1610 | For Rel-13 to Rel-15 UE, the field used for sibType29-v1610 is considered as a not comprehended, non-critically extended value according to TS 36.331 [17]. |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v890-IE0073 | For Rel-16 |
| lateNonCriticalExtension { |  | SystemInformationBlockType1-v8h0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v9e0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v10j0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v10l0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v10x0-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v12j0-IEs |  |
| schedulingInfoExt-v12j0 (SIZE (1..maxSI-Message)) OF SEQUENCE { | 1 entry |  |  |
| si-Periodicity-r12[1] | rf32 |  |  |
| sib-MappingInfo-v12j0[1] SEQUENCE (SIZE (0..maxSIB-1)) OF { | 1 entry |  |  |
| SIB-Type-v12j0[1] | sibType32-v1700 | For Rel-16 UE, the field used for sibType32-v1700 is considered as a not comprehended spare value according to TS 36.331 [17]. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v920-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1130-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1250-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1310-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1320-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1350-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1360-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1430-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1450-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1530-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1540-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1610-IEs |  |
| nonCriticalExtension { |  | SystemInformationBlockType1-v1700-IEs |  |
| cellAccessRelatedInfo-NTN-r17 SEQUENCE { |  |  |  |
| cellBarred-NTN-r17 | notBarred | To simulate handling of a not comprehended nonCriticalExtension of a UE version < Rel-17. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |