3GPP TSG-RAN5 Meeting #103	R5-242338

# Fukuoka, Japan, 20th – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0345 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of initial conditions and RRC/NAS signalling in clause 5.3A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Test configurations are introduced in clause 5.2 and it shall be clarified for which configuration a particular procedure is applicable. - Information about the RRC state has been missing in the initial conditions of the test procedures. - There is a new procedure 5.4.14 for MCX communication release. - Editorial issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Applicability for test configuraions added to initial conditions of the test prcedures. - RRC state added to the initial conditions of the test procedures. - Use of new procedure 5.4.14 for MCX communication release. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.579-1 CRs 0356, 0357 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.3A	Generic test procedures for UE MCPTT operation

### 5.3A.1	MCPTT CO session establishment/modification without provisional responses other than 100 Trying

5.3A.1.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3A.1.2	Void

5.3A.1.3	Procedure

Table 5.3A.1.3-1: MCPTT CO session establishment/modification without provisional responses other than 100 Trying

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 2 | Check: Does the UE (MCPTT client) send a SIP INVITE requesting the establishment/modification of an MCPTT call? | --> | SIP INVITE | - | P |
| - | EXCEPTION: In case of session establishment in parallel to the steps below a dedicated bearer gets established as described in clause 5.4.3. | - | - | - | - |
| 3 | The SS sends a SIP 100 Trying | <-- | SIP 100 (Trying) | - | - |
| 4 | The SS (MCPTT server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | Check: Does the UE (MCPTT client) send a SIP ACK to acknowledge the session establishment/modification? | --> | SIP ACK | - | P |
| - | EXCEPTION: Steps 6a1 describes behaviour that depends on the test case requirements; the "lower case letter" identifies a step sequence that takes place if the UE requests implicit floor control in step 2 (i.e. the "mc_implicit_request" fmtp attribute included in the SDP offer and the SS responded with the "mc_implicit_request" fmtp attribute included and the “mc_granted” fmtp attribute not present in the SDP answer. (NOTE 1) | - | - | - | - |
| 6a1 | The SS (MCPTT server) sends a Floor Granted message. | <-- | Floor Granted | - | - |
| NOTE 1: Possibilities in SDP-offer/answer depend on the test case requirements a. UE sends SDP offer with media description for floor control but without implicit floor request b. UE sends SDP offer with media description for floor control and with implicit floor request i. SDP answer from SS contains “mc_implicit_request” and “mc_granted” (Floor is implicitly granted) ii. SDP answer from SS contains “mc_implicit request” and but no “mc_granted” (Floor needs to be explicitly granted at step 6a1) iii. SDP answer from SS contains no “mc_implicit_request”and no “mc_granted” (the UE needs to explicitly request the floor) c. UE sends SDP offer without media description for floor control |  |  |  |  |  |

5.3A.1.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure with the following clarifications:

Table 5.3A.1.4-1: SIP INVITE (step 2, Table 5.3A.1.3-1)

Derivation Path: Table 5.5.2.5.1-1, condition MCPTT

Table 5.3A.1.4-2: SIP 200 (OK) (step 4, Table 5.3A.1.3-1)

Derivation Path: Table 5.5.2.17.1.2-1, condition INVITE-RSP and MCPTT

### 5.3A.2	Void

### 5.3A.3	MCPTT CO call establishment using a pre-established session

5.3A.3.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

A pre-established session is established.

5.3A.3.2	Void

5.3A.3.3	Procedure

Table 5.3A.3.3-1: MCPTT CO call establishment using a pre-established session

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 2 | Check: Does the UE (MCPTT client) send a SIP REFER message to request the establishment of an MCPTT call using a pre-established session? | --> | SIP REFER | - | P |
| 3 | The SS (MCPTT server) responds with a SIP 200 (OK) message indicating that the MCPTT call has been established | <-- | SIP 200 (OK) | - | - |
| 4 | The SS sends a Connect message | <-- | Connect | - | - |
| 5 | Check: Does the UE (MCPTT client) send an Acknowledge message in response to the Connect message? | --> | Acknowledge | - | P |

5.3A.3.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3A.4	MCPTT CO call release keeping the pre-established session

5.3A.4.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state and a call is established using a pre-established session.

5.3A.4.2	Void

5.3A.4.3	Procedure

Table 5.3A.4.3-1: MCPTT CO call release keeping the pre-established session

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT client) send a SIP REFER message with method “BYE” to release the MCPTT session and keep the pre-established session? | --> | SIP REFER | - | P |
| 2 | The SS (MCPTT server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 3 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to release the RRC connection keeping the dedicated bearer. | - | - | - | - |

5.3A.4.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3A.4.4-1: SIP REFER (step 1, Table 5.3A.4.3-1)

Derivation Path: Table 5.5.2.12-1, condition METHOD-BYE

Table 5.3A.4.4-2: SIP 200 (OK) (step 2, Table 5.3A.4.3-1)

Derivation Path: Table 5.5.2.17.1.2-1, condition REFER-RSP

### 5.3A.5	MCPTT CT call release keeping the pre-established session

5.3A.5.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state and a call is established using a pre-established session.

5.3A.5.2	Void

5.3A.5.3	Procedure

Table 5.3A.5.3-1: MCPTT CT call release keeping the pre-established session

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS (MCPTT server) releases the call by sending a Disconnect message | <-- | Disconnect | - | - |
| 2 | Check: Does the UE (MCPTT client) send an Acknowledge message to accept the release of the call? | --> | Acknowledge | - | P |
| 3 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to release the RRC connection keeping the dedicated bearer. | - | - | - | - |

5.3A.5.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3A.5.4-1: Disconnect (step 1, Table 5.3A.5.3-1)

Derivation Path: Table 5.5.6.13-1, condition ACK

### 5.3A.6	MCPTT CO session modification

5.3A.6.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state and a call is established.

5.3A.6.2	Void

5.3A.6.3	Procedure

Table 5.3A.6.3-1: MCPTT CO session modification

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT client) send a SIP INVITE requesting the modification of an MCPTT call? | --> | SIP re-INVITE | - | P |
| 2 | The SS sends a SIP 100 Trying | <-- | SIP 100 (Trying) | - | - |
| 3 | The SS (MCPTT server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 4 | Check: Does the UE (MCPTT client) send a SIP ACK to acknowledge the session modification? | --> | SIP ACK | - | P |
| - | EXCEPTION: Steps 5a1-5a2 describe behaviour that depends on whether the UE has implicitly requested a grant at step 1 which has not implicitly been granted at step 3. (NOTE 1) | - | - | - | - |
| 5a1 | IF the media description for media control in the 200 OK at step 3 contains fmtp parameter mc_implicit_request but no fmtp parameter mc_granted THEN the SS (MCPTT server) sends a Floor Granted message with request for acknowledgement. | <-- | Floor Granted | - | - |
| 5a2 | Check: Does the UE (MCPTT client) send a Floor Ack message? | --> | Floor Ack | - | P |
| NOTE 1: An implicit floor control may be requested in case of upgrade to an emergency or imminent peril group call but not in case of a downgrade or any other re-INVITE |  |  |  |  |  |

5.3A.6.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3A.6.4-1: SIP 200 (OK) (step 3, Table 5.3A.6.3-1)

Derivation Path: Table 5.5.2.17.1.2-1, condition INVITE-RSP

Table 5.3A.6.4-2: Floor Granted (step 5a1, Table 5.3A.6.3-1)

Derivation Path: Table 5.5.6.3-1, condition ACK

Table 5.3A.6.4-3: Floor Ack (Step 5a2, Table 5.3A.6.3-1)

Derivation Path: Table 5.5.6.11-1, condition UPLINK

### 5.3A.7	Void

### 5.3A.8	MCPTT CT Call establishment using a pre-established session

5.3A.8.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_IDLE state.

A pre-established session is established.

5.3A.8.2	Void

5.3A.8.3	Procedure

Table 5.3A.8.3-1: MCPTT CT Call establishment using a pre-established session

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The procedure 'MCX CT communication' as described in clause 5.4.4 is started to establish an RRC connection. | - | - | - | - |
| 2 | SS initiates an on-demand pre-arranged group call with automatic commencement mode using a pre-established session by sending a Connect message | <-- | Connect | - | - |
| 3 | Check: Does the UE (MCPTT client) send an Acknowledge message to accept the incoming pre-arranged group call using a pre-established session? | --> | Acknowledge | - | P |

5.3A.8.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3A.9	Void

### 5.3A.10	Void

### 5.3A.11	MCPTT Floor Request – Floor Granted

5.3A.11.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state and a call is established.

5.3A.11.2	Void

5.3A.11.3	Procedure

Table 5.3A.11.3-1: MCPTT Floor Request – Floor Granted

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT client) send a Floor Request message? | --> | Floor Request | - | P |
| 2 | The SS (MCPTT server) sends a Floor Granted message with request for acknowledgement. | <-- | Floor Granted | - | - |
| 3 | Check: Does the UE (MCPTT client) send a Floor Ack message? | --> | Floor Ack | - | P |
| 4 | Check: Does the UE (MCPTT client) provide floor granted notification to the user? (NOTE 1) | - | - | - | P |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3A.11.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3A.11.4-1: Floor Granted (Step 2, Table 5.3A.11.3-1)

Derivation Path: Table 5.5.6.3-1, condition ACK

Table 5.3A.11.4-2: Floor Ack (Step 3, Table 5.3A.11.3-1)

Derivation Path: Table 5.5.6.11-1, condition UPLINK

### 5.3A.12	MCPTT Floor Request – Floor Queue Position Info

5.3A.12.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state and a call is established.

5.3A.12.2	Void

5.3A.12.3	Procedure

Table 5.3A.12.3-1: MCPTT Floor Request – Floor Queue Position Info

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT client) send a Floor Request message? | --> | Floor Request | - | P |
| 2 | The SS (MCPTT server) sends a Floor Queue Position Info message indicating that the Floor Request is queued. | <-- | Floor Queue Position Info | - | - |

5.3A.12.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3A.13	MCPTT Queuing Position Request

5.3A.13.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state and a call is established.

5.3A.13.2	Void

5.3A.13.3	Procedure

Table 5.3A.13.3-1: MCPTT Queuing Position Request

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT client) send a Floor Queue Position Request message? | --> | Floor Queue Position Request | - | P |
| 2 | The SS (MCPTT server) responds with a Floor Queue Position Info message. | <-- | Floor Queue Position Info | - | - |

5.3A.13.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3A.14	MCPTT Floor Request – Floor Deny

5.3A.14.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state and a call is established.

5.3A.14.2	Void

5.3A.14.3	Procedure

Table 5.3A.14.3-1: MCPTT Floor Request – Floor Deny

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT client) send a Floor Request message? | --> | Floor Request | - | P |
| 2 | The SS (MCPTT server) sends a Floor Deny message | <-- | Floor Deny | - | - |
| 3 | Check: Does the UE (MCPTT client) provide floor deny notification to the user? (NOTE 1) | - | - | - | P |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3A.14.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3A.15	MCPTT Floor Release – Floor Idle

5.3A.15.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state and a call is established.

5.3A.15.2	Void

5.3A.15.3	Procedure

Table 5.3A.15.3-1: MCPTT Floor Release – Floor Idle

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT client) send a Floor Release message? | --> | Floor Release | - | P |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE requests an acknowledgement to the Floor Release message. | - | - | - | - |
| 2a1 | The SS (MCPTT server) sends a Floor Ack message. | <-- | Floor Ack | - | - |
| 3 | The SS (MCPTT server) sends a Floor Idle message. | <-- | Floor Idle | - | - |

5.3A.15.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3A.15.4-1: Floor Ack (Step 2a1, Table 5.3A.15.3-1)

Derivation Path: Table 5.5.11.3-1, condition DOWNLINK

### 5.3A.16	MCPTT Floor Release – Floor Taken

5.3A.16.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_CONNECTED state and a call is established.

5.3A.16.2	Void

5.3A.16.3	Procedure

Table 5.3A.16.3-1: MCPTT Floor Release – Floor Taken

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT client) send a Floor Release message? | --> | Floor Release | - | P |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE requests an acknowledgement to the Floor Release message. | - | - | - | - |
| 2a1 | The SS (MCPTT server) sends a Floor Ack message. | <-- | Floor Ack | - | - |
| 3 | The SS (MCPTT server) sends a Floor Taken message. | <-- | Floor Taken | - | - |

5.3A.16.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3A.16.4-1: Floor Ack (Step 2, Table 5.3A.16.3-1)

Derivation Path: Table 5.5.11.3-1, condition DOWNLINK