Discussion on test point selection for Carrier Leakage for shared spectrum channel access in FR1

# Introduction

A default baseline for test point selection for TRx test cases was presented in [1] at RAN5#1 5G NR Adhoc meeting, where some proposals for environmental conditions, test frequencies, channel bandwidths, subcarrier spacing and RB allocation for FR1 were based on LTE. Finally, it was stated that each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for Carrier Leakage for shared spectrum channel access test case in FR1.

# Discussion

Carrier Leakage for FR1 is defined in Power Control section in [3] where its scope is described as follows:

Carrier leakage expresses itself as unmodulated sine wave with the carrier frequency or centre frequency of aggregated transmission bandwidth configuration. It is an interference of approximately constant amplitude and independent of the amplitude of the wanted signal.Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

## Test Environment

General rule for shared spectrum channel access applies. Use the same test environment as in test case 6.4.2.2, Carrier Leakage.

Proposal 1: Define Test Environment as Normal Conditions (NC) for Carrier Leakage for shared channel spectrum access.

## Test Subcarrier Spacing

General rule for shared spectrum channel access applies. Use the same test subcarrier spacing as in test case 6.4.2.2, Carrier Leakage.

Proposal 2: Select Lowest supported SCS for UE for Carrier Leakage for shared channel spectrum access.

  - Test Frequencies

General rule for shared spectrum channel access applies. Use the same test frequencies as in test case 6.4.2.2, Carrier Leakage.

Proposal 3: Test Low, Mid and High Frequency for Carrier Leakage for shared channel spectrum access.

## Test Channel Bandwidth

General rule for shared spectrum channel access applies. Use the same test channel bandwidths as in test case 6.4.2.2, Carrier Leakage.

Proposal 4: Test the Mid Channel Bandwidth for Carrier Leakage for shared channel spectrum access.

## Modulations

General rule for shared spectrum channel access could apply. However, due to the limitation of DFT-s-OFDM in partial allocation in shared spectrum channel access, only CP-OFDM could be tested.

Follow the general rule and select QPSK modulation only.

Proposal 5: Use CP-OFDM QPSK in the Carrier Leakage for shared spectrum channel access.

## Uplink Configuration

General rule for shared spectrum channel access could apply. However, the RB allocation for shared spectrum channel access needs to be adapted for interlaced allocation.

Edge_1RB allocation is not possible for shared spectrum channel access due to interlaced allocation. Use Partial_Left/Partial_Right, as defined in Table 6.F-1 in [3] for RB change in addition to Full allocation.

Proposal 6: Use Partial_Left/Partial_Right RB allcoation for Carrier Leakage for shared channel spectrum access.

# Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 Carrier Leakage test case definition:

Proposal 1: Define Test Environment as Normal Conditions (NC) for Carrier Leakage for shared channel spectrum access.

Proposal 2: Select Lowest supported SCS for UE for Carrier Leakage for shared channel spectrum access.

Proposal 3: Test Low, Mid and High Frequency for Carrier Leakage for shared channel spectrum access.

Proposal 4: Test the Mid Channel Bandwidth for Carrier Leakage for shared channel spectrum access.

Proposal 5: Use CP-OFDM QPSK in the Carrier Leakage for shared spectrum channel access.

Proposal 6: Use Partial_Left/Partial_Right RB allcoation for Carrier Leakage for shared channel spectrum access.

# References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] 3GPP TS 38.101-1 v16.18.0  User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16)

[3] 3GPP TS 38.521-1 v18.1.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 18)