3GPP TSG-RAN5 Meeting #103	R5-242623

# Fukuoka, Japan,  20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1534 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New MO speech EVS test case 12.23a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At the RAN5#102 meeting the discussion paper R5-241310 explaining that the same requirements e.g. for voice and the EVS default configuration applies for IR.92 and NG.114. The conclusion was that RAN5 was willing to consider adding test coverage for EVS default configuration A.2 in IR.92 from Rel-15. This CR proposes to introduce a new Rel-15 test case for MO speech EVS call. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new test case, 12.23a MO MTSI speech call / EVS / Default configuration, was added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no test coverage for EVS default configuration A.2 in IR.92 from Rel-15. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.23a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 34.229-2 CR 0341 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 12.23.5	Test requirements

The UE shall send requests and responses as described in clause 12.23.4.

## 12.23a	MO MTSI speech call / EVS / Default configuration

### 12.23a.1	Definition

Test to verify that the UE correctly performs IMS mobile originated voice call setup with EVS when using IMS Multimedia Telephony. This process is described in 3GPP TS 24.229 [10], clauses 5.1.3 and 6.1, TS 24.173 [65] and TS 26.114 [66].

### 12.23a.2	Conformance requirement

Same as 12.23 except:

[IR.92, clause 2.4.3.3]:

…

The configuration of the EVS payload type to be included first in the initial SDP offer for EVS is defined by the EVS/Br and EVS/Bw parameters as specified in Annex C.3, which must be configured to one of the five above EVS Configurations.

…

[IR.92, Annex C.3]:

…

Table C.3.1 contains the configuration parameters with their default values that must be supported by the UE and the network. The UE must use the default value for each parameter in Table C.3.1 unless configured differently by any of the methods as described in section C.2.

…

| EVS/Br | 5.9-24.4 | Defined in clause 15.2 of 3GPP Release 13 TS 26.114 [35] (/<X>/Speech/<X>/EVS/Br) and clause 5 of 3GPP Release 13 TS 26.441 [76] (Table 1) | 2.4.3.3 |
| --- | --- | --- | --- |
| EVS/Bw | nb-swb | Defined in clause 15.2 of 3GPP Release 13 TS 26.114 [35] (/<X>/Speech/<X>/EVS/Bw) and clause 5 of 3GPP Release 13 TS 26.441 [76] (Table 1) | 2.4.3.3 |

Reference(s)

GSMA PRD IR.92 [133], clause 2.4.3.3 and Annex C.3.

### 12.23a.3	Test purpose

1)	To verify that when initiating MO MTSI speech call and SS needs to reserve resources, the UE performs correct exchange of SIP protocol signalling messages for setting up the session.

2)	To verify that within SIP signalling the UE performs the correct exchange of SIP header and parameter contents.

3)	To verify that within SIP signalling the UE performs the correct exchange of SDP contents.

4)	To verify that the UE is able to release the call.

### 12.23a.4	Method of test

Initial conditions

UE contains ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with EN-DC support (TS 34.229-2 [5] A.18/1)

1-14)	The UE executes the procedure described in TS 36.508 [94] table 4.5A.19.3-1 steps 1 to 14.

NOTE:	No NR cell is required to be setup in the preamble

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Step 1 defined in annex C.44 | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.19.3-1 for a UE with EN-DC support. |
| 2 |  |  | INVITE | UE sends INVITE with the first SDP offer. |
| 3 |  |  | Step 3 defined in annex C.44 | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.19.3-1 for a UE with EN-DC support. |
| 4 |  |  | 183 Session Progress | SS sends an SDP answer. |
| 5-6 |  |  | Steps 5-6 defined in annex C.44 | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.19.3-1 for a UE with EN-DC support. |
| 7 |  |  | UPDATE | UE sends UPDATE with second SDP offer |
| 8-13 |  |  | Steps 8-13 defined in annex C.44 | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.19.3-1 for a UE with EN-DC support. |
| 14 |  |  | The UE is triggered by MMI to release the call |  |
| 15 |  |  | BYE | The UE releases the call with BYE |
| 16 |  |  | 200 OK | The SS sends 200 OK for BYE |

Specific Message Contents

INVITE (Step 2)

Use the default message “INVITE for MO Call” in annex A.2.1 with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) [Note 9] - b=RR: (bandwidth-value) [Note 9] Attributes for media: - a=rtpmap: (payload type) EVS/16000 [Note 5, 10] - a=fmtp: (format) br=5.9-24.4; bw=nb-swb; max-red= (att-field) [Note 6, 7] - a=rtpmap: (payload type) AMR-WB/16000 [Note 5, 10] - a=fmtp: (format) mode-change-capability=2; max-red= (att-field) [Note 6, 8] - a=rtpmap: (payload type) telephone-event/16000 - a=fmtp: (format) - a=rtpmap: (payload type) AMR/8000 [Note 5, 10] - a=fmtp: (format) mode-change-capability=2; max-red= (att-field) [Note 6, 8] - a=rtpmap: (payload type) telephone-event/8000 - a=fmtp: (format) - a=ecn-capable-rtp: leap ect=0 [Note 2] - a=rtcp-fb:* nack ecn [Note 2] - a=rtcp-xr:ecn-sum [Note 2] - a=rtcp-rsize [Note 2] - a=ptime:20 - a=maxptime:240 Attributes for media security mechanism: - a=3ge2ae: requested [Note 3] - a=a=crypto:1 AES_CM_128_HMAC_SHA1_80inline:WVNfX19zZW1jdGwgKCkgewkyMjA7fQp9CnVubGVz¦2^20¦ 1:4FEC_ORDER=FEC_SRTP" [Note 3] Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: Attributes for ECN Capability may be present if the UE supports Explicit Congestion Notification. Note 3: Attributes for media plane security are present if the use of end-to-access-edge security is supported by UE. Note 4: Void Note 5: The channel number shall be “/1” or omitted. Note 6: Values from 0 to 220 are allowed in the att-field. Note 7: The parameters dtx, dtx-recv and evs-mode-switch shall not be present. Note 8: The parameters mode-set, mode-change-period, mode-change-neighbor, crc, robust-sorting and interleaving shall not be present. Note 9: The RR value must be greater than 0. The RS value can be any value. Note 10: The ordering of payload types shall be as listed, i.e., EVS before AMR-WB before AMR. |

183 Session Progress (Step 4)

Use the default message "183 Session Progress" in annex A.2.3 with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:65 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 1, 4] - b=AS:65 - b=RS: (bandwidth-value) [Note 5] - b=RR: (bandwidth-value) [Note 5] Attributes for media: - a=rtpmap: (payload type) EVS/16000/1 [Note 1] - a=fmtp: (format) br=5.9-24.4; bw=nb-swb; max-red=220 - a=ecn-capable-rtp: leap ect=0 [Note 2] - a=rtcp-fb:* nack ecn [Note 2] - a=rtcp-xr:ecn-sum [Note 2] - a=ptime:20 - a=maxptime:240 Attributes for media security mechanism: - a=3ge2ae: requested [Note 3] - a=crypto:1 AES_CM_128_HMAC_SHA1_80inline:PS1uQCVeeCFCanVmcjkpPywjNWhcYD0mXXtxaVBR¦2^20¦1:4 [Note 3] Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv - a=conf:qos remote sendrecv Note 1: The values for fmt, payload type and format are copied from step 2. Note 2: Attributes for ECN Capability are present if the UE supports Explicit Congestion Notification. Note 3: Attributes for media plane security are present if the use of end-to-access-edge security is supported by UE. Note 4: Transport port is the port number of the SS (see RFC 3264 clause 6). Note 5: The bandwidth-value is copied from step 2. . |

UPDATE (Step 7)

Use the default message “UPDATE” in annex A.2.5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Message-body | Header optional Contents if present: The following SDP types and values shall be present. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 2] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 3] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) EVS/16000 [Note 3] [Note 5] - a=fmtp: (format) [Note 3][Note 4] Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one Note 3: The value for fmt, payload type and format is not checked Note 4: Parameters for the codec are not checked Note 5: The channel number shall be “/1” or omitted. |

### 12.23a.5	Test requirements

The UE shall send requests and responses as described in clause 12.23a.4.