3GPP TSG-RAN5 Meeting #103	R5-242737

# Fukuoka, Japan, 20th May 2024 – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1537 | rev | - | Current version: | 17.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Annex A.5.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The contents of Message-body are not align with XML specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add “s” in the contents of Message-body. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are still errors in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

A.5.3	NOTIFY for conference event package

| Header/param |  | Cond |  | Value/remark |  | Rel |  | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request-Line |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| Method |  |  |  | NOTIFY |  |  |  |  |  |
| Request-URI |  |  |  | UE’s contact address in SIP URI form, as provided in the Contact header within the SUBSCRIBE creating the dialog |  |  |  |  |  |
| SIP-Version |  |  |  | SIP/2.0 |  |  |  |  |  |
| Via |  |  |  | order of the parameters in this header must be like in this table |  |  |  | RFC 3261 [15] |  |
| via-parm1: |  |  |  |  |  |  |  |  |  |
| sent-protocol |  |  |  | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |  |  |  |  |  |
| sent-by |  | A1 |  | IP address and protected server port of SS |  |  |  |  |  |
| sent-by |  | A2 |  | IP address and unprotected server port of SS (optional) |  |  |  |  |  |
| via-branch |  |  |  | value starting with ‘z9hG4bK’ (NOTE 1) |  |  |  |  |  |
| via-parm2: |  |  |  |  |  |  |  |  |  |
| sent-protocol |  |  |  | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |  |  |  |  |  |
| sent-by |  |  |  | scscf.3gpp.org |  |  |  |  |  |
| via-branch |  |  |  | value starting with ‘z9hG4bK’ (NOTE 1) |  |  |  |  |  |
| From |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | sip:final@conf-factory. appended with px_IMS_HomeDomainName |  |  |  |  |  |
| tag |  |  |  | tag value corresponding to the SIP URI in the From header |  |  |  |  |  |
| To |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | any IMPU within the set of IMPUs on ISIM |  |  |  |  |  |
| tag |  |  |  | tag value corresponding to the SIP URI in the To header |  |  |  |  |  |
| Call-ID |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| callid |  |  |  | same as value received in SUBSCRIBE message |  |  |  |  |  |
| CSeq |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | value of CSeq sent by the SS within its previous request in the same dialog but increased by one |  |  |  |  |  |
| method |  |  |  | NOTIFY |  |  |  |  |  |
| Contact |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | sip:final@conf-factory. appended with px_IMS_HomeDomainName |  |  |  |  |  |
| Event |  |  |  |  |  |  |  | RFC 6665 [140] RFC 4575 [86] |  |
| event-type |  |  |  | conference |  |  |  |  |  |
| Max-Forwards |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | 69 |  |  |  |  |  |
| Subscription-State |  |  |  |  |  |  |  | RFC 6665 [140] |  |
| substate-value |  | A3 OR A5 |  | active |  |  |  |  |  |
| expires |  | A3 OR A5 |  | 7200 |  |  |  |  |  |
| substate-value |  | A4 |  | terminated |  |  |  | RFC 4575 [86] |  |
| reason |  | A4 |  | noresource |  |  |  | RFC 6665 [140] |  |
| Content-Type |  | A3 OR A5 |  |  |  |  |  | RFC 3261 [15] |  |
| media-type |  |  |  | application/conference-info+xml |  |  |  | RFC 4575 [86] |  |
| Content-Length |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | length of message-body |  |  |  |  |  |
| Message-body |  | A3 |  | <?xml version="1.0" encoding="UTF-8"?> <conference-info xmlns="urn:ietf:params:xml:ns:conference-info"> entity=sip:final@conf-factory. appended with px_IMS_HomeDomainName state="full" version="0"> <users> <user entity="any IMPU within the set of IMPUs on ISIM"> <endpoint entity=" Contact URI of the UE"> <status>connected</status> <joining-method>dialed-in</joining-method> <media id="1"> <type>audio</type> <label>34567</label> <src-id>SSRC of UE's RTP packets</src-id> <status>sendrecv</status> </media> </endpoint> </user> </users> </conference-info> |  |  |  | RFC 4575 [86] |  |
|  |  | A5 |  | <?xml version="1.0" encoding="UTF-8"?> <conference-info xmlns="urn:ietf:params:xml:ns:conference-info"> entity=sip:final@conf-factory. appended with px_IMS_HomeDomainName state="full" version="0"> <users> <user entity="any IMPU within the set of IMPUs on ISIM"> <endpoint entity=" Contact URI of the UE"> <status>connected</status> <joining-method>dialed-in</joining-method> <media id="1"> <type>audio</type> <label>34567</label> <src-id>SSRC of UE's RTP packets</src-id> <media id="2"> <type>video</type> <label>34568</label> <src-id>SSRC of UE's RTP packets</src-id> <status>sendrecv</status> </media> </endpoint> </user> </users> </conference-info> |  |  |  |  |  |

| Condition |  | Explanation |  |
| --- | --- | --- | --- |
| A1 |  | IMS security (A.6a/2 TS 34.229-2 [5])) |  |
| A2 |  | GIBA (A.6a/1 TS 34.229-2 [5])) |  |
| A3 |  | SS sends NOTIFY to indicate that UE is now subscribed to the conference event package |  |
| A4 |  | SS sends NOTIFY to indicate that UE’s subscription to conference event is terminated now |  |
| A5 |  | SS sends NOTIFY to indicate that UE is now subscribed to the video conference event package |  |

NOTE 1:	Branch parameter values sent by SS are different within a test case execution.

<End of modified section>