3GPP TSG-RAN5 Meeting # 103	R5-242756

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0594 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test case 8.38 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to discussion paper R5-240643, it is expected that 18x messages should be sent reliably as per NG.114 default configuration. Currently, multiple test cases consider SIP PRACK and 200 OK to be optional based upon the presence of 100rel option tag in 18x messages. Hence, changes are required in the test cases to make the steps mandatory. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SIP PRACK and 200 OK are made mandatory. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.38 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

## 8.38	Communication Waiting and cancelling the voice call / 5GS

8.38.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and having initiated an MO voice call with preconditions }

ensure that {

when { UE receives INVITE for MT voice call with preconditions }

then { UE continues voice call initiation until 180 Ringing (including PRACK/200 OK) }

}

(2)

with { UE having continued initiation of incoming voice call until 180 Ringing }

ensure that {

when { UE receives CANCEL for incoming voice call }

then { UE responds with 200 OK and 487 Request Terminated responses }

}

8.38.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.615 clause 1]:

The Communication Waiting (CW) service enables a user to be informed, that very limited resources are available for an incoming communication. The user then has the choice of accepting, rejecting or ignoring the waiting call (as per basic call procedures).

[TS 24.615 clause 4.2.1]:

When a communication arrives at the destination user, the UE validates the status of the user. If the user is already involved in one or more communications, the terminal notifies the served user of a communication waiting situation.

[TS 24.615 clause 4.5.5.3.2]:

The UE shall insert an Alert-Info header field set to "<urn:alert:service:call-waiting>", specified in RFC 7462 [8] in the 180 (Ringing) response, in accordance with the provisional response procedures described in 3GPP TS 24.229.

[TS 24.615 clause 4.5.5.3.4]:

If user B's UE receives a CANCEL request or BYE request from User C during a CW condition, user B's UE shall:

-	stop timer TUE-CW (if necessary);

-	stop providing the CW indication to User B; and

-	apply the terminating UE procedures upon receipt of CANCEL or BYE as described in 3GPP TS 24.229.

8.38.3	Test description

8.38.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	The UE contains either ISIM and USIM applications or only USIM application on UICC.

-	The UE is configured to register for IMS after switch on.

-	The UE is configured to use preconditions.

Preamble:

-	The UE has registered to IMS and set up the MO voice call, by executing the generic test procedure in Annex A.2 up to the last step and thereafter executing the generic test procedure in A.4.1a.8.38.3.2	Test procedure sequence

Table 8.38.3.2-1: Main Behaviour

| St |  | Procedure |  | Message Sequence |  |  |  | TP |  | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | U - S |  | Message |  |  |  |  |  |
| 1-7 |  | MT Voice call setup takes place according to steps 1-7 of Annex A.5.1a |  | - |  | - |  | - |  | - |  |
| 8 |  | Step 8 of Annex A.5.1a happens |  | --> |  | 180 Ringing |  | 1 |  | P |  |
| 9 |  | Step 9 of Annex A.5.1a happens |  | <-- |  | PRACK |  | - |  | - |  |
| 10 |  | Step 10 of Annex A.5.1a happens |  | --> |  | 200 OK |  | - |  | - |  |
| 11 |  | SS sends CANCEL request to terminate INVITE transaction |  | <-- |  | CANCEL |  | - |  | - |  |
| 12 |  | UE acknowledges CANCEL with 200 OK |  | --> |  | 200 OK |  | - |  | - |  |
| 13 |  | The UE responds to INVITE with a 487 Request Terminated final response after transaction was terminated. |  | --> |  | 487 Request Terminated |  | 2 |  | P |  |
| 14 |  | SS acknowledges the receipt of 487 Request Terminated |  | <-- |  | ACK |  | - |  | - |  |
| 15 |  | The UE is made to release the call |  | - |  | - |  | - |  | - |  |
| 16-17 |  | UE releases the original MO call (Annex A.7) |  | - |  | - |  | - |  | - |  |

8.38.3.3	Specific message contents

Table 8.38.3.3-1: INVITE (Step 1, table 8.38.3.2-1)

| Derivation Path: Annex A.5.1a, Step 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Message-body |  | o=- 1111111112 1111111111 IN (addrtype) (unicast-address for SS) |  | RFC 4566 [38] |

Table 8.38.3.3-2: UPDATE (Step 6, table 8.38.3.2-1)

| Derivation Path: Annex A.5.1a, step 6 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Message-body |  | o=- 1111111112 1111111112 IN (addrtype) (unicast-address for SS) |  | RFC 4566 [38] |

Table 8.38.3.3-3: 180 Ringing (step 8, table 8.38.3.2-1)

| Derivation path: Step 8 of Annex A.5.1a |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Alert-Info |  | <urn:alert:service:call-waiting> |  | RFC 7462 [39] |

Table 8.38.3.3-4: CANCEL (step 11, table 8.38.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.2.15

Table 8.38.3.3-5: 200 OK (step 12, table 8.38.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.3.1 applying conditions A5 and A11

Table 8.38.3.3-6: 487 Request Terminated (step 13, table 8.38.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.2.16

Table 8.38.3.3-7: ACK (step 14, table 8.38.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.2.7 applying conditions A2 and A4

<End of modified section>