3GPP TSG-RAN5 Meeting #103	R5-242828

# Fukuoka, Japan, 20th May 2024 – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4420 | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MUSIM gap TC 8.1.2.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Issue#1: When network provides periodic gap, network always signals the musim-GapLength and musim-GapRepetitionAndOffset as indicated by the UE's preferred MUSIM gap configuration in the UAI, however this TC directly configure MUSIM gap without UAI. Solution for issue#1: UAI trigger procedure is added before MUSIM gap configuration. UE only need to report one MUSIM period gap in the UAI. UAI can be triggered by (1) AT CMD or MMI or (2) Set MUSIM UAI Test function. If UAI is triggered by AT CMD or MMI, MUSIM gap (MGL/MGRP/Gap offset) is chosen by UE. If UAI is triggered by test funcation, MUSIM gap (MGL/MGRP/Gap offset) in the Set MUSIM UAI Request is set based on pc_MUSIMPeriodGapPattern. TC configured and test MUSIM gap based on the UAI message content reported by UE. Issue#2: After MUSIM gap is configured in NW A(NR Cell 1), it is not correct to test that UE cannot receive traffic in NW A during MUSIM gap. It is more reasonable to check UE receive paging or measurement SSB in NW B(NR Cell 12). Solution for issue#1: Delete the TP1/2/4 and add TP5 to test UE can receive paging in NW B (i.e UE behavior in NW B). When UE receive paging in NW B, UE switch from NW A to NW B. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete TP1/2/4 and related test procedure(step11-16 and step 21-24 and T0/T3) - Add TP5 to test UE behaviour in NW B - Change SS1 to NR Cell 1 and SS2 to NR Cell 12 in the TP. - Add Table 8.1.2.1.6.3.2-1A to illustrate the MUSIM Gap parameter used in SET MUSIM UAI REQUEST. The MUSIM gap requested in SET MUSIM UAI REQUEST can include the Paging occasion in NW B but not include the Paging occasion in NW A. - Add UAI procedure in step 8A to 8D before MUSIM gap configuration procedure. - Add paging and response procedure in NW B to check UE correctly use the MUSIM gap. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Open issue exist in the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 4420 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.1.2.1.6	RRC reconfiguration/ MUSIM / MUSIM- gap / Addition / Modification / Release

8.1.2.1.6.1	Test Purpose (TP)

(1)

Void

(2)

Void

(3)

with { UE in NR RRC_CONNECTED state}

ensure that {

when { UE receives the RRCReconfiguration message includes the musim-GapConfig with periodic musim-GapID involved in musim-GapToReleaseList in NR Cell 1}

then { UE performs normal interworking with NR Cell 1 during the released gaps periodically }

}

(4)

void

(5)

with { UE in NR RRC_CONNECTED state in SS1 and UE in NR RRC_IDLE state in SS1}

ensure that

when { UE receives the RRCReconfiguration message includes the musim-GapConfig with periodic musim-GapID involved in musim-GapToAddModList and the gap configuration is active in NR Cell 1}

then { UE correctly uses MUSIM Gap configured in musim-GapConfig to monitor CN paging in NR Cell 12 }

}

8.1.2.1.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clauses  5.3.5.3, 5.3.5.9a and TS 38.133 clause 9.1.10. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO, CPA or CPC):

…

1>	if the RRCReconfiguration message includes the musim-GapConfig:

2>	perform the MUSIM gap configuration procedure as specified in 5.3.5.9a;

[TS 38.331, clause 5.3.5.9a]

The UE shall:

1>	if musim-GapConfig is set to setup:

2>	for each musim-GapId included in the received musim-GapToReleaseList:

3>	release the periodic MUSIM gap configuration associated with the musim-GapId;

2>	for each MUSIM-Gap included in the received musim-GapToAddModList:

3>	setup periodic MUSIM gap configuration indicated by the MUSIM-Gap in accordance with the received musim-GapRepetitionAndOffset (providing musim-GapRepetition and Offset value for the following condition) i.e. the first subframe of each periodic MUSIM gap occurs at an SFN and subframe of the NR PCell meeting the following condition:

SFN mod T = FLOOR(Offset/10);

subframe = Offset mod 10;

with T = musim-GapRepetition/10;

2>	if musim-AperiodicGap is included:

3>	setup aperiodic MUSIM gap configuration indicated by the musim-AperiodicGap in accordance with the received musim-Starting-SFN-AndSubframe, i.e. the first subframe of aperiodic MUSIM gap occurs at an SFN and subframe of the NR PCell meeting the following condition:

SFN = starting-SFN;

subframe = startingSubframe;

1>	else if musim-GapConfig is set to release:

[TS 38.133, clause 9.1.10]

If the UE requires gap patterns for MUSIM purpose, such as cell identification and measurement, paging monitoring, SIB acquisition, and/or on-demand SI request of the target cell in the target network, then the network may provide one or more per-UE MUSIM gap pattern(s) for concurrent monitoring of all frequency layers for MUSIM via MUSIM-GapConfig [2]. The UE can be configured with no more than three periodic MUSIM gap patterns and/or one aperiodic MUSIM gap pattern for MUSIM via MUSIM-GapConfig [2]. The MUSIM gap patterns specified in Table 9.1.10-1 are applicable only for MUSIM operation.

The UE is not required to perform cell identification and measurement, paging monitoring, SIB acquisition, and/or on-demand SI request of the target cell in the target network that is outside the MUSIM gaps.

UE supporting MUSIM capability shall support the MUSIM gap patterns listed in Table 9.1.10-1 based on UE’s capability specified in TS38.306[14] and the applicability specified in Table 9.1.10-2

UE determines MUSIM gap timing based on gap offset configuration from serving cell provided by higher layer signalling as specified in TS 38.331 [2].

Table 9.1.10-1: MUSIM Gap Pattern Configurations

| MUSIM Gap Pattern Id | MUSIM Gap Length (MGL, ms) | MUSIM Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 10 | 80 |
| 13 | 20 | 160 |
| 14 | 6 | 320 |
| 15 | 6 | 640 |
| 16 | 6 | 1280 |
| 17 | 6 | 2560 |
| 18 | 10 | 320 |
| 19 | 10 | 640 |
| 20 | 10 | 1280 |
| 21 | 10 | 2560 |
| 22 | 20 | 320 |
| 23 | 20 | 640 |
| 24 | 20 | 1280 |
| 25 | 20 | 2560 |
| 26 | 20 | 5120 |
| 27 | 10 | NA |
| 28 | 20 | NA |
| Note 1: Measurement gap pattern #27, #28 are the aperiodic gap pattern without MGRP. |  |  |

Table 9.1.10-2: Applicability for MUSIM Gap Pattern Configurations supported by the UE with NR standalone operation (with single carrier, NR CA configuration)

| MUSIM gap pattern configuration | Serving cell | Gap Purpose | Applicable MUSIM Gap Pattern Id |
| --- | --- | --- | --- |
| Per-UE | FR1, FR2, or | MUSIM Note1 | 0-13, 14-26, 27, 28 |
| MUSIM gap | FR1 + FR2 |  |  |
| NOTE 1: Inclusion of MUSIM procedures for per-UE MUSIM gaps only in NR single carrier, NR CA mode: MUSIM purpose which includes cell identification and measurement, paging monitoring, SIB acquisition, and/or on-demand SI request of the target cell in the target network. |  |  |  |

8.1.2.1.6.3	Test description

8.1.2.1.6.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1(home PLMN1) and NR Cell 12 (home PLMN2) are configured according to table 4.4.2-3 in TS 38.508-1 [18] and belong to the different frequency.

-	NR Cell 1 (belongs to TAI-1, home PLMN1, MCC=001,MNC=01) is set to ''Serving cell''.

-	NR Cell 12 (belongs to TAI-3, home PLMN2, MCC=001,MNC=02) is set to '' Serving cell''.

UE:

-	the UE is a MUSIM UE;

-	the UE is equipped with two USIMs with configuration as defined in TS 38.508-1 [4] Tables 6.4.1-27 and 6.4.1-28.

Preamble:

-	The UE performs a successful registration for MUSIM according to Table 4.9.36.2.2-1 of TS 38.508-1 [4] with following exceptions.

-	NGC Cell A is replaced by NR Cell 1

-	NGC Cell G is replaced by NR Cell 12

8.1.2.1.6.3.2	Test procedure sequence

Table 8.1.2.1.6.3.2-1 illustrates a  message to be sent for NR Cell 1 at various time instants of the test execution. Table 8.1.2.1.6.3.2-1A illustrates the MUSIM Gap parameter used in SET MUSIM UAI REQUEST

Table 8.1.2.1.6.3.2-1: Time instances of sending a message in conducted test environment

|  | Remark | Comment |
| --- | --- | --- |
| T1 | SFN mod T = FLOOR(Offset/10); subframe = Offset mod 10; with T = musim-GapRepetition/10; T1 = SFN + subframe + musim-GapLength + 2ms | T1 is always outside the gap configured in Step 10U. |
| T2 | SFN mod T = FLOOR(Offset/10); subframe = Offset mod 10; with T = musim-GapRepetition/10; T2 = SFN + subframe + 2ms | If the gap is not released successfully, T2 is in the gap in step 10U. |

Table 8.1.2.1.6.3.2-1A: MUSIM Gap parameter used in SET MUSIM UAI REQUEST

| MUSIM Gap Pattern Id(pc_MUSIMPeriodGapPattern) | MUSIM Gap Length (MGL, ms) | C2..C0 in Table 8.1.2.1.6.3.3-0D | MUSIM Gap Repetition Period (MGRP, ms) | D3..D0 in Table 8.1.2.1.6.3.3-0D | MUSIM Gap offset for FDD(ms) (Note 1,Note 2) | MUSIM Gap offset for TDD (ms) (Note1, Note 2) |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 6 | 2 | 40 | 1 | 9 | 29 |
| 1 | 6 | 2 | 80 | 2 | 9 | 69 |
| 2 | 3 | 0 | 40 | 1 | 9 | 29 |
| 3 | 3 | 0 | 80 | 2 | 9 | 69 |
| 4 | 6 | 2 | 20 | 0 | 9 | 9 |
| 5 | 6 | 2 | 160 | 3 | 89 | 149 |
| 6 | 4 | 1 | 20 | 0 | 9 | 9 |
| 7 | 4 | 1 | 40 | 1 | 9 | 29 |
| 8 | 4 | 1 | 80 | 2 | 9 | 69 |
| 9 | 4 | 1 | 160 | 3 | 89 | 149 |
| 10 | 3 | 0 | 20 | 0 | 9 | 9 |
| 11 | 3 | 0 | 160 | 3 | 9 | 149 |
| 12 | 10 | 3 | 80 | 2 | 9 | 69 |
| 13 | 20 | 4 | 160 | 3 | 89 | 139(Note 3) |
| 14 | 6 | 2 | 320 | 4 | 249 | 149 |
| 15 | 6 | 2 | 640 | 5 | 569 | 469 |
| 16 | 6 | 2 | 1280 | 6 | 1209 | 1109 |
| 17 | 6 | 2 | 2560 | 7 | 2489 | 2389 |
| 18 | 10 | 3 | 320 | 4 | 249 | 149 |
| 19 | 10 | 3 | 640 | 5 | 569 | 469 |
| 20 | 10 | 3 | 1280 | 6 | 1209 | 1109 |
| 21 | 10 | 3 | 2560 | 7 | 2489 | 2389 |
| 22 | 20 | 4 | 320 | 4 | 249 | 139(Note 3) |
| 23 | 20 | 4 | 640 | 5 | 569 | 459(Note 3) |
| 24 | 20 | 4 | 1280 | 6 | 1209 | 1099(Note 3) |
| 25 | 20 | 4 | 2560 | 7 | 2489 | 2379(Note 3) |
| Note 1: E13..E0 in Table 8.1.2.1.6.3.3-0D Note 2: MUSIM Gap offset is designed to make sure that paging occasion for NR Cell 12 is included in MUSIM Gap. MUSIM Gap offset = (((PF of NR Cell 12 - SFN-offset of NR Cell 12 - 1) mod 1024)*10 + 9 ) mod MGRP. Where SFN-offset of NR Cell 12 is defined in 38.523-3 Table 7.1.5.2-1, PF: SFN mod defaultPagingCycle = 2*(UE_ID mod (defaultPagingCycle/2)), UE_ID is 632 based on 5G-TMSI=’C2345678’O. defaultPagingCycle of NR Cell 12 is 2560ms. Note 3: MUSIM Gap offset minus extra 10 ms in order to avoid that paging occasion for NR Cell 1 is also included in MUSIM Gap. |  |  |  |  |  |  |

Table 8.1.2.1.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-8 | The UE performs the generic procedure by executing steps 1 to 8 to enter the NR RRC_CONNECTED state as described in TS 38.508-1 [4] table 4.5.4.2-3 on NR Cell 1 | - | - | - | - |
| 8A | The SS transmits an RRCReconfiguration message to configure musim-GapAssistanceConfig-r17 IE. | <-- | NR RRC: RRCReconfiguration | - | - |
| 8B | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| - | EXCEPTION: Steps 8Ca1 and 8Ca2 are executed if UE supports test function specified in TS 38.509 [9] (pc_Set_MUSIM_UAI_Info_NR specified in TS 38.508-2[5]) ELSE Step 8Cb1 is executed (Note 1). | - | - | - | - |
| 8Ca1 | The SS transmits a SET MUSIM UAI REQUEST message. | <-- | SET MUSIM UAI REQUEST | - | - |
| 8Ca2 | The UE transmits a SET MUSIM UAI RESPONSE message. | --> | SET MUSIM UAI RESPONSE | - | - |
| 8Cb1 | Make the UE send UE Assistance information. | - | - | - | - |
| 8D | The UE transmit UEAssistanceInformation message to indicate musim-GapLength-r17 and musim-GapRepetitionAndOffset-r17 within MUSIM-GapPreferenceList-r17 IE. | --> | NR RRC: UEAssistanceInformation | - | - |
| 9 | The SS transmits an RRCReconfiguration message to configure musim-GapLength-r17 and musim-GapRepetitionAndOffset-r17 on NR Cell 1 | <-- | NR RRC: RRCReconfiguration | - | - |
| 10 | UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 10A | The SS transmits a Paging message on NR Cell 12. | <-- | NR RRC: Paging | - | - |
| 10B | Configure the UE to accept Paging on NR Cell 12.(Note 2) | - | - | - | - |
| 10C | The SS releases the RRC connection on NR Cell 1.(Note 3) | <-- | NR RRC: RRCRelease | - | - |
| 10D-10J | Steps 2 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure on NR Cell 12. (Note 4) | - | - | 5 | - |
| 10K | The SS releases the RRC connection on NR Cell 12. | <-- | NR RRC: RRCRelease | - | - |
| 10L-10T | The UE performs the generic procedure by executing steps 1 to 8 to enter the NR RRC_CONNECTED state as described in TS 38.508-1 [4] table 4.5.4.2-3 on NR Cell 1 | - | - | - | - |
| 10U | The SS transmits an RRCReconfiguration message to configure musim-GapLength-r17 and musim-GapRepetitionAndOffset-r17 on NR Cell 1 | <-- | NR RRC: RRCReconfiguration | - | - |
| 10V | UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 11-16 | Void |  |  |  |  |
| 17 | The SS transmits an RRCReconfiguration message to configure musim-GapId-r17 = ‘0’ with musim-GapToReleaseList-r17 IE to UE on NR Cell 1 at T1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 18 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 19 | The SS sends UECapabilityEnquiry message to the UE on NR Cell 1 at T2. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 20 | Check: Does the UE send a UECapabilityInformation message on NR Cell 1? | --> | NR RRC: UECapabilityInformation | 3 | P |
| Note 1: The test function is optional, it can be replaced by alternative solutions (e.g AT command or MMI) due to UE implementation specific behavior. Note 2: The configuration to accept Paging may be performed by MMI or AT Command. Note 3: UE may transmit a SERVICE REQUEST message to release NAS signalling connection or UEAssistanceInformation message to leave RRC_CONNECTED mode on NR Cell 1 before RRCRelease. Note 4: UE responds to the Paging message transmitted on NR Cell 12 at step 10A. |  |  |  |  |  |

8.1.2.1.6.3.3	Specific message contents

Table 8.1.2.1.6.3.3-0: SIB1 for NR Cell 12 (preamble and all steps, Table 8.1.2.1.6.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 8.1.2.1.6.3.3-0A |  |
| } |  |  |  |  |

Table 8.1.2.1.6.3.3-0A: ServingCellConfigCommonSIB (Table 8.1.2.1.6.3.3-0)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 8.1.2.1.6.3.3-0B |  |
| } |  |  |  |

Table 8.1.2.1.6.3.3-0B: DownlinkConfigCommonSIB (Table 8.1.2.1.6.3.3-0)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| pcch-Config SEQUENCE { |  |  |  |
| defaultPagingCycle | rf256 |  |  |
| nAndPagingFrameOffset CHOICE { |  |  |  |
| halfT | 0 |  |  |
| } |  |  |  |
| ns | one |  |  |
| firstPDCCH-MonitoringOccasionOfPO | Not present |  |  |
| nrofPDCCH-MonitoringOccasionPerSSB-InPO-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.1.6.3.3-0C: RRCReconfiguration (step 8A, Table 8.1.2.1.6.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1700 SEQUENCE { |  |  |  |
| musim-GapAssistanceConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| musim-GapProhibitTimer-r17 | s10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.1.6.3.3-0D: SET MUSIM UAI REQUEST (step 8Ca1, Table 8.1.2.1.6.3.2-2)

| Derivation Path: 38.509 clause 6.13.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1’B |  |  |
| Skip indicator | 0 0 0 0’B |  |  |
| Message type | 1 0 1 1 0 0 1 0’B |  |  |
| musim-PreferredRRC-State | 0 0 0 0 0 0 1 1’B | Reserved |  |
| musim-GapPreferenceList |  |  |  |
| musim-GapPreferenceList Value type | 0 0 0 0 0 0 0 1’B |  |  |
| Length of musim-GapPreferenceList Value contents in bytes | 0 0 0 0 0 1 0 1’B | 5 bytes |  |
| musim-GapPreferenceList IE #0 |  |  |  |
| C2..C0 | Set according to MUSIM Gap Length in the Table 8.1.2.1.6.3.2-1A |  |  |
| D3..D0 | Set according to MUSIM Gap Repetition Period in the Table 8.1.2.1.6.3.2-1A |  |  |
| E13..E0 | Set according to MUSIM Gap offset in the Table 8.1.2.1.6.3.2-1A |  |  |

Table 8.1.2.1.6.3.3-0E: UEAssistanceInformation (Step 8D, Table 8.1.2.1.6.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| musim-Assistance-r17 SEQUENCE { |  |  |  |
| musim-PreferredRRC-State-r17 | Not present |  |  |
| musim-GapPreferenceList-r17 SEQUENCE (SIZE (1..4)) OF MUSIM-GapInfo-r17{ |  |  |  |
| MUSIM-GapInfo-r17 [1] SEQUENCE { | entry 1 |  |  |
| musim-Starting-SFN-AndSubframe-r17 | Not present |  |  |
| musim-GapLength-r17 | Any allowed value |  |  |
| musim-GapRepetitionAndOffset-r17 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.1.6.3.3-1: RRCReconfiguration (step 9, step 10U, step 17, Table 8.1.2.1.6.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| musim-GapConfig-r17 CHOICE { |  |  |  |
| setup | MUSIM-GapConfig-r17 |  | step 9, step 10U |
|  | MUSIM-GapConfig-Rel-r17 |  | step 17 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.1.6.3.3-2:MUSIM-GapConfig-r17(Table 8.1.2.1.6.3.3-1)

| Derivation Path: TS 38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MUSIM-GapConfig-r17 ::= SEQUENCE { |  |  |  |
| musim-GapToReleaseList-r17 | Not Present |  |  |
| musim-GapToAddModList-r17 SEQUENCE (SIZE (1..3)) OF MUSIM-Gap-r17{ |  |  |  |
| MUSIM-Gap-r17[1] SEQUENCE { |  |  |  |
| musim-GapId-r17 | 0 |  |  |
| musim-GapInfo-r17 SEQUENCE { |  |  |  |
| musim-Starting-SFN-AndSubframe-r17 | Not Present |  |  |
| musim-GapLength-r17 | Same as MUSIM-GapInfo-r17 [1].musim-GapLength-r1 in the Table 8.1.2.1.6.3.3-0E |  |  |
| musim-GapRepetitionAndOffset-r17 | Same as MUSIM-GapInfo-r17 [1]. musim-GapRepetitionAndOffset-r17 in the Table 8.1.2.1.6.3.3-0E |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| musim-AperiodicGap-r17 | Not Present |  |  |
| } |  |  |  |

Table 8.1.2.1.6.3.3-3: MUSIM-GapConfig-Rel-r17(Table 8.1.2.1.6.3.3-1)

| Derivation Path: TS 38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MUSIM-GapConfig-r17 ::= SEQUENCE { |  |  |  |
| musim-GapToReleaseList-r17 SEQUENCE (SIZE (1..3)) OF MUSIM-GapId-r17 { |  |  |  |
| MUSIM-GapId-r17[1] SEQUENCE{ |  |  |  |
| MUSIM-Gap-r17[1] SEQUENCE { |  |  |  |
| musim-GapId-r17 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| musim-GapToAddModList-r17 | Not Present |  |  |
| musim-AperiodicGap-r17 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.1.6.3.3-4: Void

<End of modified section 1>

<Start of modified section 2>

<End of modified section 2>