3GPP TSG-RAN5 Meeting #103	R5-242855

# Fukuoka, Japan, May 20-24, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1774 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 7.3B.2 reference sensitivity for inter-band EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The latest TS 38.101-3, the minimum requirements for reference sensitivity of inter-band EN-DC has been replaced with new tables that include uplink/downlink configurations. But the RAN5 specification still remain the old style of minimum requirements. - According to clause 7.3B.1, For operations with 4 Rx antenna ports in an E-UTRA band or an NR band, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,4R in Table 7.3.1-1a of TS 36.101[5] for the E-UTRA band or in Table 7.3.2-2 of TS 38.101-1 [2] for the NR band when MSD > 0. The reference sensitivity requirements of 2Rx is REFSENS + MSD, and the reference sensitivity requirements of 4Rx is REFSENS + ΔRIB,4R + MSD + ¦ΔRIB,4R ¦, where REFSENS refers to the reference sensitivity values for single carrier of 2Rx and ΔRIB,4R is a negative value. The REFSENS impact caused by more antennas is eliminated for EN-DC. So the 4Rx EN-DC should share the same reference sensitivity as that of 2Rx EN-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the minimum requirements for reference sensitivity to align with RAN4 specification. - Remove 4Rx test requirements and void the corresponding note from the test requirements table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case would remain unaligned with core specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.0.3, 7.3B.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

7.3B.2.0.3	Inter-band EN-DC within FR1

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

7.3B.2.0.3.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

Sensitivity degradation is allowed for different combinations of UL configurations and DL channel bandwidths if a band if it is impacted by UL harmonic interference from another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (high) and uplink/downlink configurations due to UL harmonic from a PC3 aggressor UL band (low) for either single band uplink or PC3 or PC2 EN-DC are specified in Table 7.3B.2.0.3.1-1. For these exceptions, only the listed test points in Table 7.3B.2.0.3.1-1 need to be tested.

Table 7.3B.2.0.3.1-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band |  | UL RB Allocation |  |  |  | DL BW |  | MSD |  | UL/DL fc condition |  |  | UL/DL harmonic order |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) |  | LCRB |  |  |  | (MHz) |  | (dB) |  |  |  |  |  |  |  |
| 1 | n77 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 10 |  | 23.9 |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| 1 | n77 | 20 | 15 |  | 100 (RBstart=0) |  |  |  | 100 |  | 13.8 |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| 1 | n77 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 10 |  | 1.1 |  | NOTE 6 |  |  | UL2/DL1 near-miss |  |  |
| 2 | n77 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 10 |  | 23.9 |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| 2 | n77 | 10 | 15 |  | 50 (RBstart=0) |  |  |  | 100 |  | 13.8 |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| 2 | n77 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 10 |  | 1.1 |  | NOTE 6 |  |  | UL2/DL1 near-miss |  |  |
| 2 | n78 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 10 |  | 23.9 |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| 2 | n78 | 10 | 15 |  | 50 (RBstart=0) |  |  |  | 100 |  | 13.8 |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| 2 | n78 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 10 |  | 1.1 |  | NOTE 6 |  |  | UL2/DL1 near-miss |  |  |
| 3 | n77 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 10 |  | 23.9 |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| 3 | n77 | 10 | 15 |  | 50 (RBstart=0) |  |  |  | 100 |  | 13.8 |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| 3 | n77 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 10 |  | 1.1 |  | NOTE 6 |  |  | UL2/DL1 near-miss |  |  |
| 3 | n78 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 10 |  | 23.9 |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| 3 | n78 | 10 | 15 |  | 50 (RBstart=0) |  |  |  | 100 |  | 13.8 |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| 3 | n78 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 10 |  | 1.1 |  | NOTE 6 |  |  | UL2/DL1 near-miss |  |  |
| 5 | n778 | 5 | 15 |  | 16 (RBstart=0) |  |  |  | 10 |  | 10.5 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 5 | n778 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 1.4 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 5 | n778 | 5 | 15 |  | 10 (RBstart=0) |  |  |  | 10 |  | 10.4 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 5 | n778 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 0.7 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 5 | n78 | 5 | 15 |  | 16 (RBstart=0) |  |  |  | 10 |  | 10.5 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 5 | n78 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 1.4 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 8 | n41 | 5 | 15 |  | 8 (RBstart=0) |  |  |  | 10 |  | 13 |  | NOTE 3 |  |  | UL3/DL1 direct-hit |  |  |
| 8 | n41 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 3.5 |  | NOTE 3 |  |  | UL3/DL1 direct-hit |  |  |
| 8 | n77 | 5 | 15 |  | 16 (RBstart=0) |  |  |  | 10 |  | 10.8 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 8 | n77 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 1.4 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 8 | n78 | 5 | 15 |  | 16 (RBstart=0) |  |  |  | 10 |  | 10.8 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 8 | n78 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 1.4 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 8 | n79 | 5 | 15 |  | 10 (RBstart=0) |  |  |  | 10 |  | 12.0 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 8 | n79 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 4.4 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 12 | n66 | 5 | 15 |  | 8 (RBstart=0) |  |  |  | 5 |  | 10 |  | NOTE 3 |  |  | UL3/DL1 direct-hit |  |  |
| 12 | n66 | 5 | 15 |  | 20 (RBstart=0) |  |  |  | 40 |  | 2.4 |  | NOTE 3 |  |  | UL3/DL1 direct-hit |  |  |
| 12 | n77 | 5 | 15 |  | 10 (RBstart=0) |  |  |  | 10 |  | 10.4 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 12 | n77 | 5 | 15 |  | 20 (RBstart=0) |  |  |  | 100 |  | 0.7 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 12 | n78 | 5 | 15 |  | 10 (RBstart=0) |  |  |  | 10 |  | 10.4 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 12 | n78 | 5 | 15 |  | 20 (RBstart=0) |  |  |  | 100 |  | 0.7 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 13 | n77 | 5 | 15 |  | 10 (RBstart=0) |  |  |  | 10 |  | 10.4 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 13 | n77 | 5 | 15 |  | 20 (RBstart=0) |  |  |  | 100 |  | 0.7 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 14 | n77 | 5 | 15 |  | 10 (RBstart=0) |  |  |  | 10 |  | 10.4 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 14 | n77 | 5 | 15 |  | 15 (RBstart=0) |  |  |  | 100 |  | 0.7 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 18 | n77 | 5 | 15 |  | 10 (RBstart=0) |  |  |  | 10 |  | 10.4 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 18 | n77 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 0.7 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 18 | n78 | 5 | 15 |  | 10 (RBstart=0) |  |  |  | 10 |  | 10.4 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 18 | n78 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 0.7 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 19 | n77 | 5 | 15 |  | 10 (RBstart=0) |  |  |  | 10 |  | 10.4 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 19 | n77 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 0.7 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 19 | n77 | 5 | 15 |  | 10 (RBstart=0) |  |  |  | 10 |  | 10.5 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 19 | n77 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 1.4 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 19 | n78 | 5 | 15 |  | 10 (RBstart=0) |  |  |  | 10 |  | 10.5 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 19 | n78 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 1.4 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 20 | n77 | 5 | 15 |  | 16 (RBstart=0) |  |  |  | 10 |  | 10.8 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 20 | n77 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 1.4 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 20 | n78 | 5 | 15 |  | 16 (RBstart=0) |  |  |  | 10 |  | 10.8 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 20 | n78 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 1.4 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 26 | n41 | 5 | 15 |  | 16 (RBstart=0) |  |  |  | 10 |  | 10.3 |  | NOTE 3 |  |  | UL3/DL1 direct-hit |  |  |
| 26 | n41 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 2.7 |  | NOTE 3 |  |  | UL3/DL1 direct-hit |  |  |
| 26 | n77 | 5 | 15 |  | 16 (RBstart=0) |  |  |  | 10 |  | 10.8 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 26 | n77 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 1.0 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 26 | n78 | 5 | 15 |  | 16 (RBstart=0) |  |  |  | 10 |  | 10.8 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| 26 | n78 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 1.0 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| n28 | 1 | 5 | 15 |  | 8 (RBstart=0) |  |  |  | 5 |  | 10.2 |  | NOTE 3 |  |  | UL3/DL1 direct-hit |  |  |
| n28 | 1 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 20 |  | 5.3 |  | NOTE 3 |  |  | UL3/DL1 direct-hit |  |  |
| n28 | 2112 | NA | NA |  | NA |  |  |  | NA |  | NA |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| n28 | 2112 | NA | NA |  | NA |  |  |  | NA |  | NA |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| 28 | n51 | 5 | 15 |  | 12 (RBstart=0) |  |  |  | 5 |  | 27.8 |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| 28 | n51 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 5 |  | 1.9 |  | NOTE 6 |  |  | UL2/DL1 near-miss |  |  |
| 28 | n77 | 5 | 15 |  | 10 (RBstart=0) |  |  |  | 10 |  | 10.4 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 28 | n77 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 0.7 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 28 | n78 | 5 | 15 |  | 10 (RBstart=0) |  |  |  | 10 |  | 10.4 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 28 | n78 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 100 |  | 0.7 |  | NOTE 5 |  |  | UL5/DL1 direct-hit |  |  |
| 66 | n77 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 10 |  | 23.9 |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| 66 | n77 | 20 | 15 |  | 100 (RBstart=0) |  |  |  | 100 |  | 13.8 |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| 66 | n77 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 10 |  | 1.1 |  | NOTE 6 |  |  | UL2/DL1 near-miss |  |  |
| 66 | n78 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 10 |  | 23.9 |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| 66 | n78 | 20 | 15 |  | 100 (RBstart=0) |  |  |  | 100 |  | 13.8 |  | NOTE 2 |  |  | UL2/DL1 direct-hit |  |  |
| 66 | n78 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 10 |  | 1.1 |  | NOTE 6 |  |  | UL2/DL1 near-miss |  |  |
| 71 | n29 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 5 |  | 4.6 |  | NOTE 3 |  |  | UL3/DL1 direct-hit |  |  |
| 71 | n210 | 5 | 15 |  | 8 (RBstart=0) |  |  |  | 5 |  | 1.7 |  | NOTE 3 |  |  | UL3/DL1 direct-hit |  |  |
| 71 | n29 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 20 |  | 0.6 |  | NOTE 3 |  |  | UL3/DL1 direct-hit |  |  |
| 71 | n210 | 5 | 15 |  | 8 (RBstart=0) |  |  |  | 20 |  | 0.6 |  | NOTE 3 |  |  | UL3/DL1 direct-hit |  |  |
| n71 | 29 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 5 |  | 4.6 |  | NOTE 3 |  |  | UL3/DL1 direct-hit |  |  |
| n71 | 210 | 5 | 15 |  | 8 (RBstart=0) |  |  |  | 5 |  | 1.7 |  | NOTE 3 |  |  | UL3/DL1 direct-hit |  |  |
| n71 | 29 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 20 |  | 0.6 |  | NOTE 3 |  |  | UL3/DL1 direct-hit |  |  |
| n71 | 210 | 5 | 15 |  | 8 (RBstart=0) |  |  |  | 20 |  | 0.6 |  | NOTE 3 |  |  | UL3/DL1 direct-hit |  |  |
| n71 | 7 | 5 | 15 |  | 8 (RBstart=0) |  |  |  | 5 |  | 14.6 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| n71 | 7 | 5 | 15 |  | 25 (RBstart=0) |  |  |  | 20 |  | 9 |  | NOTE 4 |  |  | UL4/DL1 direct-hit |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL EARFCN or NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 7: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 10: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 11: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 from TS 36.101-1 apply unless otherwise specified). NOTE 12: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL. This band is subject to 2nd harmonic fall in B21 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.1-2: Void

Table 7.3B.2.0.3.1-3: Reference sensitivity QPSK PREFSENS (EN-DC with n46)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n46 | 482,4 | 22.6 | 19.5 | 17.8 | 16.6 |  |  |  |  |  |  |  |
| NOTE 1: FFS NOTE 2: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the downlink transmission bandwidth of a victim (lower) band. NOTE 3: FFS NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.1-4: Void

Table 7.3B.2.0.3.1-5: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC paring with n46

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n46 | 48 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |

7.3B.2.0.3.2	Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

Sensitivity degradation is allowed for different combinations of UL configurations and DL channel bandwidths if a band if it is impacted by receiver harmonic mixing due to another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (low) and uplink/downlink configurations due to UL harmonic from a PC3 aggressor UL band (high) for either single band uplink or PC3 or PC2 EN-DC are specified in Table 7.3B.2.0.3.2-1.

Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor UL for PC2 EN-DC are specified in Table 7.3B.2.3.2-1a.

For these exceptions, only the listed test points in Table 7.3B.2.3.2-1 and Table 7.3B.2.3.2-1a need to be tested.

Table 7.3B.2.0.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| fULHBfULHB fULHBfULHB fDLLB=fULHB/0.150.1fDLLB=fULHB/0.150.1fULHBfULHB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band |  | DL band |  | UL BW |  |  | SCS of UL band |  |  | UL RB Allocation |  |  |  |  |  |  |  |  | DL BW |  | MSD |  | UL/DL fc condition |  |  | UL/DL harmonic order |  |
|  |  |  |  | (MHz) |  |  | (kHz) |  |  | LCRB |  |  |  |  |  |  |  |  | (MHz) |  | (dB) |  |  |  |  |  |  |
| 2 |  | n71 |  | 5 |  |  | 15 |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 26.8 |  | NOTE 4 |  |  | UL1/DL3 |  |
| 2 |  | n71 |  | 20 |  |  | 15 |  |  | 50 (RBstart=0) |  |  |  |  |  |  |  |  | 20 |  | 15.6 |  | NOTE 4 |  |  | UL1/DL3 |  |
| n2 |  | 71 |  | 5 |  |  | 15 |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 26.8 |  | NOTE 4 |  |  | UL1/DL3 |  |
| n2 |  | 71 |  | 20 |  |  | 15 |  |  | 50 (RBstart=0) |  |  |  |  |  |  |  |  | 20 |  | 15.6 |  | NOTE 4 |  |  | UL1/DL3 |  |
| n41 |  | 26 |  | 10 |  |  | 15 |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 24.3 |  | NOTE 4 |  |  | UL1/DL3 |  |
| n41 |  | 26 |  | 15 |  |  | 15 |  |  | 75 (RBstart=0) |  |  |  |  |  |  |  |  | 15 |  | 22.5 |  | NOTE 4 |  |  | UL1/DL3 |  |
| n77 |  | 2 |  | 10 |  |  | 15 |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 6.1 |  |  |  |  |  |  |
| n77 |  | 2 |  | 20 |  |  | 15 |  |  | 100 (RBstart=0) |  |  |  |  |  |  |  |  | 20 |  | 3.7 |  |  |  |  |  |  |
| n77 |  | 3 |  | 10 |  |  | 15 |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 5.7 |  |  |  |  |  |  |
| n77 |  | 3 |  | 20 |  |  | 15 |  |  | 100 (RBstart=0) |  |  |  |  |  |  |  |  | 20 |  | 2.7 |  |  |  |  |  |  |
| n77 |  | 12 |  | 10 |  |  | 15 |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 31 |  | NOTE 2 |  |  | UL1/DL5 |  |
| n77 |  | 12 |  | 10 |  |  | 15 |  |  | 50 (RBstart=0) |  |  |  |  |  |  |  |  | 10 |  | 28 |  | NOTE 2 |  |  | UL1/DL5 |  |
| n77 |  | 13 |  | 10 |  |  | 15 |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 31 |  | NOTE 2 |  |  | UL1/DL5 |  |
| n77 |  | 13 |  | 10 |  |  | 15 |  |  | 50 (RBstart=0) |  |  |  |  |  |  |  |  | 10 |  | 28 |  | NOTE 2 |  |  | UL1/DL5 |  |
| n77 |  | 14 |  | 10 |  |  | 15 |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 31 |  | NOTE 2 |  |  | UL1/DL5 |  |
| n77 |  | 14 |  | 10 |  |  | 15 |  |  | 50 (RBstart=0) |  |  |  |  |  |  |  |  | 10 |  | 28 |  | NOTE 2 |  |  | UL1/DL5 |  |
| n77 |  | 28 |  | 10 |  |  | 15 |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 28 |  | NOTE 2 |  |  | UL1/DL5 |  |
| n77 |  | 28 |  | 20 |  |  | 15 |  |  | 100 (RBstart=0) |  |  |  |  |  |  |  |  | 20 |  | 22 |  | NOTE 2 |  |  | UL1/DL5 |  |
| n77 |  | 41 |  | 10 |  |  | 15 |  |  | 12 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 10.4 |  | NOTE 8 |  |  | UL2/DL3 |  |
| n77 |  | 41 |  | 10 |  |  | 15 |  |  | 50 (RBstart=0) |  |  |  |  |  |  |  |  | 20 |  | 10.4 |  | NOTE 8 |  |  | UL2/DL3 |  |
| n78 |  | 3 |  | 10 |  |  | 15 |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 5.7 |  |  |  |  |  |  |
| n78 |  | 3 |  | 20 |  |  | 15 |  |  | 100 (RBstart=0) |  |  |  |  |  |  |  |  | 20 |  | 2.7 |  |  |  |  |  |  |
| n78 |  | 19 |  | 10 |  |  | 15 |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 7.2 |  |  |  |  |  |  |
| n78 |  | 19 |  | 15 |  |  | 15 |  |  | 75 (RBstart=0) |  |  |  |  |  |  |  |  | 15 |  | 3.8 |  |  |  |  |  |  |
| n78 |  | 41 |  | 10 |  |  | 15 |  |  | 12 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 10.4 |  | NOTE 8 |  |  | UL2/DL3 |  |
| n78 |  | 41 |  | 20 |  |  | 15 |  |  | 50 (RBstart=0) |  |  |  |  |  |  |  |  | 20 |  | 10.4 |  | NOTE 8 |  |  | UL2/DL3 |  |
| n79 |  | 11 |  | 10 |  |  | 15 |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 39.3 |  | NOTE 4 |  |  | UL1/DL3 |  |
| n79 |  | 11 |  | 15 |  |  | 15 |  |  | 75 (RBstart=0) |  |  |  |  |  |  |  |  | 15 |  | 34.5 |  | NOTE 4 |  |  | UL1/DL3 |  |
| n79 |  | 19 |  | 10 |  |  | 15 |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 29.5 |  | NOTE 2 |  |  | UL1/DL5 |  |
| n79 |  | 19 |  | 15 |  |  | 15 |  |  | 75 (RBstart=0) |  |  |  |  |  |  |  |  | 15 |  | 24.7 |  | NOTE 2 |  |  | UL1/DL5 |  |
| n79 |  | 21 |  | 10 |  |  | 15 |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 39.3 |  | NOTE 4 |  |  | UL1/DL3 |  |
| n79 |  | 21 |  | 15 |  |  | 15 |  |  | 75 (RBstart=0) |  |  |  |  |  |  |  |  | 15 |  | 34.5 |  | NOTE 4 |  |  | UL1/DL3 |  |
| n79 |  | 26 |  | 10 |  |  | 15 |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  | 5 |  | 27 |  | NOTE 2 |  |  | UL1/DL5 |  |
| n79 |  | 26 |  | 15 |  |  | 15 |  |  | 75 (RBstart=0) |  |  |  |  |  |  |  |  | 15 |  | 22.2 |  | NOTE 2 |  |  | UL1/DL5 |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 9: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case. NOTE 10: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. NOTE 12: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.2-1a: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| fULHB fULHBfULHB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band |  | DL band |  | UL BW |  | SCS of UL band |  |  |  |  | UL RB Allocation |  |  |  |  |  |  |  |  |  |  |  | DL BW |  |  |  |  |  |  | MSD |  |  | UL/DL fc condition |  |  | UL/DL harmonic order |  |  |
|  |  |  |  | (MHz) |  | (kHz) |  |  |  |  | LCRB |  |  |  |  |  |  |  |  |  |  |  | (MHz) |  |  |  |  |  |  | (dB) |  |  |  |  |  |  |  |  |
| n77 |  | 2 |  | 10 |  | 15 |  |  |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  | 9.1 |  |  |  |  |  |  |  |  |
| n77 |  | 2 |  | 20 |  | 15 |  |  |  |  | 100 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  | 6.7 |  |  |  |  |  |  |  |  |
| n77 |  | 3 |  | 10 |  | 15 |  |  |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  | 8.1 |  |  |  |  |  |  |  |  |
| n77 |  | 3 |  | 20 |  | 15 |  |  |  |  | 100 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  | 5.7 |  |  |  |  |  |  |  |  |
| n77 |  | 12 |  | 10 |  | 15 |  |  |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  | 34 |  |  | NOTE 1 |  |  | UL1/DL5 |  |  |
| n77 |  | 12 |  | 10 |  | 15 |  |  |  |  | 50 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 10 |  |  |  |  |  |  | 31 |  |  | NOTE 1 |  |  | UL1/DL5 |  |  |
| n77 |  | 13 |  | 10 |  | 15 |  |  |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  | 34 |  |  | NOTE 1 |  |  | UL1/DL5 |  |  |
| n77 |  | 13 |  | 10 |  | 15 |  |  |  |  | 50 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 10 |  |  |  |  |  |  | 31 |  |  | NOTE 1 |  |  | UL1/DL5 |  |  |
| n77 |  | 14 |  | 10 |  | 15 |  |  |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  | 34 |  |  | NOTE 1 |  |  | UL1/DL5 |  |  |
| n77 |  | 14 |  | 10 |  | 15 |  |  |  |  | 50 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 10 |  |  |  |  |  |  | 31 |  |  | NOTE 1 |  |  | UL1/DL5 |  |  |
| n77 |  | 28 |  | 10 |  | 15 |  |  |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  | 32 |  |  | NOTE 1 |  |  | UL1/DL5 |  |  |
| n77 |  | 28 |  | 20 |  | 15 |  |  |  |  | 100 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  | 25 |  |  | NOTE 1 |  |  | UL1/DL5 |  |  |
| n78 |  | 19 |  | 10 |  | 15 |  |  |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  | 9.8 |  |  |  |  |  |  |  |  |
| n78 |  | 19 |  | 15 |  | 15 |  |  |  |  | 75 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 15 |  |  |  |  |  |  | 5.8 |  |  |  |  |  |  |  |  |
| n79 |  | 19 |  | 10 |  | 15 |  |  |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  | 32.5 |  |  | NOTE 1 |  |  | UL1/DL5 |  |  |
| n79 |  | 19 |  | 15 |  | 15 |  |  |  |  | 75 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 15 |  |  |  |  |  |  | 27.7 |  |  | NOTE 1 |  |  | UL1/DL5 |  |  |
| n79 |  | 21 |  | 10 |  | 15 |  |  |  |  | 25 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  | 42.3 |  |  | NOTE 3 |  |  | UL1/DL3 |  |  |
| n79 |  | 21 |  | 15 |  | 15 |  |  |  |  | 75 (RBstart=0) |  |  |  |  |  |  |  |  |  |  |  | 15 |  |  |  |  |  |  | 37.5 |  |  | NOTE 3 |  |  | UL1/DL3 |  |  |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 3: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 4: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.2-2: Void1

7.3B.2.0.3.3	Void

7.3B.2.0.3.4	Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same EN-DC configuration due to cross band isolation issues. Reference sensitivity exceptions for the victim band are specified only for the specific uplink and downlink test points specified in Table 7.3B.2.0.3.4-1 and Table 7.3B.2.0.3.4-1a2.

Table 7.3B.2.0.3.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation |  | DL Fc |  |  | DL BW |  | MSD |  | Cross-band Interference source |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB |  | (MHz) |  |  | (MHz) |  | (dB) |  |  |  |  |
| n1 | 3 | 1922.5 | 5 | 15 | 25 (RBSTART=0) |  | 1877.5 |  |  | 5 |  | 3 |  | >ACLR2 |  |  |
| n1 | 3 | 1945 | 50 | 15 | 128 (RBSTART=0) |  | 1877.5 |  |  | 5 |  | 19.7 |  | ACLR1 |  |  |
| 1 | n3 | 1930 | 20 | 15 | 100 (RBSTART=0) |  | 1877.5 |  |  | 5 |  | [7.9] |  | >ACLR2 |  |  |
| 1 | n3 | 1922.5 | 5 | 15 | 25 (RBSTART=0) |  | 1877.5 |  |  | 5 |  | 3 |  | >ACLR2 |  |  |
| 1 | n40 | 1970 | 20 | 15 | 100 (RBSTART=0) |  | 2302.5 |  |  | 5 |  | 6.6 |  | >ACLR2 |  |  |
| 1 | n41 | 1970 | 20 | 15 | 100 (RBSTART=0) |  | 2501 |  |  | 10 |  | 6.1 |  | >ACLR2 |  |  |
| 3 | n41 | 1775 | 20 | 15 | 50 (RBSTART=50) |  | 2501 |  |  | 10 |  | 0.7 |  | >ACLR2 |  |  |
| 3 | n51 | 1720 | 20 | 15 | 50 (RBSTART=0) |  | 1429.5 |  |  | 5 |  | 6.4 |  | >ACLR2 |  |  |
| 3 | n51 | 1712.5 | 5 | 15 | 25 (RBSTART =0) |  | 1429.5 |  |  | 5 |  | 6.4 |  | >ACLR2 |  |  |
| n5 | 28 | 834 | 20 | 15 | 20 (RBSTART=0) |  | 800.5 |  |  | 5 |  | 17.5 |  | ACLR2 |  |  |
| 30 | n66 | 2310 | 10 | 15 | 25 (RBSTARTt=0) |  | 2197.5 |  |  | 5 |  | 8.3 |  | >ACLR2 |  |  |
| n40 | 1 | 2350 | 100 | 30 | 270 (RBSTART=0) |  | 2167.5 |  |  | 5 |  | [21.9] |  | ACLR2 |  |  |
| n41 | 1 | 2546 | 100 | 30 | 270 (RBSTART=0) |  | 2167.5 |  |  | 5 |  | 9.1 |  | >ACLR2 |  |  |
| n41 | 2 | 2546 | 100 | 30 | 270 (RBSTART=0) |  | 1987.5 |  |  | 5 |  | 0.6 |  | >ACLR2 |  |  |
| n41 | 3 | 2546 | 100 | 30 | 270 (RBSTART=0) |  | 1877.5 |  |  | 5 |  | 0.6 |  | >ACLR2 |  |  |
| n41 | 25 | 2546 | 100 | 30 | 270 (RBSTART=0) |  | 1992.5 |  |  | 5 |  | 0.6 |  | >ACLR2 |  |  |
| n41 | 661 | 2546 | 100 | 30 | 270 (RBSTART=0) |  | 2197.5 |  |  | 5 |  | 3.5 |  | >ACLR2 |  |  |
| 41 | n77 | 2680 | 20 | 15 | 100 (RBSTART=0) |  | 3305 |  |  | 10 |  | 8.3 |  | >ACLR2 |  |  |
| 41 | n78 | 2680 | 20 | 15 | 100 (RBSTART=0) |  | 3305 |  |  | 10 |  | 8.3 |  | >ACLR2 |  |  |
| n46 | 48 | 5190 | 80 | 30 | 216 (RBSTART=0) |  | 3697.5 |  |  | 5 |  | 13.3 |  | >ACLR2 |  |  |
| 48 | n46 | 3690 | 20 | 15 | 100 (RBSTART=0) |  | 5160 |  |  | 20 |  | 7 |  | >ACLR2 |  |  |
| n77 | 411 | 3350 | 100 | 30 | 270 (RBSTART=0) |  | 2687.5 |  |  | 5 |  | 4.5 |  | >ACLR2 |  |  |
| n78 | 71 | 3350 | 100 | 30 | 270 (RBSTART=0) |  | 2687.5 |  |  | 5 |  | 4.5 |  | >ACLR2 |  |  |
| n78 | 38 | 3350 | 100 | 30 | 270 (RBSTART=0) |  | 2617.5 |  |  | 5 |  | 3.3 |  | >ACLR2 |  |  |
| n78 | 411 | 3350 | 100 | 30 | 270 (RBSTART=0) |  | 2687.5 |  |  | 5 |  | 4.5 |  | >ACLR2 |  |  |
| n78 | 46 | 3750 | 100 | 30 | 270 (RBSTART=3) |  | 5160 |  |  | 20 |  | 13.5 |  | >ACLR2 |  |  |
| n79 | 426 | 4550 | 100 | 30 | 270 (RBSTART=0) |  | 3597.5 |  |  | 5 |  | 2.6 |  | >ACLR2 |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2515 – 2690 MHz. NOTE 3: Void. NOTE 4: The NR DL band should be configured using the lowest SCS that is compatible with the specified DL CBW. NOTE 5: Void. NOTE 6: The requirements only apply for UEs supporting inter-band DC_42_n79 ENDC with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. These restrictions are applicable to related higher order configurations. NOTE 7: The MSD exceptions are applicable to the case that interference of UL band 3rd order IMD product falls into the affected DL channels. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.4-1a: Reference sensitivity exceptions (MSD) due to cross band isolation and uplink/downlink configurations for PC2 EN-DC in NR FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| 3 | n41 | 1775 | 20 | 15 | 50 (RBstart=50) | 2501 | 10 | 0.7 | >ACLR2 |
| n41 | 3 | 2546 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 | 2.3 | >ACLR2 |
| n77 | 2 | 3350 | 100 | 30 | 270 (RBstart=0) | 1987.5 | 5 | 1.0 | >ACLR2 |
| n77 | 66 | 3350 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 1.0 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.4-2: Void

<Unchanged Skipped>

7.3B.2.3.5	Test requirement

For inter-band EN-DC configurations affected by reference sensitivity exceptions, when test points without NOTE 8, 9 and 11 in Table 7.3B.2.3.4.2.1-6 are tested, the throughput of each CG shall be ≥ 95% of the maximum throughput for the reference receive power level specified in Table 7.3B.2.3.5-1, Table 7.3B.2.3.5-2, Table 7.3B.2.3.5-3, Table 7.3B.2.3.5-3a, Table 7.3B.2.3.5-4 and Table 7.3B.2.3.5-5 for MSDs due to uplink harmonic, harmonic mixing, cross band isolation for PC3 EN-DC, cross band isolation for PC2 EN-DC, dual uplinks for PC3 EN-DC and dual uplink operation for PC2 EN-DC, respectively. For a given EN-DC combo, if more than one category of MSD applies, UE shall pass all requirement.

For test points with NOTE 8, 9 or 11 in Table 7.3B.2.3.4.2.1-6, reference sensitivity test requirements are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC, and Table 7.3.5-1 in TS 36.521-1 [10] for E-UTRA CC.

For non- exception requirements (LTE anchor agnostic approach) of all EN-DC configurations, test requirements for NR carrier are the same as 7.3.2.5 in TS 38.521-1 [8].

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable two, three, four and five bands operation.

Table 7.3B.2.3.5-1: Reference sensitivity due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) |  | 5 MHz (dBm) | 10 MHz (dBm) |  | 15 MHz (dBm) |  | 20 MHz (dBm) |  | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 3 | n772 | 15 |  |  | -71.4 +TT |  | -71.4 +TT |  | -71.3 +TT |  |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  |  | -71.7 +TT |  | -71.5 +TT |  | -71.5 +TT |  |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  |  | -72.1 +TT |  | -71.8 +TT |  | -71.7 +TT |  |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  |  | -94.2 +TT |  | -92.7 +TT |  | -91.9 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.5 +TT |  | -92.8 +TT |  | -92.1 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -94.9 +TT |  | -93.1 +TT |  | -92.3 +TT |  |  |  |  |  |  |  |  |  |
| 2 | n772, 13 | 15 |  |  | -71.4 +TT |  | -71.4 +TT |  | -71.3 +TT |  |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  |  | -71.7 +TT |  | -71.5 +TT |  | -71.5 +TT |  |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  |  | -72.1 +TT |  | -71.8 +TT |  | -71.7 +TT |  |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  |  | -94.2 +TT |  | -92.7 +TT |  | -91.9 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.5 +TT |  | -92.8 +TT |  | -92.1 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -94.9 +TT |  | -93.1 +TT |  | -92.3 +TT |  |  |  |  |  |  |  |  |  |
| 2 | n782 | 15 |  |  | -71.9+TT |  | -71.9 +TT |  | -71.8 +TT |  |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  |  | -72.2 +TT |  | -72.0 +TT |  | -72.0 +TT |  |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  |  | -72.6 +TT |  | -72.3 +TT |  | -72.2 +TT |  |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
| 2 | n783 | 15 |  |  | -94.7 +TT |  | -93.2 +TT |  | -92.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.9 +TT |  | -93.3 +TT |  | -92.6 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | --95.4 +TT |  | -93.6 +TT |  | -92.8 +TT |  |  |  |  |  |  |  |  |  |
| 3 | n782 | 15 |  |  | -71.9 +TT |  | -71.9 +TT |  | -71.8 +TT |  |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  |  | -72.2 +TT |  | -72.0 +TT |  | -72.0 +TT |  |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  |  | -72.6 +TT |  | -72.3 +TT |  | -72.2 +TT |  |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
|  | n783 | 15 |  |  | -94.7 +TT |  | -93.2 +TT |  | -92.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -95.0 +TT |  | -93.3 +TT |  | -92.6 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -95.4 +TT |  | -93.6 +TT |  | -92.8 +TT |  |  |  |  |  |  |  |  |  |
| 5 | n78 | 15 |  |  | -84.2 +TT |  | -84.4 +TT |  |  |  |  |  | -84.2 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  |  | -84.3 +TT |  | -84.5 +TT |  | -84.4 +TT |  |  |  | -84.3 +TT | -84.5 +TT | -84.4 +TT | -84.3 +TT | -84.0 +TT | -84.2 +TT |
|  |  | 60 |  |  | -84.5 +TT |  | -84.6 +TT |  | -84.5 +TT |  |  |  | -84.5 +TT | -84.6 +TT | -84.5 +TT | -84.4 +TT | -84.1 +TT | -84.3 +TT |
| 8 | n776,7 n786,7 | 15 |  |  | -84.5 +TT |  | -84.4 +TT |  | -84.2 +TT |  |  |  | -84.0 +TT | -83.9 +TT |  |  |  |  |
|  |  | 30 |  |  | -84.8 +TT |  | -84.5 +TT |  | -84.4 +TT |  |  |  | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.8 +TT | -83.5 +TT | -83.7 +TT |
|  |  | 60 |  |  | -85.2 +TT |  | -84.8 +TT |  | -84.6 +TT |  |  |  | -84.3 +TT | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.6 +TT | -83.8 +TT |
| 8 | n41 | 15 |  |  | -81.8 +TT |  | -81.7 +TT |  | -81.7 +TT |  |  |  | -81.6 +TT | -81.5 +TT |  |  |  |  |
|  |  | 30 |  |  | -82.1 +TT |  | -81.8 +TT |  | -81.9 +TT |  |  |  | -81.7 +TT | -81.6 +TT | -81.4 +TT | -81.3 +TT | -81.2 +TT | -81.2 +TT |
|  |  | 60 |  |  | -82.5 +TT |  | -82.1 +TT |  | -81.1 +TT |  |  |  | -81.9 +TT | -81.7 +TT | -81.6 +TT | -81.3 +TT | -81.2 +TT | -81.2 +TT |
| 12 | n66 | 15 |  | -89.5 +TT | -88.8 +TT |  | -88.3 +TT |  | -87.8 +TT |  |  |  | -87.7 +TT |  |  |  |  |  |
|  |  | 30 |  |  | -89.1 +TT |  | -88.4 +TT |  | -88.0 +TT |  |  |  | -87.8 +TT |  |  |  |  |  |
|  |  | 60 |  |  | -89.5 +TT |  | -88.7 +TT |  | -88.2 +TT |  |  |  | -87.9 +TT |  |  |  |  |  |
| 12 | n774, 5 | 15 |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  |  | -85.6 +TT |  | -85 +TT |  | -84.8 +TT |  |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 12 | n78 | 15 |  |  | -85.4 +TT |  | -85.1 +TT |  | -84.9 +TT |  | -84.6+TT | -84.4+TT | -84.9 +TT | -84.9 +TT |  |  |  |  |
|  |  | 30 |  |  | -85.7 +TT |  | -85.2 +TT |  | -85.1 +TT |  | -84.7+TT | -84.5+TT | -85.0 +TT | -85.0 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT |
|  |  | 60 |  |  | -86.1 +TT |  | -85.5 +TT |  | -85.3 +TT |  | -84.9+TT | -84.6+TT | -85.2 +TT | -85.1 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT |
| 13 | n774, 5 | 15 |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  |  | -85.6 +TT |  | -85 +TT |  | -84.8 +TT |  |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 14 | n774, 5 | 15 |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  |  | -85.6 +TT |  | -85 +TT |  | -84.8 +TT |  |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 18 | n774,5 n784,5 | 15 |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  |  | -85.6 +TT |  | -85.0 +TT |  | -84.8 +TT |  |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 19 | n774,5 |  | 15 |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  |  | 30 |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  |  | 60 |  |  | -85.6 +TT |  | -85.0 +TT |  | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
|  | n776,7 n786,7 |  | 15 |  |  | -84.8 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  | -83.7 +TT | -83.9 +TT |  |  |  |  |
|  |  |  | 30 |  |  | -85.1 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  | -83.8 +TT | -84.0 +TT | -83.9 +TT | -83.8 +TT | -83.5 +TT | -83.7 +TT |
|  |  |  | 60 |  |  | -85.5 +TT |  | -85.0 +TT |  | -84.8 +TT |  |  | -84.0 +TT | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.6 +TT | -83.8 +TT |
| 21 | n28 | 15 |  | -96.0 +TT | -93.0 +TT |  | -91.0 +TT |  | -88.3 +TT |  |  | -76.0 +TT |  |  |  |  |  |  |
|  |  | 30 |  |  | -93.1 +TT |  | -91.1 +TT |  | -89.5 +TT |  |  | -76.1 +TT |  |  |  |  |  |  |
| 28 | n774,5 n784,5 | 15 |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  |  | -85.6 +TT |  | -85.0 +TT |  | -84.8 +TT |  |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 20 | n776,7 n786,7 | 15 |  |  | -84.5 +TT |  | -84.4 +TT |  | -84.2 +TT |  |  |  | -83.1 +TT |  |  |  |  |  |
|  |  | 30 |  |  | -84.8 +TT |  | -84.5 +TT |  | -84.4 +TT |  |  |  | -83.2 +TT |  |  |  |  |  |
|  |  | 60 |  |  | -85.2 +TT |  | -84.8 +TT |  | -84.6 +TT |  |  |  | -83.4 +TT |  |  |  |  |  |
| 26 | n41 | 15 |  |  | -84.5 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  | -83.6 +TT | -83.3 +TT | 3.9 +TT | 3.1 +TT | 2.7 +TT |  |
|  |  | 30 |  |  | -84.8 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  | -83.7 +TT | -83.4 +TT | -83.0 +TT | -82.5 +TT | -82.4 +TT |  |
|  |  | 60 |  |  | -85.2 +TT |  | -85.0 +TT |  | -84.8 +TT |  |  |  | -83.9 +TT | -83.5 +TT | -83.2 +TT | -82.5 +TT | -82.4 +TT |  |
| 26 | n776,7 n786,7 | 15 |  |  | -85.0 +TT |  | -84.9 +TT |  | -84.7 +TT |  |  |  | -83.6 +TT | -84.6 +TT |  |  |  |  |
|  |  | 30 |  |  | -85.3 +TT |  | -85.0+TT |  | -84.9 +TT |  |  |  | -83.7 +TT | -84.7 +TT | -84.7 +TT | -84.6 +TT | -84.6 +TT | -84.6 +TT |
|  |  | 60 |  |  | -85.7 +TT |  | --85.3 +TT |  | --85.1 +TT |  |  |  | -83.9 +TT | -84.8 +TT | -84.8 +TT | -84.7 +TT | -84.7 +TT | -84.7 +TT |
| 26 | n774,5 | 15 |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -85.6 +TT | -84.4 +TT |
|  |  | 60 |  |  | -85.6 +TT |  | -85.0 +TT |  | -84.8 +TT |  |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -85.7 +TT | -84.5 +TT |
| n28 | 18,9,10 | N/A |  | -89.1 | -88.7 |  | -88.3 |  | -88.0 |  |  |  |  |  |  |  |  |  |
| 71 | n211 | 15 |  | -93.4 +TT | -93.8 +TT |  | -92.3 +TT |  | -91.2 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 +TT |  | -92.4 +TT |  | -91.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -94.5 +TT |  | -92.7 +TT |  | -91.6 +TT |  |  |  |  |  |  |  |  |  |
| 71 | n212 | 15 |  | -96.3 +TT | -93.8 +TT |  | -92.3 +TT |  | -91.2 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 +TT |  | -92.4 +TT |  | -91.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -94.5 +TT |  | -92.7 +TT |  | -91.6 +TT |  |  |  |  |  |  |  |  |  |
| n71 | 211 | N/A |  | -92.7 | -93.3 |  | -91.8 |  | -90.7 |  |  |  |  |  |  |  |  |  |
| n71 | 212 | N/A |  | -95.6 | -93.3 |  | -91.8 |  | -90.7 |  |  |  |  |  |  |  |  |  |
| n71 | 7 | N/A |  | -82.7 | -82.6 |  | -82.4 |  | -82.3 |  |  |  |  |  |  |  |  |  |
| 66 | n772, 13 | 15 |  |  | -71.4 +TT |  | -71.4 +TT |  | -71.3 +TT |  |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  |  | -71.7 +TT |  | -71.5 +TT |  | -71.5 +TT |  |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  |  | -72.1 +TT |  | -71.8 +TT |  | -71.7 +TT |  |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  |  | -94.2 +TT |  | -92.7 +TT |  | -91.9 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.5 +TT |  | -92.8 +TT |  | -92.1 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -94.9 +TT |  | -93.1 +TT |  | -92.3 +TT |  |  |  |  |  |  |  |  |  |
| 66 | n782 | 15 |  |  | -71.9 +TT |  | -71.9 +TT |  | -71.8 +TT |  |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  |  | -72.2+TT |  | -72.0 +TT |  | -72.0 +TT |  |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  |  | -72.6 +TT |  | -72.3 +TT |  | -72.2 +TT |  |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
|  | n783 | 15 |  |  | -94.7 +TT |  | -93.2 +TT |  | -92.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -95.0 +TT |  | -93.3 +TT |  | -92.6 +TT |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -95.4 +TT |  | -93.6 +TT |  | -92.8 +TT |  |  |  |  |  |  |  |  |  |
| n66 | 48,2, 13 | N/A |  | -70.7 | -70.6 |  | -70.8 |  | -70.8 |  |  |  |  |  |  |  |  |  |
| n66 | 48,3 | N/A |  | -96.1 | -93.6 |  | -92.3 |  | -91.6 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 9 The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 10: Void NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_42_n3, DC_48_n25, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28 and DC_28_n50, DC_28_n51, DC_66_n78, DC_25_n77, DC_25_n78. NOTE 14: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 15: Void NOTE 16: Void NOTE 17: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-1a: Test Tolerance (TT) for RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-2 with uplink configuration specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k for each specific EN-DC combination scenario.

Table 7.3B.2.3.5-2: Reference sensitivity due to receiver harmonic mixing for PC3 EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | n714 | 15 | -70.4 +TT | -70.4 +TT | -70.4 +TT | -70.4 +TT |  |  |  |  |  |  |  |
|  |  | 30 |  | -70.7 +TT | -70.7 +TT | -71.8 +TT |  |  |  |  |  |  |  |
|  |  | 60 |  | -72.4 +TT | -72.7 +TT | -77.0 +TT |  |  |  |  |  |  |  |
| n2 | 71 | N/A | -69.7 | -69.9 | -70.1 | -71.2 |  |  |  |  |  |  |  |
| n41 | 264 | N/A | -72.5 | -69.5 | -69.5 | N/A |  |  |  |  |  |  |  |
| n77 | 2 | N/A | -91.2 | -89.3 | -88.5 | -87.6 |  |  |  |  |  |  |  |
| n77 | 13 | N/A |  | -65.3 | -65.3 |  |  |  |  |  |  |  |  |
| n77 | 12 | N/A | -65.3 | -65.3 |  |  |  |  |  |  |  |  |  |
| n77 | 418 | N/A | -86.9 | -83.9 | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n77 | 14 | N/A | -65.3 | -65.3 |  |  |  |  |  |  |  |  |  |
| n77 | 282 | N/A | -69.8 | -69.8 | -69.8 | -68.3 |  |  |  |  |  |  |  |
| n78 | 3 | N/A | -90.6 | -89.3 | -88.5 | -87.6 |  |  |  |  |  |  |  |
| n78 | 19 | N/A | -92.1 | -91.3 | -90.7 |  |  |  |  |  |  |  |  |
| n78 | 40 | N/A | -86.9 | -83.9 | -82.1 | -80.9 |  |  |  |  |  |  |  |
| n78 | 418 | N/A | -86.9 | -83.9 | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n79 | 192 | N/A | -69.8 | -69.8 | -69.8 |  |  |  |  |  |  |  |  |
| n79 | 214 | N/A | -60.0 | -60.0 | -60.0 |  |  |  |  |  |  |  |  |
| n79 | 262 | N/A | -69.8 | -69.8 | -69.8 | N/A | N/A | N/A | N/A | N/A | N/A |  | N/A |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band andfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void. NOTE 6: Void. NOTE 7: The requirements should be verified for DL EARFCN of the victim (higher) band (superscript HB) such that fDLHB=50×fULLB0.03 with fDLHB the DL carrier frequency in the higher band and fULLB the UL carrier frequency in the lower band, both in MHz. NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band andfULHBfULHBfDLLB=fULHB/0.150.1 the UL carrier frequency in the higher band, both in MHz. NOTE 9: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 10: Void |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-2a: Reference sensitivity due to receiver harmonic mixing for PC2 EN-DC in NR FR11

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | 2 | N/A | -88.2 | -86.3 | -85.5 | -84.6 |  |  |  |  |  |  |  |
|  |  |  | -90.910 | -8910 | -88.210 | -87.310 |  |  |  |  |  |  |  |
| n77 | 3 | N/A | -66.2 | -66.4 | -66.4 | -69.4 |  |  |  |  |  |  |  |
| n77 | 132 | N/A |  | -62.3 | -62.3 |  |  |  |  |  |  |  |  |
| n77 | 12 | N/A | -65.3 | -65.3 |  |  |  |  |  |  |  |  |  |
| n77 | 14 | N/A | -65.3 | -65.3 |  |  |  |  |  |  |  |  |  |
| n77 | 282 | N/A | -66.8 | -66.8 | -66.8 | -65.3 |  |  |  |  |  |  |  |
| n78 | 19 | N/A | -89.5 | -89.1 | -88.7 |  |  |  |  |  |  |  |  |
| n79 | 192 | N/A | -66.8 | -66.8 | -66.8 |  |  |  |  |  |  |  |  |
| n79 | 214 | N/A | -57.0 | -57.0 | -57.0 |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band andfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void. NOTE 6: Void. NOTE 7: The requirements should be verified for DL EARFCN of the victim (higher) band (superscript HB) such that fDLHB=50×fULLB0.03 with fDLHB the DL carrier frequency in the higher band and fULLB the UL carrier frequency in the lower band, both in MHz. NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band andfULHBfULHBfDLLB=fULHB/0.150.1 the UL carrier frequency in the higher band, both in MHz. NOTE 9: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 10: Void |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-3 with uplink configuration specified in Table 7.3B.2.3.4.2.1-4a to Table 7.3B.2.3.4.2.1-4n.

Table 7.3B.2.3.5-3: Reference sensitivity exceptions due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| n1 | 3 | N/A | -93.3 | -91 | -89.5 | -88.5 |  |  |  |  |  |  |  |  |  |
| 1 | n3 | 15 | -94 +TT | -91.6 +TT | -90.1 +TT | -89.1 +TT | -88.1 +TT | -87.4 +TT | -86.2 +TT |  |  |  |  |  |  |
| 1 | n41 | 30 |  | -89.0 +TT | -87.0 +TT | -85.9 +TT |  | -83.9 +TT | -82.6 +TT | -81.6 +TT | -80.8 +TT |  | -79.5 +TT | -79.0 +TT | -78.6 +TT |
| 3 | n41 | 30 |  | -94.4 +TT | -92.4 +TT | -91.3 +TT |  | -89.3 +TT | -88.0 +TT | -87 +TT | -86.2 +TT |  | -84.9 +TT | -84.4 +TT | -84.0 +TT |
| n5 | 28 | 15 | -93.3 | -91.8 | -90.8 | -90 |  |  |  |  |  |  |  |  |  |
| n41 | 1 | 15 | -90.2 | -87.2 | -85.4 | -84.2 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | N/A | -96.7 | -93.7 | -91.9 | -90.7 |  |  |  |  |  |  |  |  |  |
| n41 | 3 | N/A | 3 | n41 | 30 | 3 |  |  |  |  |  |  |  |  |  |
| n41 | 25 | N/A | -95.2 | -92.2 | -90.4 | -89.2 |  |  |  |  |  |  |  |  |  |
| n77 | 411 | N/A | -92.8 | -89.8 | -88.0 | -86.8 |  |  |  |  |  |  |  |  |  |
| n41 | 66 | N/A | -95.3 | -92.3 | -90.5 | -89.3 |  |  |  |  |  |  |  |  |  |
| 41 | n773 | 30 |  | -87.3 +TT | -85.3 +TT | -84.1 +TT |  |  | -82.9 +TT | -82.9 +TT | -82.9 +TT |  | -82.1 +TT | -81.7 +TT | -81.3 +TT |
| 30 | n66 | 15 | -91.2 +TT | -88.0 +TT | -86.2 +TT | -85.0 +TT |  |  |  | -81.8 +TT |  |  |  |  |  |
| n78 | 71 | N/A | -93.5 | -90.5 | -88.7 | -87.5 |  |  |  |  |  |  |  |  |  |
| n78 | 38 | N/A | -96.7 | -93.7 | -91.9 | -90.7 |  |  |  |  |  |  |  |  |  |
| n78 | 411 | N/A | -92.8 | -89.8 | -88.0 | -86.8 |  |  |  |  |  |  |  |  |  |
| 41 | n78 | 30 |  | -87.8 +TT | -85.8 +TT | -84.6 +TT |  |  | -83.4 +TT | -83.4 +TT | -83.4 +TT |  | -82.6 +TT | -82.2 +TT | -81.8 +TT |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 3: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 4: Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-3a: Reference sensitivity exceptions due to cross band isolation for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band |  | DL band |  | SCS (kHz) |  | 5 MHz (dBm) |  | 10 MHz (dBm) |  | 15 MHz (dBm) |  | 20 MHz (dBm) |  | 25 MHz (dBm) |  | 30 MHz (dBm) |  | 40 MHz (dBm) |  | 50 MHz (dBm) |  | 60 MHz (dBm) |  | 70 MHz (dBm) |  | 80 MHz (dBm) |  | 90 MHz (dBm) |  |  | 100 MHz (dBm) |
| 3 |  | n41 |  | 30 |  | -94.4 +TT |  | -92.4 +TT |  | -91.3 +TT |  |  |  | -89.3 +TT |  | -88.0 +TT |  | -87.0 +TT |  | -86.2 +TT |  |  |  | -84.9 +TT |  | -84.4 +TT |  | -84.0 +TT |  |  |  |
| n41 |  | 3 |  | N/A |  | -94.0 |  | -91.0 |  | -89.2 |  | -88.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 |  | 2 |  | N/A |  | -98.0 +1.0 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 |  | 66 |  | N/A |  | -99.5 +1.0 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: TT is the same as defined in Table 7.3B.2.3.5-1a. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-4 with uplink configuration specified in Table 7.3B.2.3.4.2.1-6.

Table 7.3B.2.3.5-4: Reference sensitivity exceptions due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order | Duplex mode |
| DC_1A_n3A | 1 | N/A | [-76.3] | - | - | - | - | IMD3 | FDD |
|  | n3 | 15 | REFSENS |  | - | - | - | N/A | TDD |
| DC_1A_n8A | 1 | N/A | -93.3 | - | - | - | - | IMD4 | FDD |
|  | n8 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_1A_n77A | 1 | N/A | -69.5 | - | - | - | - | IMD23 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n77A | 1 | N/A | -91.3 | - | - | - | - | IMD4- | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n78A | 1 | N/A | -91.3 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n66A | 2 | N/A | -77.3 | - | - | - | - | IMD3 |  |
|  | n66 | 15 | REFSENS | - | - | - | - | N/A |  |
| DC_2A_n66A | 2 | N/A | REFSENS | - | - | - | - | N/A |  |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 |  |
| DC_2A_n77A. DC_2A_n77(2A) | 2 | N/A | -71.3 | - | - | - | - | IMD2 | FDD |
| DC_2A-2A_n77A DC_2A_n77(2A) DC_2A-2A_n77(2A) | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 2 | N/A | -89.3 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 2 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n78A | 2 | N/A | -71.3 | - | - | - | - | IMD23 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | - | TDD |
| DC_2A_n78A | 2 | N/A | -89.3 | - | - | - | - | N/A | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | IMD43 | TDD |
| DC_3A_n1A | 3 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -74.0+TT | - | - | - | - | IMD3 | FDD |
| DC_3A_n5A | 3 | N/A |  | -89.3 | - | - | - | IMD4 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 3 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -74.0+TT | - | - | - | - | IMD23 | FDD |
| DC_3A_n7A | 3 | N/A | REFSENS | - | - | - | - | - |  |
|  | n7 | 15 | - | -84.6 +TT5 | - | - | - | - |  |
| DC_3A_n8A | 3 | N/A | - | REFSENS | - | - | - | N/A | FDD |
|  | n8 | 15 | -88.3 | - | - | - | - | IMD43 | FDD |
|  | 3 | N/A | - | -86.9 | - | - | - | IMD5 | FDD |
|  | n8 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_3A_n41A | 3 | N/A | -88.1 | - | - | - | - | IMD4 | FDD |
|  | n41 |  | REFSENS | - | - | - | - |  |  |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -70.3 | - | - | - | - | IMD23 | FDD |
|  |  |  | -70.67 | - | - | - | - |  |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n77A, DC_3A_n77(2A), DC_3C_n77A, DC_3C_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n66A | 5 | N/A | -67.3 | - | - | - | - | IMD23 | FDD |
|  | n66 |  | REFSENS | - | - | - | - | N/A | FDD |
| DC_5A_n77A8e | 5 | N/A | -89 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 5 | N/A | -91.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n78A | 5 | N/A | -89.0 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_7A_n3A | 7 | N/A | - | -81.3 | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_7A_n5A | 7 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -86.0+TT | - | - | - | - | IMD33 | FDD |
| DC_7A_n66A | 7 | N/A | - | -79.3 | - | - | - | IMD4 | FDD |
|  | n66 | 15 | REFSENS | - |  | - | - | N/A | TDD |
| DC_7A_n77A | 7 | N/A | -90.2 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n1A | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -94.0 + TT | - | - | -- | - | IMD4 | FDD |
| DC_8A_n3A | 8 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  | n3 | 15 | - | REFSENS | - | - | - | N/A | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | - | -87.4+ TT | - | - | - | IMD5 | FDD |
| DC_8A_n20A | n20 | 15 | -71.3 | - | - | - | - | IMD3 | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n20 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 8 | N/A | -71.3 | - | - | - | - | IMD3 | FDD |
| DC_8A_n41A | 8 | N/A | -84.2 | - | - | - | - | IMD33 | FDD |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n77A DC_8A_n78A DC_8A-SUL_n78A-n81A | 8 | N/A | -88.0 | - | - | - | - | IMD4 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | H4 | TDD |
| DC_12A_n77A DC_12A_n77(2A) | 12 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_12_n78 | 12 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_13A_n77A | 13 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_14A_n77A DC_14A_n77(2A) | 14 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_19A_n78A | 19 | N/A | -85.7 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | FDD |
| DC_20A_n3A | 20 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | -93.0 +TT | - | - | - | - | IMD4 | FDD |
|  | 20 | N/A | -87.3 | - | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_20_n7 | 20 | N/A | -84.3 | - | - | - | - | IMD3 | FDD |
|  | n7 | 15 | - | REFSENS | - | - | - | N/A | FDD |
| DC_20_n8 | 20 | N/A | -71.3 | - | - | - | - | IMD3 | FDD |
|  | n8 | 15 | -71.3 | - | - | - | - | IMD3 | FDD |
| DC_20A_n78A, DC_20A-SUL_n78A-n82A | 20 | N/A | -74.6 | - | - | - | - | IMD44 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_21A_n28A | 21 | N/A | -96.8 | - | - | - | - | IMD5 | FDD |
|  | n28 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_21A_n79A | 21 | N/A | -80.9 | - | - | - | - | IMD3 | FDD |
|  | n79 | 15 | - | - | - | - | REFSENS | N/A | TDD |
| CA_28A_n77A, CA_28A_n78A, DC_28A-SUL_n78A-n83A | 28 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_30A_n77A DC_30A_n77(2A) | 30 | N/A | -92.8 | - | - | - | - | IMD3 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_48A_n66A | 48 | N/A | - | REFSENS | - | - | - | N/A | TDD |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 | FDD |
| DC_66A_n2A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n2 | 15 | -78.0+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n2 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n5A | 66 | N/A | -68.8 | - | - | - | - | IMD23 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n25A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n25 | 15 | -76.5+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -75.8 | - | - | - | - | IMD3 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n71A | 66 | N/A | -93.8 | - | - | - | - | IMD4 | FDD |
|  | n71 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n77A DC_66A_n77(2A) DC_66A-66A_n77A DC_66A-66A_n77(2A) | 66 | N/A | -67.8 | - | - | - | - | IMD2 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_66A_n78A | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_71A_n66A | 71 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n66 | 15 | -94.5 +TT | - | - | - | - | IMD4 | FDD |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: Void NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Void. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-5: Reference sensitivity exceptions due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm)) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order) | Duplex mode |
| DC_1A_n78A | 1 | N/A | REFSENS-17.8 | - | - | - | - | IMD4 |  |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n77A | 2 | N/A | -65.2 | - | - | - | - | IMD2 | FDD |
| DC_2A-2A_n77A |  |  | -65.157 |  |  |  |  |  |  |
| DC_2A_n77C | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A-2A_n77C | 2 | N/A | -78.2 | - | - | - | - | IMD49 | FDD |
|  |  |  | -78.157 |  |  |  |  |  |  |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n41A | 3 | N/A | -77.9 | - | - | - | - | IMD4 | FDD |
|  |  |  | -80.67 | - | - | - | - |  |  |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n78A | 3 | N/A | -64.4 | - | - | - | - | IMD2 | FDD |
|  |  |  | -67.17 | - | - | - | - |  |  |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n78A | 3 | N/A | -77.8 | - | - | - | - | IMD4 | FDD |
|  |  |  | -80.57 | - | - | - | - |  |  |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n77A8 | 5 | N/A | -78.7 | - | - | - | - | IMD49 | FDD |
| DC_5A_n77C8 | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_12A_n77A | 12 | N/A | -84.6 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_13A_n77A | 13 | N/A | -80.93 | - | - | - | - | IMD5 | FDD |
| DC_13A_n77C | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_14A_n77A | 14 | N/A | -84.6 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_19A_n78A | 19 | N/A | -74.0 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | FDD |
| DC_21A_n79A | 21 | N/A | -65.9 | - | - | - | - | IMD3 | FDD |
|  | n79 | 15 | - | REFSENS |  | - | - | N/A | TDD |
| DC_28A_n77A | 28 | N/A | -78.6 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_30A_n77A | 30 | N/A | -81.4 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_66A_n77A DC_66A-66A_n77A DC_66A-66A-66A_n77A DC_66A_n77C DC_66A-66A_n77C DC_66A-66A-66A_n77C | 66 | N/A | -64.47 | - | - | - | - | IMD2 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 66 | N/A | -87.53 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: For UEs only indicating support of Single UL, this requirement is verified with non-simultaneous uplink transmissions on the E-UTRA and NR CGs. NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |

<End of Changes>