3GPP TSG-RAN5 Meeting #103	R5-242861

# Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5321 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction for Specific message contents in 9.2.3.1.22 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to TS.36.523-1, there are editorial corrections for test case 9.2.3.1.22 “Normal tracking area update / Abnormal case / access barred due to access class control or NAS signalling connection establishment rejected by the network”. Editorioal correction point is as follows. Mistakes in Information Element on Table Table 9.2.3.1.22.3.3-1: SystemInformationBlockType2 for Cell A (step 1, Table 9.2.3.1.22.3.2-1) and Table 9.2.3.1.22.3.3-2: SystemInformationBlockType2 for Cell B (step 11, Table 9.2.3.1.22.3.2-1) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial correction for Specific message contents in 9.2.3.1.22 “Normal tracking area update / Abnormal case / access barred due to access class control or NAS signalling connection establishment rejected by the network”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will make a misreading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

##### 9.2.3.1.22	Normal tracking area update / Abnormal case / access barred due to access class control or NAS signalling connection establishment rejected by the network

9.2.3.1.22.1	Test Purpose (TP)

(1)

with { The UE is in the state EMM-REGISTERED }

ensure that {

when { Access is barred for signalling in the cell UE is camping [Access Class barred in System information] }

then { the UE will not initiate the tracking area updating procedure on the current cell }

}

(2)

with { The UE is in the state EMM-REGISTERED }

ensure that {

when { Access is barred for signalling in the cell UE is camping [T302 running due to RRCConnectionReject message reception] }

then { the UE will not initiate the tracking area updating procedure on the current cell }

}

(3)

with { The UE is in the state EMM-REGISTERED }

ensure that {

when { Access is not barred for signalling in the cell UE is camping }

then { the UE will initiate the tracking area updating procedure on the current cell }

}

(4)

with { The UE is in the state EMM-REGISTERED }

ensure that {

when { Access was barred for signalling in the cell and UE has reselected an new cell where access for "signalling" is granted }

then { the UE will initiate the tracking area updating procedure on the new cell }

}

9.2.3.1.22.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.3.2.6 and TS 36.331, clause 5.3.3.2

[TS 24.301, clause 5.5.3.2.6]

The following abnormal cases can be identified:

a)	Access barred because of access class barring or NAS signalling connection establishment rejected by the network

If access is barred for "signalling" (see 3GPP TS 36.331 [22]), the tracking area updating procedure shall not be started. The UE stays in the current serving cell and applies the normal cell reselection process. The tracking area updating procedure is started as soon as possible and if still necessary, e.g. when access for "signalling" is granted on the current cell or when the UE moves to a cell where access for "signalling" is granted.

[TS 36.331, clause 5.3.3.2]

…

1>	else (the UE is establishing the RRC connection for mobile originating signalling):

2>	if timer T302 or T305 is running:

3>	consider access to the cell as barred;

2>	else if SystemInformationBlockType2 includes the ac-BarringInfo and the ac-BarringForMO-Signalling is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	for at least one of these Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Signalling is set to zero:

4>	consider access to the cell as not barred;

3>	else:

4>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

4>	if 'rand' is lower than the value indicated by ac-BarringFactor included in ac-BarringForMO-Signalling:

5>	consider access to the cell as not barred;

4>	else:

5>	consider access to the cell as barred;

2>	else:

3>	consider access to the cell as not barred;

9.2.3.1.22.3	Test description

9.2.3.1.22.3.1	Pre-test conditions

System Simulator:

-	cell A and cell B are configured according to table 6.3.2.2-1 in TS 36.508 [18].

-	cell A belongs to TAI-1 (home PLMN)

-	cell B belongs to TAI-2 (home PLMN)

-	cell D belongs to TAI-4 (home PLMN)

UE:

None;

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell B according to TS 36.508 [18].

9.2.3.1.22.3.2	Test procedure sequence

Table 9.2.3.1.22.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sets the cell type of cell A to the ''Serving cell'', sets the cell type of cell B to the '' Non-Suitable cell'', and sets SystemInformationBlockType2 parameters as described below. | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | Check: for 60 seconds if UE initiates the tracking area updating procedure on cell A? | - | - | 1 | F |
| 3 | The SS transmits a Paging message including systemInfoModification. | - | - | - | - |
| 4 | The SS changes SystemInformationBlockType2 parameters to default parameters defined in [18]. | - | - | - | - |
| 5 | The UE transmits RRC Connection Request | - | - | - | - |
| 6 | SS responds with RRCConnectionReject message with IE waitTime set to 10 seconds(Max Value). | - | - | - | - |
| 7 | Check: for 10 seconds if UE initiates the tracking area updating procedure and hence transmits RRC Connection Request? | - | - | 2 | F |
| 8 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST? | --> | TRACKING AREA UPDATE REQUEST | 3 | P |
| 9 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 10 | The UE sends TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 10A | The SS releases the RRC connection. | - | - | - | - |
| 11 | The SS set SystemInformationBlockType1 and SystemInformationBlockType2 parameters as described below. | - | - | - | - |
| 12 | Void | - | - |  |  |
| 13 | The SS sets the cell type of cell B to the '' serving cell'', cell A to “non-suitable off Cell” , and cell D to “Suitable neighbour intra-frequency cell”. |  |  |  |  |
| 13A | Check: For 60 seconds if the UE initiates the tracking area update procedure on Cell B? | - | - | 1 | F |
| 13B | The SS sets the cell type of cell B to “Suitable neighbour intra-frequency cell” and cell D to “serving cell” | - | - | - | - |
| - | The following messages are to be observed on Cell D unless explicitly stated otherwise. | - | - | - | - |
| 14 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST? | --> | TRACKING AREA UPDATE REQUEST | 4 | P |
| 15 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 16 | The UE sends TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508 | - | - | - | - |

9.2.3.1.22.3.3	Specific message contents

Table 9.2.3.1.22.3.3-1: SystemInformationBlockType2 for Cell A (step 1, Table 9.2.3.1.22.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringInfo SEQUENCE { |  |  |  |
| ac-BarringForEmergency | FALSE |  |  |
| ac-BarringForMO-Signalling SEQUENCE { |  |  |  |
| ac-BarringFactor | p00 |  |  |
| ac-BarringTime | s4 |  |  |
| ac-BarringForSpecialAC | ‘11111’B |  |  |
| } |  |  |  |
| ac-BarringForMO-Data | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.1.22.3.3-1A: SystemInformationBlockType1 for Cell A (step 4, Table 9.2.3.1.22.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 |  |  |
| } |  |  |  |

Table 9.2.3.1.22.3.3-1AA: SystemInformationBlockType1-BR-r13 for Cell A (step 4 when UE under test is CAT-M1, Table 9.2.3.1.22.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | UECAT0 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | n entries | The same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | SystemInfoValueTagSI-r13[k]=1, where k is the entry corresponding to the system info including SystemInformationBlockType2. For all other entries the value is set to 0 (same as in the default SystemInformationBlockType1-BR-r13 message). |  |  |
| SystemInfoValueTagSI-r13[n] |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.1.22.3.3-2: SystemInformationBlockType2 for Cell B (step 11, Table 9.2.3.1.22.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringInfo SEQUENCE { |  |  |  |
| ac-BarringForEmergency | FALSE |  |  |
| ac-BarringForMO-Signalling SEQUENCE { |  |  |  |
| ac-BarringFactor | p00 |  |  |
| ac-BarringTime | s4 |  |  |
| ac-BarringGorSpecialACClassBarringList SEQUENCE (SIZE (maxAC)) OF SEQUENCE { | ‘11111’B |  |  |
| } |  |  |  |
| ac-BarringForMO-Data | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.1.22.3.3-3: SystemInformationBlockType1 for Cell B (step 11, Table 9.2.3.1.22.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| intraFreqReselection | allowed |  |  |
| } |  |  |  |
| systemInfoValueTag | 1 |  |  |
| } |  |  |  |

Table 9.2.3.1.22.3.3-4: SystemInformationBlockType1-BR-r13 for Cell B (step 11, Table 9.2.3.1.22.3.2-1 when UE under test is CAT-M1)

| Derivation Path: 36.508, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| intraFreqReselection | allowed |  |  |
| } |  |  |  |
| systemInfoValueTag | 1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | UECAT0 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | n entries | The same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | SystemInfoValueTagSI-r13[k]=1, where k is the entry corresponding to the system info including SystemInformationBlockType2. For all other entries the value is set to 0 (same as in the default SystemInformationBlockType1-BR-r13 message). |  |  |
| SystemInfoValueTagSI-r13[n] |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>