3GPP TSG-RAN5 Meeting #103	R5-242867

# Fukuoka, Japan, May 20th 2024 - May 24th 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3129 | rev | - | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MCE RRM test case 6.5.7D.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add minimum requirements for Tx switching across 3 or 4 bands tests; - To add MCE RRM test case 6.5.7D.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Minimum requirements for Tx switching across 3 or 4 bands tests are added. MCE RRM test case 6.5.7D.1 is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.7D.0 (new), 6.5.7D.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 CR 3132 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.5.7D		DL interruptions at UE switching across three or four uplink bands

6.5.7D.0	Minimum conformance requirements

The DL interruption requirements at dynamic switching across three or four uplink bands specified in this clause are applicable for an NR inter-band CA configuration and SUL band configuration with inter-band CA when the capability BandCombination-UplinkTxSwitch-v1800 is present, and are only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [9]. In the NR inter-band CA configuration and SUL band configuration with inter-band CA, the number of NR uplink bands with different carrier frequencies is three or four. NR UL carrier(s) in each of the three or four uplink bands are capable of one or two transmit antenna connector(s), according to the UE capability.

When dynamic switching across three or four uplink bands is conducted, UE is allowed to cause DL interruption of X OFDM symbols in NR downlink carrier(s) as indicated by uplinkTxSwitching-DL-Interruption [2]. The DL interruption starts from the first OFDM symbol which fully or partially overlaps with the UL Tx switching period located in uplink NR band(s) as indicated in RRC signalling [2]. The DL interruption lengths of X for single TAG and for 2-TAG are defined in Table 6.5.7D.0-1 and Table 6.5.7D.0-2.

Note:	If one downlink carrier is indicated to be interrupted by two band pairs for dynamic switching simultaneously, the DL interruption length on the victim carrier is determined by the maximum of uplink switching periods of the two band pairs. If UplinkTxSwitchingAdditionalPeriodDualUL-r18 is reported, DL interruption length is determined by the uplink Tx switching period, which is indicated by UplinkTxSwitchingAdditionalPeriodDualUL-r18. If periodOnULBands-r18 is reported, DL interruption length is determined by the uplink Tx switching period, which is indicated by periodOnULBands-r18.

No DL interruption is allowed in the NR downlink carrier(s) which is not indicated by uplinkTxSwitching-DL-Interruption. No DL interruption is allowed for some inter-band UL CA configurations and SUL band configurations with inter-band CA as specified in TS 38.101-1 [2].

Table 6.5.7D.0-1: DL interruption length on NR carrier(s) in the unit of OFDM symbols (X) for switching across three or four uplink bands for single TAG

|  | NR Slot length (ms) | Uplink Tx switching period Note1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | 7 |
| Note 1: Uplink Tx switching period depends on UE capability. Note 2: RTD=3us is assumed to derive the DL interruption length. |  |  |  |  |

Table 6.5.7D.0-2: DL interruption length on NR carrier(s) in the unit of OFDM symbols (X) for switching across three or four uplink bands for two TAGs

|  | NR Slot length (ms) | Uplink Tx switching period Note1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | 8 |
| Note 1: Uplink Tx switching period depends on UE capability. Note 2: RTD=9 us is assumed to derive the DL interruption length. |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 8.2.2.2.10D.

6.5.7D.1	NR SA FR1 DL interruptions at switching across three uplink bands in TDD-TDD CA for single TAG

Editor’s Note: This test case is incomplete in following aspects:

-	Message contents are still missing.

-	TT analysis is still missing.

6.5.7D.1.1	Test purpose

The purpose of this test is to verify DL interruption requirements during UE dynamic switching across three uplink bands for single TAG defined in TS 38.133 [6] clause 8.2.2.2.10D.

6.5.7D.1.2	Test applicability

This test applies to all types of NR UE release 18 onwards supporting of inter-band UL CA (three bands), dynamic UL Tx switching across up to 4 bands and simultaneous transmission and reception in TDD-TDD inter-band NR CA.

6.5.7D.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.7D.0.

The normative reference for this requirement is TS 38.133 [6] A.6.5.7D.1

6.5.7D.1.4	Test description

The test consists of three active FR1 NR cells: PCell (Cell 1), SCell (Cell 2) and SCell (Cell 3), where

- Cell 1, Cell 2 and Cell 3 are on NR UL carrier 1 in band A, NR UL carrier 2 in band B and NR UL carrier 3 in band C, respectively;

- Cell 1, Cell 2 and Cell 3 belong to the same TAG;

- Band A, B and C are three different TDD bands.

- UE is capable to transmit with one, one and two transmiting antenna connector(s) on Cell 1, Cell 2 and Cell 3, respectively;

The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, UplinkTxSwitchingMoreBands-r18 is indicated to UE.

6.5.7D.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.7D.1.4.1-1.

Table 6.5.7D.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 6.5.7D.1-1 | NR Cell 1: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 6.5.7D.1.4.1-2.

Table 6.5.7D.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.7D.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 for Cell 1 and Cell 2, A.3.1.8.2a for Cell 3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | Without LTE link Without Fader |  |  |

1. The general test parameter settings are set up according to Table 6.5.7D.1.4.1-3.

2. Message contents are defined in clause 6.5.7D.1.4.3.

3. The test scenario comprises of three NR Cells. (Cell 1, Cell 2 and Cell 3). Cell 1, Cell 2 and Cell 3 are configured according to Annex C.1.2 and C.1.3.

Table 6.5.7D.1.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | Three radio channels are used for this test. |
| Active cell |  | Cell 1: FR1 PCell Cell 2: FR1 SCell Cell 3: FR1 SCell | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 FR1 SCell on RF channel number 3 |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Cell 1: CSI-RS.2.5 TDD Cell 2: CSI-RS.2.5 TDD Cell 3: CSI-RS.2.5 TDD |  |
| T1 | s | 5 |  |

6.5.7D.1.4.2	Test procedure

- Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

- The SS transmits an RRCReconfiguration message with UplinkTxSwitchingMoreBands-r18 to the UE to configure Cell 2 and Cell 3 as SCells according to Table 6.5.7D.1.5-1.

- The UE shall transmit an RRCReconfigurationComplete message.

- The SS activates Cell 2 and Cell 3.

- Set the parameters according to T1 in Table 6.5.7D.1.4.1-3 and Table 6.5.7D.1.5-1. Propagation conditions are set according to Annex C clauses C.2.2. T1 starts.

- For Cell 1 and Cell 2, the SS triggers aperiodic CSI-RS for L1-RSRP reporting with power boost of 6dB on the following symbol in the 1st special slot of every radio frame:

-	symbol #10 if UE does not report uplinkTxSwitching-DL-Interruption-r18;

-	otherwise,

-	symbol #4 if UE capability uplink Tx switching period is 210 us, or

-	symbol #5 if UE capability uplink Tx switching period is 140us, or

-	symbol #8 if UE capability uplink Tx switching period is 35us.

- For Cell 3, the SS triggers aperiodic CSI-RS for L1-RSRP reporting with power boost of 6dB on the following symbol in the 2nd special slot of every radio frame:

-	symbol #10 if UE does not report uplinkTxSwitching-DL-Interruption-r18;

-	otherwise,

-	symbol #4 if UE capability uplink Tx switching period is 210 us, or

-	symbol #5 if UE capability uplink Tx switching period is 140us, or

-	symbol #8 if UE capability uplink Tx switching period is 35us.

- If the UE sends L1-RSRP report containing valid L1-RSRP results for Cell 1, Cell 2 and Cell 3 and each L1-RSRP result meet the test requirements given in Table 6.5.7D.1.5-3, increase the number of passed iterations by one otherwise increase the number of failed iterations by one.

- SS transmits in Cell 1 an RRCReconfiguration message to release all SCells and ensures UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the reconfiguration fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5)

or

switches off and on the UE and ensures UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

- Repeat step 2-9 until a test verdict has been achieved.

6.5.7D.1.4.3	Message contents

TBD

6.5.7D.1.5	Test requirements

Table 6.5.7D.1.5-1 defines the primary level settings including test tolerances for NR SA FR1 DL interruptions at switching across three uplink bands in TDD-TDD CA for single TAG.

Table 6.5.7D.1.5-1: Cell specific test parameters

| Parameter | Unit | Cell 1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- |
| Frequency Range |  | FR1 | FR1 | FR1 |
| Duplex mode |  | TDD | TDD | TDD |
| TDD configuration |  | TDDConf.2.1 except that S=’11DL: 1GP :2UL’ | TDDConf.2.1 except that S=’11DL: 1GP :2UL’ | TDDConf.2.2 |
| BWchannel | MHz | 40: NRB,c = 106 | 40: NRB,c = 106 | 40: NRB,c = 106 |
| Initial BWP Configuration |  | DLBWP.0.1 | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration |  | DLBWP.1.1 | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration |  | ULBWP.1.1 | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration |  | SRSConf.1 in Table 6.5.7D.1.5-1 | SRSConf.1 in Table 6.5.7D.1.5-1 | SRSConf.2 in Table 6.5.7D.1.5-1 |
| PDSCH RMC |  | SR.2.1 TDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET RMC |  | CR.2.1 TDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET RMC |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  | SMTC.1 | SMTC.1 | SMTC.1 |
| SSB Configuration |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| NocNote 2 | dBm/ 15 kHz | -104+TT | -104+TT | -104+TT |
| SS-RSRP Note 3 | dBm/ SCS | -87+TT | -87+TT | -87+TT |
| Ês/Iot | dB | 14+TT | 14+TT | 14+TT |
| Ês/Noc | dB | 14+TT | 14+TT | 14+TT |
| NocNote 2 | dBm/ SCS | -101+TT | -101+TT | -101+TT |
| IoNote3 | dBm/ 38.16 MHz | -55.79+TT | -55.79+TT | -55.79+TT |
| Time offset to Cell1 Note 4 | s | - | 0 | 0 |
| Propagation Condition |  | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between slot boundaries of signals received from the three cells at the UE antenna connector including time alignment error between the three cells. |  |  |  |  |

Table 6.5.7D.1.5-2: SRS Configurations

|  | Field | SRSConf.1 | SRSConf.2 |
| --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |
|  | srs-ResourceIdList | 0 | 0 |
|  | resourceType | Periodic | Periodic |
|  | Usage | Codebook | Codebook |
|  | SRS-ResourceSetId | 0 | 0 |
| SRS-Resource | nrofSRS-Ports | Port1 | Port1 |
|  | transmissionComb | n2 | n2 |
|  | combOffset-n2 | 0 | 0 |
|  | cyclicShift-n2 | 0 | 0 |
|  | resourceMapping startPosition | 0 | 0 |
|  | resourceMapping nrofSymbols | n2 | n2 |
|  | resourceMapping repetitionFactor | n1 | n1 |
|  | freqDomainPosition | 0 | 0 |
|  | freqDomainShift | 0 | 0 |
|  | freqHopping c-SRS | 25 | 25 |
|  | freqHopping b-SRS | 0 | 0 |
|  | freqHopping b-hop | 0 | 0 |
|  | groupOrSequenceHopping | Neither | Neither |
|  | resourceType | Periodic | Periodic |
|  | periodicityAndOffset-p | sl20,5 | sl20,3 |
|  | sequenceId | 0 | 0 |

Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 6.5.7D.1.5-3.

Table 6.5.7D.1.5-3: L1-RSRP absolute accuracy requirements for the reported values of CSI-RS

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value | 75-TT |
| Highest reported value | 75+TT |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.