3GPP TSG-RAN5 Meeting #103	R5-242925

# Fukuoka, Japan, May 20-24, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2788 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test cases 6.2L.3.x for inter-band CA with TxD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case 6.2L.3.2 MPR, 6.2L.3.3 A-MPR and 6.2L.3.4 configured transmitted power for inter-band CA with TxD are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the new test cases 6.2L.3.2 MPR, 6.2L.3.3 A-MPR and 6.2L.3.4 configured transmitted power for inter-band CA with TxD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2L.3.2(new), 6.2L.3.3(new), 6.2L.3.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2L.3.2	UE maximum output power reduction for inter-band UL CA with Tx Diversity

6.2L.3.2.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified maximum output power with MPR and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2L.3.2.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with Tx Diversity.

6.2L.3.2.3	Minimum conformance requirements

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the requirements in clause 6.2G.2 apply for the component carrier configured with Tx Diversity and the requirements in clause 6.2.2 apply for the other component carrier.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2L.3.2.

6.2L.3.2.4	Test procedure

This test is covered by clauses 6.2.2 UE maximum output power reduction and 6.2G.2 UE maximum output power reduction for Tx Diversity.

6.2L.3.2.5	Test requirements

Same test requirement as 6.2G.2 for the band supporting Tx Diversity and 6.2.2 for the other band.

6.2L.3.3	UE additional maximum output power reduction for inter-band UL CA with Tx Diversity

6.2L.3.3.1	Test purpose

Additional emission requirements for CA can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

6.2L.3.3.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with Tx Diversity.

6.2L.3.3.3	Minimum conformance requirements

For inter-band UL CA with Tx Diversity in one of the two frequency bands, unless specified in Table 6.2A.3.0.3-1, the requirements in clause 6.2.3 apply only to the indicated carrier.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2L.3.3.

6.2L.3.3.4	Test procedure

This test is covered by clauses 6.2.3 UE additional maximum output power reduction and 6.2G.3 UE additional maximum output power reduction for Tx Diversity.

6.2L.3.3.5	Test requirements

Same test requirement as 6.2G.3 for the band supporting Tx Diversity and 6.2.3 for the other band.

6.2L.3.4	Configured transmitted power for inter-band UL CA with Tx Diversity

6.2L.3.4.1	Test purpose

To verify that the total measured UE configured maximum output power PUMAX in two uplink carrier aggregation is within the specified bounds.

6.2L.3.4.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with Tx Diversity.

6.2L.3.4.3	Minimum conformance requirements

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the requirements in clause 6.2A.4.1.3 apply except that:

-	PPowerClass,CA is the maximum UE power specified in Table 6.2L.3.1-1 without taking into account the tolerance;

-	MPRc and A-MPRc are specified in clause 6.2G.2 and clause 6.2G.3 respectively for the component carrier configured with Tx Diversity.

-	ΔPPowerClass,CA:

–	For a power class 2 UE, it is 3dB when the requirements of default power class are applied as specified in sub-clause 6.2.J.3.1, otherwise ΔPPowerClass, CA = 0 dB;

–	For a power class 1.5 UE, it is 6dB when the requirements of default power class are applied as specified in sub-clause 6.2.J.3.1; and it is 3dB when the requirements of power class 2 are applied as specified in sub-clause 6.2.J.3.1; otherwise ΔPPowerClass, CA = 0 dB;

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2L.3.4.

6.2L.3.4.4	Test procedure

This test is covered by clauses 6.2.4 Configured transmitted power and 6.2G.4 Configured transmitted power for Tx Diversity.

6.2L.3.4.5	Test requirements

Same test requirement as 6.2G.4 for the band supporting Tx Diversity and 6.2.4 for the other band.

<End of Changes>