3GPP TSG-RAN5 Meeting #103	R5-242942

Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

Source:			CATT

Title:	Discussion Paper for Rel-17 eNS_Ph2 WI NSAC Session management tests case applicability

Document for:	Discussion and Endorsement

Agenda Item:	6.4.x

- Introduction

The Rel-17 work item on Enhancement of Network Slicing Phase 2 was introduced for the purpose of Network Slice Admission Control (NSAC), including the following UE requirements:

- Restriction on the maximum number of UEs per network slice with a proper cause and a backoff timer.

- Restriction on the maximum number of PDU sessions per network slice with a proper cause and a backoff timer.

- Support of subscription-based mechanism ensuring that a UE is only allowed to be registered with compatible network slices.

In R5-241479, Analysis on NSAC / Session management test cases shows:

Observation#2 - None of the SM test case introduced are testing unique UE requirement specific to R-17 eNS_Ph2 WI. This may cause even earlier UE release to PASS this test case but more importantly the UE requirements are not part of R-17 eNS_Ph2 WI

- Discussion

Observation: Most updates for R-17 eNS_Ph2 WI on Session management aspects are enhanced based on the existing rejection cause values: #67 "insufficient resources for specific slice and DNN", #67 "insufficient resources for specific slice and DNN", #69 "insufficient resources for specific slice" and Back-off timer…which have already been defined since Rel-15. It is hard to describe the scenario for conformance testing on enhancement of NSAC along. The issue observed in R5-241479 exists.

- Proposals

Proposal#1：Update the applicability of test cases in 10.1.8 NSAC / Session management aspects from Rel-17 to Rel-15, related WI code from eNS_Ph2-UEConTest to 5GS_NR_LTE-UEConTest.

Proposal#2：A new Pics based on UE General Capability of supporting NSAC will be defined to limit the test cases to be applied only after this feature is commercialized along with network IOT.

4.	References

[1] R5-236570	Discussion Paper for Rel-17 eNS_PH2 WI Test case applicability

[2] R5-241479	Discussion Paper for Rel-17 eNS_Ph2 WI tests case applicability