3GPP TSG-RAN5 Meeting #103	R5-242979

# Fukuoka City, Japan, 20th - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3167 | rev | - | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction of references to TS 38.101-1 in section 7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_CLI-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 38.533, section 2 References, the following references are defined [2] 3GPP TS 38.101-1, [18] 3GPP TS 38.521-2 There are several wrong references in TS 38.533, referring to 38.101-1 [18] |  |  |  |  |  |  |  |  |
| Summary of change: |  | References to TS 38.101-1 [18] have been corrected to TS 38.101-1 [2] in section 7 of TS 38.533 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | References to TS 38.101-1 and TS 38.521-2 will remain inconsitent in TS 38.533 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

#### 7.7.5.0	Minimum conformance requirements

##### 7.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy

The SRS-RSRP measurement reported by the UE shall fulfil the accuracy requirements defined in Table 7.7.5.0.1-1 for FR1 and Table 5.7.7.0.1-2 for FR2, provided that the following conditions are met. The accuracy requirements in this clause are derived based on AWGN radio propagation conditions.

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for SRS-RSRP measurements are fulfilled according to Annex B.2.z for a corresponding Band for each relevant SRS resource configured for measurement.

-	The time difference between UE’s DL reference timing in the serving cell and SRS arrival time is no larger than Terror_SRS_RSRP, where

-	Terror_SRS_RSRP = TC × NTA_offset + 4.67us for FR1

-	Terror_SRS_RSRP = TC × NTA_offset + 3.67us for FR2

-	NTA_offset is defined in Table 7.1.2-2

-	TC is 0.509ns

-	The number of SRS ports in the SRS resource configured for measurement is 1,

-	The number of symbols in the SRS resource configured for measurement is 1,

-	The number of repetitions in the SRS resource configured for measurement is 1,

-	Frequency hopping, sequence group hopping or sequence hopping is disabled in the SRS resource configured for measurement,

-	The bandwidth of the SRS resource is 48 PRBs.

-	One of the following conditions is met

-	There is no other SRS resource with the same root sequence and on the same symbol and with same comb as the relevant SRS resource.

-	If multiple SRS resources are on the same symbol and with same comb, the distance between cyclic shifts of any two resources is no less than 6 if transmissionComb = n4, and no less than 4 if transmissionComb = n2.

Table 7.7.5.0.1-1: SRS-RSRP absolute accuracy in FR1

| Accuracy |  |  |  |  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  |  | Extreme condition |  |  | SRS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB |  |  |  |  |  | dB |  | dBm / SCSSRS |  |  | dBm/BW Channel | dBm/BW Channel |
| SCSSRS (kHz) |  |  | SCSSRS (kHz) |  |  |  |  |  |  |  |  |  |
| 15 | 30 | 60 | 15 | 30 | 60 |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |  |
| 3 | 3.5 | 5 | 7.5 | 8 | 9.5 | 1 | NR_TDD_FR1_A, | -120 | -117 | -114 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_C | -119 | -116 | -113 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_E | -118 | -115 | -112 | N/A | -70 |
| 6.5 | 7 | 8.5 | 9.5 | 10 | 11.5 | 1 | NR_TDD_FR1_A, NR_TDD_FR1_C, NR_TDD_FR1_D, NR_TDD_FR1_E | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.7.5.0.1-2: SRS-RSRP absolute accuracy in FR2

| Accuracy |  |  |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  | Extreme condition |  | SRS Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  |  |  | Minimum Io |  |  | Maximum Io |
| dB |  |  |  | dB | dBm / SCSSRS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
| SCSSRS (kHz) |  | SCSSRS (kHz) |  |  | SCSSRS = | SCSSRS = |  |  |
| 60 | 120 | 60 | 120 |  | 60kHz | 120kHz |  |  |
| 6 | 8.5 | 9 | 11.5 | ≥1 | Same value as SRS_RP in Table B.2.7-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 9 | 11.5 | 11 | 13.5 | ≥1 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |  |

7.7.5.0.1.1	SRS-RSRP report mapping

The reporting range of SRS-RSRP is defined from -140 dBm to -44 dBm with 1 dB resolution. The mapping of measured quantity is defined in Table 5.7.7.0.1.2-1. The range in the signalling may be larger than the guaranteed accuracy range.

Table 5.7.7.0.1.2-1: SRS-RSRP measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| SRS-RSRP_0 | SRS-RSRP<-140 | dBm |
| SRS-RSRP_1 | -140≤ SRS-RSRP<-139 | dBm |
| SRS-RSRP_2 | -139≤ SRS-RSRP<-138 | dBm |
| SRS-RSRP_3 | -138≤ SRS-RSRP<-137 | dBm |
| SRS-RSRP_4 | -137≤ SRS-RSRP<-136 | dBm |
| .. | .. | … |
| SRS-RSRP_95 | -46≤ SRS-RSRP<-45 | dBm |
| SRS-RSRP_96 | -45≤ SRS-RSRP<-44 | dBm |
| SRS-RSRP_97 | -44≤ SRS-RSRP | dBm |
| SRS-RSRP_98 | Infinity |  |
| Note: ‘Infinity’ means that UE cannot detect SRS due to too strong signal to measure. |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.22.1.

< End of changes >