3GPP TSG-RAN5 Meeting #103	R5-242997

# Fukuoka, Japan, 20th May – 24th May 2024

Title:							MU discussion on FR2c

Source:			Anritsu

Agenda Item:			5.4.20

Document for:			Endorsement

# 1.	Introduction

There is an action point AP#99.21 to define MTSU of FR2 RF test cases for FR2c (n259). In the previous RAN5 meetings, we have agreed on MTSU/TT/relaxation values for Frequency error [1][2], MOP (EIRP, TRP), REFSENS, EIS spherical coverage [3], MOP (Spherical coverage), MPR, Minimum output power, SEM, ACS/In-band blocking, and Tx/Rx Spurious emissions [4][5][6][7]. Note that [ ] remains in MTSU/TT/relaxation values for MPR, Minimum output power, and ACS/In-band blocking. This paper provides testability analysis on the remaining Transmit OFF power and ACLR, and propose to remove the remaining [ ]. Unless otherwise commented, observations and proposals in this paper are for PC3, CBW  400 MHz, NTC, IFF, max device size  40 cm.

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#99.21 | RF | Define MTSU for regulatory test cases (MOP, EIRP, TRP, Tx OFF power, Freq error, OBW, SEM, ACLR, Ref Sense, ACS, Inband Blocking, Tx/Rx Spurious emissions) for FR2 Band n259 | Anritsu, R&S, KEYS, DCM | R5-232983 R5-233174 R5-233971-5 R5-235140 R5-234891-3 R5-237725/7 R5-237731 R5-236067 R5-237733 R5-237734 R5-237723 R5-241083/4/5/6 R5-240401/2/3/4/5 R5-241344/5 | RAN5#103 | Open |

Table 1 Summary of the current agreement on MTSU and relaxation for FR2c (NTC)

| Test case | MTSU | Relaxation |
| --- | --- | --- |
| MOP (EIRP) | 6.64 dB | - |
| MOP (TRP) | 6.16 dB | - |
| MOP (Spherical coverage) | 6.84 dB | - |
| MPR | [6.89] dB | - |
| Minimum output power | [7.34] dB | [14.5] dB for 400 MHz |
| Transmit OFF power | No agreement | No agreement |
| Frequency error | 0.01 ppm | - |
| OBW | No agreement | No agreement |
| SEM | 6.86 dB | - |
| ACLR | No agreement | No agreement |
| REFSENS | 6.34 dB | - |
| EIS spherical coverage | 6.04 dB | - |
| ACS/In-band blocking | [9.46] dB | [6.8] dB for 100 MHz |
| Tx Spurious |  |  |
| Up to 80 GHz | Same as FR2b | Same as FR2b |
| 80 GHz to 87 GHz | 8.14 dB | - |
| Rx Spurious |  |  |
| Up to 80 GHz | Same as FR2b | Same as FR2b |
| 80 GHz to 87 GHz | 8.31 dB | 33.1 dB |

# 2.	Discussion

Since we have no concerns on the current MTSU/TT/relaxation values for MPR, Minimum output power, and ACS/In-band blocking, we propose to remove [ ] from them.

Proposal 1: Remove [ ] from MTSU/TT value of MPR for FR2c.

Proposal 2: Remove [ ] from MTSU/TT/relaxation value of Minimum output power for FR2c.

Proposal 3: Remove [ ] from MTSU/relaxation value of ACS/In-band blocking for FR2c.

## 2.1.	Transmit OFF power

Transmit OFF power has testability issue due to low UL power. To test with influence of noise = 1.0 dB, required relaxation values are  27.5 dB for CBW = 50 MHz, 30.5 dB for CBW = 100 MHz, 33.5 dB for CBW = 200 MHz, and 36.5 dB for CBW = 400 MHz based on the analysis in [7]. We propose to not test these unless there is a clear demand for testing. In addition to FR2c, there is also no clear demand for testing transmit OFF power for FR2b, with [ ] remaining in its relaxation values.

Proposal 4: Do not test Transmit OFF power for FR2b and FR2c.

## 2.2.	ACLR

Following table shows the testability at each test point and each CBW. In the table, test points with ΔSNR ≤ 1.0 dB or 1.0 dB < ΔSNR ≤ 1.2 dB are respectively marked in green or orange, and others are marked in red.

Table 2 Testability of ACLR test points for FR2c

| Test ID | Adjacent channel level [dBm] |  | SNR [dB] |  |  |  | Influence of noise [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz – 200 MHz | 400 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 1 | -2.3 | -2.8 | 11.2 | 8.2 | 5.2 | 1.7 | 0.32 | 0.61 | 1.14 | 2.24 |
| 2 | -2.3 | -2.8 | 11.2 | 8.2 | 5.2 | 1.7 | 0.32 | 0.61 | 1.14 | 2.24 |
| 3 | -1.3 | -2.8 | 12.2 | 9.2 | 6.2 | 1.7 | 0.25 | 0.49 | 0.93 | 2.24 |
| 4 | -2.3 | -2.8 | 11.2 | 8.2 | 5.2 | 1.7 | 0.32 | 0.61 | 1.14 | 2.24 |
| 5 | -2.3 | -2.8 | 11.2 | 8.2 | 5.2 | 1.7 | 0.32 | 0.61 | 1.14 | 2.24 |
| 6 | -1.3 | -2.8 | 12.2 | 9.2 | 6.2 | 1.7 | 0.25 | 0.49 | 0.93 | 2.24 |
| 7 | -4.3 | -6.3 | 9.2 | 6.2 | 3.2 | -1.8 | 0.49 | 0.93 | 1.70 | 4.00 |
| 8 | -4.3 | -6.3 | 9.2 | 6.2 | 3.2 | -1.8 | 0.49 | 0.93 | 1.70 | 4.00 |
| 9 | -4.3 | -6.3 | 9.2 | 6.2 | 3.2 | -1.8 | 0.49 | 0.93 | 1.70 | 4.00 |
| 10 | -8.3 | -9.3 | 5.2 | 2.2 | -0.8 | -4.8 | 1.14 | 2.04 | 3.42 | 6.04 |
| 11 | -8.3 | -9.3 | 5.2 | 2.2 | -0.8 | -4.8 | 1.14 | 2.04 | 3.42 | 6.04 |
| 12 | -8.3 | -9.3 | 5.2 | 2.2 | -0.8 | -4.8 | 1.14 | 2.04 | 3.42 | 6.04 |
| 13 | -4.8 | -6.8 | 8.7 | 5.7 | 2.7 | -2.3 | 0.55 | 1.03 | 1.86 | 4.31 |
| 14 | -4.8 | -6.8 | 8.7 | 5.7 | 2.7 | -2.3 | 0.55 | 1.03 | 1.86 | 4.31 |
| 15 | -4.8 | -6.8 | 8.7 | 5.7 | 2.7 | -2.3 | 0.55 | 1.03 | 1.86 | 4.31 |
| Note: SNR is calculated from the minimum peak EIRP = 18.7 dBm, MPR + T(MPR) + MBR at each test point, ACLR requirement = 16 dB, and TE noise floor = -4.5 dBm/400 MHz. |  |  |  |  |  |  |  |  |  |  |

If relaxation is not allowed, test points with low SNR are not testable. There are two options depending on what dB is considered as low SNR. Note that Option 1 is the same approach with FR2b.

Option 1: Test the green marked test points in Table 2 without relaxation, and do not test others. Influence of noise = 1.0 dB is used in MTSU calculation.

Option 2: Test the green or orange marked test points in Table 2 without relaxation, and do not test others. Influence of noise = 1.14 dB is used in MTSU calculation.

On the other hand, all test points can be tested if relaxation is allowed. Following shows two options depending on what dB of influence of noise is achieved by relaxation. Note that Option 3 is the same approach with FR2a.

Option 3: Test all test points with the relaxation values on the left side in Table 3. Influence of noise = 1.0 dB is used in MTSU calculation.

Option 4: Test all test points with the relaxation values on the right side in Table 3. Influence of noise = 1.2 dB is used in MTSU calculation.

Table 3 Relaxation of ACLR for FR2c

| Test ID | Relaxation [dB] for Option 3 (Note 1) |  |  |  | Relaxation [dB] for Option 4 (Note 2) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 1 | 0 | 0 | 0.8 | 4.3 | 0 | 0 | 0 | 3.3 |
| 2 | 0 | 0 | 0.8 | 4.3 | 0 | 0 | 0 | 3.3 |
| 3 | 0 | 0 | 0 | 4.3 | 0 | 0 | 0 | 3.3 |
| 4 | 0 | 0 | 0.8 | 4.3 | 0 | 0 | 0 | 3.3 |
| 5 | 0 | 0 | 0.8 | 4.3 | 0 | 0 | 0 | 3.3 |
| 6 | 0 | 0 | 0 | 4.3 | 0 | 0 | 0 | 3.3 |
| 7 | 0 | 0 | 2.8 | 7.8 | 0 | 0 | 1.8 | 6.8 |
| 8 | 0 | 0 | 2.8 | 7.8 | 0 | 0 | 1.8 | 6.8 |
| 9 | 0 | 0 | 2.8 | 7.8 | 0 | 0 | 1.8 | 6.8 |
| 10 | 0.8 | 3.8 | 6.8 | 10.8 | 0 | 2.8 | 5.8 | 9.8 |
| 11 | 0.8 | 3.8 | 6.8 | 10.8 | 0 | 2.8 | 5.8 | 9.8 |
| 12 | 0.8 | 3.8 | 6.8 | 10.8 | 0 | 2.8 | 5.8 | 9.8 |
| 13 | 0 | 0.3 | 3.3 | 8.3 | 0 | 0 | 2.3 | 7.3 |
| 14 | 0 | 0.3 | 3.3 | 8.3 | 0 | 0 | 2.3 | 7.3 |
| 15 | 0 | 0.3 | 3.3 | 8.3 | 0 | 0 | 2.3 | 7.3 |
| Note 1: Relaxation value is calculated to achieve SNR = 6.0 dB (influence of noise = 1.0 dB). Note 2: Relaxation value is calculated to achieve SNR = 5.0 dB (influence of noise = 1.2 dB). |  |  |  |  |  |  |  |  |

In TR 38.903 [8], expanded uncertainty of ACLR for FR2c (40.8 GHz to 44.3 GHz) has been already defined as 6.61 dB. For each option, MTSU can be obtained by adding influence of noise to the expanded uncertainty, and TT is calculated by the same approach in FR2a and FR2b. MTSU/TT and characteristics of each option are summarized below.

Table 4 Summary of ACLR for FR2c

|  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- |
| Number of testable test points | 23 | 33 | 60 (all) | 60 (all) |
| Relaxation | - | - | ≤ 10.8 dB | ≤ 9.8 dB |
| Influence of noise | 1.0 dB | 1.14 dB | 1.0 dB | 1.2 dB |
| MTSU (Note 1) | 7.61 dB | 7.75 dB | 7.61 dB | 7.81 dB |
| TT (Note 2) |  |  |  |  |
| CBW = 50 MHz, Test ID 1-2, 4-5 | 5.61 dB |  |  |  |
| CBW = 50 MHz, Test ID 3, 6 | 5.55 dB |  |  |  |
| CBW = 50 MHz, Test ID 7-9 | 5.79 dB |  |  |  |
| CBW = 50 MHz, Test ID 10-12 | - | 6.44 dB | 5.64 dB | 6.44 dB |
| CBW = 50 MHz, Test ID 13-15 | 5.84 dB |  |  |  |
| CBW = 100 MHz, Test ID 1-2, 4-5 | 5.91 dB |  |  |  |
| CBW = 100 MHz, Test ID 3, 6 | 5.79 dB |  |  |  |
| CBW = 100 MHz, Test ID 7-9 | 6.23 dB |  |  |  |
| CBW = 100 MHz, Test ID 10-12 | - | - | 3.54 dB | 4.54 dB |
| CBW = 100 MHz, Test ID 13-15 | - | 6.33 dB | 6.03 dB | 6.33 dB |
| CBW = 200 MHz, Test ID 1-2, 4-5 | - | 6.44 dB | 5.64 dB | 6.44 dB |
| CBW = 200 MHz, Test ID 3, 6 | 6.23 dB |  |  |  |
| CBW = 200 MHz, Test ID 7-9 | - | - | 4.19 dB | 5.19 dB |
| CBW = 200 MHz, Test ID 10-12 | - | - | 1.92 dB | 2.92 dB |
| CBW = 200 MHz, Test ID 13-15 | - | - | 3.86 dB | 4.86 dB |
| CBW = 400 MHz, Test ID 1-6 | - | - | 3.24 dB | 4.24 dB |
| CBW = 400 MHz, Test ID 7-9 | - | - | 1.50 dB | 2.50 dB |
| CBW = 400 MHz, Test ID 10-12 | - | - | 1.00 dB | 1.54 dB |
| CBW = 400 MHz, Test ID 13-15 | - | - | 1.31 dB | 2.31 dB |
| Note 1: MTSU = Expanded uncertainty 6.61 dB + Influence of noise Note 2: TT = max(R, ΔSNRmr+0.65 x (MTSUIFF - Influence of noise)) -R + TT due to metric change - R: Relaxation values defined in Table 3 - ΔSNRmr: Systematic offset due to noise when measuring ACP at minimum requirement level. Concrete values are shown on the right side in Table 2. - MTSUIFF - Influence of noise: Expanded uncertainty 6.61 dB - TT due to metric change: 1.0 dB |  |  |  |  |

Considering the consistency with FR2b, we recommend Option 1 for the approach of FR2c.

Proposal 5: For FR2c ACLR, adopt Option 1 with MTSU/TT values in Table 4.

# 3.	Conclusion

RAN5 is asked to endorse the following proposals for FR2c, PC3, CBW  400 MHz, IFF, max device size  40 cm.

Proposal 1: Remove [ ] from MTSU/TT value of MPR for FR2c.

Proposal 2: Remove [ ] from MTSU/TT/relaxation value of Minimum output power for FR2c.

Proposal 3: Remove [ ] from MTSU/relaxation value of ACS/In-band blocking for FR2c.

Proposal 4: Do not test Transmit OFF power for FR2b and FR2c.

Proposal 5: For FR2c ACLR, adopt Option 1 with MTSU/TT values in Table 4.

Options for FR2c ACLR are shown below.

Option 1: Test the green marked test points in Table 2 without relaxation, and do not test others. Influence of noise = 1.0 dB is used in MTSU calculation.

Option 2: Test the green or orange marked test points in Table 2 without relaxation, and do not test others. Influence of noise = 1.14 dB is used in MTSU calculation.

Option 3: Test all test points with the relaxation values on the left side in Table 3. Influence of noise = 1.0 dB is used in MTSU calculation.

Option 4: Test all test points with the relaxation values on the right side in Table 3. Influence of noise = 1.2 dB is used in MTSU calculation.

# 4.	References

[1] R5-233971, “Testability and MU analysis for FR2c”, Keysight Technologies UK Ltd, RAN5#100

[2] R5-235753, “MU discussion on FR2c”, Anritsu, RAN5#100

[3] R5-237724, “MU discussion on FR2c”, Anritsu, RAN5#101

[4] R5-237723, “Discussion on MU and TT analysis for FR2c”, Keysight Technologies UK Ltd, RAN5#101

[5] R5-240401, “Discussion on MU and TT analysis for FR2c”, Keysight Technologies UK Ltd, RAN5#102

[6] R5-241344, “On the MU for n259”, Rohde & Schwarz, RAN5#102

[7] R5-241858, “MU discussion on FR2c”, Anritsu, RAN5#102

[8] TR 38.903, “Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”, V.18.2.0 (2024-03)