3GPP TSG-RAN5 Meeting #103	R5-243002

# Fukuoka, Japan, 20th May – 24th May 2024

Title:							QoQZ for QZ size 40 cm

Source:			Anritsu

Agenda Item:			5.4.20

Document for:			Endorsement

# 1.	Introduction

In the last meeting [1][2][3], QoQZ value up to FR2d was defined for QZ size 30 cm (NTC, ETC) and QZ size 40 cm (NTC). This paper provides our proposals for the remaining work below.

- The applicability to FR2d of the note which specifies that QoQZ value for QZ size 30 cm is used for MTSU calculation for QZ size 40 cm

- QoQZ value up to FR2d for QZ size 40 cm (ETC)

- QoQZ value for Spurious emissions for QZ size 40 cm

# 2.	Discussion

The tables in annex show the currently agreed QoQZ values in [4]. The applicability of Note 2 to FR2d, and QoQZ values for ETC or Spurious emissions for QZ size 40 cm have not yet been defiend. To define the remaining QoQZ values, we provide the following measurement results of QoQZ. Note that the values for in-band measurement are the same as [3].

Table 1: Standard Deviations for EIRP/EIS metric (QZ size = 40 cm, ETC)

| Positions | QoQZ MU [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 23.450 GHz | 32.125 GHz | 40.800 GHz | 44.300 GHz | 49.000 GHz |
| P1-P7 (Stage 2) | 0.87 | 0.85 | 0.53 | 0.73 | 0.68 |
| P1 only (Stage 1) | 0.58 | 0.38 | 0.15 | 0.16 | 0.41 |

Table 2: Standard Deviations for TRP metric (QZ size = 40 cm, ETC)

| Positions | QoQZ MU [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 23.450 GHz | 32.125 GHz | 40.800 GHz | 44.300 GHz | 49.000 GHz |
| P1-P7 (Stage 2) | 0.33 | 0.31 | 0.35 | 0.50 | 0.56 |
| P1 only (Stage 1) | 0.25 | 0.13 | 0.20 | 0.49 | 0.22 |

Table 3: Standard Deviations for Spurious measurement (QZ size = 40 cm)

| Positions | QoQZ MU [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 6.000 GHz | 12.750 GHz | 23.450 GHz | 40.800 GHz | 66.000 GHz | 87.000 GHz |
| P1-P7 (Stage 2) | 0.99 | 0.58 | 0.61 | 0.38 | 0.50 | 0.48 |

Considering the above measurement results and commonality with the already endorsed QoQZ values, we propose the follwing QoQZ values for QZ size = 40 cm.

Table 4: Endorsed/Proposed QoQZ MU for EIRP/EIS and TRP metric for P1-P7 (Stage 2)

| Condition | QoQZ MU [dB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | FR2a | FR2b | FR2c | FR2d |
| QZ size ≤ 30 cm and NTC | 0.6 | 0.6 | 0.7 | 0.7 |
| QZ size ≤ 30 cm and ETC | 0.9 | 0.9 | 0.9 | 0.9 |
| QZ size = 40 cm and NTC | 0.7 (Note 2) | 0.7 (Note 2) | 0.8 (Note 2) | 0.8 (Note 2) |
| QZ size = 40 cm and ETC | 0.9 | 0.9 | 0.9 | 0.9 |
| Note 1: Black text indicates endorsed values, and green text indicates proposed values in this paper. Note 2: QoQZ value for QZ size ≤ 30 cm is used for MTSU calculation. |  |  |  |  |

Table 5: Endorsed/Proposed QoQZ MU for EIRP/EIS and TRP metric for P1 only (Stage 1)

| Condition | QoQZ MU [dB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | FR2a | FR2b | FR2c | FR2d |
| QZ size ≤ 40 cm and NTC | 0.4 | 0.4 | 0.5 | 0.5 |
| QZ size ≤ 30 cm and ETC | 0.6 | 0.6 | 0.6 | 0.6 |
| QZ size = 40 cm and ETC | 0.6 | 0.6 | 0.6 | 0.6 |
| Note 1: Black text indicates endorsed values, and green text indicates proposed values in this paper. |  |  |  |  |

Table 6: Endorsed/Proposed QoQZ MU for Spurious measurement for Stage 1 and Stage 2

| Condition | QoQZ MU [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 6 GHz to 12.75 GHz | 12.75 GHz to 23.45 GHz | 23.45 GHz to 40.8 GHz | 40.8 GHz to 66 GHz | 66 GHz to 87 GHz |
| QZ size ≤ 30 cm | 0.7 | 0.6 | 0.6 | 0.6 | 0.6 |
| QZ size = 40 cm | 1.0 | 0.6 | 0.6 | 0.6 | 0.6 |
| Note 1: Black text indicates endorsed values, and green text indicates proposed values in this paper. |  |  |  |  |  |

Proposal 1: Use the QoQZ MU values in Table 4, Table 5, and Table 6 for MTSU calculation.

# 3.	Conclusion

RAN5 is asked to endorse following proposals.

Proposal 1: Use the QoQZ MU values in Table 4, Table 5, and Table 6 for MTSU calculation.

# 4.	References

[1] R5-240605, “On 30cm QoQZ MU Topics”, Keysight Technologies UK Ltd, RAN5#102

[2] R5-241078, “On the QoQZ MU for n262 and TRP”, ROHDE & SCHWARZ, RAN5#102

[3] R5-241087, “QoQZ up to FR2d”, Anritsu, RAN5#102

[4] TR 38.903, “Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”, V18.2.0 (2024-03)

# Annex Agreed QoQZ in TR 38.903

The following tables show the currently agreed QoQZ values in [4].

Table B.2.2.3-1: Uncertainty value for quality of quiet zone for IFF

| QZ size | Power class | Condition | Test case | Frequency range | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| All | All | All | ACLR (relative measurement) | FR2a, FR2b, FR2c | 0.52 | Actual | 1.00 | 0.52 |
| ≤30cm | PC1, PC3, PC5, PC6 | NC | NOTE1 | FR2a, FR2b | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | FR2c, FR2d | 0.7 | Actual | 1.00 | 0.7 |
|  |  |  | SE | 6GHz to 12.75GHz | 0.7 | Actual | 1.00 | 0.7 |
|  |  |  |  | 12.75GHz to 23.45GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 23.45GHz to 40.8GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 40.8GHz to 66GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 66GHz to 87GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  | ETC | NOTE1 | FR2a, FR2b, FR2c, FR2d | 0.9 | Actual | 1.00 | 0.9 |
| 40cm | PC1, PC3, PC5, PC6 | NC | NOTE1 | FR2a, FR2b | 0.7 (NOTE 2) | Actual | 1.00 | 0.7 |
|  |  |  |  | FR2c, FR2d | 0.8 (NOTE 2) | Actual | 1.00 | 0.8 |
|  |  |  | SE | 6GHz to 12.75GHz | TBD | Actual | 1.00 | TBD |
|  |  |  |  | 12.75GHz to 23.45GHz | TBD | Actual | 1.00 | TBD |
|  |  |  |  | 23.45GHz to 40.8GHz | TBD | Actual | 1.00 | TBD |
|  |  |  |  | 40.8GHz to 66GHz | TBD | Actual | 1.00 | TBD |
|  |  |  |  | 66GHz to 87GHz | TBD | Actual | 1.00 | TBD |
|  |  | ETC | NOTE1 | FR2a, FR2b | TBD | Actual | 1.00 | TBD |
| NOTE 1: The uncertainty in current row applies to maximum output power with EIRP and TRP, EIRP spherical coverage, MPR, configured output power with power boost, minimum output power, transmit OFF power, spectrum emission mask, reference sensitivity, adjacent selectivity, in-band blocking. NOTE 2: For Uncertainty assessment in tables across clause B and MTSU calculation, QoQZ uncertainty value for QZ ≤ 30cm is used also for 40cm for FR2a through FR2c. The applicability to FR2d is FFS. NOTE 3: Void |  |  |  |  |  |  |  |  |

Table B.2.2.19-1: Uncertainty value for quiet zone for calibration process for IFF

| QZ size | Power class | Condition | Test case | Frequency range | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| All | All | All | ACLR (relative measurement) | FR2a, FR2b, FR2c | 0.32 | Actual | 1.00 | 0.32 |
| ≤ 30cm | PC1, PC3, PC5, PC6 | NC | NOTE1 | FR2a, FR2b | 0.4 | Actual | 1.00 | 0.4 |
|  |  |  |  | FR2c, FR2d | 0.5 | Actual | 1.00 | 0.5 |
|  |  |  | SE | 6GHz to 12.75GHz | 0.7 | Actual | 1.00 | 0.7 |
|  |  |  |  | 12.75GHz to 23.45GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 23.45GHz to 40.8GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 40.8GHz to 66GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 66GHz to 87GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  | ETC | NOTE1 | FR2a, FR2b, FR2c, FR2d | 0.6 | Actual | 1.00 | 0.6 |
| 40cm | PC1, PC3, PC5, PC6 | NC | NOTE1 | FR2a, FR2b | 0.4 | Actual | 1.00 | 0.4 |
|  |  |  |  | FR2c, FR2d | 0.5 | Actual | 1.00 | 0.5 |
|  |  |  | SE | 6GHz to 12.75GHz | TBD | Actual | 1.00 | TBD |
|  |  |  |  | 12.75GHz to 23.45GHz | TBD | Actual | 1.00 | TBD |
|  |  |  |  | 23.45GHz to 40.8GHz | TBD | Actual | 1.00 | TBD |
|  |  |  |  | 40.8GHz to 66GHz | TBD | Actual | 1.00 | TBD |
|  |  |  |  | 66GHz to 80GHz | TBD | Actual | 1.00 | TBD |
|  |  | ETC | NOTE 1 | FR2a, FR2b, FR2c, FR2d | TBD | Actual | 1.00 | TBD |
| NOTE 1: The uncertainty in current row applies to maximum output power with EIRP and TRP, EIRP spherical coverage, MPR, configured output power with power boost, minimum output power, transmit OFF power, spectrum emission mask, reference sensitivity, adjacent selectivity, in-band blocking. NOTE 2: Void |  |  |  |  |  |  |  |  |