3GPP TSG-RAN5 Meeting #103	R5-243075

# Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3197 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of a new section for IDC Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, CATT, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_ENDC_SON_MDT_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add of a new section for IDC Configuration |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a new section for IDC Configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No IDC test configuration |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.508-2 CR 0640, 38.523-1 CR 4432 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The section number “4.8.X” highlighted in yellow need to be assigned by MCC, and the “4.8.X” used in 38.508-2 CR 0640, 38.523-1 CR 4432 need to be replaced by the assigned number. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

4.8.5	URSP configurations

4.8.5.1	General

FFS

4.8.5.2	UE Route Selection Policy Rules

Table 4.8.5.2-1: UE Route Selection Policy Rule #1

| Derivation Path: TS 24.526, Figure 5.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| URSP rules | 1 entry |  |  |
| URSP rule |  |  |  |
| Rule Precedence | 1 |  |  |
| Traffic descriptor |  |  |  |
| Traffic descriptor component type identifier | ‘1000 1000’B | DNN type |  |
| Traffic descriptor component |  |  |  |
| Application descriptors | Not present |  |  |
| IP descriptors | Not present |  |  |
| Domain descriptors | Not present |  |  |
| Non-IP descriptors | Not present |  |  |
| DNN | FFS |  |  |
| Connection Capabilities | Not present |  |  |
| List of Route Selection Descriptors | Route Selection Descriptor #1 | Table 4.8.5.3-1 |  |

4.8.5.3	Route Selection Descriptors

Table 4.8.5.3-1: Route Selection Descriptor #1

| Derivation Path: TS 24.526, Figure 5.2.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Route Selection Descriptor Precedence | 1 |  |  |
| Route selection descriptor contents |  |  |  |
| Route selection descriptor component type identifier | ‘0000 0010’B |  | S-NSSAI type |
|  | ‘0000 0100’B |  | DNN type |
| Route selection descriptor component |  |  |  |
| SSC Mode Selection | Not present |  |  |
| Network Slice Selection | FFS |  | S-NSSAI type |
| DNN Selection | FFS |  | DNN type |
| PDU Session Type Selection | Not present |  |  |
| Non-Seamless Offload indication | Not present |  |  |
| Access Type preference | Not present |  |  |

| Condition | Explanation |
| --- | --- |
| S-NSSAI type | The Route selection descriptor component type is S-NSSAI |
| DNN type | The Route selection descriptor component type is DNN |

4.8.X	IDC Configuration

The present subclause provides configurations required for IDC mehchanism testing. Table 4.8.X-1 provides time instances configurations for testing IDC mechanism. When “T0” applies, UE does not transmit UEAssistanceInformation message including IDC-Assistance-r16 as UE cannot detect IDC problem with “T0” configuration. When “T1” applies, UE transmits UEAssistanceInformation with IDC-Assistance-r16 as UE can detect IDC problem with “T1” configuration if it was configured with idc-AssistanceConfig-r16.

Table 4.8.X-1: IDC Configuration, time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 14 | WLAN AP 1 (Cell 27) | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  |  |
|  | BeaconRSSI | dBm |  | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  |  |
|  | BeaconRSSI | dBm |  | -80 |  |

<End of modified section>