3GPP TSG RAN5 Meeting #103	 	R5-243100

Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

Source:		Rohde & Schwarz

Title:	On the MU for n259

Agenda Item:		5.4.20

Document for:	Discussion and Endorsement

# Introduction

In previous meetings, the MU for FR2c has been discussed [1 to 10]. However, some MU values have not been finalized. We continue the discussion with this contribution.

# Discussion

In this paper, we focus our analysis on PC3, CHBW ≤ 400 MHz, IFF with a maximum device size of 40cm.

## Occupied Bandwidth

We have assessed the MU for OBW of our test system following the same analysis as outlined in [11].

For a CHBW of 50 MHz and 100 MHz, we propose a value of 0.4 [%CBW] for the MTSU and to not relax the requirement. The analysis for CHBWs of 200 MHz and 400 MHz is work in progress.

Proposal 1: For OBW, apply no relaxation for CHBWs of 50 MHz and 100 MHz and use a MTSU of 0.4 [%CBW] for FR2c, IFF and PC3. Keep the MU/testability of 200 MHz and 400 MHz CHBW open.

## ACS and In-band Blocking

In the last meeting a value of [9.46 dB] has been agreed for the total MU of the in-band blocking and ACS TCs for CHBW ≤ 100 MHz, IFF and PC3. In addition relaxations for the interferer power levels have been agreed within square brackets. In order to progress, we propose to remove these square brackets.

Proposal 2: Remove the square brackets around the MU value and relaxation of the interferer power level for in-band blocking and ACS for FR2c, CHBW ≤ 100 MHz, IFF and PC3.

## Minimum Output power

In the last meeting relaxation values within square brackets for minimum output power have been agreed. We propose to remove the square brackets in order to progress.

Proposal 3: Remove the square brackets around the relaxation values for minimum output power for FR2c and PC3.

## MPR

In the last meeting a value of [0.55 dB] has been agreed for the influence of noise. In order to progress, we propose to remove these square brackets.

Proposal 4: Remove the square brackets around the influence of noise and MU value for MPR for FR2c, IFF and PC3.

## ACLR

Based on the noise level of our test system, we consider an MPR of 2.5 dB as testable with a SNRtotal of 6 dB for a CHBW of 200 MHz and propose to restrict testing to CHBWs ≤ 200 MHz.

Table 3 provides a summary of the Test IDs, power levels, SNR and influence of noise. ACP denotes the adjacent channel power. Red and green shading represent testable and not testable ID & CHBW combinations, respectively.

Table 3: Testability, SNR and influence of noise for ACLR

|  | ACP | ACP | SNR (dB) |  |  |  | Influence of noise (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | 50 MHz – 200 MHz | 400 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 1 | -2.3 | -2.8 | 12 | 9 | 6 | 2.5 | 0.27 | 0.51 | 0.97 | 1.94 |
| 2 | -2.3 | -2.8 | 12 | 9 | 6 | 2.5 | 0.27 | 0.51 | 0.97 | 1.94 |
| 3 | -1.3 | -2.8 | 13 | 10 | 7 | 2.5 | 0.21 | 0.41 | 0.79 | 1.94 |
| 4 | -2.3 | -2.8 | 12 | 9 | 6 | 2.5 | 0.27 | 0.51 | 0.97 | 1.94 |
| 5 | -2.3 | -2.8 | 12 | 9 | 6 | 2.5 | 0.27 | 0.51 | 0.97 | 1.94 |
| 6 | -1.3 | -2.8 | 13 | 10 | 7 | 2.5 | 0.21 | 0.41 | 0.79 | 1.94 |
| 7 | -4.3 | -6.3 | 10 | 7 | 4 | -1 | 0.41 | 0.79 | 1.46 | 3.54 |
| 8 | -4.3 | -6.3 | 10 | 7 | 4 | -1 | 0.41 | 0.79 | 1.46 | 3.54 |
| 9 | -4.3 | -6.3 | 10 | 7 | 4 | -1 | 0.41 | 0.79 | 1.46 | 3.54 |
| 10 | -8.3 | -9.3 | 6 | 3 | 0 | -4 | 0.97 | 1.76 | 3.01 | 5.46 |
| 11 | -8.3 | -9.3 | 6 | 3 | 0 | -4 | 0.97 | 1.76 | 3.01 | 5.46 |
| 12 | -8.3 | -9.3 | 6 | 3 | 0 | -4 | 0.97 | 1.76 | 3.01 | 5.46 |
| 13 | -4.8 | -6.8 | 9.5 | 6.5 | 3.5 | -1.5 | 0.46 | 0.88 | 1.60 | 3.82 |
| 14 | -4.8 | -6.8 | 9.5 | 6.5 | 3.5 | -1.5 | 0.46 | 0.88 | 1.60 | 3.82 |
| 15 | -4.8 | -6.8 | 9.5 | 6.5 | 3.5 | -1.5 | 0.46 | 0.88 | 1.60 | 3.82 |

Proposal 5: Select the Test IDs for FR2c per CHBW based on the green shaded cells in Table 2. For FR2c, apply an influence of noise of 1 dB for ACLR, in the frequency range from 40.8 GHz to 44.3 GHz, CHBW ≤ 200 MHz, IFF and PC3.

Accompanying CRs in Refs. [12] and [13] document the MU assessment in TR 38.903 [14] and TS 38.521-2 [15], respectively.

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | FR2c |
| #2 | Size of QZ for IFF | 40 cm |
| #3 | UE power class | PC3 |
| #4 | Max EIRP | EIRP MAX= +43 dBm (rms) |
| #5 | Wanted signal PAPR | 13 dB |
| #6 | Temperature range of the test equipment | 20°C – 35°C |
| #7 | Channel bandwidth | Default: 400 MHz ACS & IBB: 100 MHz ACLR: 200 MHz |

# Conclusion

In summary, we have assessed the uncertainty in the frequency range of FR2c and make the following proposals:

Proposal 1: For OBW, apply no relaxation for CHBWs of 50 MHz and 100 MHz and use a MTSU of 0.4 [%CBW] for FR2c, IFF and PC3. Keep the MU/testability of 200 MHz and 400 MHz CHBW open.

Proposal 2: Remove the square brackets around the MU value and relaxation of the interferer power level for in-band blocking and ACS for FR2c, CHBW ≤ 100 MHz, IFF and PC3.

Proposal 3: Remove the square brackets around the relaxation values for minimum output power for FR2c and PC3.

Proposal 4: Remove the square brackets around the influence of noise and MU value for MPR for FR2c, IFF and PC3.

Proposal 5: Select the Test IDs for FR2c per CHBW based on the green shaded cells in Table 2. For FR2c, apply an influence of noise of 1 dB for ACLR, in the frequency range from 40.8 GHz to 44.3 GHz, CHBW ≤ 200 MHz, IFF and PC3.

# References

- R5-233174, On the MU for n259, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #99, May 2023

- R5-232983, MU discussion on FR2c, Anritsu, 3GPP TSG RAN WG 5 Meeting #99, May 2023

- R5-235140, On the MU for n259, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #100, August 2023

- R5-235753, MU discussion on FR2c, Anritsu, 3GPP TSG RAN WG 5 Meeting #100, August 2023

- R5-237725, On the MU for n259, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #101, November 2023

- R5-237723, Discussion on MU and TT analysis for FR2c, Keysight Technologies UK Ltd, 3GPP TSG RAN WG 5 Meeting #101, November 2023

- R5-237724, MU discussion on FR2c, Anritsu, 3GPP TSG RAN WG 5 Meeting #101, November 2023

- R5-241344, On the MU for n259, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #102, February 2024

- R5-241858, MU discussion on FR2c, Anritsu, 3GPP TSG RAN WG 5 Meeting #102, February 2024

- R5-240401, Discussion on MU and TT analysis for FR2c, Keysight Technologies UK Ltd, 3GPP TSG RAN WG 5 Meeting #102, February 2024

- R5-213311, On the MU of FR2 OBW, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #91-e, May 2021

- R5-243101, Documentation of MU for n259, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #103, May 2024

- R5-243102, Update of MU for n259, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #103, May 2024

- TR 38.903, V18.2.0, 3GPP TSG RAN WG5, March 2024

- TS 38.521-2, V18.2.0, 3GPP TSG RAN WG5, March 2024