3GPP TSG RAN5 Meeting #103	 	R5-243103

Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

Source:		Rohde & Schwarz

Title:	On the MU and TT for CA and MIMO test cases

Agenda Item:		5.4.20

Document for:	Discussion and Endorsement

# Introduction

For several Rel-15 CA or UL MIMO test cases the MU and TT has not been specified which blocks the progress of these test cases. In this contribution, we propose MU and TT for some of these test cases.

# Discussion

In this paper, we focus our analysis on PC3, CHBW ≤ 400 MHz, IFF with a maximum device size of 40cm.

## CA test cases

For an aggregated CHBW ≤ 400 MHz, the MU is the same for a CA test case as for the associated single carrier test case as long as the power levels are the same. Based on our analysis this is the case for the test cases comprised in Table 1.

Table 1: CA test cases

| Clause | Test case |
| --- | --- |
| 6.3A.4.2.1 | Absolute power tolerance for CA (2UL CA) |
| 6.3A.4.2.2 | Absolute power tolerance for CA (3UL CA) |
| 6.3A.4.2.3 | Absolute power tolerance for CA (4UL CA) |
| 6.3A.4.4.1 | Aggregate power tolerance for CA (2UL CA) |
| 6.3A.4.4.2 | Aggregate power tolerance for CA (3UL CA) |
| 6.3A.4.4.3 | Aggregate power tolerance for CA (4UL CA) |

Proposal 1: We propose to reuse the MU and TT of the single carrier test cases for the CA test cases in Table 1.

## UL MIMO test cases

When the power level of the minimum conformance requirement for an UL MIMO TC is the same as the one of the associated SISO TC, the MU and TT can directly be leveraged. Based on our review this is the case for the test cases comprised in Table 2.

Table 2: UL MIMO test cases

| Clause | Test case |
| --- | --- |
| 6.5D.3.1 | Transmitter Spurious emissions for UL MIMO |
| 6.5D.3.2 | Spurious emission band UE co-existence for UL MIMO |
| 6.5D.3.3 | Additional spurious emissions for UL MIMO |

Proposal 2: We propose to reuse the MU and TT of the SISO test cases for the UL MIMO test cases in Table 2.

Accompanying CRs in Refs. [1] and [2] document the MU assessment in TR 38.903 [3] and TS 38.521-2 [4], respectively. CR in [5] updates the status of the associated EN-DC TCs in TS 38.521-3 [6]. The applicability in TS 38.522 [7] is updated with CR in Ref. [8].

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | FR2c |
| #2 | Size of QZ for IFF | 40 cm |
| #3 | UE power class | PC3 |
| #4 | Max EIRP | EIRP MAX= +43 dBm (rms) |
| #5 | Wanted signal PAPR | 13 dB |
| #6 | Temperature range of the test equipment | 20°C – 35°C |
| #7 | Channel bandwidth | 400 MHz |

# Conclusion

In summary, we have assessed the uncertainty and make the following proposals:

Proposal 1: We propose to reuse the MU and TT of the single carrier test cases for the CA test cases in Table 1.

Proposal 2: We propose to reuse the MU and TT of the SISO test cases for the UL MIMO test cases in Table 2.

# References

- R5-243104, Documentation of MU for UL MIMO, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #103, May 2024

- R5-243105, Update of MU and TT for CA and MIMO, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #103, May 2024

- TR 38.903, V18.2.0, 3GPP TSG RAN WG5, March 2024

- TS 38.521-2, V18.2.0, 3GPP TSG RAN WG5, March 2024

- R5-243106, Update of status of FR2 CA and UL MIMO test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #103, May 2024

- TS 38.521-3, V18.2.0, 3GPP TSG RAN WG5, March 2024

- TS 38.522, V18.2.0, 3GPP TSG RAN WG5, March 2024

- R5-243107, Update of applicability of FR2 CA and MIMO test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #103, May 2024