3GPP TSG-RAN5 Meeting #103	R5-243115

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0025 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of frequency error test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The applicability of the frequency error test cases is not clear whether the frequency error test cases apply to UEs that support both GSO and NGSO. The maximum positive Doppler/2 has a negative value in Table 6.4B.1_1.4.3-3a. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The applicability of the frequency error test cases has been aligned to the definition in TS 36.521-2. The sign of maximum positive Doppler/2 has been corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will remain inconsistent and ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4A.1_1.2, 6.4A.1_1.4.3, 6.4A.1_2.2, 6.4B.1_1.2, 6.4B.1_1.4.3, 6.4B.1_2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 6.4A.1_1	Frequency error with GSO ephemeris for UE category M1

6.4A.1_1.1	Test purpose

This test verifies the ability of both, the receiver and the transmitter, to process frequency correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

6.4A.1_1.2	Test applicability

This test case applies to all types of E-UTRA  FDD UE release 17 and forward of UE category M1 that support satellite access operation and only GSO or both GSO and NGSO.

< Unchanged sections omitted >

6.4A.1_1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6 with the condition CEModeA and 5.6.2, the exceptions in subclause 7.3A.4.3.

Table 6.4A.1_1.4.3-1a: SystemInformationBlockType31- eMTC NTN Ephemeris Information for GSO satellites (maximum positive Doppler)

| Derivation Path: TS 36.508 [12] Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -17104941 |  |  |
| positionY-r17 | 27550229 |  |  |
| positionZ-r17 | -607219 |  |  |
| velocityVX-r17 | 258 |  |  |
| velocityVY-r17 | 299 |  |  |
| velocityVZ-r17 | 6277 |  |  |
| } |  |  |  |
| } |  |  |  |
| k-Offset-r17 | 264 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 26.15 degrees, one-way delay equal to 129.93 ms and Doppler equal to 0.17 ppm |  |  |  |

Table 6.4A.1_1.4.3-1b: Void

Table 6.4A.1_1.4.3-2a: SystemInformationBlockType31-eMTC NTN Ephemeris Information for GSO satellites (maximum negative Doppler)

| Derivation Path: TS 36.508 [12] Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -17061001 |  |  |
| positionY-r17 | 27582763 |  |  |
| positionZ-r17 | -276165 |  |  |
| velocityVX-r17 | 361 |  |  |
| velocityVY-r17 | 160 |  |  |
| velocityVZ-r17 | -6335 |  |  |
| } |  |  |  |
| } |  |  |  |
| k-Offset-r17 | 264 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 26.78 degrees, one-way delay equal to 129.74 ms and Doppler equal to -0.17 ppm. |  |  |  |

Table 6.4A.1_.4.3-2b: Void

Table 6.4A.1_1.4.3-3a: SystemInformationBlockType31-eMTC NTN Ephemeris Information for GSO  satellites (maximum positive Doppler/2)

| Derivation Path: TS 36.508 [12] Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -17062164 |  |  |
| positionY-r17 | 27354696 |  |  |
| positionZ-r17 | -3544856 |  |  |
| velocityVX-r17 | -360 |  |  |
| velocityVY-r17 | 164 |  |  |
| velocityVZ-r17 | 2993 |  |  |
| } |  |  |  |
| } |  |  |  |
| k-Offset-r17 | 264 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 20.61 degrees, one-way delay equal to 131.70 ms and Doppler equal to 0.085 ppm. |  |  |  |

< Unchanged sections omitted >

### 6.4A.1_2	Frequency error with NGSO ephemeris for UE category M1

6.4A.1_2.1	Test purpose

Same test purpose as in clause 6.4A.1_1.1.

6.4A.1_2.2	Test applicability

This test case applies to all types of E-UTRA FDD UE release 17 and forward of UE category M1 that support satellite access operation and only NGSO or both GSO and NGSO.

< Unchanged sections omitted >

### 6.4B.1_1	Frequency error with GSO ephemeris for UE category NB1 and NB2

6.4B.1_1.1	Test purpose

This test verifies the ability of both, the receiver and the transmitter, to process frequency correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

6.4B.1_1.2	Test applicability

This test case applies to all types of NB-IoT FDD UE release 17 and forward of UE category NB1 and NB2 that support satellite access operation and only GSO or both GSO and NGSO.

< Unchanged sections omitted >

6.4B.1_1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6 and 5.6.2

Table 6.4B.1_1.4.3-1a: SystemInformationBlockType31-NB NB-IoT NTN Ephemeris Information for GSO satellites (maximum positive Doppler) for NB-IoT NTN

| Derivation Path: TS 36.508 [12] Table 8.2.2.1.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -17104941 |  |  |
| positionY-r17 | 27550229 |  |  |
| positionZ-r17 | -607219 |  |  |
| velocityVX-r17 | 258 |  |  |
| velocityVY-r17 | 299 |  |  |
| velocityVZ-r17 | 6277 |  |  |
| } |  |  |  |
| } |  |  |  |
| k-Offset-r17 | 264 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 26.15 degrees, one-way delay equal to 129.93 ms and Doppler equal to 0.17 ppm |  |  |  |

Table 6.4B.1_1.4.3-1b: Void

Table 6.4B.1_1.4.3-2a: SystemInformationBlockType31-NB NB-IoT NTN Ephemeris Information for GSO satellites (maximum negative Doppler)

| Derivation Path: TS 36.508 [12] Table 8.2.2.1.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -17061001 |  |  |
| positionY-r17 | 27582763 |  |  |
| positionZ-r17 | -276165 |  |  |
| velocityVX-r17 | 361 |  |  |
| velocityVY-r17 | 160 |  |  |
| velocityVZ-r17 | -6335 |  |  |
| } |  |  |  |
| } |  |  |  |
| k-Offset-r17 | 264 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 26.78 degrees, one-way delay equal to 129.74 ms and Doppler equal to -0.17 ppm. |  |  |  |

Table 6.4B.1_1.4.3-2b: Void

Table 6.4B.1_1.4.3-3a: SystemInformationBlockType31-NB NB-IoT NTN Ephemeris Information for GSO  satellites (maximum positive Doppler/2)

| Derivation Path: TS 36.508 [12] Table 8.2.2.1.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -17062164 |  |  |
| positionY-r17 | 27354696 |  |  |
| positionZ-r17 | -3544856 |  |  |
| velocityVX-r17 | -360 |  |  |
| velocityVY-r17 | 164 |  |  |
| velocityVZ-r17 | 2993 |  |  |
| } |  |  |  |
| } |  |  |  |
| k-Offset-r17 | 264 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 20.61 degrees, one-way delay equal to 131.70 ms and Doppler equal to 0.085 ppm. |  |  |  |

< Unchanged sections omitted >

### 6.4B.1_2	Frequency error with NGSO ephemeris for UE category NB1 and NB2

6.4B.1_2.1	Test purpose

Same test purpose as in clause 6.4B.1_1.1.

6.4B.1_2.2	Test applicability

This test case applies to all types of NB-IoT FDD UE release 17 and forward of UE category NB1 and NB2 that support satellite access operation and only NGSO or both GSO and NGSO.

< End of changes >