Title:	Discussion on test point selection for NS_43 A-MPR in FR1

Document for:	Discussion and approval

Background

The purpose of this paper is to provide an analysis of the parameters for the A-MPR test configuration table with a selection of test points that minimize testing and still achieve good test coverage for NS_43. The intention with NS_43 is to protect the downlink band of n18/n19 (DL 860-890MHz) as indicated in Table 1 below.

Table 1: Additional requirement for "NS_43" (from [4])

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15 MHz |  |
| 860 ≤ f ≤ 890 | -40 | 1 MHz |
| NOTE 1: Applicable for 5 MHz and 15 MHz channel BW confined between 900 MHz and 915 MHz and for 10 MHz channel BW confined between 905 MHz and 915 MHz |  |  |

Discussion

The general agreement in RAN5 is that FR1 testing should follow what is being tested for LTE. This is also valid for test point selection for A-MPR in FR1.

The selection of test points below is partly based on approved test point analyzes for SEM [2, 3] for FR1. For NS_43 the upper part of operating band n8 and n81 (UL 880-915 MHz) has specific A-MPR requirements to protect the downlink band of n18/n19 (DL 860-890 MHz), see Table 2 ,Table 3 and Table 4 for PC3 and PC2.

Table 2: A-MPR regions for NS_43 (Power class 3 and 2)(from [4])

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 MHz | 902.5 ≤ FC < 912.5 |  | > 2.7 MHz/12/SCS | A1 |  |  |  |
| 10 MHz | FC = 910 |  | > 7.2 MHz/12/SCS | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  | > 8.1 MHz/12/SCS | A3 |  | > 7.2 MHz/12/SCS | A5 |
| 15 MHz | FC = 907.5 | ≤ 1.8 MHz /12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | ≥ 7.2 MHz/12/SCS | A6 |
|  |  | ≥ 12.24 MHz/12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | < 7.2 MHz/12/SCS | A7 |
|  |  |  |  |  | ≥ 6.12 MHz/12/SCS ≤ 7.2 MHz/12/SCS | > 5.4 MHz/12/SCS | A7 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.6-2 and Table 6.2.3.6-2a. NOTE 2: Void NOTE 3: Void |  |  |  |  |  |  |  |

The A-MPR requirements in Table 2 are split into 2 A-MPR regions for 5, 10 and 15 MHz channel bandwidths. The UL RB allocation for testing of these A-MPR regions are allocated to the upper part of operating band n8 and n81. The start of the UL RB allocation and number of RB’s depends on both carrier center frequency and channel bandwidth.

Table 3: A-MPR requirements for NS_43 (Power class 3)(from [4])

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer / Inner |
| DFT-s-OFDM | Pi/2 BPSK |  | N/A | ≤ 1.5 | N/A |  |  |  | N/A |  | N/A | ≤ 9 |
|  | QPSK | ≤ 2 |  |  |  |  |  | ≤ 2.5 |  |  |  | ≤ 9 |
|  | 16 QAM |  |  |  |  |  |  |  |  | ≤ 2.5 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 2.5 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |
| CP-OFDM | QPSK | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 4 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |

Table 4: A-MPR for NS_43 (Power Class 2) (from [4])

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) | A7 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer / Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 | N/A | ≤ 1.5 | N/A |  |  |  | N/A |  | N/A | ≤ 12 | ≤ 4 | 3 |
|  | QPSK | ≤ 3.5 |  |  |  |  |  | ≤ 3.5 |  | ≤ 5.5 |  | ≤ 12 | ≤ 4 | 3 |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  | ≤ 3.5 |  | ≤ 5.5 |  | ≤ 12 | ≤ 5 | 4 |
|  | 64 QAM | ≤ 3.5 |  | ≤ 4.5 |  | ≤ 5 |  |  |  |  |  | ≤ 12 | ≤ 5.5 | 5.5 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 12 | ≤ 6.5 | 5.5 |
| CP-OFDM | QPSK | ≤ 4 |  |  |  |  |  | ≤ 4 |  | ≤ 6 |  | ≤ 12 | ≤ 6 | 4.5 |
|  | 16 QAM | ≤ 4 |  |  |  |  |  | ≤ 4.5 |  | ≤ 6 |  | ≤ 12 | ≤ 6 | 5 |
|  | 64 QAM | ≤ 4 |  | ≤ 5 |  | ≤ 6.5 |  |  |  |  |  | ≤ 12 | ≤ 6.5 | 6 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 12 | ≤ 7 | 7 |

Test Environment

Based on the baseline agreed in [2], the test environment for NR A-MPR measurement for FR1 can be defined as in LTE.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR NS_43.

Test Frequencies

UL test carrier centre frequencies are defined in Table 2 for the NS_43 A-MPR regions. Select 902.5 MHz  910 MHz and 907.5 MHz as test frequency for 5, 10 and 15 MHz channel bandwidth, since these test frequencies are closest to the DL band to be protected.

PROPOSAL 2: Select 902.5 MHz, 910 MHz and 907.5 MHz as test frequencies for NS_43 A-MPR tests.

Test Channel Bandwidth

The minimum requirements for NS_43 apply for all channel bandwidths between 5 MHz to 15 MHz.

PROPOSAL 3: If supported, select 5, 10 and 15 MHz channel bandwidth.

  - Test Subcarrier Spacing

From the perspective of testing the maximum spectrum utilization and minimum guardband, lowest SCS shall be tested.

PROPOSAL 4: Select lowest subcarrier spacing.

  - UL Modulation

The selection of UL modulation can be based on the agreed proposals for SEM [2], i.e. all defined UL waveforms (5 DFT-s OFDM and 4 CP-OFDM waveforms).

PROPOSAL 5: Test all UL modulation waveforms supported by the UE for NS_43 A-MPR in FR1.

  - Uplink RB allocation

The UL RB allocation can be based on the agreed proposals for SEM [2], i.e. Edge_1RB_left, Edge_1RB_Right and Outer_Full. These common uplink configurations are listed in [5]. The AMPR for NS_43 has some Region specific RB allocations as specified in Table 2. The AMPR requirements and RB allocations can be interpreted as LCRB@RBStart as in table 4.

Table 4: UL RB allocations for NS_43

| Ch BW (MHz) |  |  | Region A |  |  |  | Region B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Fc (MHz) |  | SCS |  | RBstart |  |  | LCRB | LCRB@RBstart |  | Proposal |  | RBstart |  | LCRB |  | LCRB@RBstart |  | Proposal |  |
| 5 |  | 902.5 |  | 15 |  | - |  |  | >15 | Min16@0 Max 25@0 |  | Outer_Full (A1) |  | - |  | - |  | - |  | - |  |
|  |  |  |  | 30 |  | - |  |  | >7.5 | Min 8@0 Max 11@0 |  | Outer_Full (A1) |  | - |  | - |  | - |  | - |  |
|  |  |  |  | 60 |  | - |  |  | >3.75 | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  |  |  | 15 |  | - |  |  | >40 | Min 41@0 Max 52@0 |  | Outer_Full (A2) |  | - |  | >30 |  | Min 31@0 Max 52@0 |  | Outer_Full (A4) |  |
|  |  |  |  | 30 |  | - |  |  | >20 | Min 21@0 Max 24@0 |  | Outer_Full (A2) |  | - |  | >15 |  | Min 16@0 Max 24@0 |  | Outer_Full (A4) |  |
|  |  |  |  | 60 |  | - |  |  | >10 | 11@0 |  | Outer_Full (A2) |  | - |  | >7.5 |  | Min 8@0 Max 11@0 |  | Outer_Full (A4) |  |
| 10 |  | 910 |  | 15 |  | - |  |  | >45 | Min 46@0 Max 52@0 |  | Outer_Full (A3) |  | - |  | >40 |  | Min 41@0 Max 52@0 |  | Outer_Full (A5) |  |
|  |  |  |  | 30 |  | - |  |  | >22.5 | Min 23@0 Max 24@0 |  | Outer_Full (A3) |  | - |  | >20 |  | Min 21@0 Max 24@0 |  | Outer_Full (A5) |  |
|  |  |  |  | 60 |  | - |  |  | >11.3 | - |  | - |  | - |  | >10 |  | 11@0 |  | Outer_Full (A5) |  |
|  |  | 15 |  |  | Min 1@0 Max 79@0 | Edge_1RB_Left + Outer_Full (A6) | >10 <34 | >40 |  | Skip test (not low edge) (A6) |
|  |  | 30 | <5 | >0 | Min 1@0 Max 38@0 | Edge_1RB_Left + Outer_Full (A6) | >5 <17 | >20 | 32@6 |  |
|  |  | 60 |  |  | Min 1@0 Max 18@0 | Edge_1RB_Left + Outer_Full (A6) | >2.5 <8.5 | >10 | 15@3 |  |
|  |  | 15 | >68 | >0 | 10@69 | Skip test (not low edge) (A6) | >10 <34 | >40 | 1@78 | Skip test (not low edge) (A7) |
| 15 | 907.5 | 30 | >34 | >0 | 3@35 |  | >5 <17 | >20 | 1@37 |  |
|  |  | 60 | >17 | >0 | - |  | >2.5 <8.5 | >10 | 1@17 |  |
|  |  | 15 | - | - | - | - | >34 <40 | >30 | 44@35 | Skip test (not low edge) (A7) |
|  |  | 30 | - | - | - | - | >17 <20 | >15 | 20@18 |  |
|  |  | 60 | - | - | - | - | >8.5 <10 | >7.5 | 9@9 |  |

The proposals for LCRB@RBstart in Table 4 are all based on allocating as many RBs as possible close to the upper band to be protected (DL 860-890 MHz) as it can be considered a worst-case scenario for spurious emissions towards the DL band. Four RB allocations for 15 MHz band within Table 4 can be skipped since they are not close to the DL band. Worst case for 15 MHz channel bandwidth is Edge_1RB_Left and Outer_Full since they are closer to the DL band to be protected.

PROPOSAL 6: Select Outer_Full for all tests, with addition of Edge_1RB_Left for 15 MHz Region A.

2.7.	Number of Test Points

The maximum number of test points for A-MPR tests for NS_43:

| Environment conditions | Number SCS | Number of ChBW | Maximum Number of Modulations | Maximum Number RB allocations | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3 | 9 | 2 | 28 for PC3 38 for PC2 |

Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_43 A-MPR test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR in FR1.

PROPOSAL 2: Select 902.5 MHz, 910 MHz and 907.5 MHz as test frequencies for NS_43 A-MPR tests.

PROPOSAL 3: If supported, select 5, 10 and 15 MHz channel bandwidth.

PROPOSAL 4: Select lowest subcarrier spacing.

PROPOSAL 5: Test all UL modulation waveforms supported by the UE for NS_43 A-MPR in FR1.

PROPOSAL 6: Select Outer_Full for all tests, with addition of Edge_1RB_Left for 15 MHz Region A.

References

- R5-180065, "Discussion about Test parameters for NR TRx TCs " (NTT DOCOMO).

- R5-180886, " Discussion on test point selection for NR SEM in FR1” (Keysight Technologies).

- R5-182019, “Discussion of NR FR1 Test Point for TX Spurious Emission test cases”, (Qualcomm Inc.)

- 3GPP TS 38.101-1, V18.5.0 (2024-03), Table 6.2.3.6-1

- 3GPP TS 38.521-1, V18.2.0 (2024-03), Table 6.1-1