3GPP TSG-RAN5 Meeting #103	R5-243210

# Fukuoka, Japan, May 20-24, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2837 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding new test case Time mask for switching across four uplink bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adding new test case for Time mask for switching across four uplink bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test case 6.3A.3.7 Time mask for switching across three uplink bands |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Time mask for switching across four uplink bands can’t be tested for UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.7(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0913 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is implemented into specification after implementing R5-243209 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.3A.3.7	Time mask for switching across four uplink bands

Editor’s Note:	This test case is incomplete due to the following aspects are not yet determined:

-	Brackets still exists in the test case

-	Test procedure is incomplete

-	Connection diagram and Message contents are still TBD

-	Test requirement is incomplete

6.3A.3.7.1	Test purpose

To verify that the time mask for switching across three uplink bands meets the requirements given in 6.3A.3.0.3.6 and  6.3A.3.0.3.6a.

The time mask for switching across three uplink bands defines the transient period(s) allowed when the capability [supportedBandPairListNR-r18] is present.

6.3A.3.7.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support 4 bands CA configuration with UL CA with dynamic UL Tx switching.

6.3A.3.7.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3A.3.0.3.6 and 6.3A.3.0.3.6a.

6.3A.3.7.4	Test description

6.3A.3.7.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in table 6.3A.3.7.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3A.3.7.4.1-1: Test Configuration Table for switching across four uplink bands

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.1.3 for inter band CA in FR1 |  |  | Mid range for PCC and SCCs |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest NRB on PCC and Highest NRB_agg on SCCs |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  | Highest |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCCs | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 2) |  |
|  |  |  | PCC | SCC1 / SCC2 / SCC3 |
| 1 | N/A for this test | CP-OFDM QPSK | Inner Full | Inner Full |
| NOTE 1: For 3 bands configuration CA_nWA-nXA-nYA-nZA, PCC is located on band W, SCC1 is located on band X, SCC2 is located on band Y and SCC3 is located on band Z. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A.3-1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure TBD for TE diagram and section TBD for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3A.3.7.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3A.3.7.4.3.

6.3A.3.7.4.2	Test procedure

In this test case, one component carrier (PCC) is configured in band W, one component carrier (SCC1) is configured in band X, one component carrier (SCC2) is configured in band Y, and one component carrier (SCC3) is configured in band Z.

Sub test 1 is to verify the time mask for switching between band W and band X with the switching period located in band W, and switching between band X and band Y with the switching period located in band X, and switching between band Y and band Z with the switching period located in band Z. (Figure 6.3A.3.0.3.6-1 and 6.3A.3.0.3.6-2). UE is capable of uplink transmission on one band of the band pair in the band combination before and after uplink switching according to the scheduling commands. Component carriers on Band W, X, Y and Z are capable of two transmit antenna connectors.

Sub test 2 is to verify the time mask for switching from band pair [W,X] to band pair [Y,Z]. UE is capable of simultaneous uplink transmission on the two NR UL bands from the band pair for which dualUL is declared in the band combination according to the scheduling commands. Component carriers on band W, X,Y and band Z are with one transmit antenna connector and one antenna port.

- Sub test 1

  - Configure SCC1, SCC2 and SCC3 according to Annex C.0, C.1, C.2 for all downlink physical channels.

  - The SS shall configure SCC1, SCC2 and SCC3 as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.3A.3.7.4.3 with the switching period location is configured on PCC, SCC1 and SCC3.

  - SS activates SCCs by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133 [19], clause 9.3).

1.3a	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.7.4.1-1 on PCC only. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PCMAX,c level on PCC.

1.3b	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.7.4.1-1 on SCC1, SCC2 and SCC3. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PCMAX,c level on SCC1, SCC2 and SCC3.

  - SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.7.4.1-1 on PCC on slot n-1, where slot n is an uplink slot for SCC1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

  - The SS sends uplink scheduling information via DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.7.4.1-1 on SCC1 on slot n and slot m, with both slot n and slot m being uplink slots for SCC1, and slot m+1 being uplink slot for SCC2, and m ≥ n+20 when SCS=15kHz (m ≥ n+40 when SCS=30 kHz, m ≥ n+80 when SCS=60 kHz). Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

  - SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.7.4.1-1 on SCC2 on slot m+1 and k, where slot m+1and k are uplink slots for SCC2, and slot k+1 being uplink slot for SCC3, and k ≥ m+20 when SCS=15kHz (k ≥ m+40 when SCS=30 kHz, k ≥ m+80 when SCS=60 kHz). Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

  - SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.7.4.1-1 on SCC3 on slot k+1, where slot k+1 is an uplink slot for SCC3. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

  - Measure the sum of output power of UE PUSCH transmission on PCC over all antenna connectors during slot n-1 excluding a transient period of 10 µs and a Switching period X µs in the end of slot n-1. The length of uplink switching period X is indicated by UE capability [switchingPeriodFor2T] in the band pair.

  - Measure the sum of output power of UE PUSCH transmission on SCC1 over all antenna connectors during slot n and slot m excluding a transient period of 10 µs in the beginning of slot n, and excluding a a transient period of 10 µs and Switching period X µs in the end of slot m. The length of uplink switching period X is indicated by UE capability [switchingPeriodFor2T] in the band pair

  - Measure the sum of output power of UE PUSCH transmission on SCC2 over all antenna connectors during slot m+1 and slot k excluding a transient period of 10 µs in the beginning of slot m+1 and in the end of slot k.

  - Measure the sum of output power of UE PUSCH transmission on SCC3 over all antenna connectors during slot k+1 excluding a transient period of 10 µs and a Switching period X µs in the beginning of slot k+1. The length of uplink switching period X is indicated by UE capability [switchingPeriodFor2T] in the band pair.

- Sub test 2

TBD

6.3A.3.7.4.3		Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exceptions:

TBD

6.3A.3.7.5	Test requirement

The requirement for the power measured in step 1.8, 1.9, 1.0, 1.11 of the test procedures shall not exceed the values specified in table 6.3A.3.7.5-1.

Table 6.3A.3.7.5-1: Time mask for switching across four bands (On power)

|  | Measured output power |  |
| --- | --- | --- |
| Transmit ON power | Same as Test ID 1 of Table 6.2D.2.5-1 as appropriate | PCC and SCC1 and SCC2 and SCC3 in Test step 1.8, 1.9, 1.0, 1.11 |
|  | TBD | TBD |
| NOTE1: TT or each frequency and channel bandwidth of Transmit ON power is specified in Table 6.2A.2.1.5-3 |  |  |

<End of Changes>