3GPP TSG RAN WG5 Meeting #103	R5-243234

Fukuoka, Japan, 20th May 2024 – 25th May 2024

Agenda Item:	6.2

Source:	Orange, Qualcomm

Title:	Proposal for new test cases related to P-CSCF Restoration procedures

Document for:	Endorsement

# 1	Introduction

With 3G/2G sunset, in some countries CS Fall Back won’t be an option anymore. Failures may occur during call establishment when issue happens in IMS core network. VoIMS service must be resilient.

A set of standardized procedures for restoration of IMS connectivity after node or link failure are defined in TS 23.380 [1] and TS 24.229 [2]. These procedures are also included in IR92 [3] and NG114 [4] in P-CSCF Discovery section.

# 2	Discussion

The discussion is about adding the following test cases in RAN5 specifications to cover IMS/P-CSCF restoration procedures bellow:

Proposal 2-1: Verify UE behaviour when P-CSCF addresses are included in PDU-Session Modification Command when UE is connected to 5GC

As described in section U.2.2.1C of TS 24.229 [2] if the UE receives one or more P-CSCF address(es) in the extended Protocol Configuration Options information element of a PDU SESSION MODIFICATION COMMAND message and the one or more P-CSCF addressee(s) do not include the address of the currently used P-CSCF, then the UE shall acquire a different P-CSCF address from the one or more P-CSCF addresse(s) in the PDU SESSION MODIFICATION COMMAND message. If more than one P-CSCF address with the same container identifier (i.e. "P-CSCF IPv6 Address" or "P-CSCF IPv4 Address") are included, then the UE shall assume that the more than one P-CSCF addresses with the same container identifier are prioritised with the first P-CSCF address with the same container identifier within the Protocol Configuration Options information element as the P-CSCF address with the highest priority

Proposal 2-2: Verify UE behavior when P-CSCF address are included in Modify EPS Bearer Context Request when UE is connected to EPS

As described in section L.2.2.1C of TS 24.229 [2] if the UE used method II for P-CSCF discovery and if the UE receives one or more P-CSCF address(es) in the Protocol Configuration Options information element of a Modify EPS Bearer Context Request message the one or more P-CSCF addresse(s) do not include the address of the currently used P-CSCF, then the UE shall acquire a different P-CSCF address from the one or more P-CSCF addresse(s) in the Modify EPS Bearer Context Request message. If more than one P-CSCF address with the same container identifier (i.e. “P-CSCF IPv6 Address” or “P-CSCF Ipv4 Address”) are included, then the UE shall assume that the more than one P-CSCF addresses with the same container identifier are prioritised with the first P-CSCF address with the same container identifier within the Protocol Configuration Options information element as the P-CSCF address with the highest priority.

Proposal 2-3: Verify UE behaviour when P-CSCSF address are included in IKEv2 INFORMATIONAL exchange when UE is connected to Non 3GPP Access

As described in section 5.6.5.2 of TS 23.380 [1] if the UE has sent its capability the indication of the support of P-CSCF restoration to the ePDG by including the P-CSCF_RESELECTION_SUPPORT Notify payload within an IKE_AUTH request message (as defined in TS 24.302 [5]) and if the UE receives an IKEv2 informational exchange message containing a list of P-CSCF addresses, the UE shall first send a response to the ePDG, and then the UE must acquire a different P-CSCF from the currently used P-CSCF, and initiate a new initial SIP registration

Proposal 2-4:

Verify UE behaviour for P-CSCF discovery after PDU Session Release command 5GSM Cause #39 Reactivation requested when UE is connected to 5GC

Verify UE behaviour for P-CSCF discovery after EPS Bearer Deactivation ESM Cause #39 Reactivation requested when UE is connected to MME

Verify UE behaviour for P-CSCF discovery after DELETE payload REACTIVATION_REQUESTED_CAUSE when UE is connected to Non 3GPP Access

As defined in TS 23.380 [1], upon receipt of a request to release an IMS PDN/PDU connection with the cause "reactivation requested" over the 3GPP or the WLAN access, the UE shall locally release the IMS PDN/PDU connection. As a result of the release of the IMS PDN/PDU connection, the UE shall re-establish the IMS PDN/PDU connection, and also perform a new P-CSCF discovery (as the IMS PDN connection was lost). Then the UE shall perform a new initial IMS registration after the new P-CSCF discovery.

# 3	Proposal

It’s suggested to add test cases for all the proposals with the following integration:

| Proposal | Impacted RAN5 specs and new test case clauses and titles |
| --- | --- |
| 2-1 | One test case to be added in NR spec TS 38.523-1: TC 10.1.2.3: Network-requested PDU session modification / new P-CSCF address / Initial IMS registration CR R5-242937 and CR R5-242938 in RAN5#103 meeting |
| 2-2 | One test case to be added in LTE spec TS 36.523-1: TC 10.3.2: EPS Bearer context modification / new P-CSCF address / Initial IMS registration CR R5-242940 and CR R5-242941 in RAN5#103 meeting |
| 2-3 | One test case in IMS spec TS 34.229-1: TC G.8.2: Initial Registration on new P-CSCF / WLAN To be defined in RAN5#104 meeting |
| 2-4 | One test case to be added in NR spec TS 38.523-1: TC 10.1.3.3: Network-requested PDU session release / reactivation requested / new P-CSCF address / Initial IMS registration To be defined in RAN5#104 meeting One test case to be added in LTE spec TS 36.523-1: TC 10.4.3: EPS bearer context deactivation / reactivation requested / new P-CSCF address / Initial IMS registration To be defined in RAN5#104 meeting One test case in IMS spec TS 34.229-1: TC G.8.3: Initial Registration on new P-CSCF / Reactivation requested by network / WLAN To be defined in RAN5#104 meeting |

# 4	References

[1] 3GPP TS 23.380: 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; IMS Restoration Procedures (Release 17)

[2] 3GPP TS 24.229: 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3 (Release 18)

[3] GSMA PRD IR.92: "IR.92 IMS Profile for Voice and SMS, Version 19.0".

[4] GSMA PRD NG.114: "IMS Profile for Voice, Video and Messaging over 5GS, Version 6.0".

[5] 3GPP TS 24.302: 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3 (Release 18)