3GPP TSG-RAN5 Meeting #103	R5-243267

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0059 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new TC 6.5.1 Location Information Request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There was no existing test case for MCData Location Information Request |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added clause 6.5 and new test case On-network / Location/ Location Information Request |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Devices could fail in the commercial field without testing the capability |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5, 6.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

## 6.5	Location

### 6.5.1	On-network / Location/ Location Information Request

6.5.1.1	Test Purpose (TP)

(1)

with { UE (MCData Client)registered and authorized for MCData service and not in an emergency state and having received a location information configuration message providing configuration parameters for both NonEmergencyLocationInformation and EmergencyLocationInformation }

ensure that {

when { UE (MCData Client) receives a location information request before the minimumReportInterval timer has expired  }

then { UE (MCData Client)does not wait for the minimumReportInterval timer to expire, and,  sends "immediately" a SIP MESSAGE containing location information report including information in accordance with the NonEmergencyLocationInformation configuration and the <ReportID> attribute set to the value of the <RequestID> attribute in the received Request, and, resets the minimumReportInterval timer }

}

(2)

with { UE (MCData Client)registered and authorized for MCData service and not in an emergency state and having received a location information configuration message providing configuration parameters for both NonEmergencyLocationInformation and EmergencyLocationInformation }

ensure that {

when { UE (MCData Client) determines that the periodic location reporting trigger has fired and the minimumReportInterval timer has expired }

then { UE (MCData Client)sends a SIP MESSAGE containing location information report including information in accordance with the NonEmergencyLocationInformation configuration and the <TriggerID> attribute set to the value of the <TriggerID> attribute in the received location information configuration message, and, resets the minimumReportInterval timer }

}

(3)

with { UE (MCData Client)registered and authorized for MCData service and not in an emergency state and having received a location information configuration message }

ensure that {

when { UE (MCData Client) receives a new location information configuration message and ConfigScope set to 'Full' }

then { UE (MCData Client) replies with a SIP 200 (OK) message, and stores the contents of the <Configuration> elements thereby overriding the previous configuration }

}

6.5.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 17.3.1, 17.3.2, 17.3.3, 17.3.4.1, 17.3.4.2. Unless otherwise stated, these are Rel-16 requirements.

[TS 24.282 clause 17.3.1]

The MCData client sends a location report when one of the trigger criteria is fulfilled or when it receives a request from the participating MCData function to send a location report. To send the location report the MCData client can use an appropriate SIP message that it needs to send for other reasons, or it can include the location report in a SIP MESSAGE request.

To send a location report, the MCData client includes in the SIP MESSAGE request an application/vnd.3gpp.mcdata-location-info+xml MIME body as specified in clause D.4. The MCData client populates the elements in accordance with its reporting configuration. Further location information may also be included in the P-Access-Network-Info header field.

[TS 24.282 clause 17.3.2]

Upon receiving a SIP MESSAGE request containing:

1)	an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref set to the value "urn:urn-7:3gpp-service.ims.icsi.mcdata";

2)	a Content-Type header field set to "application/vnd.3gpp.mcdata-location-info+xml"; and

3)	an application/vnd.3gpp.mcdata-location-info+xml MIME body with a <Configuration> root element included in the <location-info> root element;

the MCData client:

1)	shall store the contents of the <Configuration> elements;

2)	shall set the location reporting triggers accordingly; and

3)	shall start the minimumReportInterval timer.

[TS 24.282 clause 17.3.3]

Upon receiving a SIP MESSAGE request containing:

1)	an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref set to the value "urn:urn-7:3gpp-service.ims.icsi.mcdata";

2)	a Content-Type header field set to "application/vnd.3gpp.mcdata-location-info+xml"; and

3)	an application/vnd.3gpp.mcdata-location-info+xml MIME body with a <Request> element included in the <location-info> root element;

the MCData client:

1)	shall send a location report as specified in subclause 17.3.4; and

2)	shall reset the minimumReportInterval timer.

[TS 24.282 clause 17.3.4.1]

If a location reporting trigger fires, the MCData client checks if the minimumReportInterval timer is running. If the timer is running the MCData client waits until the timer expires. When the minimumReportInterval timer expires, the MCData client:

1)	shall, if any of the reporting triggers are still true, send a location information report as specified in subclause 17.3.4.2.

If the MCData client receives a location information request as specified in subclause 17.3.3, the MCData client shall send a location report as specified in subclause 17.3.4.2.

[TS 24.282 clause 17.3.4.2]

If the MCData client needs to send a SIP request anyway (i.e. for reasons other than explicit location reporting request or the firing of a configured location trigger), the MCData client:

1)	shall include an application/vnd.3gpp.mcdata-location-info+xml MIME body and in the <location-info> root element the MCData client shall include:

a)	a <Report> element and, if the Report was triggered by a location request, include the <ReportID> attribute set to the value of the <RequestID> attribute in the received Request;

b)	<TriggerId> child elements, if triggers have fired, where each element is set to the value of the <Trigger-Id> attribute associated with the triggers that have fired; and

c)	the location reporting elements corresponding to the triggers that have fired, if at least one trigger has fired;

2)	shall set the minimumReportInterval timer to the minimumReportInterval time and start the timer; and

3)	shall reset all triggers.

If the MCData client does not need to send a SIP request for reasons other than explicit location reporting request or the firing of a configured location trigger, the MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6]. The MCData client;

1)	shall include in the Request-URI, the SIP URI received in the P-Asserted-Identity header field in the received SIP MESSAGE request for location report configuration;

2)	shall include a Content-Type header field set to "application/vnd.3gpp.mcdata-location-info+xml";

3)	shall include an application/vnd.3gpp.mcdata-location-info+xml MIME body and in the <location-info> root element include:

a)	a <Report> element and if the Report was triggered by a location request include the <ReportID> attribute set to the value of the <RequestID> attribute in the received Request;

b)	<TriggerId> child elements, if triggers have fired, where each element is set to the value of the <Trigger-Id> attribute associated with the triggers that have fired; and

c)	the location reporting elements corresponding to the triggers that have fired, if at least one trigger has fired;

4)	shall include an Accept-Contact header field with the media feature tag g.3gpp.mcdata along with parameters "require" and "explicit" in accordance with IETF RFC 3841 [8];

5)	shall set the minimumReportInterval timer to the minimumReportInterval time and start the timer;

6)	shall reset all triggers; and

7)	shall send the SIP MESSAGE request as specified in 3GPP TS 24.229 [5].

6.5.1.3	Test description

6.5.1.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 36.579-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 36.579-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

6.5.1.3.2	Test procedure sequence

Table 6.5.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-3 | Steps 1a1-3 of procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 are performed to configure location reporting with minimumIntervalLength timer set to 20s and PeriodicReport reporting interval set to 15s. | - | - | - | - |
| 4 | Start timer t_Wait=10s (NOTE 1) | - | - | - | - |
| 5 | Start timer t_PeriodicReport=15s (PeriodicReport timer set in step 1). | - | - | - | - |
| 6 | The procedure 'MCX communication release' as described in TS 36.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| 7 | t_Wait expires. | - | - | - | - |
| 8-10 | Steps 1a1-3 of procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 are performed to request a location report. | - | - | - | - |
| 11-12 | Check: Does the UE (MCData Client) correctly perform steps 2-3 of procedure 'MCX SIP MESSAGE CO' as described in TS 36.579-1 [2] Table 5.3.32.3-1 before t_PeriodicReport expires to provide location information for the NonEmergencyLocationInformation configured at step 2? | - | - | 1 | P |
| 13 | Start timer t_MinimumReportInterval=20s | - | - | - | - |
| 14 | Stop timer t_PeriodicReport | - | - | - | - |
| 15 | The procedure 'MCX communication release' as described in TS 36.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| 16 | t_MinimumReportInterval expires | - | - | - | - |
| 17-19 | Check: Does the UE (MCData Client) correctly perform steps 1a1-3 of procedure 'MCX SIP MESSAGE CO' as described in TS 36.579-1 [2] Table 5.3.32.3-1 to provide periodic location information? | - | - | 2 | P |
| 20-21 | Check: Does the UE (MCData Client) correctly perform step 2-3 of procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to remove the periodic trigger criteria from the location reporting configuration? | - | - | 3 | P |
| 22-23 | Steps 2-3 of procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 are performed to request a location report. | - | - | - | - |
| 24-25 | Check: Does the UE (MCData Client) correctly perform steps 2-3 of procedure 'MCX SIP MESSAGE CO' as described in TS 36.579-1 [2] Table 5.3.32.3-1 to provide location information for the configuration done at steps 18-19? | - | - | 1 | P |
| 26 | Check: Does the UE (MCData Client) send any SIP MESSAGE in the next 25s? | --> | SIP MESSAGE | 2 | F |
| 27 | The procedure 'MCX communication release' as described in TS 36.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| NOTE 1: The value of 10s is arbitrary chosen. |  |  |  |  |  |

6.5.1.3.3	Specific message contents

Table 6.5.1.3.3-1: SIP MESSAGE from the SS (Step 2, Table 6.5.1.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition LOCATION-CONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-info as described in Table 6.5.1.3.3-2 |  |  |  |
| MIME body part |  | Location-info | TS 24.282 [31] clause D.4 |  |
| MIME-part-body | Location-info as described in Table 6.5.1.3.3-3 |  |  |  |

Table 6.5.1.3.3-2: MCData-Info in SIP MESSAGE (Table 6.5.1.3.3-1, 6.5.1.3.3-4, 6.5.1.3.3-10, 6.5.1.3.3-12)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdataoinfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-user-id | not present |  |  |  |

Table 6.5.1.3.3-3: Location-Info in SIP MESSAGE (Table 6.5.1.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Configuration |  |  |  |  |
| NonEmergencyLocationInformation |  |  |  |  |
| ServingEcgi | not present |  |  |  |
| NeighbouringEcgi | not present | . |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| GeographicalCoordinate | present | Provision of <GeographicalCoordinate> in the report is requested. |  |  |
| minimumIntervalLength | "20" |  |  |  |
| TriggeringCriteria |  |  |  |  |
| PeriodicReport |  |  |  |  |
| extension base | "15" | periodicity of the reports (in seconds) |  |  |
| TiggerID | "PERIODIC" |  |  |  |

Table 6.5.1.3.3-4: SIP MESSAGE from the SS (Step 9, Table 6.5.1.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition LOCATION-INFO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-info as described in Table 6.5.1.3.3-2 |  |  |  |
| MIME body part |  | Location-info | TS 24.282 [31] clause D.4 |  |
| MIME-part-body | Location-info as described in Table 6.5.1.3.3-5 |  |  |  |

Table 6.5.1.3.3-5: Location-Info in SIP MESSAGE (Table 6.5.1.3.3-4)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.3-3

Table 6.5.1.3.3-6: SIP MESSAGE from the UE (Step 11, Table 6.5.1.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.32.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition LOCATION_REPORT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-URI | tsc_MCData_PublicServiceId_PF_A | same URI as provided in the Asserted-Identity header field of the SIP MESSAGE for location reporting configuration |  |  |
| Message-body |  |  |  |  |
| MIME body part | Not present | MCData-Info |  |  |
| MIME body part |  | Location-info | TS 24.282 [31] clause D.4 |  |
| MIME-part-body | Location-info as described in Table 6.5.1.3.3-7 |  |  |  |

Table 6.5.1.3.3-7: Location-Info in SIP MESSAGE (Table 6.5.1.3.3-6)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportID attribute | "1" | The same as the ID in the request message. |  |  |
| ReportType attribute | "NonEmergency" |  |  |  |
| CurrentLocation |  | Information corresponding to the location reporting configuration of table 6.5.1.3.3-3 |  |  |
| CurrentServingEcgi | not present |  |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| CurrentCoordinate |  |  |  |  |
| longitude | Encrypted <longitude> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-3 |  |  |
| latitude | Encrypted <latitude> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-3 |  |  |

Table 6.5.1.3.3-8: SIP MESSAGE from the UE (Step 18, Table 6.5.1.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.32.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition LOCATION-INFO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-URI | tsc_MCData_PublicServiceId_PF_A | same URI as provided in the Asserted-Identity header field of the SIP MESSAGE for location reporting configuration |  |  |
| Message-body |  |  |  |  |
| MIME body part | Not present | MCData-Info |  |  |
| MIME body part |  | Location-info | TS 24.282 [31] clause D.4 |  |
| MIME-part-body | Location-info as described in Table 6.5.1.3.3-9 |  |  |  |

Table 6.5.1.3.3-9: Location-Info in SIP MESSAGE (Table 6.5.1.3.3-8)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportType attribute | "NonEmergency" |  |  |  |
| TriggerID | "PERIODIC" |  |  |  |
| CurrentLocation |  | Information corresponding to the location reporting configuration of table 6.5.1.3.3-3 |  |  |
| CurrentServingEcgi | not present |  |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| CurrentCoordinate |  |  |  |  |
| longitude | Encrypted <longitude> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-2 |  |  |
| latitude | Encrypted <latitude> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-2 |  |  |

Table 6.5.1.3.3-10: SIP MESSAGE from the SS (Step 20, Table 6.5.1.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition LOCATION-CONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-info as described in Table 6.5.1.3.3-2 |  |  |  |
| MIME body part |  | Location-info | TS 24.282 [31] clause D.4 |  |
| MIME-part-body | Location-info as described in Table 6.5.1.3.3-11 |  |  |  |

Table 6.5.1.3.3-11: Location-Info in SIP MESSAGE (Table 6.5.1.3.3-10)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Configuration |  |  |  |  |
| NonEmergencyLocationInformation |  |  |  |  |
| ServingEcgi | present | Provision of <ServingEcgi> in the report is requested. |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| GeographicalCoordinate | not present |  |  |  |

Table 6.5.1.3.3-12: SIP MESSAGE from the SS (Step 22, Table 6.5.1.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition LOCATION-INFO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-info as described in Table 6.5.1.3.3-2 |  |  |  |
| MIME body part |  | Location-info | TS 24.282 [31] clause D.4 |  |
| MIME-part-body | Location-info as described in Table 6.5.1.3.3-13 |  |  |  |

Table 6.5.1.3.3-13: Location-Info in SIP MESSAGE (Table 6.5.1.3.3-12)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.3-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Request |  |  |  |  |
| RequestID | "2" | The RequestID that the MCData client shall reference in the Report |  |  |

Table 6.5.1.3.3-14: SIP MESSAGE from the UE (Step 24, Table 6.5.1.3.2-1;
step 2, TS 36.579-1 [2], Table 5.3.32.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition LOCATION-INFO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-URI | tsc_MCData_PublicServiceId_PF_A | same URI as provided in the Asserted-Identity header field of the SIP MESSAGE for location reporting configuration |  |  |
| Message-body |  |  |  |  |
| MIME body part | Not present | MCData-Info |  |  |
| MIME body part |  | Location-info | TS 24.282 [31] clause D.4 |  |
| MIME-part-body | Location-info as described in Table 6.5.1.3.3-15 |  |  |  |

Table 6.5.1.3.3-15: Location-Info in SIP MESSAGE (Table 6.5.1.3.3-14)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info | present |  |  |  |
| Report | present |  |  |  |
| ReportID | "2" | The same as the ID in the request message. |  |  |
| ReportType | "NonEmergency" |  |  |  |
| CurrentLocation | present | Information corresponding to the location reporting configuration of table 6.5.1.3.3-8 |  |  |
| CurrentServingEcgi | Encrypted <CurrentServingEcgi> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-2 |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| CurrentCoordinate | not present |  |  |  |

<END OF CHANGES>