3GPP TSG-RAN5 Meeting #103	R5-243343

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3218 | rev | - | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to FR1 RedCap E-UTRA - NR inter - RAT Handover TC 18.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Wrong SSB configuration in case of NR 15 KHZ. Test Configurations 1,2,4,5,6,8 for NR 15 KHz are being configured using SSB.1 redCap FR1 which is defined for SSB SCS 30 KHZ per Annex A.3B.1 SSB configuration for FR1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated NR neighbout Cell specific test parameters for NR table to correct SSB configuration in case of configuration 1,2,4,5,6,8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance UE may fail the test procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 CR R4-2407447 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 18.2.1.1	E-UTRA - NR FR1 handover  for 2Rx UE

18.2.1.1.1	Test purpose

To verify the E-UTRAN to NR FR1 handover requirements for 2RX RedCap as specified in TS 36.133 [23] clause 5.3.4B.

18.2.1.1.2	Test applicability

This test applies to all E-UTRA UE from release 17 onwards and capable of NR RedCap with 2Rx.

18.2.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 18.2.1.0

The normative reference for this requirement is TS 38.133 [6] clause A.18.2.1.1.

18.2.1.1.4	Test description

Same as the test description given in clause 8.3.1.1.4.

18.2.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 8.3.1.1.4.1 with following exceptions:

-	Table 8.3.1.1.4.1-1 is replaced by Table 18.2.1.1.4.1-1.

-	Table 8.3.1.1.4.1-2 is replaced by Table 18.2.1.1.4.1-2.

-	Table 8.3.1.1.4.1-3 is replaced by Table 18.2.1.1.4.1-3.

Table 18.2.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 18.2.1.1-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 18.2.1.1-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 18.2.1.1-3 | LTE FDD, NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 18.2.1.1-4 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| 18.2.1.1-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 18.2.1.1-6 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 18.2.1.1-7 | LTE TDD, NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 18.2.1.1-8 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 18.2.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 18.2.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 18.2.1.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | E-UTRAN cell |
|  | Neighbouring cell |  | Cell 2 | NR cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  | RSRP |  |
| b2-Threshold1 |  | dBm | -83 | Absolute E-UTRAN RSRP threshold for event B2 |
| b2-Threshold2NR |  | dBm | As specified in Table 18.2.1.1.5-2 | Absolute NR SS-RSRP threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 8.1.2.1-1 started before T2 starts [23] |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

18.2.1.1.4.2	Test procedure

Same as the test procedure given in clause 8.3.1.1.4.2.

18.2.1.1.4.3	Message contents

Same as the message contents given in clause 8.3.1.1.4.3 with following exceptions.

-	Table 7.3.1-3 in TS 38.508-1 [14] is used with condition SMTC.1 RedCap.

18.2.1.1.5	Test requirement

Same as the test requirements given in clause 8.3.1.1.5 with following exceptions:

-	Table 8.3.1.1.5-1 is replaced by Table 18.2.1.1.5-1.

-	Table 8.3.1.1.5-2 is replaced by Table 18.2.1.1.5-2.

Table 18.2.1.1.5-1: Cell specific test parameters for E-UTRA PCell (Cell 1)

| Parameter | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1,2,3,4,5,6,7,8 | 2 |  |  |
| Duplex mode |  | 1,2,3,4 | FDD |  |  |
|  |  | 5,6,7,8 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 5,6,7,8 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 5,6,7,8 | 1 |  |  |
| BWchannel | MHz | 1,2,3,4,5,6,7,8 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PRACH ConfigurationNote2 |  | 1,2,3,4 | 4 |  |  |
|  |  | 5,6,7,8 | 53 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1,2,3,4 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |  |
|  |  | 5,6,7,8 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1,2,3,4 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  |  | 5,6,7,8 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote3 |  | 1,2,3,4 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |  |
|  |  | 5,6,7,8 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |  |
| PBCH_RA | dB | 1,2,3,4,5,6,7,8 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -98 |  |  |
| Ês/Noc | dB | 1,2,3,4,5,6,7,8 | 7 | 5.45 Note 8 | 5.45 Note 8 |
| Ês/IotNote6 | dB | 1,2,3,4,5,6,7,8 | 7 | 5.45 | 5.45 |
| RSRPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -91 | -92.55 | -92.55 |
| SCH_RPNote6 | dBm/15kHz | 1,2,3,4,5,6,7,8 | -91 | -92.55 | -92.55 |
| IoNote6 | dBm/9MHz | 1,2,3,4,5,6,7,8 | -62.43 | -63.68 | -63.68 |
| Propagation Condition |  | 1,2,3,4,5,6,7,8 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1,2,3,4,5,6,7,8 | 1x2 Low |  |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. NOTE 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. NOTE 8: Including test tolerance given in Annex F. |  |  |  |  |  |

Table 18.2.1.1.5-2: NR neighbour Cell specific test parameters for NR neighbour (Cell 2)

| Parameter |  | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1,2,3,4,5,6,7,8 | 1 |  |  |
| Duplex mode |  |  | 1,5 | FDD |  |  |
|  |  |  | 2,3,6,7 | TDD |  |  |
|  |  |  | 4.8 | HD-FDD |  |  |
| TDD Configuration |  |  | 2 6 | TDDConf.1.1 |  |  |
|  |  |  | 3,7 | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | 1,4,5,8 | 10: NRB,c = 52 (FDD) |  |  |
|  |  |  | 2,6 | 10: NRB,c = 52 (TDD) |  |  |
|  |  |  | 3,7 | 20: NRB,c = 51 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1,4,5,8 | SR.1.1 FDD |  |  |
|  |  |  | 2,6 | SR.1.1 TDD |  |  |
|  |  |  | 3,7 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1,4,5,8 | CR.1.1 FDD |  |  |
|  |  |  | 2,6 | CR.1.1 TDD |  |  |
|  |  |  | 3,7 | CR.2.1 TDD |  |  |
| PRACH configuration |  |  | 1,2,3,4,5,6,7,8 | PRACH.1 FR1 |  |  |
| OCNG patternNote1 |  |  | 1,2,3,4,5,6,7,8 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1,2,3,4,5,6,7,8 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1,2,3,4,5,6,7,8 | SMTC.1 RedCap |  |  |
| SSB configuration |  |  | 1,2,4,5,6,8 | SSB.4 RedCap FR1 |  |  |
|  |  |  | 3,7 | SSB.2 RedCap FR1 |  |  |
| b2-Threshold2NR |  | dBm | 1,2,4,5,6,8 | -106 |  |  |
|  |  |  | 3,7 | -103 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1,2,3,4,5,6,7,8 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1,2,3,4,5,6,7,8 | -98 |  |  |
| NocNote2 |  | dBm/SCS | 1,2,4,5,6,8 | -98 |  |  |
|  |  |  | 3,7 | -95 |  |  |
| Ês/Noc |  | dB | 1,2,3,4,5,6,7,8 | -infinity | 1.55 Note 4 | 1.55 Note 4 |
| Ês/IotNote3 |  | dB | 1,2,3,4,5,6,7,8 | -infinity | 1.55 | 1.55 |
| SS-RSRPNote3 |  | dBm/SCS | 1,2,4,5,6,8 | -infinity | -96.45 | -96.45 |
|  |  |  | 3,7 | -infinity | -93.44 | -93.44 |
| IoNote3 |  | dBm/9.36 MHz | 1,2,4,5,6,8 | -70.05 | -66.19 | -66.19 |
|  |  | dBm/18.36 MHz | 3,7 | -67.13 | -63.27 | -63.27 |
| Propagation condition |  |  | 1,2,3,4,5,6,7,8 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1,2,3,4,5,6,7,8 | 1x2 Low |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Including test tolerance given in Annex F. |  |  |  |  |  |  |

## <<< Skip unchanged sections >>>

## <<< END OF CHANGES >>>