3GPP TSG-RAN5 Meeting #103	R5-243345

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

Agenda item:	5.4.20

Source:	Keysight Technologies

Title:	Discussion on PRACH FR2 power measurement

Document for:	Discussion and Endorsement

# Introduction

During RAN5#102, several discussion papers were presented to continue progressing on FR2 testability aspects about Rel-18 Beam Correspondence for Initial access. During the discussion, some companies raised the concern that, before discussing this Rel-18 topic, RAN5 should solve first the testability of FR2 RRM PRACH test case (TS 38.533 [1] TC 7.3.2.2.X for SA and 5.3.2.2.X for NSA) where FR2 PRACH absolute power is measured in some specific subtests.

This contribution analyses differences/commonalities between Rel-18 Beam Correspondence for Initial Access and FR2 RRM Random Access test cases.

# Discussion

Following sections describes, in high level manner, differences and commonalities between different PRACH requirements across RRM [1] and RF [3] specs:

- Rel-18 Beam Correspondence for Initial Access

- Rel-15 FR2 PRACH power measurements in RRM random access test cases

- Rel-15 FR2 PRACH time mask test case

## Test cases analysis

#### Rel-18 Beam Correspondence for Initial Access

Rel-18 Beam Correspondence for Initial Access requires following aspects for PRACH power measurements:

- UE achives Pcmax before PRACH measurements.

- To measure an average power across several ms.

- To measure PRACH after UE power is stable. Hence, several first preambles needs to be discarded.

- Optimal Tx beam selection is required.

- UE beam lock is needed.

- Metric: Spherical coverage.

Observation 1. Rel-18 BC for Initial access requires to measure at Pcmax across stable Tx preambles.

#### Rel-15 FR2 RRM random access test cases

- Rel-15 FR2 RRM Random access test cases are not concluded for those subtests where PRACH power measurement is required. Extracted from [1]:

- PRACH power characteristics in Rel-15 FR2 random access test cases:

- First preamble for absolute power. Extracted from 5.3.2.2.1 test case in [1]:

- Expected Power level: 0.6dBm is the expected PRACH power level based on test requirements:

- Absolute power tolerance needs to be added:

  - Absolute power tolerance requirement defined in [2] as follows:

Observation 2. Rel-15 RRM random access test cases requires to measure first preamble around 0.6dBm within absolute power tolerance requirement as defined in TS 38.101-2 Table 6.3.4.2-1.

Observation 3. Absolute power tolerance of ±14 dB needs to be considered in PRACH absolute power measurements in Rel-15 FR2 random access TCs.

Observation 4. Rel-15 FR2 Random Access test cases will checked PRACH absolute power level as 0.6 dBm ± 14 dB.

#### Rel-15 PRACH time mask test case

- Rel-15 PRACH time mask test case is not concluded yet in [3].

- PRACH power characteristics in Rel-15 PRACH time mask test case:

- PRACH power level based on open loop power control parameters.

- Requirement does not specify first preamble.

- The period of the measurement shall be the slot during the PRACH preamble transmission.

- Requirement does not specify PRACH power level.

Observation 5. Rel-15 PRACH time mask requirement does not specify which preamble and expected power level needs to be measured.

Observation 6. Rel-15 PRACH time mask test case is not completed due to open testability aspects not concluded in terms of UE beamforming for PRACH transmission: TE can’t control UE beamforming for PRACH transmission.

## Differences between Rel-18 and Rel-15 PRACH test cases

As described in section 2.1, there are differences between Rel-18 Beam correspondence for Initial Access requirement and Rel-15 PRACH test cases.

Differences:

- Number of preambles to be measured.

- Which preamble to be measured.

- Expected PRACH power level.

- UE beam lock function.

- FR2 metric: Spherical coverage vs EIRP

Proposal 1. Considering differences between Rel-15 and Rel-18 PRACH requirements it is proposed to decouple the analysis and way forward between Rel-18 Beam Correspondence for Initial access from Rel-15 PRACH test cases.

## Way forward on Rel-15 RRM random access

In Rel-15 RRM random access test cases, it is observed in the lab a variability on PRACH absolute power level for first preamble across different run of same TC in same test platform for same device under test. The variability in real measurements is around expected power level (0.6dBm) ± 13 dB. This variability could be related to common topic treated in previous section: TE can’t control UE beamforming for PRACH transmission.

However, in section 2.1.2 of this document clarifies that the absolute PRACH power, for the first preamble in RRM FR2 random access test cases, shall have an accuracy within Absolute power tolerance requirement (±14 dB). Additionally, test tolerance on top of Absolute power tolerance needs to be added based on test system Measurement uncertainty. Hence, this variability in real results looks aligned with Absolute power tolerance requirements.

Observation 7. PRACH absolute power level results in the lab for first preamble in Rel-15 RRM random access test cases are aligned with Absolute power tolerance requirements. Hence, there is not a need to control UE beamforming for PRACH transmission in Rel-15 RRM random access test case.

Proposal 2. To consider Rel-15 RRM Random access subtest for PRACH absolute power level testable.

Observation 8. Pending to conclude with Test Tolerances based on Measurement Uncertainty for Uplink absolute power measurement term in [1] Annex F.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1. Rel-18 BC for Initial access requires to measure at Pcmax across stable Tx preambles.

Observation 2. Rel-15 RRM random access test cases requires to measure first preamble around 0.6dBm within absolute power tolerance requirement as defined in TS 38.101-2 Table 6.3.4.2-1.

Observation 3. Absolute power tolerance of 14 dB needs to be considered in PRACH absolute power measurements in Rel-15 FR2 random access TCs.

Observation 4. Rel-15 FR2 Random Access test cases will checked PRACH absolute power level as 0.6 dBm ± 14 dB.

Observation 5. Rel-15 PRACH time mask requirement does not specify which preamble and expected power level needs to be measured.

Observation 6. Rel-15 PRACH time mask test case is not completed due to open testability aspects not concluded in terms of UE beamforming for PRACH transmission: TE can’t control UE beamforming for PRACH transmission.

Observation 7. PRACH absolute power level results in the lab for first preamble in Rel-15 RRM random access test cases are aligned with Absolute power tolerance requirements. Hence, there is not a need to control UE beamforming for PRACH transmission in Rel-15 RRM random access test case.

Observation 8. Pending to conclude with Test Tolerances based on Measurement Uncertainty for Uplink absolute power measurement term in [1] Annex F.

Proposal 1. Considering differences between Rel-15 and Rel-18 PRACH requirements it is proposed to decouple the analysis and way forward between Rel-18 Beam Correspondence for Initial access from Rel-15 PRACH test cases.

Proposal 2. To consider Rel-15 RRM Random access subtest for PRACH absolute power level testable.

# References

- TS 38.533, “User Equipment (UE) conformance specification; Radio Resource Management (RRM)”, v18.2.0 (2024-04)

- TS 38.101-2, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”, v15.25.0 (2024-04)

- TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”, v18.2.0 (2024-04)