# Title:	Test Tolerances for 5G NR UE transmit timing accuracy Tests

Source:	Qualcomm, Huawei, Hisilicon

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for RRM transmit timing accuracy tests.

The test cases in question, from TS 38.533, are:

- 4.4.1.1, EN-DC FR1 UE transmit timing accuracy

- 6.4.1.1, NR UE Transmit Timing Test for FR1

- 10.2.1.1, EN-DC FR1 UE Transmit Timing Test with PSCell under DL CCA

- 11.3.1.1, NR SA FR1 UE Transmit Timing Test with PCell under DL CCA

- 16.4.1.1, NR SA FR1 NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE

- 16.4.1.2, NR SA FR1 NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE

This document describes the process to derive the Test Tolerances for Configuration 1 of Table A.6.4.1.1.1-1. In section 4 the analysis is extended to other test Configurations and other test cases.

# 2. 	Test case in TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v15.25.0:

#### A.6.4.1.1	NR UE Transmit Timing Test for FR1

##### A.6.4.1.1.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1.2.

Supported test configurations are shown in Table A.6.4.1.1.1-1.

Table A.6.4.1.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NR TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

For this test a single NR cell is used. Table A.6.4.1.1.1-2 defines the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.6.4.1.1.1-3.

Table A.6.4.1.1.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 | Test2 |
| --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1,2,3 | 1 | 1 |
| TDD configuration |  | 1 | Not Applicable |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |
| DRx Cycle | ms | 1,2,3 | N/A | DRX.8Note5 |
| PDSCH Reference measurement channel |  | 1 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  |
| SSB configuration |  | 1,2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC Configuration |  | 1,2 | SMTC.1 |  |
|  |  | 3 | SMTC.2 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3 | -98 | -98 |
| Note2 | dBm/SCS | 1,2 | -98 | -98 |
|  |  | 3 | -95 | -95 |
|  |  | 1,2,3 | 3 | 3 |
|  |  | 1,2,3 | 3 | 3 |
| SS-RSRPNote3 | dBm/SCS | 1,2 | -95 | -95 |
|  |  | 3 | -92 | -92 |
| IoNote3 | dBm/9.36MHz | 1,2 | -65.2 | -65.2 |
|  | dBm/38.1MHz | 3 | -59.2 | -59.2 |
| Propagation condition |  | 1,2,3 | AWGN |  |
| SRS Config |  | 1,2 | SRSConf.1Note6 | SRSConf.3Note6 |
|  |  | 3 | SRSConf.1Note6 | SRSConf.2Note6 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: DRx related parameters are given in Table A.3.3.8-1 Note 6: SRS configs are given in Table A.6.4.1.1.1-3 |  |  |  |  |

Table A.6.4.1.1.1-3: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 | SRSConf.2 | SRSConf.3 | Comments |
| --- | --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 | 0 |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 | 0 |  |
|  | freqHopping c-SRS | 14 for test configuration 1,2 25 for test configuration 3 | 25 | 14 | Matches NRB,c |
|  | freqHopping b-SRS | 0 | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither | Neither |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1, 0 | sl640, 0 | sl320, 0 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | 0 | Any 10 bit number |

Table A.6.4.1.1.1-4: Void

##### A.6.4.1.1.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

1) Setup NR PCell according to parameters given in Table A.6.4.1.1.1-1.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB.

a.	The NTA offset value (in Tc units) is 25600

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1

3)	The test system shall adjust the timing of the DL path by values given in Table A.6.4.1.1.2-1

Table A.6.4.1.1.2-1: Adjustment Value for DL Timing

| SCS of SSB signals (KHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 15 | +64*64Tc | +32*64Tc |
| 30 | +32*64Tc | +16*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in clause 7.1.2 Table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first path (in time) of DL SSB used by the UE to determine downlink timing is received from the reference cell at the UE antenna.  Skip this step for test 2 with DRX configured.

5)	The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first path (in time) of DL SSB used by the UE to determine downlink timing is received from the reference cell at the UE antenna. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment

# 3. 	Calculation of Test Tolerances

The test consists of two subtests with one cell configured; the difference between the Subtests is related to DRX cycle and other parameters. Each Subtest consists of one time period. The downlink timing of Cell 1 is changed and the changes in UE transmit timing are observed. The transmit timing is verified by the UE transmitting SRS used as a measurement reference facilitating the SS timing estimation. The test requires the UE to pass in one of the supported test configurations in FR1, namely different duplex modes.

3.1	Calculations for Transmit Timing

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in TS 38.133 v15.25.0 Table 7.1.2-1:

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

The measurement uncertainty of the test system is the only additional effect to be considered. The Test tolerance for uplink signal transmit timing relative to downlink is 1.75 * 64* Tc, considered to be a reasonable uncertainty value for the Test system measurement of transmit timing. This was derived from taking 25% of the tightest UE core requirement for FR1, which is 7*64*Tc for ≥SCS for 30KHz, giving a ±1.75 * 64* Tc uncertainty. This has been reflected in Annex F measurement uncertainty definition for Uplink signal transmit timing relative to downlink: ±112 Tc = 1.75 * 64* Tc.

For Test 1 and Test 2 (10MHz Channel BW) the test requirements are:

Formula: Upper limit + TT, Lower limit – TT:

Normal conditions (±7*64*Tc ±1.75 * 64* Tc) = ± 8.75*64*Tc

These values are therefore can be used in TS 38.533 in the table for Test requirements under article 4.4.1.1.

The UE shall be capable of changing the transmission timing according to the received downlink frame. When the transmission timing error between the UE and the reference timing exceeds ±Te the UE is required to adjust its timing to within ±Te. The reference timing shall be before the downlink timing.

All adjustments made to the UE uplink timing shall follow these rules, from TS 38.133 v15.25.0 clause 7.1.2:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq.

2)	The minimum aggregate adjustment rate shall be Tp  per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200ms.

where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 7.1.2-3.

Table 7.1.2-3: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (KHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| 2 | 60 | 2.5*64*Tc | 2.5*64*Tc |
|  | 120 | 2.5*64*Tc | 2.5*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

For the time adjustment test, as well as the measurement uncertainty of the test system two additional effects need to be considered. Both the allowed UE frequency error and the UE time adjustment quantisation can affect the result of the test, and it is not possible to test the UE Transmit Timing Accuracy in isolation from these core requirements. They are therefore taken into account when setting the Test Tolerances and the Test Requirements.

The measurement uncertainty of the test system is the only additional effect to be considered. The Test tolerance for uplink signal transmit timing relative to downlink is 1.75 * 64* Tc, considered to be a reasonable uncertainty value for the Test system measurement of transmit timing. This was derived from taking 25% of the tightest UE core requirement for FR1, which is 7*64*Tc for ≥SCS ofr 30KHz, giving a ±1.75 * 64* Tc uncertainty.

Test system uncertainty

The maximum test system uncertainty for relative transmit timing during adjustment is 1.75 * 64* Tc, considered to be a reasonable uncertainty value for the Test system measurement of relative transmit timing during adjustment. This was derived from taking 25% of the tightest original UE core requirement in TS 38.133 v15.25.0 Table 7.1.2-2, which was 7*64*Tc for SCS of 60KHz, giving a 1.75 * 64* Tc uncertainty.

Allowed UE frequency error

In addition to the effect of test system uncertainty for relative transmit timing, the effect of allowed UE frequency error specified in TS 38.101 clause 6.4.1 should be taken into account.

### 6.4.1	Frequency error

The UE modulated carrier frequency shall be accurate to within ±0.1 PPM observed over a period of 1 ms compared to the carrier frequency received from the NR Node B.

The effect of allowed UE frequency error on the timing adjustment rules is calculated as follows for FDD Configuration1:

Rule 1: The maximum amount of the magnitude of the timing change in one adjustment shall be Tq (5.5*64*Tc).

As the SRS is configured to be transmitted every 1ms for FDD for configuration 1, the UE timing change from frequency error could be (0.001s x (0.1 x 10-6)) = 0.1ns. This is negligible compared to the core requirement and does not need to be considered for the rule 1 requirement.

Rule 2: The minimum aggregate adjustment rate shall be Tp (5.5*64*Tc) per second.

During 1s, the UE timing change from frequency error could be (1s x (0.1 x 10-6)) = 100ns. As Ts = 1/(15000 x 2048) s or 32.6ns, the UE timing change from frequency error could be 3.1*64*Tc for FR1.

Rule 3: The maximum aggregate adjustment rate shall be Tq (5.5*64*Tc) per 200ms.

During 200ms, the UE timing change from frequency error could be (0.2s x (0.1 x 10-6)) = 20ns. As Ts = 1/(15000 x 2048) s or 32.6ns, the UE timing change from frequency error could be ± 0.6*64*Tc for FR1.

UE time adjustment quantisation

Although not explicitly stated in TS 38.133, the UE timing adjustment is assumed to be quantised, and a quantisation step size up to Tq should be taken into account.

The effect of quantised UE timing adjustment on the timing adjustment rules is calculated as follows:

Rule 1: The maximum amount of the magnitude of the timing change in one adjustment shall be Tq seconds.

As the magnitude of the timing change in one adjustment is being directly measured, no additional Tq allowance is needed for the rule 1 requirement.

Rule 2: The minimum aggregate adjustment rate shall be Tp  per second. The minimum aggregate adjustment rate shall be 3.1*64*Tc per second for FR1.

If a time window of exactly 1s is used, and the UE makes the largest allowed timing steps with the longest allowed time between changes as shown in the diagram, the SS could catch one, two or three UE timing changes depending on the synchronisation between the SS measurement period of 1s and the UE timing change. As this part of the test aims to check the UE minimum adjustment rate, the sliding time window is increased by the minimum amount of one sample (1ms for FDD configuration 1. This ensures that the SS will not miss any timing changes occurring during the 1s time period specified in the core requirements.

The case shown in the diagram, for Configuration 1 with 10MHz channel Bandwidth, is a UE making the largest allowed timing steps of 2.75*64*Tc with the longest allowed time between changes (2 changes in 1s to meet the minimum aggregate adjustment rate of 5.5*64*Tc per second) for FR1.

Rule 3: The maximum aggregate adjustment rate shall be Tq per 200ms.

If a time window of exactly 200ms is used, and the UE makes the largest allowed timing steps with the longest allowed time between changes as shown in the diagram, the SS could catch zero, one or two UE timing changes depending on the synchronisation between the SS measurement period of 200ms and the UE timing change. As this part of the test aims to check the UE maximum adjustment rate, the sliding time window is decreased by the minimum amount of one sample (1ms for FDD configuration 1). This ensures that the SS will not include any extra timing changes occurring during the 200ms time period specified in the core requirements.

The case shown in the diagram, for FDD with 10MHz channel Bandwidth, is a UE making the largest allowed timing steps of 2.75*64*Tc with the longest allowed time between changes (1 change every 200ms to meet the minimum aggregate adjustment rate of 2.75*64*Tc per 200ms).

Combined effect of allowed UE frequency error and UE time adjustment quantisation

The diagram above shows the combined effect of allowed UE frequency error, UE time adjustment quantisation and the use of a sliding measurement window (for simplicity the test system timing measurement uncertainty is not included in the diagram).

The dashed green and blue lines show the specified maximum and minimum aggregate adjustment rates respectively for 10MHz channel bandwidth. The stepped green and blue lines show possible behaviour if the UE uses the largest possible time quantisation step, and changes the timing infrequently, which gives the worst case for measurement. Smaller or more frequent steps are also allowed.

The effect of frequency error is shown by dashed black lines, and can make the UE Tx timing earlier or later than expected. The combined effect is shown by the green and blue lines with sloping steps.

For the maximum adjustment rate measurement, the 198ms window “Position A” shows a typical measurement, where the test system would expect to see one timing quantisation step, together with a frequency error which may increase the timing change. In window “Position B” the test system would see no timing quantisation steps, but as the Test requirement is a one-sided limit this is unimportant.

For the minimum adjustment rate measurement, the 1.002s window “Position C” shows a typical measurement, where the test system would expect to see two timing quantisation steps, together with a frequency error which may decrease the timing change. In window “Position D” the test system could see three timing quantisation steps, but as the Test requirement is a one-sided limit this is unimportant.

When calculating the Test requirements, the test system uncertainty, the effect of allowed UE frequency error where relevant, and the effect of UE time adjustment quantisation where relevant, are included.

For Test 1 with Configuration 1 (FDD SCS of 15KHz) the test requirements are:

- The Maximum timing change in one adjustment Tq calculated as shown:

|  | Lower limit | Upper limit |
| --- | --- | --- |
| Core requirement | Not applicable | +2.75 * 64 * TC |
| Test system uncertainty | Not applicable | +0.5 * 64 * Tc |
| UE frequency error allowance | Not applicable | Not applicable |
| Test requirement | Not applicable | +3.25 *64 * Tc |

- The Minimum aggregate adjustment rate per second calculated as shown:

|  | Lower limit | Upper limit |
| --- | --- | --- |
| Core requirement | +5.5*64*Tc | Not applicable |
| Test system uncertainty | -0.5*64*Tc | Not applicable |
| UE frequency error allowance | -3.1*64*Tc | Not applicable |
| Test requirement | 1.9*64*Tc | Not applicable |

- The Maximum aggregate adjustment rate per 200 ms calculated as shown:

|  | Lower limit | Upper limit |
| --- | --- | --- |
| Core requirement | Not applicable | +2.75 * 64 * Tc |
| Test system uncertainty | Not applicable | +0.5 * 64 * Tc |
| UE frequency error allowance | Not applicable | +0.6 * 64 * Tc |
| Test requirement | Not applicable | +3.85 * 64 * Tc |

3.2	Downlink Signal Level Uncertainties

The SS provides 1 cell with AWGN to the UE. We propose to control the following parameters:

- AWGN absolute power, Noc1 ±1.5 dB averaged over BWConfig

- Ratio of cell 1 signal / AWGN, Ês1 / Noc1 0.3 dB averaged over BWConfig

The downlink uncertainty values are chosen from the 38.533 Annex F.

3.3	Key Signal level parameters in test case

To ensure that the SNR does not fall below the core requirement specified in Annex A of TS 38.133, the Es/Noc values are increased by the amount of the SNR uncertainty, 0.3dB. Therefore, the test requirements are:

- = 3+0.3 dB =3.3dB

- = -98 dBm/15 kHz

- = 3+0.3 dB =3.3dB

- Io1 (Test 1) = -65.2 dBm/9.36 MHz

# 4. Treatment of other test cases

TS 38.533 also contains similar test cases 16.4.1.1.1/16.4.1.1.2 for 1RX and 2 Rx RedCap UEs in clause 16.4.1. Same level of uncertainties applies to these test cases since Rel-15 legacy UE and Rel-17 RedCap UE with 1 RX and 2 Rx share the same requirements according to 38.133. Therefore, the same test tolerances for A.4.4.1.1 apply.

TS 38.533 also contains similar test cases 10.2.1.1 and 11.3.1.1, which are the equivalent version of tests 4.4.1.1 and 6.4.1.1 when testing on an unlicensed band subject to CCA. There is no additional measurement uncertainty derived from the channel being subject to CCA, therefore same level of uncertainties applies to these test cases since Rel-15 legacy UE and Rel-16 NR-U UE share the same requirements according to 38.133.