3GPP TSG-RAN5 Meeting #103	R5-243416

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | 0058 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex F updates for NTN RF test on Spectrum emission mask |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Benelux B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test Requirements for NTN SEM test are incomplete due to missing test tolerances |  |  |  |  |  |  |  |  |
| Summary of change: |  | Annex F is updated with MU and TT values and test requirements is also updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NTN SEM Test will stay incomplete due to missing MU and TT values |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.2, Annex F.1.2, Annex F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

#### 6.5.2.2	Spectrum emission mask

##### 6.5.2.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth

##### 6.5.2.2.2	Test applicability

The requirements of this test apply to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation.

<Unchanged sections skipped>

##### 6.5.2.2.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables 6.2.2.5-1, and the power of any UE emission shall fulfil requirements in Table 6.5.2.2.5-1.

Table 6.5.2.2.5-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | 5 | 10, 15, 20 |  |
| ± 0-1 | -13+TT | -13+TT | 1 % of channel BW |
| ± 1-5 | -10+TT | -10+TT | 1 MHz |
| ± 5-6 | -13+TT |  |  |
| ± 6-10 | -25+TT |  |  |
| ± 5-BWChannel |  | -13+TT |  |
| ± BWChannel-(BWChannel+5) |  | -25+TT |  |

Table 6.5.2.2.5-2: Test Tolerance (Spectrum Emission Mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 1.5 dB | 1.8 dB | 1.8 dB |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

<Unchanged sections skipped>

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.3.1 Minimum output power | Same as clause 6.3.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. |  |
| 6.3.2 Transmit OFF power | Same as clause 6.3.2 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. |  |
| 6.4.1_1 Frequency error with GSO ephemeris | Same as clause 6.4.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. |  |
| 6.4.1_2 Frequency error with NGSO ephemeris | Same as clause 6.4.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. |  |
| 6.5.2.2 Spectrum Emission Mask | Same as clause 6.5.2.2 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz.. |  |
| 6.5.3.1 General spurious emissions | Same as clause 6.5.3.1 in TS 38.521-1 [2]. |  |
| 6.5.3.2 Spurious emission for UE co-existence | Same as clause 6.5.3.2 in TS 38.521-1 [2]. |  |
| 6.5.3.3 Additional spurious emissions | Same as clause 6.5.3.3 in TS 38.521-1 [2]. |  |
| 6.5.4 Transmit intermodulation | Same as clause 6.5.4 in TS 38.521-1 [2]. |  |

<Unchanged sections skipped>

## F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.3.1 Minimum output power | Same as clause 6.3.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. | Minimum requirement + TT |
| 6.3.2 Transmit OFF power | Same as clause 6.3.2 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. | Minimum requirement + TT |
| 6.4.1_1 Frequency error with GSO ephemeris | Same as clause 6.4.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. | Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: REFSENS + TT |
| 6.4.1_2 Frequency error with NGSO ephemeris | Same as clause 6.4.1 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz. | Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: REFSENS + TT |
| 6.5.2.2 Spectrum Emission Mask | Same as clause 6.5.2.2 in TS 38.521-1 [2] for FDD band with f ≤ 3 GHz.. | Minimum requirement + TT |
| 6.5.3.1 General spurious emissions | Same as clause 6.5.3.1 in TS 38.521-1 [2]. | Minimum requirement + TT |
| 6.5.3.2 Spurious emission for UE co-existence | Same as clause 6.5.3.2 in TS 38.521-1 [2]. | Minimum requirement + TT |
| 6.5.3.3 Additional spurious emissions | Same as clause 6.5.3.3 in TS 38.521-1 [2]. | Minimum requirement + TT |
| 6.5.4 Transmit intermodulation | Same as clause 6.5.4 in TS 38.521-1 [2]. | Minimum requirement + TT |

<End of changes>