3GPP TSG-RAN5 Meeting #103	R5-243436

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0847 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Table G.1.5-1 for entries corresponding to FR1.15-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Incorrect MNAS to MNS scaling factor is used in Table G.1.5-1 for PDSCH reference channels from Tables A.3.2.2.1-2 and A.3.2.2.1-3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the MNAS to MNS scaling factor in Table G.1.5-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MNS value in Table G.1.5-1 will be unnecesarily longer resulting in longer test time |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

### A.3.2.2	TDD

#### A.3.2.2.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.2.2.1-1: PDSCH Reference Channel for TDD UL-DL pattern FR1.15-1 and LTE-NR coexistence scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-1.1 TDD | R.PDSCH.1-1.2 TDD | R.PDSCH.1-1.3 TDD |  |  |
| Channel bandwidth | MHz | 10 | 10 | 20 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |
| Allocated resource blocks | PRBs | 52 | 52 | 106 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} |  | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} |  | 9 | 11 | 9 |  |  |
| Allocated slots per 2 frames |  | 7 | 7 | 7 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 4 | 4 | 13 |  |  |
| Modulation |  | QPSK | QPSK | 16QAM |  |  |
| Target Coding Rate |  | 0.30 | 0.30 | 0.48 |  |  |
| Number of MIMO layers |  | 1 | 1 | 1 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} |  | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} |  | 12 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 18 | 18 | 18 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | Bits | 2472 | 3240 | 15880 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | Bits | 16 | 16 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | CBs | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | CBs | 1 | 1 | 2 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 10, 11 | Bits | 7760 | 10256 | 32672 |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,9,12,…,19} | Bits | 8384 | 10880 | 33920 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 0.865 | 1.134 | 5.558 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: No user data is scheduled on slots with LTE PBCH/PSS/SSS |  |  |  |  |  |  |

Table A.3.2.2.1-2: PDSCH Reference Channel for TDD CC with UL-DL pattern FR1.15-1 and CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-2.1 TDD | R.PDSCH.1-2.2 TDD | R.PDSCH.1-2.3 TDD | R.PDSCH.1-2.4 TDD | R.PDSCH.1-2.5 TDD |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 25 | 52 | 79 | 106 | 133 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 8 | 8 | 8 | 8 | 8 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 15 | 15 | 15 | 15 | 15 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 13 | 13 | 13 | 13 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 | 0.48 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 12 | 12 | 12 | 12 | 12 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 8064 | 16896 | 25608 | 33816 | 43032 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 12552 | 26120 | 39936 | 53288 | 67584 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 24 | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | CBs | 1 | 3 | 4 | 5 | 6 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | CBs | 2 | 4 | 5 | 7 | 9 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 10, 11 | Bits | 25200 | 52416 | 79632 | 106848 | 134064 |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 16800 | 34944 | 53088 | 71232 | 89376 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,9,12,…,19} | Bits | 26400 | 54912 | 83424 | 111936 | 140448 |
| Max. Throughput averaged over 2 frames | Mbps | 8.516 | 17.745 | 27.086 | 36.072 | 45.778 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.1-3: PDSCH Reference Channel for TDD CC with UL-DL pattern FR1.15-1 and CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-3.1 TDD | R.PDSCH.1-3.2 TDD | R.PDSCH.1-3.3 TDD |  |  |
| Channel bandwidth | MHz | 30 | 40 | 50 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |
| Number of allocated resource blocks | PRBs | 160 | 216 | 270 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 8 | 8 | 8 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 | 12 |  |  |
| Allocated slots per 2 frames | Slots | 15 | 15 | 15 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 13 | 13 | 13 |  |  |
| Modulation |  | 16QAM | 16QAM | 16QAM |  |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 |  |  |
| Number of MIMO layers |  | 2 | 2 | 2 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 12 | 12 | 12 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 51216 | 69672 | 86040 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 79896 | 108552 | 135296 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 24 | 24 | 24 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | CBs | 7 | 9 | 11 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | CBs | 10 | 13 | 17 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 10, 11 | Bits | 161280 | 217728 | 272160 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 107520 | 145152 | 181440 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,9,12,…,19} | Bits | 168960 | 228096 | 285120 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 54.186 | 73.638 | 91.621 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<< Skipped sections >>

## G.1.5	Minimum Test time

<< Skipped sections >>

Table G.1.5-1: Minimum Test time for PDSCH demodulation

| TDD UL-DL pattern |  |  | Reference Channel |  |  | Propagation condition |  |  | Demodulation scenario (doppler speed) |  |  | Minimum number of active subframes (MNAS) |  |  | MNAS to MNS Scaling factor (Note 3) |  |  | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NA |  |  | R.PDSCH.1-8.1 FDD |  |  | HST-750 |  |  | 750 Hz |  |  | 6000 (Note 1) |  |  | 1.0526 |  |  | 6400 |  |  |
| NA |  |  | R.PDSCH.1-8.2 FDD |  |  | HST-972 |  |  | 972 Hz |  |  | 6000 (Note 1) |  |  | 1.0526 |  |  | 6400 |  |  |
| NA |  |  | R.PDSCH.1-8.1 FDD |  |  | TDLC300-600 |  |  | 600 Hz |  |  | 8000 (Note 1) |  |  | 1.0526 |  |  | 9000 |  |  |
| NA |  |  | R.PDSCH.1-1.1 FDD |  |  | TDLB100-400 |  |  | 400 Hz |  |  | 10000 (Note 1) |  |  | 1.0526 |  |  | 11000 |  |  |
| NA |  |  | R.PDSCH.1-1.1 HD-FDD |  |  | TDLB100-400 |  |  | 400 Hz |  |  | 10000 (Note 1) |  |  | 1.3333 |  |  | 14000 |  |  |
| NA |  |  | R.PDSCH.1-1.2 FDD, R.PDSCH.1-2.1 FDD, R.PDSCH.1-5.1 FDD, R.PDSCH.5-1.1 FDD |  |  | TDLC300-100 |  |  | 100 Hz |  |  | 20000 (Note 1) |  |  | 1.0526 |  |  | 22000 |  |  |
| NA |  |  | R.PDSCH.1-1.2 HD-FDD |  |  | TDLC300-100 |  |  | 100 Hz |  |  | 20000 (Note 1) |  |  | 1.3333 |  |  | 27000 |  |  |
| NA |  |  | R.PDSCH.1-1.3 FDD, R.PDSCH.1-2.2 FDD, R.PDSCH.1-2.3 FDD, R.PDSCH.1-2.4 FDD, R.PDSCH.1-2.5 FDD, R.PDSCH.1-3.1 FDD, R.PDSCH.1-3.2 FDD, R.PDSCH.1-3.3 FDD, R.PDSCH.1-3.4 FDD, R.PDSCH.1-4.1 FDD, R.PDSCH.1-4.2 FDD, R.PDSCH.1-12.1 FDD, R.PDSCH.2-1.1 FDD, R.PDSCH.5-1.2 FDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.0526 |  |  | 79000 |  |  |
| NA |  |  | R.PDSCH.1-1.3 HD-FDD, R.PDSCH.1-1.4 HD-FDD, R.PDSCH.1-1.5 HD-FDD, R.PDSCH.1-2.1 HD-FDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.3333 |  |  | 100000 |  |  |
| NA |  |  | R.PDSCH.1-7.1 FDD, R.PDSCH.1-7.2 FDD R.PDSCH.1-18.1 FDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.25 |  |  | 94000 |  |  |
| NA |  |  | R.PDSCH.1-17.1 FDD |  |  | TDLD30-5 |  |  | 5 Hz |  |  | 70000 (Note 1) |  |  | 1.0526 |  |  | 74000 |  |  |
| NA |  |  | R.PDSCH.1-8.3 FDD, R.PDSCH.1-13.1 FDD, R.PDSCH.1-13.2 FDD, R.PDSCH.1-13.3 FDD, R.PDSCH.1-13.4 FDD, R.PDSCH.1-13.5 FDD, R.PDSCH.1-14.1 FDD, R.PDSCH.1-14.2 FDD, R.PDSCH.1-14.3 FDD, R.PDSCH.1-14.4 FDD |  |  | HST-SFN |  |  | 870 Hz |  |  | 30000 (Note 1) |  |  | 1.0526 |  |  | 32000 |  |  |
| NA |  |  | R.PDSCH.1-8.5 FDD |  |  | HST-SFN Scheme A HST-SFN Scheme B |  |  | 870 Hz |  |  | [30000 (Note 1)] |  |  | 1.0526 |  |  | [32000] |  |  |
| NA |  |  | R.PDSCH.1-8.4 FDD, R.PDSCH.1-15.1 FDD, R.PDSCH.1-15.2 FDD, R.PDSCH.1-15.3 FDD, R.PDSCH.1-15.4 FDD, R.PDSCH.1-15.5 FDD, R.PDSCH.1-16.1 FDD, R.PDSCH.1-16.2 FDD, R.PDSCH.1-16.3 FDD, R.PDSCH.1-16.4 FDD |  |  | HST-DPS |  |  | 870 Hz |  |  | 30000 (Note 1) |  |  | 1.0526 |  |  | 32000 |  |  |
| NA |  |  | R.PDSCH.1-9.1 FDD, R.PDSCH.1-9.2 FDD, R.PDSCH.1-9.3 FDD, R.PDSCH.1-9.4 FDD, R.PDSCH.1-9.5 FDD, R.PDSCH.1-10.1 FDD, R.PDSCH.1-10.2 FDD |  |  | TDLA30-10 |  |  | 10Hz |  |  | 75000 (Note 1) |  |  | 1.0526 |  |  | 79000 |  |  |
| FR1.15-1 |  |  | R.PDSCH.1-1.1 TDD, R.PDSCH.1-1.2 TDD |  |  | TDLA30-10 |  |  | 10Hz |  |  | 75000 (Note 1) |  |  | 2.8571 |  |  | 215000 |  |  |
| FR1.15-1 |  |  | R.PDSCH.1-2.1 TDD, R.PDSCH.1-2.2 TDD, R.PDSCH.1-2.3 TDD, R.PDSCH.1-2.4 TDD, R.PDSCH.1-2.5 TDD, R.PDSCH.1-3.1 TDD, R.PDSCH.1-3.2 TDD, R.PDSCH.1-3.3 TDD |  |  | TDLA30-10 |  |  | 10Hz |  |  | 75000 (Note 1) |  |  | 1.3333 |  |  | 100000 |  |  |
| FR1.30-1A |  |  | R.PDSCH.2-1.1 TDD, R.PDSCH.2-1.5 TDD |  |  | TDLB100-400 |  |  | 400 Hz |  |  | 10000 (Note 1) |  |  | 1.2903 |  |  | 13000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-1.2 TDD, R.PDSCH.2-2.1 TDD, R.PDSCH.2-7.1 TDD, R.PDSCH.2-26.1 TDD, R.PDSCH.7-1.1 TDD |  |  | TDLC300-100 |  |  | 100 Hz |  |  | 20000 (Note 1) |  |  | 1.2903 |  |  | 26000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-2.2 TDD, R.PDSCH.2-2.3 TDD, R.PDSCH.2-2.4 TDD, R.PDSCH.2-2.5 TDD, R.PDSCH.2-3.1 TDD, R.PDSCH.2-3.2 TDD, R.PDSCH.2-3.3 TDD, R.PDSCH.2-3.4 TDD, R.PDSCH.2-3.5 TDD, R.PDSCH.2-4.1 TDD, R.PDSCH.2-4.3 TDD, R.PDSCH.7-1.2 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.2903 |  |  | 97000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-1.3 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.4815 |  |  | 112000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-25.1 TDD |  |  | TDLD30-5 |  |  | 5 Hz |  |  | 70000 (Note 1) |  |  | 1.2903 |  |  | 91000 |  |  |
| FR1.30-2 |  |  | R.PDSCH.2-5.1 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.2903 |  |  | 97000 |  |  |
| FR1.30-2 |  |  | R.PDSCH.2-17.1 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 5 |  |  | 375000 |  |  |
| FR1.30-3 |  |  | R.PDSCH.2-6.1 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.4815 |  |  | 112000 |  |  |
| FR1.30-4 |  |  | R.PDSCH.2-9.1 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.2903 |  |  | 97000 |  |  |
| FR1.30-5 |  |  | R.PDSCH.2-11.1 TDD |  |  | TDLB100-400 |  |  | 400Hz |  |  | 10000 (Note 1) |  |  | 1.2903 |  |  | 13000 |  |  |
| FR1.30-6 |  |  | R.PDSCH.2-12.1 TDD |  |  | TDLB100-400 |  |  | 400Hz |  |  | 10000 (Note 1) |  |  | 1.2903 |  |  | 13000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-10.1 TDD |  |  | HST-1000 |  |  | 1000 Hz |  |  | 15000 (Note 1) |  |  | 1.4815 |  |  | 23000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-10.1 TDD |  |  | HST-1667 |  |  | 1667 Hz |  |  | 15000 (Note 1) |  |  | 1.4815 |  |  | 23000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-10.4 TDD, R.PDSCH.2-19.1 TDD, R.PDSCH.2-19.2 TDD, R.PDSCH.2-19.3 TDD, R.PDSCH.2-19.4 TDD, R.PDSCH.2-19.5 TDD, R.PDSCH.2-20.1 TDD, R.PDSCH.2-20.2 TDD, R.PDSCH.2-20.3 TDD, R.PDSCH.2-20.4 TDD, R.PDSCH.2-20.5 TDD, R.PDSCH.2-21.1 TDD |  |  | HST-SFN |  |  | 1667 Hz |  |  | 30000 (Note 1) |  |  | 1.4815 |  |  | 45000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-10.5 TDD, R.PDSCH.2-22.1 TDD, R.PDSCH.2-22.2 TDD, R.PDSCH.2-22.3 TDD, R.PDSCH.2-22.4 TDD, R.PDSCH.2-22.5 TDD, R.PDSCH.2-23.1 TDD, R.PDSCH.2-23.2 TDD, R.PDSCH.2-23.3 TDD, R.PDSCH.2-23.4 TDD, R.PDSCH.2-23.5 TDD, R.PDSCH.2-24.1 TDD |  |  | HST-DPS |  |  | 1667 Hz |  |  | 30000 (Note 1) |  |  | 1.4815 |  |  | 45000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-30.1 TDD |  |  | HST-SFN Scheme A HST-SFN Scheme B |  |  | 1667 Hz |  |  | [30000 (Note 1)] |  |  | 1.4815 |  |  | [45000] |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-13.1 TDD, R.PDSCH.2-13.2 TDD, R.PDSCH.2-13.3 TDD, R.PDSCH.2-13.4 TDD, R.PDSCH.2-13.5 TDD, R.PDSCH.2-14.1 TDD, R.PDSCH.2-14.2 TDD, R.PDSCH.2-14.3 TDD, R.PDSCH.2-14.4 TDD, R.PDSCH.2-14.5 TDD, R.PDSCH.2-15.1 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.2903 |  |  | 97000 |  |  |
| FR1.30-7 |  |  | R.PDSCH.2-18.1 TDD, R.PDSCH.2-18.2 TDD, R.PDSCH.2-18.3 TDD, R.PDSCH.2-18.4 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 2) |  |  | 1.6129 (Note 4) |  |  | [121000] |  |  |
| FR2.60-1 |  |  | R.PDSCH.4-1.1 TDD |  |  | TDLA30-75 |  |  | 75 Hz |  |  | 20000 (Note 2) |  |  | 1.33 |  |  | 27000 |  |  |
| FR2.120-1A |  |  | R.PDSCH.5-1.1 TDD |  |  | TDLC60-300 |  |  | 300 Hz |  |  | 10000 (Note 2) |  |  | 1.25 |  |  | 13000 |  |  |
| FR2.120-1 |  |  | R.PDSCH.5-2.1 TDD, R.PDSCH.5-2.2 TDD, R.PDSCH.5-2.3 TDD, R.PDSCH.5-3.1 TDD |  |  | TDLA30-300 |  |  | 300 Hz |  |  | 10000 (Note 2) |  |  | 1.25 |  |  | 13000 |  |  |
| FR2.120-1 |  |  | R.PDSCH.5-1.2 TDD |  |  | TDLA30-75 |  |  | 75 Hz |  |  | 20000 (Note 2) |  |  | 1.25 |  |  | 25000 |  |  |
| FR2.120-2 |  |  | R.PDSCH.5-4.1 TDD, R.PDSCH.5-5.1 TDD, R.PDSCH.5-5.2 TDD, R.PDSCH.5-6.1 TDD |  |  | TDLA30-75 |  |  | 75 Hz |  |  | 20000 (Note 2) |  |  | 1.33 |  |  | 27000 |  |  |
| FR2.120-1 |  |  | R.PDSCH.5-10.1 TDD |  |  | TDLD30-75 |  |  | 75 Hz |  |  | 20000 (Note 2) |  |  | 1.26 |  |  | 26000 |  |  |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNAS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots. Note 4: To account for the LBT failure probability of 0.25, MNS scaled up by 25% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< End of Changes >>