3GPP TSG-RAN5 Meeting #103	R5-243468

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5299 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 8.1.3.6b Redirection to UTRAN-Redir-policy bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone GmbH, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2024-04-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case 8.1.3.6b RRC connection release / Redirection from E-UTRAN to UTRAN / redir-policy bit includes wrong 3GPP release 17 and was corrected to Rel-18, according to referred CT1 and RAN2 CR’s. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Release of test case 8.1.3.6b RRC connection release / Redirection from E-UTRAN to UTRAN / redir-policy bit has been corrected to Rel-18 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage for redir-policy bit remains incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.6b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1447 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | MCC TF160 added as co-sourcing company |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 8.1.3.6b	RRC connection release / Redirection from E-UTRAN to UTRAN / Redir-policy bit

8.1.3.6b.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionRelease message including an IE redirectedCarrierInfo with utra-CarrierFreq UTRA frequency and upper layers indicate that redirect to UTRAN without AS security is not allowed }

then { UE discards redirection to UTRAN }

}

8.1.3.6b.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.1.2.4 and 5.5.3.2.4, TS 36.331, clause 5.3.8.3 and 5.3.3.4b. Unless otherwise stated these are Rel-18 requirements.

[TS 24.301, clause 5.5.1.2.4]

…

If the redir-policy bit is set to "Unsecured redirection to GERAN or UTRAN not allowed" in the Network policy IE of the ATTACH ACCEPT message, the UE shall set the network policy on unsecured redirection to GERAN or UTRAN for the current PLMN to "Unsecured redirection to GERAN or UTRAN not allowed" and indicate to the lower layers that unsecured redirection to a GERAN or UTRAN cell is not allowed. If the redir-policy bit is set to "Unsecured redirection to GERAN or UTRAN allowed" or if the Network policy IE is not included in the ATTACH ACCEPT message, the UE shall set the network policy for the current PLMN to "Unsecured redirection to GERAN or UTRAN allowed" and indicate to the lower layers that unsecured redirection to a GERAN or UTRAN cell is allowed. The UE shall set the network policy on unsecured redirection to GERAN or UTRAN to "Unsecured redirection to GERAN or UTRAN not allowed" and indicate this to the lower layers when any of the following events occurs:

-	the UE initiates an EPS attach or tracking area updating procedure in a PLMN different from the PLMN where the UE performed the last successful EPS attach or tracking area updating procedure;

-	the UE is switched on; or

-	the UICC containing the USIM is removed.

…

[TS 24.301, clause 5.5.3.2.4]

…

If the redir-policy bit is set to "Unsecured redirection to GERAN or UTRAN not allowed" in the Network policy IE of the TRACKING AREA UPDATE ACCEPT message, the UE shall set the network policy on unsecured redirection to GERAN for the current PLMN to "Unsecured redirection to GERAN or UTRAN not allowed" and indicate to the lower layers that unsecured redirection to a GERAN or UTRAN cell is not allowed. If the redir-policy bit is set to "Unsecured redirection to GERAN or UTRAN allowed" or if the Network policy IE is not included in the TRACKING AREA UPDATE ACCEPT message, the UE shall set the network policy on unsecured redirection to GERAN or UTRAN for the current PLMN to "Unsecured redirection to GERAN or UTRAN allowed" and indicate to the lower layers that unsecured redirection to a GERAN or UTRAN cell is allowed. The UE shall set the network policy on unsecured redirection to GERAN or UTRAN to "Unsecured redirection to GERAN or UTRAN not allowed" and indicate this to the lower layers when any of the following events occurs:

-	the UE initiates an EPS attach or tracking area updating procedure in a PLMN different from the PLMN where the UE performed the last successful EPS attach or tracking area updating procedure;

-	the UE is switched on; or

-	the UICC containing the USIM is removed.

…

[TS 36.331, clause 5.3.8.3]

The UE shall:

…

1>	if the RRCConnectionRelease message includes redirectedCarrierInfo indicating redirection to geran, utra-FDD, utra-TDD or utra-TDD-r10;  or

1>	if the RRCConnectionRelease message includes idleModeMobilityControlInfo including freqPriorityListGERAN or freqPriorityListUTRA-FDD or freqPriorityListUTRA-TDD:

2>	if AS security has not been activated; and

2>	if upper layers indicate that redirect to GERAN or UTRAN without AS security is not allowed (see TS 24.301):

3>	ignore the content of the RRCConnectionRelease;

3>	perform the actions upon leaving RRC_CONNECTED or RRC_INACTIVE as specified in 5.3.12, with release cause 'other', upon which the procedure ends;

…

[TS 36.331, clause 5.3.3.4b]

The UE shall:

…

1>	if the RRCEarlyDataComplete message includes redirectedCarrierInfo indicating redirection to geran, utra-FDD or utra-TDD and

1>	if upper layers indicate that redirect to GERAN or UTRAN without AS security is not allowed:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other', upon which the procedure ends;

…

8.1.3.6b.3	Test description

8.1.3.6b.3.1	Pre-test conditions

System Simulator:

-	2 cells, one E-UTRA and one UTRA cell and different location areas:

-	Cell 1 E-UTRA serving cell

-	Cell 5 suitable neighbour UTRA cell

-	Cell power levels are selected according to [18] so that camping on Cell 1 is guaranteed

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) according to [18] and Redir policy set to ‘Unsecured redirection to GERAN or UTRAN not allowed’.

8.1.3.6b.3.2	Test procedure sequence

Table 8.1.3.6b.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-4 | Steps 2-5 of the generic procedure for Generic Radio Bearer establishment specified in TS 36.508 [18], subclause 4.5.3.3 in Cell 1 take place. |  |  | - | - |
| 5 | SS transmits an RRCConnectionRelease message (IE redirectedCarrierInfo including UTRA-CarrierFreq of Cell 5). | <-- | RRCConnectionRelease | - | - |
| 6 | Check: Does the UE transmit a RRC CONNECTION REQUEST message on Cell 5 within 5s from step 5?. | --> | RRC: RRC CONNECTION REQUEST | 1 | F |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 [18] subclause 6.4.2.2 indicate that the UE is in RRC IDLE on E-UTRAN cell 1? | - | - | 1 | - |

8.1.3.6b.3.3	Specific message or IE contents

Table 8.1.3.6b.3.3-1: SystemInformationBlockType6 for cell 1 (preamble and all steps, Table 8.1.3.6b.3.2-1)

| Derivation Path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 3 | Lower priority than E-UTRA |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 3 | Lower priority than E-UTRA |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.1.3.6b.3.3-2: Message ATTACH ACCEPT (preamble)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/Remark |  | Comment | Condition |
| Network policy |  |  |  |  |  |
| Redir-policy | 0001’B |  | Unsecured redirection to GERAN or UTRAN not allowed |  |  |

Table 8.1.3.6b.3.3-3: RRCConnectionRelease message (step 5, Table 8.1.3.6b.3.2-1)

| Derivation Path: 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellInfoList-r9 ::= CHOICE {} | Not present | redirection with SIB is optional |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.1.3.6a.3.3-4: System Information Block type 19 for Cell 5 (preamble and all steps, Table 8.1.3.6a.3.2-1)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| Utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>