3GPP TSG-RAN5 Meeting #103	R5-243475

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-3 | CR | 3679 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of cl 8.5.4.7 Inter RAT handover to UTRAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160, ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2024-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current Prose, there is no clause that outlines the handling of case where ciphering has been activated with EEA0 in Inter-RAT handover to UTRAN. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced a new section 8.5.4.7.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of specification will remain |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.7.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.523-1 CR#5310 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Associated with R5s240281 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.5.4.7	Inter RAT handover to UTRAN

##### 8.5.4.7.1	ciphering has not been activated

ciphering has not been started in the radio access technology from which inter RAT handover is performed. TM mode radio bearer will be established in the UTRAN.

1.	Sending HANDOVER TO UTRAN COMMAND in a RAT different from UTRAN

2.	After receiving HANDOVER TO UTRAN COMPLETE message

CMAC_SecurityMode_Config_REQ

startValue = value received in HANDOVER TO UTRAN COMPLETE message

cn_DomainIdentity = CS or PS

CMAC_Ciphering_Activate_REQ (CN domain concerned)

incHFN = NotInc

cipheringModeCommand = NULL

activationTimeForDPCH = now

CRLC_SecurityMode_Config_REQ

startValue = value received in HANDOVER TO UTRAN COMPLETE

cn_DomainIdentity = CS or PS

CRLC_Ciphering_Activate_REQ (CN domain concerned)

cipheringModeCommand = NULL

rb_DL_CiphActivationTimeInfo = now (RB1, RB2, RB3, RB4)

incHFN = Inc	CRLC_Ciphering_Activate_REQ (CN domain concerned)

rb_UL_CipheringActivationTimeInfo = now (RB1, RB2, RB3, RB4)

incHFN = Inc

##### 8.5.4.7.2	ciphering has been activated

ciphering has been started in the radio access technology from which inter RAT handover is performed. TM mode radio bearer will be established in the UTRAN.

1.	Before sending HANDOVER TO UTRAN COMMAND

CRLC_SecurityMode_Config_REQ

startValue = "START" value included in the IE "UE security information" in the variable "INTER_RAT_HANDOVER_INFO_TRANSFERRED"

cipheringKey = value generated in authentication procedure in GRAN

cn_DomainIdentity = CS or PS

CRLC_Ciphering_Activate_REQ (CN domain concerned)

cipheringModeCommand = Start/Restart (algorithm in HANDOVER TO UTRAN COMMAND)

rb_DL_CiphActivationTimeInfo = now (RB1, RB2, RB3, RB4)

incHFN = NotInc

CRLC_Ciphering_Activate_REQ (CN domain concerned)

rb_UL_CipheringActivationTimeInfo = now (RB1, RB2, RB3, RB4)

incHFN = NotInc

CMAC_SecurityMode_Config_REQ

startValue = "START" value included in the IE "UE security information" in the variable "INTER_RAT_HANDOVER_INFO_TRANSFERRED"

cipheringKey = value generated in authentication procedure in GRAN

cn_DomainIdentity = CS or PS

CMAC_Ciphering_Activate_REQ (CN domain concerned)

incHFN = NotInc

cipheringModeCommand = Start/Restart (algorithm in HANDOVER TO UTRAN COMMAND)

activationTimeForDPCH = now

2.	Sending HANDOVER TO UTRAN COMMAND in a RAT different from UTRAN

3.	After receiving HANDOVER TO UTRAN COMPLETE message

CMAC_SecurityMode_Config_REQ

startValue = value received in the response message

cipheringKey = value maintained by TTCN

cn_DomainIdentity = CS or PS

CMAC_Ciphering_Activate_REQ (CN domain concerned)

cipheringModeCommand = Start/Restart (algorithm) in HANDOVER TO UTRAN COMMAND)

activationTimeForDPCH = value in "COUNT-C activation time"

incHFN = IncByOne_IncPerCFN_Cycle

CRLC_SecurityMode_Config_REQ

startValue = value received in HANDOVER TO UTRAN COMPLETE

cipheringKey = value generated in authentication procedure in GRAN

cn_DomainIdentity = CS or PS

CRLC_Ciphering_Activate_REQ (CN domain concerned)

cipheringModeCommand = Start/Restart (algorithm in HANDOVER TO UTRAN COMMAND)

rb_DL_CiphActivationTimeInfo = now (RB1, RB2, RB3, RB4)

incHFN = Inc

CRLC_Ciphering_Activate_REQ (CN domain concerned)

rb_UL_CipheringActivationTimeInfo = now (RB1, RB2, RB3, RB4)

incHFN = Inc

##### 8.5.4.7.3	ciphering has been activated with EEA0

ciphering has been started in the radio access technology with EEA0 from which inter RAT handover is performed. TM mode radio bearer will be established in the UTRAN with ciphering off.