3GPP TSG-RAN5 Meeting #103	R5-243484

# Fukuoka, Japan, 20th May 2024 – 24th May 2024

Source:	Huawei, HiSilicon,

Title:	Discussion to add new RedCap HD-FDD TC

Agenda item:	6.4.7

Document for:	Endorsement

Discussion

Observation 1: Some requirement related to HD-FDD RedCap UE highlight below haven’t been tested.

[TS 38.331 clause 5.2.2.4.2]

Upon receiving the SIB1 the UE shall:

…

1>	if the UE is a RedCap UE and it is in RRC_IDLE or in RRC_INACTIVE, or if the RedCap UE is in RRC_CONNECTED while T311 is running:

2>	if intraFreqReselectionRedCap is not present in SIB1:

…

2> else:

…

3>	if the halfDuplexRedCapAllowed is not present in the acquired SIB1 and the UE supports only half-duplex FDD operation:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform barring based on intraFreqReselectionRedCap as specified in TS 38.304 [20], upon which the procedure ends;

…

1>	if in RRC_CONNECTED while T311 is not running:

…

1>	else:

…

2>	if the UE is not a RedCap UE, or for TDD if the UE is a RedCap UE, or for FDD if the UE is a RedCap UE and halfDuplexRedCapAllowed is present, or if the UE is a RedCap UE and the RedCap UE supports full-duplex FDD operation on this band:

3>	if neither trackingAreaCode nor trackingAreaList is provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];

3>	else if UE is IAB-MT and if iab-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list nor the selected SNPN nor the registered SNPN:

4>	consider the cell as barred in accordance with TS 38.304 [20];

3>	else:

…

4>	select the first frequency band in the frequencyBandList, for FDD from frequencyBandList for uplink, or for TDD from frequencyBandList for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present, and for RedCap UEs in FDD, if the halfDuplexRedCapAllowed is not present, for which the UE supports full-duplex FDD operation;

…

2>	else:

3>	consider the cell as barred in accordance with TS 38.304 [20]; and

3>	perform barring as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, is set to notAllowed;

Proposal

Based on the above discussion, the followings are proposed:

Proposal 1: Add new TC for HD-FDD RedCap to enhance the test coverage:

- If UE support half-duplex on one FDD band, RedCap UE will bar the Cell which does not allow half-duplex RedCap UE in this FDD band.

- MFBI Cell broadcast more than one bands. If UE support half-duplex on one FDD band but support full-duplex on the other FDD  band, RedCap UE can access to MFBI Cell which does not allow half-duplex RedCap UE

If this proposal is agreed, following new test case can be added.

6.1.2.29	Cell Selection / RedCap / halfDuplexRedCapAllowed / MFBI

# test case:

R5-243585

#Test applicability:

R5-243586

#new pics:

R5-243495