3GPP TSG-RAN5 Meeting #103	R5-243516

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4385 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for MBS TC 14.2.4.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The reference tables of Table 14.2.4.2.1.3.3-14 and Table 14.2.4.2.1.3.3-19 are incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The reference tables of Table 14.2.4.2.1.3.3-14 and Table 14.2.4.2.1.3.3-19 have been corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The error will exist in the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-242594r1 |  |  |  |  |  |  |  |  |

14.2.4.2.1	MBS Multicast / RRC / MRB Reconfiguration / Establishment / Modification / Release / Success

14.2.4.2.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state and UE has joined one MBS multicast session }

ensure that {

when { UE receives a Paging with matched TMGI and moves to RRC_CONNECTED state and receives RRCReconfiguration message including a MRB-ToAddModList to add an UM MRB with one UM RLC for PTM transmission as MRB-Identity 1 and an AM MRB with one AM RLC for PTP transmission as MRB-Identity 2 }

then { UE successfully establishes the MRBs }

}

(2)

with { UE in NR RRC_CONNECTED state and MRB-Identity 1 and MRB-Identity 2 have established }

ensure that {

when { UE receives RRCReconfiguration message including a MRB-ToReleaseList to release a MRB-Identity which is not part of current UE configuration }

then { UE does not consider the message as erroneous and replies RRCReconfigurationComplete }

}

(3)

with { UE in NR RRC_CONNECTED state and MRB-Identity 1 and MRB-Identity 2 have established }

ensure that {

when { UE receives RRCReconfiguration message including a MRB-ToReleaseList to release MRB-Identity 2 and including rlc-BearerToaddModList to modify MRB-Identity 1 by adding a RLC-AM for PTP transmission }

then { UE releases PDCP entity for MRB-Identity 2 and MRB-Identity 2 and changes MRB-Identity 1 to AM MRB with one UM RLC for PTM transmission and one AM RLC for PTP transmission and could receives MBS data of MRB-Identity 1 with PTM transmission and PTP transmission }

}

(4)

with { UE in NR RRC_CONNECTED state and MRB-Identity 1 have established }

ensure that {

when { UE receives RRCReconfiguration message including a MRB-ToAddModList to update the MRB-Identity 1 to the MRB-Identity 3 by setting the value of MRB-IdentityNew to 3 }

then { UE updates the MRB-Identity to the value MRB-IdentityNew }

}

(5)

with { UE in NR RRC_CONNECTED state and MRB(s) have established }

ensure that {

when { UE receives RRCReconfiguration message including a MRB-ToReleaseList to release all the MRB configured with same mbs-SessionId }

then { UE releases PDCP entity and the MRB-Identity and indicate release of the user plane resources for mbs-SessionId to upper layers }

}

14.2.4.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clause 5.3.5.5.4, 5.3.5.6.6 and 5.3.5.6.7. Unless otherwise stated these are Rel-17 requirements.

[TS 38. 331, clause 5.3.5.5.4]

For each RLC-BearerConfig received in the rlc-BearerToAddModList IE the UE shall:

1>	if the UE's current configuration contains an RLC bearer with the received logicalChannelIdentity/LogicalChannelIdentityExt within the same cell group:

2>	if the RLC bearer is associated with an DAPS bearer, or

2>	if any DAPS bearer is configured and the RLC bearer is associated with an SRB:

…

2>	else:

3>	if reestablishRLC is received:

4>	re-establish the RLC entity as specified in TS 38.322 [4];

3>	reconfigure the RLC entity or entities in accordance with the received rlc-Config;

3>	reconfigure the logical channel in accordance with the received mac-LogicalChannelConfig;

3>	if servedMBS-RadioBearer is received:

4>	associate this logical channel with the PDCP entity identified by servedMBS-RadioBearer;

NOTE 1:	For DRB and SRB, the network does not re-associate an already configured logical channel with another radio bearer. Hence servedRadioBearer is not present in this case. For MRB, the network does not re-associate an already configured logical channel with DRB or SRB or another MRB (i.e. MRB with another PDCP entity). Hence multicastRLC-BearerConfig is not present in this case.

…

1>	else (a logical channel with the given logicalChannelIdentity/LogicalChannelIdentityExt is not configured within the same cell group, including the case when full configuration option is used):

2>	if the servedRadioBearer associates the logical channel with an SRB and rlc-Config is not included:

3>	establish an RLC entity in accordance with the default configuration defined in 9.2 for the corresponding SRB;

2>	else:

3>	establish an RLC entity in accordance with the received rlc-Config;

2>	if the servedRadioBearer associates the logical channel with an SRB and if mac-LogicalChannelConfig is not included:

3>	configure this MAC entity with a logical channel in accordance to the default configuration defined in 9.2 for the corresponding SRB;

2>	else:

3>	configure this MAC entity with a logical channel in accordance to the received mac-LogicalChannelConfig;

2>	associate this logical channel with the PDCP entity identified by servedRadioBearer or servedMBS-RadioBearer.

[TS 38. 331, clause 5.3.5.6.6]

The UE shall:

1>	for each mrb-Identity value included in the mrb-ToReleaseList that is part of the current UE configuration; or

1>	for each mrb-Identity value that is to be released as the result of full configuration according to 5.3.5.11:

2>	release the PDCP entity and the mrb-Identity;

2>	if there is no other multicast MRB configured with the same mbs-SessionId as configured for the released multicast MRB:

3>	indicate the release of the user plane resources for the mbs-SessionId to upper layers.

NOTE 1:	The UE does not consider the message as erroneous if the mrb-ToReleaseList includes any mrb-Identity value that is not part of the current UE configuration.

NOTE 2:	Whether or not the RLC and MAC entities associated with this PDCP entity are reset or released is determined by the CellGroupConfig.

[TS 38.331, clause 5.3.5.6.7]

The UE shall for each element in the order of entry in the list mrb-ToAddModList:

1>	if mrb-Identity value included in the mrb-ToAddModList is part of the UE configuration:

2>	if mrb-Identity value included in the mrb-ToAddModList for which mrb-IdentityNew is included (i.e., multicast MRB ID change):

3>	update the mrb-Identity to the value mrb-IdentityNew;

…

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config;

1>	else if mrb-Identity value included in the mrb-ToAddModList is not part of the UE configuration (i.e., multicast MRB establishment including the case when full configuration option is used):

2>	establish a PDCP entity and configure it in accordance with the received pdcp-Config;

2>	associate the established multicast MRB with the corresponding mbs-SessionId;

2>	if an SDAP entity with the received mbs-SessionId does not exist:

3>	establish an SDAP entity as specified in TS 37.324 [24] clause 5.1.1;

3>	if an SDAP entity with the received mbs-SessionId did not exist prior to receiving this reconfiguration:

4>	indicate the establishment of the user plane resources for the mbs-SessionId to upper layers.

NOTE 1:	When setting the reestablishPDCP flag for a radio bearer, the network ensures that the RLC receiver entities do not deliver old PDCP PDUs to the re-established PDCP entity. It does that e.g., by triggering a reconfiguration with sync of the cell group hosting the old RLC entity or by releasing the old RLC entity.

NOTE 2:	In this specification, UE configuration refers to the parameters configured by NR RRC unless otherwise stated.

NOTE 3:	When updating the mrb-Identity, the network ensures new MRBs are listed at the end of the mrb-ToAddModList if they have the same MRB ID as in the existing UE configuration.

14.2.4.2.1.3	Test description

14.2.4.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the serving cell.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-3 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

-	The UE is made interested in receiving MBS Multicast service with MBS service ID '000101'H.

14.2.4.2.1.3.2	Test procedure sequence

Table 14.2.4.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1a1-1b12a1 | Steps 1a1 to 1b12a1 of the generic procedures described in TS 38.508-1 [4] subclause 4.9.34 are performed on NR Cell 1 to establish an associated PDU Session to the MBS DNN and join in MBS Multicast session. | - | - | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 3 | The SS transmits a Paging message including a TMGI matched with MBS multicast session which the UE has joined. | <-- | NR RRC: Paging | - | - |
| 4-10 | Steps 2 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 to complete service procedure. | - | - | - | - |
| 11 | The SS transmits an RRCReconfiguration message to establish MRB-Identity 1 and MRB-Identity 2. | <-- | NR RRC: RRCReconfiguration | - | - |
| 12 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 13a1-13a2 | Steps 9a1 to 9a2 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed with condition UE TEST LOOP MODE C and Multicast MRB. | - | - | - | - |
| 14 | The SS transmits a MBS Packet via RLC-UM in MRB-Identity 1. | <-- | MBS Packet. | - | - |
| 15 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 16 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 17 | Check: Is the number of reported MBS Packets received on the MRB-Identity 1 in step 16 equal to 1? | - | - | 1 | P |
| 18 | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 19 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 20a1-20a2 | Steps 9a1 to 9a2 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed with condition UE TEST LOOP MODE C and Multicast MRB. | - | - | - | - |
| 21 | The SS transmits a MBS Packet via RLC-AM in MRB-Identity 2. | <-- | MBS Packet. | - | - |
| 22 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 23 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 24 | Check: Is the number of reported MBS Packets received on the MRB-Identity 2 in step 23 equal to 1? | - | - | 1 | P |
| 25 | The SS transmits an RRCReconfiguration message to release MRB-Identity 3. | <-- | NR RRC: RRCReconfiguration | - | - |
| 26 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | 2 | P |
| 27 | The SS transmits an RRCReconfiguration message to release MRB-Identity 2 and modify MRB-Identity 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 28 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | 3 | P |
| 29 | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 30 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 31a1-31a2 | Steps 9a1 to 9a2 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed with condition UE TEST LOOP MODE C and Multicast MRB. | - | - | - | - |
| 32 | The SS transmits a MBS Packet via RLC-AM of MRB-Identity 1. | <-- | MBS Packet. | - | - |
| 33 | The SS transmits a MBS Packet via RLC-UM of MRB-Identity 1. | <-- | MBS Packet. | - | - |
| 34 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 35 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 36 | Check: Is the number of reported MBS Packets received on the MRB-Identity 1 in step 35 equal to 2? | - | - | 3 | P |
| 37 | The SS transmits an RRCReconfiguration message to change MRB-Identity 1 to MRB-Identity 3. | <-- | NR RRC: RRCReconfiguration | - | - |
| 38 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | 4 | P |
| 39 | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 40 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 41a1-41a2 | Steps 9a1 to 9a2 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed with condition UE TEST LOOP MODE C and Multicast MRB. | - | - | - | - |
| 42 | The SS transmits a MBS Packet via RLC-AM of MRB-Identity 3. | <-- | MBS Packet. | - | - |
| 43 | The SS transmits a MBS Packet via RLC-UM of MRB-Identity 3. | <-- | MBS Packet. | - | - |
| 44 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 45 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 46 | Check: Is the number of reported MBS Packets received on the MRB-Identity 3 in step 45 equal to 2? | - | - | 4 | P |
| 47 | The SS transmits an RRCReconfiguration message to release MRB-Identity 3 and a PDU SESSION MODIFICATION COMMAND. | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| 48 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | 5 | P |
| 49 | The UE transmits an ULInformationTransfer message and a PDU SESSION MODIFICATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | - |

14.2.4.2.1.3.3	Specific message contents

Table 14.2.4.2.1.3.3-1: ACTIVATE TEST MODE (preamble, Table 14.2.4.2.1.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.2.4.2.1.3.3-2: Paging (step 3, Table 14.2.4.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-9 with condition TMGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| pagingGroupList-r17 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 { | 1 entry |  |  |
| TMGI-r17[1] | Set to the value of the TMGI UE have joined. | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.2.1.3.3-3: RRCReconfiguration (step 11, Table 14.2.4.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4],Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig | Table 14.2.4.2.1.3.3-4 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | Table 14.2.4.2.1.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.2.1.3.3-4: RadioBearerConfig (Table 14.2.4.2.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 2 entries |  |  |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI |  |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=1 | UM MRB |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB |  |  |
| } |  |  |  |
| MRB-ToAddMod-r17 [2] SEQUENCE { |  | entry 2 |  |
| mbs-SessionId-r17 | TMGI |  |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=2 | AM MRB |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.2.1.3.3-5: CellGroupConfig (Table 14.2.4.2.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 1 m=1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and PTP and MRBm | entry 2 m=2 |  |
| } |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
| reconfigurationWithSync | Not present |  |  |
| rlf-TimersAndConstants | Not present |  |  |
| rlmInSyncOutOfSyncThreshold | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig with condition MBS_Multicast |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.2.1.3.3-6: CLOSE UE TEST LOOP (steps 13a1, 20a1, 31a1 and 41a1 Table 14.2.4.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], clause 4.7A.2, condition UE TEST LOOP MODE C and Multicast MRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode C LB setup |  |  |  |
| MRB ID | ‘ 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 ’B | MRB-Identity is 1 | Step 13a1, Step 31a1 |
|  | ‘ 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 ’B | MRB-Identity is 2 | Step 20a1 |
|  | ‘ 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 ’B | MRB-Identity is 3 | Step 41a1 |

Table 14.2.4.2.1.3.3-7: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST (steps 15, 22, 34 and 44, Table 14.2.4.2.1.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-9

Table 14.2.4.2.1.3.3-8: RRCReconfiguration (steps 25 and 47, Table 14.2.4.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig | Table 14.2.4.2.1.3.3-9 |  |
| nonCriticalExtension | Not present |  | Step 25 |
| nonCriticalExtension SEQUENCE { |  |  | Step 47 |
| masterCellGroup | CellGroupConfig | Table 14.2.4.2.1.3.3-10 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} | DedicatedNAS-Message |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.2.1.3.3-9: RadioBearerConfig (Table 14.2.4.2.1.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| mrb-ToReleaseList-r17 SEQUENCE (SIZE (1..maxMRB-r17)) OF MRB-ToAddMod-r17 { | 1 entry |  |  |
| MRB-Identity-r17[1] | 3 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.2.1.3.3-10: CellGroupConfig (Table 14.2.4.2.1.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList | Not Present |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity { | 2 entries |  |  |
| logicalChannelIdentity[1] | LogicalChannelIdentity with conditions PTM and MRBm | entry 1 m=1 |  |
| logicalChannelIdentity[2] | LogicalChannelIdentity with conditions PTP and MRBm | entry 2 m=1 |  |
| } |  |  |  |
| mac-CellGroupConfig | Not Present |  |  |
| physicalCellGroupConfig | Not Present |  |  |
| spCellConfig | Not Present |  |  |
| } |  |  |  |

Table 14.2.4.2.1.3.3-11: RRCReconfiguration (Step 27, Table 14.2.4.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig | Table 14.2.4.2.1.3.3-12 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | Table 14.2.4.2.1.3.3-13 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.2.1.3.3-12: RadioBearerConfig (Table 14.2.4.2.1.3.3-11)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 1 entry |  |  |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI |  |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=1 |  |
| pdcp-Config-r17 | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| mrb-ToReleaseList-r17 SEQUENCE (SIZE (1..maxMRB-r17)) OF MRB-ToAddMod-r17 { | 1 entry |  |  |
| MRB-Identity-r17[1] | MRB-Identity with condition MRBm | m=2 |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.2.1.3.3-13: CellGroupConfig (Table 14.2.4.2.1.3.3-11)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 1 m=1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and PTP and MRBm | entry 2 m=1 |  |
| } |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity { | 1 entry |  |  |
| logicalChannelIdentity[1] | LogicalChannelIdentity with conditions MRBm and PTP | entry 1 m=2 |  |
| } |  |  |  |
| mac-CellGroupConfig | Not Present |  |  |
| physicalCellGroupConfig | Not Present |  |  |
| spCellConfig | Not Present |  |  |
| } |  |  |  |

Table 14.2.4.2.1.3.3-14: RRCReconfiguration (Step 37, Table 14.2.4.2.1.3.2-1)

| Derivation Path: TS 38.508-1, table 4.6.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig | Table 14.2.4.2.1.3.3-15 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | Table 14.2.4.2.1.3.3-16 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.2.1.3.3-15: RadioBearerConfig (Table 14.2.4.2.1.3.3-14)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 1 entry |  |  |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | Not present |  |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=1 |  |
| mrb-IdentityNew-r17 | 3 |  |  |
| reestablishPDCP-r17 | Not present |  |  |
| recoverPDCP-r17 | Not present |  |  |
| pdcp-Config-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.2.1.3.3-16: CellGroupConfig (Table 14.2.4.2.1.3.3-14)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig | entry 1 Table 14.2.4.2.1.3.3-17 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig | entry 2 Table 14.2.4.2.1.3.3-18 |  |
| } |  |  |  |
| mac-CellGroupConfig | Not Present |  |  |
| physicalCellGroupConfig | Not Present |  |  |
| spCellConfig | Not Present |  |  |
| } |  |  |  |

Table 14.2.4.2.1.3.3-17: RLC-BearerConfig (Table 14.2.4.2.1.3.3-16)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-148, conditions UM_DLonly and PTM and MRBm (m=1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| multicastRLC-BearerConfig-r17 SEQUENCE { |  |  |  |
| servedMBS-RadioBearer-r17 | 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.2.1.3.3-18: RLC-BearerConfig (Table 14.2.4.2.1.3.3-16)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-148, conditions AM and PTP and MRBm (m=1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| multicastRLC-BearerConfig-r17 SEQUENCE { |  |  |  |
| servedMBS-RadioBearer-r17 | 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.2.1.3.3-19: PDU SESSION MODIFICATION COMMAND (step 47, Table 14.2.4.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-9. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Received MBS container |  |  |  |
| Received MBS information |  |  |  |
| Rejection cause | ‘110’B | MBS session is released |  |
| MSAI | ‘00’B | MBS service area not included |  |
| MD | ‘100’B | Remove UE from MBS session |  |
| MSCI | ‘0’B | MBS security container not included |  |
| MTI | ‘00’B | No MBS timers included |  |
| IPAE | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  |  |
| MBMS Service ID | ‘000101’B |  |  |
| MCC | See table 4.4.2-3 |  |  |
| MNC | See table 4.4.2-3 |  |  |
| Source IP address information | Not present |  |  |
| Destination IP address information | Not present |  |  |
| MBS service area | Not present |  |  |
| MBS timers | Not present |  |  |
| MBS security container | Not present |  |  |