3GPP TSG-RAN5 Meeting #103	R5-243518

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4387 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction for MBS TC of 14.2.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are extra curly brackets in Paging message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. The extra brackets have been deleted. 2. Some other editorial corrections about local step number and reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The paging message content will not be aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-242596r1 |  |  |  |  |  |  |  |  |

14.2.4.1	MBS Multicast / RRC / Paging

14.2.4.1.1	MBS Multicast / RRC / Paging for group notification/ RRC_IDLE

14.2.4.1.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state and UE has joined one MBS multicast session }

ensure that {

when { UE receives a Paging message including TMGIs unmatched with MBS multicast session which the UE has joined, not including ue-Identity }

then { UE does not establish any RRC connection }

}

(2)

with { UE in NR RRC_IDLE state and UE has joined one MBS multicast session }

ensure that {

when { UE receives a Paging message including a TMGI matched with MBS multicast session which the UE has joined, not including ue-Identity }

then { UE successfully establishes the RRC connection }

}

(3)

with { UE in NR RRC_IDLE state and UE has released MBS multicast session }

ensure that {

when { UE receives a Paging message including a TMGI matched with MBS multicast session which the UE has released, not including ue-Identity }

then { UE does not establish any RRC connection }

}

14.2.4.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.300, clause 16.10.5.2; TS 38.331, clause 5.3.2.3; TS 24.501, clause 5.6.2.2.1, 5.6.1.1. Unless otherwise stated these are Rel-17 requirements.

[TS 38.300, clause 16.10.5.2]

A UE can receive data of MBS multicast session only in RRC_CONNECTED state. If the UE which joined a multicast session is in RRC_CONNECTED state and when the multicast session is activated, the gNB sends RRCReconfiguration message with relevant MBS configuration for the multicast session to the UE.

When there is (temporarily) no data to be sent to the UEs for a multicast session, the gNB may move the UE to RRC IDLE/INACTIVE state. gNBs supporting MBS use a group notification mechanism to notify the UEs in RRC IDLE/INACTIVE state when a multicast session has been activated by the CN or the gNB has multicast session data to deliver. Upon reception of the group notification, the UEs reconnect to the network. The group notification is addressed with P-RNTI on PDCCH, and the paging channels are monitored by the UE as described in clause 9.2.5. Paging message for group notification contains MBS session ID which is utilized to page all UEs in RRC IDLE and RRC INACTIVE states that joined the associated MBS multicast session, i.e., UEs are not paged individually. The UE stops monitoring for group notifications related to a specific multicast session once the UE leaves this multicast session.

[TS 38.331, clause 5.3.2.3]

1>	for each TMGI included in pagingGroupList, if any, included in the Paging message:

2>	if the UE has joined an MBS session indicated by the TMGI included in the pagingGroupList:

3>	forward the TMGI to the upper layers;

[TS 24.501, clause 5.6.2.2.1]

If TMGI is used as paging identity and the TMGI matches with MBS multicast session which the has UE joined, the UE shall respond to the paging. Otherwise, the UE shall not respond to the paging.

[TS 24.501, clause 5.6.1.1]

…

The UE shall invoke the service request procedure when:

a)	the UE, in 5GMM-IDLE mode over 3GPP access, receives a paging request from the network;

14.2.4.1.1.3	Test description

14.2.4.1.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	The SS configures the NR Cell 1 as the "Serving cell".

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-3 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

-	The UE is made interested in receiving MBS Multicast service with MBS service ID '000101'H.

14.2.4.1.1.3.2	Test procedure sequence

Table 14.2.4.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1a1-1b12a1 | Steps 1a1 to 1b12a1 of the generic procedures described in TS 38.508-1 [4] subclause 4.9.34 are performed on NR Cell 1 to establish an associated PDU Session to the MBS DNN and join in MBS Multicast session. | - | - | - | - |
| 2 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| 3 | The SS transmits a Paging message including TMGIs unmatched with MBS multicast session which the UE has joined. | <-- | NR RRC: Paging | - | - |
| 4 | Check: Does the UE transmit an RRCSetupRequest message within 10s? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 5 | The SS transmits a Paging message including a TMGI matched with MBS multicast session which the UE has joined | <-- | NR RRC: Paging | - | - |
| 6 | Check: Does the UE transmit an RRCSetupRequest message? | --> | NR RRC: RRCSetupRequest | 2 | P |
| 7 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 8 | The UE transmit an RRCSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | - |
| 9-12 | Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 to complete service procedure. | - | - | - | - |
| 13 | The SS transmits an RRCReconfiguration message to establish MRB. | <-- | NR RRC: RRCReconfiguration | - | - |
| 14 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 15a1-15a2 | Steps 9a1 to 9a2 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed with condition UE TEST LOOP MODE C and Multicast MRB. | - | - | - | - |
| 16 | The SS transmits a MBS Packet. | <-- | MBS Packet. | - | - |
| 17 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 18 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 19 | Check: Is the number of reported MBS Packets received on the MRB in step 18 equal to 1? | - | - | 2 | P |
| 20-22 | Steps 1 to 3 of the generic procedures described in TS 38.508-1 [4] subclause 4.9.37 are performed on NR Cell 1 to release MBS Multicast session. | - | - | - | - |
| 23 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| 24 | The SS transmits a Paging message including a TMGI matched with MBS multicast session which the UE has released. | <-- | NR RRC: Paging | - | - |
| 25 | Check: Does the UE transmit an RRCSetupRequest message within 10s? | --> | NR RRC: RRCSetupRequest | 3 | F |

14.2.4.1.1.3.3	Specific message contents

Table 14.2.4.1.1.3.3-1: ACTIVATE TEST MODE (preamble, Table 14.2.4.1.1.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.2.4.1.1.3.3-2: Paging (step 3, Table 14.2.4.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-9, condition TMGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| pagingGroupList-r17 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 { | 3 entries |  |  |
| TMGI-r17[1] | Set to the different value from the TMGI UE have joined. | entry 1 |  |
| TMGI-r17[2] | Set to the different value from the TMGI UE have joined. | entry 2 |  |
| TMGI-r17[3] | Set to the different value from the TMGI UE have joined. | entry 3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.1.1.3.3-3: Paging (step 5, Table 14.2.4.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-9, condition TMGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| pagingGroupList-r17 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 { | 3 entries |  |  |
| TMGI-r17[1] | Set to the different value from the TMGI UE have joined. | entry 1 |  |
| TMGI-r17[2] | Set to the different value from the TMGI UE have joined. | entry 2 |  |
| TMGI-r17[3] | Set to the value of the TMGI UE have joined. | entry 3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.1.1.3.3-4: RRCSetupRequest (step 6, Table 14.2.4.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupRequest ::= SEQUENCE { |  |  |  |
| rrcSetupRequest SEQUENCE { |  |  |  |
| establishmentCause | mt-Access |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.1.1.3.3-5: RRCReconfiguration (step 13, Table 14.2.4.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4],Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with condition MRBm and UM_PTM | m=1 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with condition MRBm and UM_PTM | m=1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.1.1.3.3-6: CLOSE UE TEST LOOP (step 15a1, Table 14.2.4.1.1.3.2-1)

Derivation Path: 38.508-1 [4], Table 4.7A-3, condition UE TEST LOOP MODE C and Multicast MRB

Table 14.2.4.1.1.3.3-7: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST (step 17, Table 14.2.4.1.1.3.2-1)

Derivation Path: 36.508 [6], Table 4.7A-9

Table 14.2.4.1.1.3.3-8: Paging (step 24, Table 14.2.4.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-9, condition TMGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| pagingGroupList-r17 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 { | 1 entries |  |  |
| TMGI-r17[1] | Set to the value of the TMGI UE have released | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

14.2.4.1.2	MBS Multicast / RRC / Paging for group notification / RRC_INACTIVE

14.2.4.1.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state and UE has joined one MBS multicast session }

ensure that {

when { UE receives a Paging message including TMGIs unmatched with MBS multicast session which the UE has joined, not including ue-Identity }

then { UE does not resume RRC connection }

}

(2)

with { UE in NR RRC_INACTIVE state and UE has joined one MBS multicast session }

ensure that {

when { UE receives a Paging message including a TMGI matched with MBS multicast session which the UE has joined, not including ue-Identity }

then { UE successfully resumes the RRC connection with resumecause set to mt-Access }

}

(3)

with { UE in NR RRC_INACTIVE state and UE has released MBS multicast session }

ensure that {

when { UE receives a Paging message including a TMGI matched with MBS multicast session which the UE has released, not including ue-Identity }

then { UE does not resume RRC connection }

}

(4)

with { UE in NR RRC_INACTIVE state and UE has joined one MBS multicast session }

ensure that {

when { UE receives a Paging message including a TMGI matched with MBS multicast session which the UE has joined, and including a matched identity fullI-RNTI }

then { UE successfully resumes the RRC connection with resumecause set to mt-Access}

}

(5)

with { UE in NR RRC_INACTIVE state and UE has joined one MBS multicast session }

ensure that {

when {  UE receives a Paging message including a TMGI matched with MBS multicast session which the UE has joined, and including a matched identity ng-5G-S-TMSI }

then { UE releases RRC connection with release cause ‘other’ and goes to NR RRC_IDLE state }

}

(6)

with { UE in NR RRC_INACTIVE state and UE has joined one MBS multicast session }

ensure that {

when { UE receives a Paging message including a TMGI matched with MBS multicast session which the UE has joined, and including a unmatched identity ng-5G-S-TMSI }

then { UE successfully resumes the RRC connection with resumecause set to mt-Access}

}

14.2.4.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.300, clause 16.10.5.2; TS 38.331, clause 5.3.2.3; TS 24.501, clause 5.6.2.2.1 and 5.3.1.4. Unless otherwise stated these are Rel-17 requirements.

[TS 38.300, clause 16.10.5.2]

When there is (temporarily) no data to be sent to the UEs for a multicast session, the gNB may move the UE to RRC IDLE/INACTIVE state. gNBs supporting MBS use a group notification mechanism to notify the UEs in RRC IDLE/INACTIVE state when a multicast session has been activated by the CN or the gNB has multicast session data to deliver. Upon reception of the group notification, the UEs reconnect to the network. The group notification is addressed with P-RNTI on PDCCH, and the paging channels are monitored by the UE as described in clause 9.2.5. Paging message for group notification contains MBS session ID which is utilized to page all UEs in RRC IDLE and RRC INACTIVE states that joined the associated MBS multicast session, i.e., UEs are not paged individually. The UE stops monitoring for group notifications related to a specific multicast session once the UE leaves this multicast session.

[TS 38.331, clause 5.3.2.3]

1>	if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the Paging message, or

…

2>	if the ue-Identity included in the PagingRecord matches the UE's stored fullI-RNTI:

…

3>	else:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

…

2>	else if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers:

3>	if upper layers indicate the support of paging cause:

4>	forward the ue-Identity, accessType (if present) and paging cause (if determined) to the upper layers;

3>	else:

4>	forward the ue-Identity and accessType (if present) to the upper layers;

3>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other';

1>	for each TMGI included in pagingGroupList, if any, included in the Paging message:

2>	if the UE has joined an MBS session indicated by the TMGI included in the pagingGroupList:

3>	forward the TMGI to the upper layers;

1>	if in RRC_INACTIVE and the UE has joined one or more MBS session(s) indicated by the TMGI included in the pagingGroupList; and

1>	if none of the ue-Identity included in any of the PagingRecord, if included in the Paging message, matches the UE identity allocated by upper layers:

2>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set as below:

…

3>	else:

4>	resumeCause is set to mt-Access.

[TS 24.501, clause 5.6.2.2.1]

If TMGI is used as paging identity and the TMGI matches with MBS multicast session which the has UE joined, the UE shall respond to the paging. Otherwise, the UE shall not respond to the paging.

[TS 24.501, clause 5.3.1.4]

…

Upon receiving AMF paging indication from the lower layers, the UE shall transition from 5GMM-CONNECTED mode with RRC inactive indication to 5GMM-IDLE mode over 3GPP access and handle the AMF paging same as the paging request received in the 5GMM-IDLE mode over 3GPP access as specified in clause 5.6.1.

14.2.4.1.2.3	Test description

14.2.4.1.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	The SS configures the NR Cell 1 as the "Serving cell".

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-3 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

-	The UE is made interested in receiving MBS Multicast service with MBS service ID '000101'H.

14.2.4.1.2.3.2	Test procedure sequence

Table 14.2.4.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1a1-1b12a1 | Steps 1a1 to 1b12a1 of the generic procedures described in TS 38.508-1 subclause 4.9.34 are performed on NR Cell 1 to establish an associated PDU Session to the MBS DNN and join in MBS Multicast session. | - | - | - | - |
| 2 | The SS transmits an RRCRelease message including suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 3 | The SS transmits a Paging message including TMGIs unmatched with MBS multicast session which the UE has joined. | <-- | NR RRC: Paging | - | - |
| 4 | Check: Does the UE transmit an RRCResumeRequest message within 10s? | --> | NR RRC: RRCResumeRequest | 1 | F |
| 5 | The SS transmits a Paging message including a TMGI matched with MBS multicast session which the UE has joined | <-- | NR RRC: Paging | - | - |
| 6 | Check: Does the UE transmit an RRCResumeRequest message? | --> | NR RRC: RRCResumeRequest | 2 | P |
| 7 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 8 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 9a1-9a2 | Steps 9a1 to 9a2 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed with condition UE TEST LOOP MODE C and Multicast MRB. | - | - | - | - |
| 10 | The SS transmits a MBS Packet. | <-- | MBS Packet | - | - |
| 11 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 12 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 13 | Check: Is the number of reported MBS Packets received on the MRB in step 12 equal to 1? | - | - | 2 | P |
| 14 | The SS transmits an RRCRelease message including suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 15 | The SS transmits a Paging message including a TMGI matched with MBS multicast session which the UE has joined and include a matched fullI-RNTI. | <-- | NR RRC: Paging | - | - |
| 16 | Check: Does the UE transmit an RRCResumeRequest message? | --> | NR RRC: RRCResumeRequest | 4 | P |
| 17 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 18 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 19 | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 20 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 21a1-21a2 | Steps 9a1 to 9a2 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed with condition UE TEST LOOP MODE C and Multicast MRB. | - | - | - | - |
| 22 | The SS transmits a MBS Packet. | <-- | MBS Packet | - | - |
| 23 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 24 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 25 | Check: Is the number of reported MBS Packets received on the MRB in step 24 equal to 1? | - | - | 4 | P |
| 26 | The SS transmits an RRCRelease message including suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 27 | The SS transmits a Paging message including a TMGI matched with MBS multicast session which the UE has joined and include a matched ng-5G-S-TMSI. | <-- | NR RRC: Paging | - | - |
| 28 | Check: Does the UE transmit an RRCSetupRequest message? | --> | NR RRC: RRCSetupRequest | 5 | P |
| 29 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 30 | The UE transmit an RRCSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | - |
| 31-34 | Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 to complete service procedure | - | - | - | - |
| 35 | The SS transmits an RRCReconfiguration message to establish MRB. | <-- | NR RRC: RRCReconfiguration | - | - |
| 36 | The UE transmit an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 37 | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 38 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 39a1-39a2 | Steps 9a1 to 9a2 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are executed with condition UE TEST LOOP MODE C and Multicast MRB. | - | - | - | - |
| 40 | The SS transmits a MBS Packet. | <-- | MBS Packet | - | - |
| 41 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 42 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 43 | Check: Is the number of reported MBS Packets received on the MRB in step 40 equal to 1? | - | - | 5 | P |
| 44 | The SS transmits an RRCRelease message including suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 45 | The SS transmits a Paging message including a TMGI matched with MBS multicast session which the UE has joined and include an unmatched identity ng-5G-S-TMSI. | <-- | NR RRC: Paging | - | - |
| 46 | Check: Does the UE transmit an RRCResumeRequest message? | --> | NR RRC: RRCResumeRequest | 6 | P |
| 47 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 48 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 49 | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 50 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 51a1-51a2 | Steps 9a1 to 9a2 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed with condition UE TEST LOOP MODE C and Multicast MRB. | - | - | - | - |
| 52 | The SS transmits a MBS Packet. | <-- | MBS Packet | - | - |
| 53 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 54 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 55 | Check: Is the number of reported MBS Packets received on the MRB in step 52 equal to 1? | - | - | 6 | P |
| 56-58 | Steps 1 to 3 of the generic procedures described in TS 38.508-1 [4] subclause 4.9.37 are performed on NR Cell 1 to release MBS Multicast session. | - | - | - | - |
| 59 | The SS transmits an RRCRelease message including suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 60 | The SS transmits a Paging message including a TMGI matched with MBS multicast session which the UE has joined. | <-- | NR RRC: Paging | - | - |
| 61 | Check: Does the UE transmit an RRCResumeRequest message within 10s? | --> | NR RRC: RRCResumeRequest | 3 | F |

14.2.4.1.2.3.3	Specific message contents

Table 14.2.4.1.2.3.3-1: ACTIVATE TEST MODE (preamble, Table 14.2.4.1.2.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.2.4.1.2.3.3-2: RRCRelease (step 2, step 14, step 26, step 44 and step 59, Table 14.2.4.1.2.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-16, condition NR_RRC_INACTIVE

Table 14.2.4.1.2.3.3-3: Paging (step 3, Table 14.2.4.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-9, condition TMGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| pagingGroupList-r17 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 { | 3 entries |  |  |
| TMGI-r17[1] | Set to the different value from the TMGI UE have joined. | entry 1 |  |
| TMGI-r17[2] | Set to the different value from the TMGI UE have joined. | entry 2 |  |
| TMGI-r17[3] | Set to the different value from the TMGI UE have joined. | entry 3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.1.2.3.3-4: Paging (step 5, Table 14.2.4.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-9, condition TMGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| pagingGroupList-r17 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 { | 3 entries |  |  |
| TMGI-r17[1] | Set to the different value from the TMGI UE have joined. | entry 1 |  |
| TMGI-r17[2] | Set to the different value from the TMGI UE have joined. | entry 2 |  |
| TMGI-r17[3] | Set to the value of the TMGI UE have joined. | entry 3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.1.2.3.3-5: RRCResumeRequest (step 6, step16 and step46, Table 14.2.4.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |
| resumeCause | mt-Access |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.1.2.3.3-6: CLOSE UE TEST LOOP (step 9a1, step 21a1, step 39a1 and step 51a1, Table 14.2.4.1.2.3.2-1)

Derivation Path: 38.508-1 [4], Table 4.7A-3, condition UE TEST LOOP MODE C and Multicast MRB

Table 14.2.4.1.2.3.3-7: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST (step 11, step 23, step 41 and step53, Table 14.2.4.1.2.3.2-1)

Derivation Path: 36.508 [6], Table 4.7A-9

Table 14.2.4.1.2.3.3-8: Paging (step 15, Table 14.2.4.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-9, condition TMGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord { | 1 entry |  |  |
| PagingRecord[1] SEQUENCE { |  | entry 1 |  |
| ue-Identity CHOICE { |  |  |  |
| fullI-RNTI | Set to the value of the I-RNTI-Value of the UE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| pagingGroupList-r17 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 { | 1 entry |  |  |
| TMGI-r17[1] | Set to the value of the TMGI UE have joined. | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.1.2.3.3-9: Paging (step 27, Table 14.2.4.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-9, condition TMGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord { | 1 entry |  |  |
| PagingRecord[1] SEQUENCE { |  | entry 1 |  |
| ue-Identity CHOICE { |  |  |  |
| ng-5G-S-TMSI | Set to the value of the NG-5G-S-TMSI of the UE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| pagingGroupList-r17 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 { | 1 entry |  |  |
| TMGI-r17[1] | Set to the value of the TMGI UE have joined. | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.1.2.3.3-10: RRCSetupRequest (step 28, Table 14.2.4.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupRequest ::= SEQUENCE { |  |  |  |
| rrcSetupRequest SEQUENCE { |  |  |  |
| establishmentCause | mt-Access |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.1.2.3.3-11: RRCReconfiguration (step 35, Table 14.2.4.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4],Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with condition MRBm and UM_PTM | m=1 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with condition MRBm and UM_PTM | m=1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.1.2.3.3-12: Paging (step 45, Table 14.2.4.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-9, condition TMGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord { | 1 entry |  |  |
| PagingRecord[1] SEQUENCE { |  | entry 1 |  |
| ue-Identity CHOICE { |  |  |  |
| ng-5G-S-TMSI | Set to the different value from the NG-5G-S-TMSI of the UE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| pagingGroupList-r17 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 { | 1 entry |  |  |
| TMGI-r17[1] | Set to the value of the TMGI UE have joined. | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.4.1.2.3.3-13: Paging (step 60, Table 14.2.4.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-9, condition TMGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| pagingGroupList-r17 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 { | 1 entry |  |  |
| TMGI-r17[1] | Set to the value of the TMGI UE have released | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |