Title: 	Discussion on test point selection for NS_47/NS_01 A-MPR for CA in FR1

Source: 	Huawei

- Introduction

As agreed in RAN5#78, each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for NS_47/NS_01 combination in Additional Maximum Power Reduction for CA test case in FR1.

In Rel-16 specification, only inter-band 2UL CA is defined in TS 38.101-1, but the analysis will be extended in the future for other configurations.

- Discussion

UE additional maximum output power reduction for CA FR1 is defined in Transmitter power for CA section in [1] where its scope is described as follows:

Unless otherwise stated, for inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in clause 6.2.3 apply for each uplink component carrier.

When NS_01 is signalled, no A-MPR requirement is applied. For NS_47, A-MPR minimum requirement in [2] is extracted as below.

When NS_47 is signalled for 30 MHz channel bandwidths A-MPR is defined in 6.2.3.3.18-2.

Table 6.2.3.3.18-2: A-MPR for modulation and waveform type

| Modulation/Waveform | A1(dB) |  | A2(dB) |  | A3(dB) |  | A4(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC3 | PC2 | PC3 | PC2 | PC3 | PC2 | PC3 | PC2 |
|  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 | ≤ 2 | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 | ≤ 2 | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 |  | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 6 | ≤ 8.5 |  | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 6 | ≤ 8.5 |  | ≤ 5 |  | ≤ 6 |
| CP-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 | ≤ 4 | ≤ 7 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 | ≤ 4 | ≤ 7 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 |  | ≤ 7 |
| CP-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  |  |  | ≤ 7 |

The MPR requirement for power class 3 is defined in 6.2.2 in [2], which is extracted as below :

Table 6.2.2.3-1: Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Edge RB allocations |  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 |  | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 |  |  | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52 | 02 |  |  |
|  | QPSK | ≤ 1 |  |  | 0 |
|  | 16 QAM | ≤ 2 |  |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |  |
| CP-OFDM | QPSK | ≤ 3 |  |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0dB MPR is 26dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40% of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |  |

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

  - Test Environment

Based on the baseline agreed in [4], test environment for UE additional maximum output power reduction for CA can be defined as in LTE regardless of number of CCs.

Proposal 1: Select Test Environment as Normal Conditions (NC) for NS_47/NS_01 for A-MPR for CA in FR1.

  - Test Frequencies

In test case 6.2.3 A-MPR, the selection of test frequencies for single carrier testing for NS_47 takes the SEM into consideration. For inter-band CA testing, it’s reasonable to select the same test frequencies for NS_47, which is PCC in this test case.

For SCC, NS_01 has no A-MPR requirement. Selecting Low range and High range for SCC will be reasonable.

Proposal 2: Select 2560 MHz as test frequency for NS_47.

  - Test Channel Bandwidth

Only 30 MHz channel bandwidth is specified for NS_47.

Proposal 3: For NS_47/NS_01 A-MPR for CA testing, select Lowest and Highest channel bandwidth for SCC. Test Channel Bandwidth for PCC is 30MHz.

  - Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for SEM, i.e. Lowest and Highest supported subcarrier spacing.

Proposal 4: Select 30kHz for PCC and SCC for NS_47/NS_01 A-MPR for CA testing.

  - Uplink Waveform, Modulation and RB allocation

According to the clause 4.1.1.2 in TR 38.905, three configurations should be tested in A-MPR for CA testing, which are :

-	Test the minimum Total power backoff value

-	Test the maximum Total power backoff value

-	Test the maximum unbalanced Total power backoff among CCs (max Pcmaxc difference).

Where the Total power backoff value means: MAX[MPR, A-MPR]

Based on the AMPR requirement for NS_47 and MPR requirement, following configurations are derived for each CC:

The minimum Total power backoff configuration:

PCC:	DFT-s-OFDM + QPSK + Edge_1RB_Right RB allocation (Backoff = 2dB)

SCC:	DFT-s-OFDM + QPSK + Inner_Full RB allocation

The maximum Total power backoff configuration:

PCC:	DFT-s-OFDM +QPSK + Edge_1RB_Left RB allocation

SCC:	CP-OFDM + 256QAM + Outer_Full RB allocation

The maximum unbalanced Total power backoff configuration:

PCC: DFT-s-OFDM +QPSK + Edge_1RB_Left RB allocation

SCC: DFT-s-OFDM + QPSK + Inner RB allocation

Based on the above analysis, the Uplink configuration should be selected as table 1.

Proposal 5: Select Uplink Waveform, Modulation and RB allocation should be selected as table 1.

Table 1 Test Parameters for NS_47/NS_01 AMPR

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  | Low range for PCC and SCC PCC specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | See Ch BW column for PCC Lowest NRB_agg, Highest NRB_agg for SCC |  |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  |  |  | 30kHz |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  | SCC |  |
|  |  | FC (MHz) | Ch BW (MHz) | Modulation |  | RB allocation | Modulation | RB allocation |
| 1 |  | Default | 30 | DFT-s-OFDM QPSK |  | Edge_1RB_Right | DFT-s-OFDM QPSK | Inner Full |
| 2 | N/A | Default | 30 | DFT-s-OFDM QPSK |  | Edge_1RB_Left | CP-OFDM 256QAM | Outer Full |
| 3 |  | Default | 30 | DFT-s-OFDM QPSK |  | Edge_1RB_Left | DFT-s-OFDM QPSK | Inner Full |

  - Number of Test Points

Based on above proposals, the total test points of NS_47/NS_01 A-MPR for CA are tabulated below.

Table 2 Total test points of NS_47/NS_01 A-MPR for CA

| Environment conditions | Test Frequencies | Test ChBW | Test SCS | Test ID | Total Test Points |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 | 1 | 1 | 3 | 3 |

- Conclusion

Proposal 1: Select Test Environment as Normal Conditions (NC) for NS_47/NS_01 for A-MPR for CA in FR1.

Proposal 2: Select 2560 MHz as test frequency for NS_47.

Proposal 3: For NS_47/NS_01 A-MPR for CA testing, select Lowest and Highest channel bandwidth for SCC. Test Channel Bandwidth for PCC is 30MHz.

Proposal 4: Select 30kHz for PCC and SCC for NS_47/NS_01 A-MPR for CA testing.

Proposal 5: Select Uplink Waveform, Modulation and RB allocation should be selected as table 1.

- References

[1] 3GPP TS 38.521-1 V16.4.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone; (Release 16)

[2] 	3GPP TS 38.101-1 v15.10.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[3]	R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[4]	3GPP TR 38.905 V16.4.0 Derivation of test points for radio transmission and reception User Equipment (UE) conformance test cases (Release 16)