3GPP TSG-RAN5 Meeting #103	R5-243655

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2778 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A Tx test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | WE Certification Oy, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A ΔTIB,c requirements and spurious emissions test are missing from 38.521-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A ΔTIB,c requirements and spurious emissions test are added to 38.521-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Tx requirements for CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A cannot be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.4.0.2.3, 6.5A.3.1.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is an update of R5-242705 and the outcome of a final revision to this document. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 6.2A.4.0.2.3	ΔTIB,c for Inter-band CA

Table 6.2A.4.0.2.3-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination |  | NR Band |  | ΔTIB,c (dB) |  |
| --- | --- | --- | --- | --- | --- |
| CA_n1-n3 |  | n1 |  | 0.3 |  |
|  |  | n3 |  | 0.3 |  |
| CA_n1-n8 |  | n1 |  | 0.3 |  |
|  |  | n8 |  | 0.3 |  |
| CA_n1-n28 |  | n1 |  | 0.3 |  |
|  |  | n28 |  | 0.6 |  |
| CA_n1-n41 |  | n1 |  | 0.5 |  |
|  |  | n41 |  | 0.5 |  |
| CA_n1-n77 |  | n1 |  | 0.6 |  |
|  |  | n77 |  | 0.8 |  |
| CA_n1-n78 |  | n1 |  | 0.3 |  |
|  |  | n78 |  | 0.8 |  |
| CA_n2-n5 |  | n2 |  | 0.3 |  |
|  |  | n5 |  | 0.3 |  |
| CA_n2-n14 |  | n2 |  | 0.3 |  |
|  |  | n14 |  | 0.3 |  |
| CA_n2-n30 |  | n2 |  | 0.5 |  |
|  |  | n30 |  | 0.3 |  |
| CA_n2-n48 |  | n2 |  | 0.6 |  |
|  |  | n48 |  | 0.8 |  |
| CA_n2-n66 |  | n2 |  | 0.5 |  |
|  |  | n66 |  | 0.5 |  |
| CA_n2-n77 |  | n2 |  | 0.6 |  |
|  |  | n77 |  | 0.8 |  |
| CA_n3-n5 |  | n3 |  | 0.3 |  |
|  |  | n5 |  | 0.3 |  |
| CA_n3-n8 |  | n3 |  | 0.3 |  |
|  |  | n8 |  | 0.3 |  |
| CA_n3-n28 |  | n3 |  | 0.3 |  |
|  |  | n28 |  | 0.3 |  |
| CA_n3-n41 |  | n3 |  | 0.5 |  |
|  |  | n41 |  | 0.34 |  |
|  |  |  |  | 0.85 |  |
| CA_n3-n77 |  | n3 |  | 0.6 |  |
|  |  | n77 |  | 0.8 |  |
| CA_n3-n78 |  | n3 |  | 0.6 |  |
|  |  | n78 |  | 0.8 |  |
| CA_n5-n30 |  | n5 |  | 0.3 |  |
|  |  | n30 |  | 0.3 |  |
| CA_n5-n48 |  | n5 |  | 0.3 |  |
|  |  | n48 |  | 0.3 |  |
| CA_n5-n66 |  | n5 |  | 0.3 |  |
|  |  | n66 |  | 0.3 |  |
| CA_n5-n77 |  | n5 |  | 0.6 |  |
|  |  | n77 |  | 0.8 |  |
| CA_n8-n78 |  | n8 |  | 0.6 |  |
|  |  | n78 |  | 0.8 |  |
| CA_n14-n30 |  | n14 |  | 0.3 |  |
|  |  | n30 |  | 0.3 |  |
| CA_n14-n66 |  | n14 |  | 0.3 |  |
|  |  | n66 |  | 0.3 |  |
| CA_n14-n77 |  | n14 |  | 0.5 |  |
|  |  | n77 |  | 0.8 |  |
| CA_n20-n78 |  | n20 |  | 0.6 |  |
|  |  | n78 |  | 0.8 |  |
|  |  | n24 |  | 0.3 |  |
| CA_n24-n41 |  | n41 |  | 0.44 |  |
|  |  |  |  | 0.95 |  |
| CA_n24-n48 |  | n24 |  | 0.6 |  |
|  |  | n48 |  | 0.8 |  |
| CA_n24-n77 |  | n24 |  | 0.6 |  |
|  |  | n77 |  | 0.8 |  |
| CA_n25-n66 |  | n25 |  | 0.5 |  |
|  |  | n66 |  | 0.5 |  |
| CA_n25-n77 |  | n25 |  | 0.6 |  |
|  |  | n77 |  | 0.8 |  |
| CA_n28-n41 |  | n28 |  | 0.3 |  |
|  |  | n41 |  | 0.3 |  |
| CA_n28-n77 |  | n28 |  | 0.5 |  |
|  |  | n77 |  | 0.8 |  |
| CA_n28-n78 |  | n28 |  | 0.5 |  |
|  |  | n78 |  | 0.8 |  |
| CA_n28-n79 |  | n28 |  | 0.5 |  |
|  |  | n79 |  | 0.8 |  |
| CA_n30-n66 |  | n30 |  | 0.5 |  |
|  |  | n66 |  | 0.8 |  |
| CA_n30-n77 |  | n30 |  | 0.3 |  |
|  |  | n77 |  | 0.8 |  |
| CA_n39-n41 |  | n39 |  | 02 |  |
|  |  | n41 |  | 02 |  |
|  |  | n39 |  | 0.53 |  |
|  |  | n41 |  | 0.53 |  |
| CA_n41-n771 |  | n41 |  | 0.3 |  |
|  |  | n77 |  | 0.8 |  |
| CA_n41-n79 |  | n41 |  | 0.3 |  |
|  |  | n79 |  | 0.8 |  |
| CA_n48-n66 |  | n48 |  | 0.8 |  |
|  |  | n66 |  | 0.6 |  |
| CA_n66-n70 |  | n66 |  | 0.5 |  |
|  |  | n70 |  | 0.5 |  |
| CA_n66-n71 |  | n66 |  | 0.3 |  |
|  |  | n71 |  | 0.3 |  |
| CA_n66-n77 |  | n66 |  | 0.6 |  |
|  |  | n77 |  | 0.8 |  |
| CA_n66-n78 |  | n66 |  | 0.6 |  |
|  |  | n78 |  | 0.8 |  |
| CA_n70-n71 |  | n70 |  | 0.3 |  |
|  |  | n71 |  | 0.6 |  |
| CA_n71-n77 |  | n71 |  | 0.5 |  |
|  |  | n77 |  | 0.8 |  |
| CA_n71-n78 |  | n71 |  | 0.5 |  |
|  |  | n78 |  | 0.8 |  |
| CA_n77-n79 |  | n77 |  | 0.5 |  |
|  |  | n79 |  | 0.5 |  |
|  |  | n78 |  | 0.5 |  |
| CA_n78-n79 |  |  |  | 1.56 |  |
|  |  | n79 |  | 0.5 |  |
|  |  |  |  | 1.56 |  |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications.NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 6: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |

Table 6.2A.4.0.2.3-2: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n1-n3-n77 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| CA_n1-n3-n78 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n8-n78 | n1 | 0.3 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n28-n77 | n1 | 0.6 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |
| CA_n1-n41-n77 | n1 | 0.5 |
|  | n41 | 0.5 |
|  | n77 | 0.8 |
| CA_n2-n5-n77 | n2 | 0.6 |
|  | n5 | 0.8 |
|  | n77 | 0.8 |
| CA_n2-n5-n48 | n2 | 0.6 |
|  | n5 | 0.3 |
|  | n48 | 0.8 |
| CA_n2-n48-n66 | n2 | 0.6 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |
| CA_n2-n48-n77 | n2 | 0.6 |
|  | n48 | 0.8 |
|  | n77 | 0.8 |
| CA_n2-n66-n77 | n2 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n3-n28-n41 | n3 | 0.5 |
|  | n28 | 0.3 |
|  | n41 | 0.31/0.82 |
| CA_n3-n28-n77 | n3 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
| CA_n3-n41-n77 | n3 | 0.6 |
|  | n41 | 0.31/0.82 |
|  | n77 | 0.8 |
| CA_n5-n48-n66 | n5 | 0.3 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |
| CA_n5-n48-n77 | n5 | 0.6 |
|  | n48 | 0.8 |
|  | n77 | 0.8 |
| CA_n5-n66-n77 | n5 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n28-n41-n79 | n28 | 0.5 |
|  | n41 | 0.3 |
|  | n79 | 0.8 |
| CA_n48-n66-n77 | n48 | 0.8 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. |  |  |

Table 6.2A.4.0.2.3-3: ΔTIB,c due to NR CA (four bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)5 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration6 |  |  |  |
| CA_n2-n5-n48-n66 | 0.6 | 0.3 | 0.8 | 0.6 |
| CA_n2-n5-n48-n77 | 0.6 | 0.3 | 0.8 | 0.8 |
| CA_n2-n5-n66-n77 | 0.5 | 0.3 | 0.5 | 0.8 |
| CA_n2-n48-n66-n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| CA_n5-n48-n66-n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| NOTE 1: FFS NOTE 2: FFS NOTE 3: FFS NOTE 4: FFS NOTE 5: “-” denotes ΔTIB,c = 0. NOTE 6: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5-n78 the band order from left to right is n1, n3, n5 and n78. |  |  |  |  |

##### ------------------------MANY UNCHANGED SECTIONS SKIPPED HERE--------------

##### 6.5A.3.1.1	General spurious emissions for CA (2UL CA)

6.5A.3.1.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5A.3.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA.

6.5A.3.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.3.1.0.

6.5A.3.1.1.4	Test description

6.5A.3.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Tables 6.5A.3.1.1.4.1-1 to 6.5A.3.1.1.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.1.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg for both PCC and SCC Highest NRB_agg for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | CP-OFDM QPSK | Outer_Full | Outer_Full |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

Table 6.5A.3.1.1.4.1-2: Intra-band contiguous CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | CP-OFDM QPSK | Outer_Full | N/A |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left | N/A |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right | N/A |
| 4 |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Right |
| 5 |  | CP-OFDM QPSK | Outer_Full | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band.. |  |  |  |  |

Table 6.5A.3.1.1.4.1-3: Intra-band non-contiguous CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 3 |  | CP-OFDM QPSK | Outer_Full | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Tables 6.5A.3.1.1.4.1-1, 6.5A.3.1.1.4.1-2 and 6.5A.3.1.1.4.1-3 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.3.1.1.4.3.

6.5A.3.1.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.1.1. Message contents are defined in clause 6.5A.3.1.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 6.5A.3.1.1.4.1-1, 6.5A.3.1.1.4.1-2 and 6.5A.3.1.1.4.1-3 as appropriate on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5A.3.1.1.5-1 for inter-band CA, table 6.5A.3.1.1.5-3 for intra-band contiguous CA and intra-band non-contiguous CA. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5A.3.1.1.5-1 for inter-band CA, table 6.5A.3.1.1.5-3 for intra-band contiguous CA and intra-band non-contiguous CA.

The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average. For power class 2 intra-band contiguous carrier aggregation, the spurious emissions is measured as the sum from both UE transmit antenna connectors when UE indicates support for dualPA-Architecture IE.

6.5A.3.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.5A.3.1.1.4.3-1: FrequencyInfoUL-SIB for inter-band CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 16 |  | Power class 3 and Inter-band CA |

Table 6.5A.3.1.1.4.3-2: FrequencyInfoUL-SIB for intra-band contiguous CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  | Power class 3 and Intra-band contiguous CA with Bandwidth class C Test ID 4 |
|  | 5 |  | Power class 3 and Intra-band contiguous CA with Bandwidth class C Test ID 5 |
|  | 8 |  | Power class 3 and Intra-band contiguous CA with Bandwidth class B Test ID 4 |
|  | 3 |  | Power class 3 and Intra-band contiguous CA with Bandwidth class B Test ID 5 |
|  | 11 |  | Power class 2 and Intra-band contiguous CA with Bandwidth class C Test ID 4 |
|  | 5 |  | Power class 3 and Intra-band contiguous CA with Bandwidth class C Test ID 5 |

6.5A.3.1.1.5	Test requirement

This clause specifies the requirements for the specified NR band for Transmitter Spurious emissions requirement with frequency range as indicated in Table 6.5A.3.1.1.5-1 for inter-band UL CA configuration and Table 6.5A.3.1.1.5-3 for intra-band contiguous and non-contiguous UL CA configuration. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For inter-band CA, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth. If for some frequency a spurious emission requirement of individual component carrier overlaps with the spectrum emission mask or channel bandwidth of another component carrier, then it does not apply.

For intra-band contiguous CA, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5A.3.1.0-1 from the edge of the aggregated channel bandwidth.

For intra-band non-contiguous CA, the spurious emission limits apply to frequency ranges that are more than FOOB in Table 6.5.3.1.3-1 away from the edges of each carrier in the gap and out of the gap.

The measured average power of spurious emission, derived in step 6, shall not exceed the described value in Table 6.5A.3.1.1.5-1 for inter-band UL CA configuration and Table 6.5A.3.1.1.5-3 for intra-band contiguous UL CA and intra-band non-contiguous configuration.

Table 6.5A.3.1.1.5-1: General spurious emissions test requirements for inter-band UL CA

| Frequency Range |  | Maximum Level |  | Measurement Bandwidth |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test requirements for CA_n1A-n3A Configuration |  |  |  |  |  |  |  |
| 135 MHz ≤ f ≤ 270 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |
| 1440 MHz ≤ f ≤ 1650 MHz 2055 MHz ≤ f ≤ 2250 MHz 3630 MHz ≤ f ≤ 3765 MHz 5340 MHz ≤ f ≤ 5550 MHz 5550 MHz ≤ f ≤ 5745 MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |
| Test requirements for CA_n1A-n8A Configuration |  |  |  |  |  |  |  |
| 90 MHz ≤ f ≤ 220 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |
| 1005 MHz ≤ f ≤ 1100 MHz 2800 MHz ≤ f ≤ 2895 MHz 2925 MHz ≤ f ≤ 3080 MHz 3680 MHz ≤ f ≤ 3810 MHz 4720 MHz ≤ f ≤ 4875 MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |
| Test requirements for CA_n1A-n28A Configuration |  |  |  |  |  |  |  |
| 424 MHz ≤ f ≤ 574 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |
| 1172 MHz ≤ f ≤ 1277 MHz 2623 MHz ≤ f ≤ 2728 MHz 3092 MHz ≤ f ≤ 3257 MHz 3326 MHz ≤ f ≤ 3476 MHz 4543 MHz ≤ f ≤ 4708 MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |
| Test requirements for CA_n1A-n78A Configuration |  |  |  |  |  |  |  |
| 40 MHz ≤ f ≤ 660 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |
| 1320 MHz ≤ f ≤ 1880 MHz 4620 MHz ≤ f ≤ 5780 MHz 7140 MHz ≤ f ≤ 7760 MHz 8520 MHz ≤ f ≤ 9580 MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |
| Test requirements for CA_n1A-n79A Configuration |  |  |  |  |  |  |  |
| 440 MHz ≤ f ≤ 1000 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |
| 1000 MHz ≤ f ≤ 1160 MHz 2420 MHz ≤ f ≤ 3080 MHz 6320 MHz ≤ f ≤ 8080 MHz 8240 MHz ≤ f ≤ 8960 MHz 10720 MHz ≤ f ≤ 11980 MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |
| Test requirements for CA_n2A-n5A Configuration |  |  |  |  |  |  |  |
| 152 MHz ≤ f ≤ 262 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1001 MHz ≤ f ≤ 1086 MHz 2674 MHz ≤ f ≤ 2759 MHz 2851 MHz ≤ f ≤ 2996 MHz 3498 MHz ≤ f ≤ 3608 MHz 4524 MHz ≤ f ≤ 4669 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n2A-n14A Configuration |  |  |  |  |  |  |  |
| 254 MHz ≤ f ≤ 334 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1052 MHz ≤ f ≤ 1122 MHz 2638 MHz ≤ f ≤ 2708 MHz 2902 MHz ≤ f ≤ 3032 MHz 3426 MHz ≤ f ≤ 3506 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n2A-n30A Configuration |  |  |  |  |  |  |  |
| 395 MHz ≤ f ≤ 465 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1385 MHz ≤ f ≤ 1515 MHz 2700 MHz ≤ f ≤ 2780 MHz 4155 MHz ≤ f ≤ 4225 MHz 6005 MHz ≤ f ≤ 6135 MHz 6460 MHz ≤ f ≤ 6540 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n2A-n48A Configuration |  |  |  |  |  |  |  |
| 9 kHz ≤ f < 150 kHz |  | -36 dBm |  | 1 kHz |  |  |  |
| 150 kHz ≤ f < 30 MHz |  | -36 dBm |  | 10 kHz |  |  |  |
| 30 MHz ≤ f < 270 MHz |  | -36 dBm |  | 100 kHz |  |  |  |
| 1640 MHz ≤ f ≤ 1850 MHz 5190 MHz ≤ f ≤ 5610 MHz 7250 MHz ≤ f ≤ 7520 MHz 8950 MHz ≤ f ≤ 9310 MHz |  | -30 dBm |  | 1 MHz |  |  |  |
| Test requirements for CA_n2A-n77A Configuration |  |  |  |  |  |  |  |
| 500 MHz ≤ f ≤ 520 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |
| 1390 MHz ≤ f ≤ 2350 MHz 4690 MHz ≤ f ≤ 6550 MHz 7000 MHz ≤ f ≤ 8020 MHz 8450 MHz ≤ f ≤ 10310 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n3A-n8A Configuration |  |  |  |  |  |  |  |
| 25 MHz ≤ f <30 MHz |  | -36 dBm |  | 10kHz |  |  |  |
| 30 MHz ≤ f ≤ 120 MHz 795 MHz ≤ f ≤ 905 MHz |  | -36 dBm |  | 100kHz |  |  |  |
| 2505 MHz ≤ f ≤ 2700 MHz 3470 MHz ≤ f ≤ 3615 MHz 4300 MHz ≤ f ≤ 4485 MHz |  | -30 dBm |  | 1MHz |  |  |  |
| Test requirements for CA_n3A-n28A Configuration |  |  |  |  |  |  |  |
| 214 MHz ≤ f ≤ 379 MHz 962 MHz ≤ f ≤ 1000 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |
| 1000 MHz ≤ f ≤ 1082 MHz 2413 MHz ≤ f ≤ 2533 MHz 2672 MHz ≤ f ≤ 2867 MHz 3116 MHz ≤ f ≤ 3281 MHz 4123 MHz ≤ f ≤ 4318 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n3A-n41A Configuration |  |  |  |  |  |  |  |
| 711 MHz ≤ f ≤ 1000 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |
| 1000 MHz ≤ f ≤ 1074 MHz 3207 MHz ≤ f ≤ 3670 MHz 4206 MHz ≤ f ≤ 4475 MHz 5916 MHz ≤ f ≤ 6260 MHz 6702 MHz ≤ f ≤ 7165 MHz |  | -30 dBm |  | 1MHz |  | To be tested with NS_01 signalled for band n41 |  |
|  |  | -25 dBm |  | 1MHz |  | To be tested with NS_04 signalled for band n41 |  |
| Test requirements for CA_n3A-n77A Configuration |  |  |  |  |  |  |  |
| 270 MHz ≤ f ≤ 780 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |
| 1515 MHz ≤ f ≤ 2490 MHz 4815 MHz ≤ f ≤ 6690 MHz 6720 MHz ≤ f ≤ 7770 MHz 8310 MHz ≤ f ≤ 10185 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |
| 270 MHz ≤ f ≤ 380 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1515MHz ≤ f ≤ 2090 MHz 4815 MHz ≤ f ≤ 5890 MHz 6720 MHz ≤ f ≤ 7370 MHz 8310 MHz ≤ f ≤ 9385 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n5A-n30A Configuration |  |  |  |  |  |  |  |
| 607 MHz ≤ f ≤ 667 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1456 MHz ≤ f ≤ 1491 MHz 3129 MHz ≤ f ≤ 3164 MHz 3761 MHz ≤ f ≤ 3806 MHz 3953 MHz ≤ f ≤ 4013 MHz 5434 MHz ≤ f ≤ 5479 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n5A-n48A Configuration |  |  |  |  |  |  |  |
| 1852 MHz ≤ f ≤ 2052 MHz 2701 MHz ≤ f ≤ 2876 MHz 4374 MHz ≤ f ≤ 4549 MHz 5198 MHz ≤ f ≤ 5398 MHz 6251 MHz ≤ f ≤ 6576 MHz 7924 MHz ≤ f ≤ 8249 MHz |  | -30 dBm |  | 1 MHz |  |  |  |
| Test requirements for CA_n5A-n66A Configuration |  |  |  |  |  |  |  |
| 12 MHz ≤ f ≤ 30 MHz |  | -36 dBm+TT |  | 10 kHz |  |  |  |
| 30 MHz ≤ f ≤ 132 MHz 861 MHz ≤ f ≤ 956 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 2534 MHz ≤ f ≤ 2736 MHz 3358 MHz ≤ f ≤ 3478 MHz 4244 MHz ≤ f ≤ 4409 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n5A-n77A Configuration |  |  |  |  |  |  |  |
| 1602 MHz ≤ f ≤ 3376 MHz 4124 MHz ≤ f ≤ 5898 MHz 5751 MHz ≤ f ≤ 9249 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |
| 1470 MHz ≤ f ≤ 2040 MHz 2385 MHz ≤ f ≤ 2920 MHz 4180 MHz ≤ f ≤4715 MHz 5060 MHz ≤ f ≤5630 MHz 5685 MHz ≤ f ≤6720 MHz 7480 MHz ≤ f ≤8515 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n14A-n30A Configuration |  |  |  |  |  |  |  |  |
| 709 MHz ≤ f ≤ 739 MHz |  |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1507 MHz ≤ f ≤ 1527 MHz 2093 MHz ≤ f ≤ 3113 MHz 3812 MHz ≤ f ≤ 3842 MHz 3881 MHz ≤ f ≤ 3911 MHz |  |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n14A-n66A Configuration |  |  |  |  |  |  |  |  |
| 114 MHz ≤ f ≤ 204 MHz 912 MHz ≤ f ≤ 992 MHz |  |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 2498 MHz ≤ f ≤ 2578 MHz 2622 MHz ≤ f ≤ 2772 MHz 3286 MHz ≤ f ≤ 3376 MHz 4208 MHz ≤ f ≤ 4358 MHz |  |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n14A-n77A Configuration |  |  |  |  |  |  |  |  |
| 1704 MHz ≤ f ≤ 3412 MHz 4088 MHz ≤ f ≤ 5796 MHz 5808 MHz ≤ f ≤ 9198 MHz |  |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n20A-n78A Configuration |  |  |  |  |  |  |  |
| 1576 MHz ≤ f ≤ 2136 MHz 2438 MHz ≤ f ≤ 2968 MHz 4132 MHz ≤ f ≤ 4662 MHz 4964 MHz ≤ f ≤ 5524 MHz 5738 MHz ≤ f ≤ 6768 MHz 7432 MHz ≤ f ≤ 8462 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n24A-n41A Configuration |  |  |  |  |  |  |  |
| 563 MHz ≤ f ≤ 825 MHz 835.5 MHz ≤ f ≤ 1000 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1000 MHz ≤ f ≤ 1063.5 MHz 3331.5 MHz ≤ f ≤ 3753.5 MHz 4122.5 MHz ≤ f ≤ 4350.5 MHz 5749 MHz ≤ f ≤ 6011 MHz 6618.5 MHz ≤ f ≤ 7040.5 MHz |  | -25 dBm+TT |  | 1 MHz |  | NS-01 signalled for n41 |  |
|  |  | -30 dBm+TT |  | 1 MHz |  | NS-04 signalled for n41 |  |
| Test requirements for CA_n24A-n48A Configuration |  |  |  |  |  |  |  |
| 229 MHz ≤ f ≤ 447 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1889.5 MHz ≤ f ≤ 2073.5 MHz 5176.5 MHz ≤ f ≤ 5360.5 MHz 5439.5 MHz ≤ f ≤ 5773.5 MHz 6803 MHz ≤ f ≤ 7021 MHz 8726.5 MHz ≤ f ≤ 9060.5 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n24A-n77A Configuration |  |  |  |  |  |  |  |
| 21 MHz ≤ f < 30 MHz |  | -36 dBm+TT |  | 10 kHz |  |  |  |
| 30 MHz ≤ f < 947 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1639.5 MHz ≤ f ≤ 2573.5 MHz 4926.5 MHz ≤ f ≤ 7521 MHz 8226.5 MHz ≤ f ≤ 10060.5 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n25A-n66A Configuration. 70 MHz ≤ f ≤ 205 MHz; -36 dBm+TT; 100 kHz. 1505 MHz ≤ f ≤ 1710 MHz 1920 MHz ≤ f ≤ 2120 MHz 3560 MHz ≤ f ≤ 3695 MHz 5270 MHz ≤ f ≤ 5610 MHz; -30 dBm+TT; 1 MHz. Test requirements for CA_n25A-n77A Configuration. 500 MHz ≤ f ≤ 530 MHz; -36 dBm+TT; 100 kHz. 1385 MHz ≤ f ≤ 2350 MHz 4685 MHz ≤ f ≤ 6550 MHz 7000 MHz ≤ f ≤ 8030 MHz 8450 MHz ≤ f ≤ 10315 MHz; -30 dBm+TT; 1 MHz. Test requirements for CA_n25A-n78A Configuration. 100 MHz ≤ f ≤ 530 MHz; -36 dBm+TT; 100 kHz. 1385 MHz ≤ f ≤ 1950 MHz 4685 MHz ≤ f ≤ 5750 MHz 7000 MHz ≤ f ≤ 7630 MHz 8450 MHz ≤ f ≤ 9515 MHz; -30 dBm+TT; 1 MHz. |
| Test requirements for CA_n26A-n66A Configuration |  |  |  |  |  |  |  |
| 12 MHz ≤ f < 30 MHz |  | -36 dBm+TT |  | 10 kHz |  |  |  |
| 30 MHz ≤ f ≤ 152 MHz 861 MHz ≤ f ≤ 966 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 2524 MHz ≤ f ≤ 2746 MHz 3338 MHz ≤ f ≤ 3478 MHz 4234 MHz ≤ f ≤ 4409 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n26A-n70A Configuration |  |  |  |  |  |  |  |
| 3 MHz ≤ f < 30 MHz |  | -36 dBm+TT |  | 10 kHz |  |  |  |
| 30 MHz ≤ f < 82 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 846 MHz ≤ f ≤ 896 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 2509 MHz ≤ f ≤ 2606 MHz 3323 MHz ≤ f ≤ 3408 MHz 4204 MHz ≤ f ≤ 4269 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n28A-n41A Configuration |  |  |  |  |  |  |  |
| 1000MHz ≤ f ≤ 1284 MHz 1748MHz ≤ f ≤ 1987 MHz 3199MHz ≤ f ≤ 3438 MHz 3902MHz ≤ f ≤ 4186 MHz 4244MHz ≤ f ≤ 4677 MHz 5695MHz ≤ f ≤ 6128MHz |  | -25 dBm+TT |  | 1MHz |  |  |  |
| Test requirements for CA_n28A-n77A Configuration |  |  |  |  |  |  |  |
| 1804 MHz ≤ f ≤ 3497 MHz 4003 MHz ≤ f ≤ 5696 MHz 5852 MHz ≤ f ≤ 9148 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n28A-n78A Configuration |  |  |  |  |  |  |  |
| 1804 MHz ≤ f ≤ 2394 MHz 2552 MHz ≤ f ≤ 3097 MHz 4003 MHz ≤ f ≤ 4548 MHz 4706 MHz ≤ f ≤ 5296 MHz 5852 MHz ≤ f ≤ 6897 MHz 7303 MHz ≤ f ≤ 8348 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n28A-n79A Configuration |  |  |  |  |  |  |  |
| 2904 MHz ≤ f ≤ 3594 MHz 3652 MHz ≤ f ≤ 4297 MHz 5103 MHz ≤ f ≤ 5748 MHz 5806 MHz ≤ f ≤ 6496 MHz 8052 MHz ≤ f ≤ 9297 MHz 9503 MHz ≤ f ≤ 10748 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n30A-n66A Configuration |  |  |  |  |  |  |  |
| 525 MHz ≤ f ≤ 605 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1105 MHz ≤ f ≤ 1255 MHz 2830 MHz ≤ f ≤ 2920 MHz 4015 MHz ≤ f ≤ 4095 MHz 5725 MHz ≤ f ≤ 5875 MHz 6320 MHz ≤ f ≤ 6410 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n30A-n77A Configuration |  |  |  |  |  |  |  |
| 410 MHz ≤ f ≤ 1000 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1000 MHz ≤ f ≤ 1895 MHz 4285 MHz ≤ f ≤ 6515 MHz 7910 MHz ≤ f ≤ 8830 MHz 8905 MHz ≤ f ≤ 10715 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n39A-n41A Configuration |  |  |  |  |  |  |  |
| 576 MHz ≤ f ≤ 810 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1070 MHz ≤ f ≤ 1344 MHz 3072 MHz ≤ f ≤ 3500 MHz 4376 MHz ≤ f ≤ 4610 MHz 6256 MHz ≤ f ≤ 6530 MHz 6872 MHz ≤f ≤7300 MHz |  | -30 dBm+TT |  | 1 MHz |  | 4 |  |
|  |  | -25 dBm +TT |  | 1MHz |  | 3 |  |
| Test requirements for CA_n41A-n77A Configuration |  |  |  |  |  |  |  |
| 610 MHz ≤ f ≤ 1000 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |
| 1000 MHz ≤ f ≤ 2080 MHz 3910 MHz ≤ f ≤ 6890 MHz 8292 MHz ≤ f ≤ 11090 MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |
| Test requirements for CA_n41A-n79A Configuration |  |  |  |  |  |  |  |
| 8 MHz ≤ f ≤30 MHz |  | -36 dBm+TT |  | 10kHz |  |  |  |
| 30 MHz ≤ f ≤980 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |
| 1710MHz ≤ f ≤ 2504MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |
| 6110MHz ≤ f ≤ 7690MHz |  |  |  |  |  |  |  |
| 9392MHz ≤ f ≤ 10380MHz |  |  |  |  |  |  |  |
| 11296MHz ≤ f ≤ 12690MHz |  |  |  |  |  |  |  |
| Test requirements for CA_n48A-n66A Configuration |  |  |  |  |  |  |  |
| 10 MHz ≤ f < 30 MHz |  | -36 dBm+TT |  | 10 kHz |  |  |  |
| 30 MHz ≤ f < 280 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1770 MHz ≤ f ≤ 1990 MHz 5260 MHz ≤ f ≤ 5690 MHz 6970 MHz ≤ f ≤ 7260 MHz 8810 MHz ≤ f ≤ 9180 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n48A-n70A Configuration |  |  |  |  |  |  |  |
| 130 MHz ≤ f ≤ 310 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1840 MHz ≤ f ≤ 2005 MHz 5245 MHz ≤ f ≤ 5705 MHz 6940 MHz ≤ f ≤ 7120 MHz 8795 MHz ≤ f ≤ 9110 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n48A-n71A Configuration |  |  |  |  |  |  |  |
| 2154 MHz ≤ f ≤ 2374 MHz 2852 MHz ≤ f ≤ 3037 MHz 4213 MHz ≤ f ≤ 4398 MHz 4876 MHz ≤ f ≤ 5096 MHz 6402 MHz ≤ f ≤ 6737 MHz 7763 MHz ≤ f ≤ 8098 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n66A-n71A Configuration |  |  |  |  |  |  |  |
| 314 MHz ≤ f < 454 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1012 MHz ≤ f ≤ 1117 MHz 2373 MHz ≤ f ≤ 2478 MHz 2722 MHz ≤ f ≤ 2897 MHz 3036 MHz ≤ f ≤ 3176 MHz 4083 MHz ≤ f ≤ 4258 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n66A-n77A Configuration |  |  |  |  |  |  |  |
| 260 MHz ≤ f ≤ 780 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |
| 1520 MHz ≤ f ≤ 2490 MHz 4820 MHz ≤ f ≤ 6690 MHz 6720 MHz ≤ f ≤ 7760 MHz 8310 MHz ≤ f ≤ 10180 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n66A-n78A Configuration |  |  |  |  |  |  |  |
| 260 MHz ≤ f ≤ 380 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |
| 1520 MHz ≤ f ≤ 2090 MHz 4820 MHz ≤ f ≤ 5890 MHz 6720 MHz ≤ f ≤ 7360 MHz 8310 MHz ≤ f ≤ 9380 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n70A-n71A Configuration |  |  |  |  |  |  |  |
| 299 MHz ≤ f < 384 MHz 997 MHz ≤ f < 1000 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1000 MHz ≤ f ≤ 1047 MHz 2358 MHz ≤ f ≤ 2408 MHz 2692 MHz ≤ f ≤ 2757 MHz 3021 MHz ≤ f ≤ 3106 MHz 4053 MHz ≤ f ≤ 4118 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n71A-n77A Configuration |  |  |  |  |  |  |  |
| 1904 MHz ≤ f ≤ 3537 MHz 3963 MHz ≤ f ≤ 5596 MHz 5902 MHz ≤ f ≤ 9098 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n71A-n78A Configuration |  |  |  |  |  |  |  |
| 1904 MHz ≤ f ≤ 2474 MHz 2602 MHz ≤ f ≤ 3137 MHz 3963 MHz ≤ f ≤ 4498 MHz 4626 MHz ≤ f ≤ 5196 MHz 5902 MHz ≤ f ≤ 6937 MHz 7263 MHz ≤ f ≤ 8298 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| NOTE 1: Applies for Band for which the upper frequency edge of the UL Band is greater than 2.55 GHz and less than or equal to 5.2 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz NOTE 3: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 38.101-3 [4] when NS_04 is signalled. NOTE 4: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in subclause 5.2B of TS 38.101-3 [4] when NS_04 is signalled. |  |  |  |  |  |  |  |

Table 6.5A.3.1.1.5-2: Test Tolerance for General spurious emissions

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0 | 0 |
| 40MHz < BW ≤ 100MHz | 0 | 0 |

--------------END OF CHANGES-------------