3GPP TSG-RAN5 Meeting #103	R5-243702

# Fukuoka City, Fukuoka, Japan, 20th May 2024 – 24th May 2024

Agenda item:	5.3.33.9

Source: 	Apple

Title: 	On Beam Correspondence initial access in Idle and FR2 RRM RACH Test

Document for:	Discussion and Endorsement

# Introduction

As per the LS [1] received from RAN4#108bis meeting, Beam correspondence during initial access in IDLE test is to verify the SSB based beam correspondence for PRACH MSG1 EIRP spherical coverage. It is intended to provide enough duration for UE 1) to select optimal Tx Beam, 2) to lock Tx Beam and keep transmitting multiple number of RACH preambles till it reach max counter of preambleTransMax during RAR holding period unlike FR2 RRM random access test cases where SS send MSG2 random access response just after receiving 5 preambles with preambleTransMax set as n6, as per RRM random access test procedures specified in TS 38.533 clause 5.3.2.2.1 and 7.3.2.2.1 [2].

FR2 RRM random access test cases have different test conditions and procedure than Beam correspondence during initial access in IDLE test.

The goal is to continue progress on Beam Correspondence test case development independently and FR2 RRM random access testability can be evaluated in parallel.

# Discussion

The intention of this paper is to discuss and highlight the differences between test conditions and procedure of Beam correspondence during initial access in IDLE test and FR2 RRM random access test.

The following table is to summarize the delta differences between them.

| Differences in Test Conditions/Procedure | Beam Correspondence during initial access in IDLE (TS38.521-2) | FR2 RRM random access Test Cases TS 38.533 clause 5.3.2.2.1.4.2 Test 3 |
| --- | --- | --- |
| Beam Lock | UE Beam Lock function is required. | Beam lock is not required. |
| Optimal Beam Selection | Optimal beam selection is needed for RACH MSG1 | Optimal Beam selection is not required |
| Spatial Coverage | It is required for PRACH Spherical coverage. | It’s a single point test. |
| Preamble Retransmissions | UE can retransmit several preambles. | Limited to 5 preambles. |
| preambleTransMax counter | This test may allow UE to use counter up to n200 | For SSB based, max. counter set at n6 |

Observation 1: Beam correspondence during initial access in IDLE and FR2 RRM test case have different test conditions and test procedure.

Proposal 1: RAN5 to continue test procedure development of Beam correspondence during initial access in IDLE independently (TS38.521-2) and evaluate RRM RACH test case (TS 38.533) testability in parallel.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1: Beam correspondence during initial access in IDLE and FR2 RRM random access test cases have different test conditions and test procedure.

Proposal 1: RAN5 to continue test procedure development of Beam correspondence during initial access in IDLE independently and evaluate RRM RACH test case testability in parallel.

# References

- R4-2317764, “LS on testability issues of beam correspondence requirements”, RAN4#108-bis, October 9 – October 13, 2023

- TS 38.533, User Equipment (UE) conformance specification; Radio Resource Management (RRM)