3GPP TSG-RAN5 Meeting #103	R5-243782

# Fukuoka, Japan, May 20th 2024 - May 24th 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3162 | rev | 1 | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RedCap RRM test case 17.7.4.1 with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To update test parameters taking test tolerance into consideration. - To update Editor’s note |  |  |  |  |  |  |  |  |
| Summary of change: |  | Issues mentioned above are solved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.7.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 CR 3165 TR 38.903 CR 0739 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT Annex F CR: R5-242905, 38.903 CR: R5-242906 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-242902r1 |  |  |  |  |  |  |  |  |

<Start of Change>

17.7.4.1	Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell

Editor’s Note: This test case has been completed for the following configurations:

-	Test frequency f ≤ 40.8 GHz

-	UE PC3

-	Normal conditions

-	The test is incomplete for UE power classes other than PC3

-	The test is incomplete for test frequencies > 40.8 GHz

-	The test is incomplete for extreme conditions

17.7.4.1.1	Test purpose

To verify that the intra-frequency SS-SINR measurement accuracy is within the specified limits for all bands.

17.7.4.1.2	Test applicability

This test applies to all types of NR RedCap UE from Release 17 onwards and supporting SS-SINR measurements.

17.7.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.7.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.17.7.4.1.

17.7.4.1.4	Test description

Same as the test description given in clause 7.7.3.1.4.

17.7.4.1.4.1	Initial conditions

Same as the initial conditions given in clause 7.7.3.1.4.1 with following exceptions:

- Table 7.7.3.1.4.1-1 is replaced by Table 17.7.4.1.4.1-1;

- Table 7.7.3.1.4.1-2 is replaced by Table 17.7.4.1.4.1-2;

Table 17.7.4.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.7.4.1-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 17.7.4.1.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | TE Part: A.3.3.1.1 DUT Part: A.3.4.1.1 | As specified in TS 38.508-1 [14] Annex A. |
| Exceptions to connection diagram | N/A |  |

17.7.4.1.4.2	Test procedure

Same as the test procedure given in clause 7.7.3.1.4.2 with following exceptions:

-	Table 7.7.3.1.5-1 is replaced by Table 17.7.4.1.5-1;

-	Table 7.7.3.1.5-2 is replaced by Table 17.7.4.1.5-2;

-	Table 7.7.3.1.5-3 is replaced by Table 17.7.4.1.5-3;

17.7.4.1.4.3	Message contents

Same as the message contents given in clause 7.7.3.1.4.3.

17.7.4.1.5	Test requirement

Same as the test requirement given in clause 7.7.3.1.5 with following exceptions:

-	Table 7.7.3.1.5-1 is replaced by Table 17.7.4.1.5-1;

-	Table 7.7.3.1.5-2 is replaced by Table 17.7.4.1.5-2;

-	Table 7.7.3.1.5-3 is replaced by Table 17.7.4.1.5-3;

Table 17.7.4.1.5-1: Cell-specific test parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| SSB ARFCN |  | Freq2 |  |
| Duplex mode |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 66 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |
| DRX cycle configuration | ms | Not applicable |  |
| TRS configuration |  | TRS.2.1 TDD |  |
| TCI state |  | TCI.State.0 |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - |
| Dedicated RMSI CORESET Reference Channel |  | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 |  |
| SMTC configuration |  | SMTC.1 |  |
| SSB configuration |  | SSB.1 FR2 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 |  |
| SS-RSSI-Measurement |  | Not Applicable |  |
| EPRE ratio of PSS to SSS |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS | dB | 0 |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation conditions |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 17.7.4.1.5-2: OTA related test parameters

| Parameter |  | Unit | Test 1 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  |  | Setup 1 according to clause A.9.1 |  | Setup 1 according to clause A.9.1 |  |
| Assumption for UE beamsNote 6 |  |  | Rough |  | Rough |  |
| Note1 |  | dBm/15kHz Note4 | -105 |  | -105 |  |
| Note1 |  | dBm/SCS Note3 | -96 |  | -96 |  |
|  |  | dB | 4.54 | 2.66 | -3 |  |
| SSB_RPNote2 |  | dBm/SCS Note4 |  | -91.46 | -93.34 | -98.8 | -98.8 |
| SS-SINR Note2 | n257, 258, n261 |  |  | -0.18 | -3.32 | -4.95 | -4.95 |
|  | n260 | dB |  | -0.31 | -3.42 | -5.20 | -5.20 |
|  | n259 |  |  | -0.39 | -3.48 | -5.48 | -5.48 |
|  | n257, 258, n261 |  | -0.18 | -3.32 | -4.95 | -4.95 |
|  | n260 | dB | -0.31 | -3.42 | -5.20 | -5.20 |
|  | n259 |  | -0.39 | -3.48 | -5.48 | -5.48 |
| IoNote2 |  |  | dBm/95.04 MHz Note4 | -59.43 |  | -63.87 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-SINR, SSB_RP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

Table 17.7.4.1.5-3: SS-SINR Intra frequency absolute accuracy requirements for the reported values

| UE Power Class 3 | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- |
| Normal Conditions |  |  |  |  |
| Lowest reported value (Cell 2) | n257, n258, n261 | SS-SINR_22 | All bands | SS-SINR_18 |
|  | n260 | SS-SINR_21 |  |  |
|  | n259 | FFS |  |  |
| Highest reported value (Cell 2) | All bands | SS-SINR_58 | n257, n258, n261 | SS-SINR_55 |
|  |  |  | n260 | SS-SINR_54 |
|  |  |  | n259 | FFS |
| Extreme Conditions |  |  |  |  |
| Lowest reported value (Cell 2) | n257, n258, n261 | FFS | All bands | FFS |
|  | n260 | FFS |  |  |
|  | n259 | FFS |  |  |
| Highest reported value (Cell 2) | All bands | FFS | n257, n258, n261 | FFS |
|  |  |  | n260 | FFS |
|  |  |  | n259 | FFS |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

<End of Change>