3GPP TSG-RAN5 Meeting #103	R5-243955

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4450 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 8.2.3.18.7 inter-SN CPC for NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The new test case for SN initiated inter-SN CPC for NR-DC was missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test case for SN initiated inter-SN CPC for NR-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SN initiated inter-SN CPC cannot be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.508-2; 38.523-2 CR 0644; 0480 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is a revision of R5-243175 and 4 revisions of it. |  |  |  |  |  |  |  |  |

-------- Start of change --------

Table 8.2.3.18.5.3.3-9: RRCReconfigurationComplete (step 5, Table 8.2.3.18.5.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-14 with Condition NR-DC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfigurationComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfigurationComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| selectedCondRRCReconfig-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CPC_NR Cell 28 | When NR Cell 28 is selected as target PSCell |
| CPC_NR Cell 29 | When NR Cell 29 is selected as target PSCell |

##### 8.2.3.18.7	SN initiated inter-SN Conditional PSCell change / Success / NR-DC

8.2.3.18.7.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with NR-DC and receiving an RRCReconfiguration message for conditional PSCell change where the execution conditions refers the SCG measurement configuration }

ensure that {

when { Any CPC execution conditions is not satisfied }

then { UE maintains connection with source PSCell and continues to evaluate the CPC execution conditions }

}

(2)

with { UE in NR RRC_CONNECTED state with NR-DC and receiving an RRCReconfiguration message for conditional PSCell change where the execution conditions refers the SCG measurement configuration }

ensure that {

when { Any CPC execution condition is satisfied }

then { UE performs conditional PSCell change to the candidate target cell }

}

(3)

with { UE in NR RRC_CONNECTED state with NR-DC and receiving an RRCReconfiguration message for conditional PSCell change where the execution conditions refers the SCG measurement configuration }

ensure that {

when { UE performs conditional PSCell change successfully }

then { UE releases stored CPC configurations }

}

8.2.3.18.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 36.331 clause 5.3.5.3, 5.3.5.9.5 and 38.331 clause 5.3.5.13.1, 5.3.5.14a and 5.3.5.3. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.5.13.1]

The network configures the UE with one or more candidate target SpCells in the conditional reconfiguration. The UE evaluates the condition of each configured candidate target SpCell. The UE applies the conditional reconfiguration associated with one of the target SpCells which fulfils associated execution condition.

…

In NR-DC, the UE may receive two independent conditionalReconfiguration:

-	a conditionalReconfiguration associated with MCG, that is included in the RRCReconfiguration message received via SRB1; and

-	a conditionalReconfiguration, associated with SCG, that is included in the RRCReconfiguration message received via SRB3, or, alternatively, included within a RRCReconfiguration message embedded in a RRCReconfiguration message received via SRB1.

…

[TS 38.331, clause 5.3.5.13.3]

For each condReconfigId received in the condReconfigToAddModList IE the UE shall:

1>	if an entry with the matching condReconfigId exists in the condReconfigToAddModList within the VarConditionalReconfig:

2>	if the entry in condReconfigToAddModList includes an condExecutionCond, condExecutionCondSCG, or condExecutionCondPSCell;

3>	replace condExecutionCond, condExecutionCondSCG, or condExecutionCondPSCell within the VarConditionalReconfig with the value received for this condReconfigId;

2>	if the entry in condReconfigToAddModList includes subsequentCondReconfig containing condExecutionCondToAddModList:

3>	for each condReconfigId received in condExecutionCondToAddModList:

4>	if an entry with the matching condReconfigId exists in the condExecutionCondToAddModList within VarConditionalReconfig;

5>	replace the entry in condExecutionCondToAddModList within VarConditionalReconfig with the value received for this condReconfigId;

4>	else:

5>	add a new entry in condExecutionCondToAddModList within VarConditionalReconfig with the value received for this condReconfigId;

2>	if the entry in condReconfigToAddModList includes subsequentCondReconfig containing condExecutionCondToReleaseList:

3>	for each condReconfigId received in condExecutionCondToReleaseList that is part of current stored condExecutionCondToAddModList within VarConditionalReconfig:

4>	remove the entry in condExecutionCondToAddModList within VarConditionalReconfig with the value received for this condReconfigId;

2>	if the entry in condReconfigToAddModList includes an securityCellSetId;

3>	replace securityCellSetId within the VarConditionalReconfig with the value received for this condReconfigId;

NOTE:	The UE should release the entry within VarServingSecurityCellSetID in case all the subsequent CPAC configurations are released.

2>	if the entry in condReconfigToAddModList includes an condRRCReconfig;

3>	replace condRRCReconfig within the VarConditionalReconfig with the value received for this condReconfigId;

1>	else:

2>	add a new entry for this condReconfigId within the VarConditionalReconfig;

1>	perform conditional reconfiguration evaluation as specified in 5.3.5.13.4;

[TS 38.331, clause 5.3.5.13.4]

The UE shall:

1>	for each condReconfigId within the VarConditionalReconfig:

2>	if the RRCReconfiguration within condRRCReconfig includes the masterCellGroup including the reconfigurationWithSync:

…

2>	else if the RRCReconfiguration within condRRCReconfig includes the secondaryCellGroup including the reconfigurationWithSync:

3>	if the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the secondaryCellGroup within the received condRRCReconfig is not the PSCell; and

4>	if the condReconfigToAddMod for the condReconfigId does not include subsequentCondReconfig; or

4>	if there is no condReconfigToAddMod which includes subsequentCondReconfig for the PSCell; or

4>	if the condReconfigToAddMod for the condReconfigId includes subsequentCondReconfig and there is a condReconfigToAddMod which includes subsequentCondReconfig with a matching condReconfigId value in condExecutionCondToAddModList for the PSCell:

5>	consider the cell to be applicable cell;

2>	if condExecutionCondSCG is configured and condExecutionCond is not configured:

3>	in the remainder of the procedure, consider each measId indicated in the condExecutionCondSCG as a measId in the VarMeasConfig associated with the SCG measConfig;

2>	for each measId included in the measIdList within VarMeasConfig indicated in the condExecutionCond, condExecutionCondSCG, or condExecutionCondPSCell associated to condReconfigId:

3>	if the condTriggerConfig is not configured with nesEvent:

4>	if the condEventId is associated with condEventT1, and if the entry condition applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell; or

4> if the condEventId is associated with condEventD1 or condEventD2, and if the entry conditions applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig; or

4>	if the condEventId is associated with condEventA3, condEventA4 or condEventA5, and if the entry condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

5>	consider the event associated to that measId to be fulfilled;

4>	if the measId for this event associated with the condReconfigId has been modified; or

4>	if the condEventId is associated with condEventT1, and if the leaving condition applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell; or

4>if the condEventId is associated with condEventD1 or condEventD2, and if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig; or

4>	if the condEventId is associated with condEventA3, condEventA4 or condEventA5, and if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

5>	consider the event associated to that measId to be not fulfilled;

2>	if condExecutionCondPSCell is not configured:

3>	if event(s) associated to all measId(s) within condTriggerConfig for the applicable cell are fulfilled:

4>	consider the applicable cell, associated to that condReconfigId, as a triggered cell;

4>	initiate the conditional reconfiguration execution, as specified in 5.3.5.13.5;

2>	else:

…

NOTE 1:	Up to 2 MeasId can be configured for each condReconfigId, if condExecutionCondPSCell is not configured. The conditional reconfiguration event of the 2 MeasId may have the same or different event conditions, triggering quantity, time to trigger, and triggering threshold.

[TS 38.331, clause 5.3.5.13.5]

The UE shall:

1>	if more than one pair of triggered PCell and associated triggered PSCell exist:

…

1>	else if only one pair of triggered PCell and associated triggered PSCell exists:

…

1>	else if more than one triggered cell exists:

2>	select one of the triggered cells as the selected cell for conditional reconfiguration execution;

1>	else:

2>	consider the triggered cell as the selected cell for conditional reconfiguration execution;

1>	for the selected cell(s) of conditional reconfiguration execution:

2>	if the subsequentCondReconfig is included in the entry in VarConditionalReconfig containing the RRCReconfiguration message for the selected cell:

…

2>	else:

3>	apply the stored condRRCReconfig of the selected cell and perform the actions as specified in 5.3.5.3;

NOTE:	If multiple NR cells are triggered in conditional reconfiguration execution, it is up to UE implementation which one to select, e.g. the UE considers beams and beam quality to select one of the triggered cells for execution.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, upon execution of the conditional reconfiguration (CHO, CPA, CPC, or subsequent CPAC), or upon execution of an LTM cell switch:

…

1>	else if the RRCReconfiguration message was received via SRB1 within the nr-SCG within mrdc-SecondaryCellGroup (UE in NR-DC, mrdc-SecondaryCellGroup was received in RRCReconfiguration or RRCResume via SRB1):

2>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution for CPC or subsequent CPAC which is configured via conditionalReconfiguration contained in nr-SCG within mrdc-SecondaryCellGroup; or

2>	if the RRCReconfiguration is applied due to an LTM cell switch execution:

3>	submit the RRCReconfigurationComplete message via the NR MCG embedded in NR RRC message ULInformationTransferMRDC as specified in clause 5.7.2a.3.

…

1>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and when MAC of an NR cell group successfully completes a Random Access procedure triggered above; or,

…

2>	if the reconfigurationWithSync was included in spCellConfig of an SCG and the CPA, CPC, or subsequent CPAC was configured:

3>	remove all the entries in the condReconfigList within the MCG and the SCG VarConditionalReconfig except for the entries in which subsequentCondReconfig is present, if any;

3>	remove all the entries within VarConditionalReconfiguration as specified in TS 36.331 [10], clause 5.3.5.9.6, if any;

…

[TS 38.331, clause 5.7.2a.3]

The UE shall set the contents of the ULInformationTransferMRDC message as follows:

1>	if there is a need to transfer MR-DC dedicated information related to NR:

2>	set the ul-DCCH-MessageNR to include the NR MR-DC dedicated information to be transferred (e.g., NR RRC MeasurementReport, UEAssistanceInformation, FailureInformation, RRCReconfigurationComplete, MCGFailureInformation, or IABOtherInformation message);

1>	else if there is a need to transfer MR-DC dedicated information related to E-UTRA:

…

1>	submit the ULInformationTransferMRDC message to lower layers for transmission, upon which the procedure ends.

8.2.3.18.7.3	Test description

8.2.3.18.7.3.1	Pre-test conditions

System Simulator:

-	NR Cell 3 is the PCell, NR Cell 1 is the source PSCell and NR Cell 2 and NR Cell 4 are the candidate target PSCell.

UE:

-	None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in state NR RRC_CONNECTED using generic procedure parameter Connectivity (NR-DC), Bearers (MCG(s) and SCG) established according to TS 38.508-1 [4], clause 4.5.4.

-	 Else, the UE is in state NR RRC_CONNECTED using generic procedure parameter Connectivity (NR-DC), Bearers (MCG(s) and SCG) established and Test Loop Function (On) with UE test loop mode B according to TS 38.508-1 [4], clause 4.5.4.

8.2.3.18.7.3.2	Test procedure sequence

Tables 8.2.3.18.7.3.2-1 and 8.2.3.18.7.3.2-2 illustrate the downlink power levels to be applied for NR Cells at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" and “T2", are applied at the point indicated in the Main behaviour description in Table 8.2.3.18.7.3.2-3.

Table 8.2.3.18.7.3.2-1: Power levels in FR1

|  | Parameter | Unit | NR Cell 3 | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -88 | -94 | -94 | Power levels are such that event A3 is not satisfied for NR Cell 2 and NR Cell 4. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -94 | -79 | -94 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 2 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -94 | -94 | -79 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 4 |

Table 8.2.3.18.7.3.2-2: Power levels in FR2

|  | Parameter | Unit | NR Cell 3 | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 | -100 | -100 | Power levels are such that event A3 is not satisfied for NR Cell 2 and NR Cell 4. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 | -82 | -91 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 2 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 | -91 | -82 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 4 |

Table 8.2.3.18.7.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration for conditional PSCell change with ConditionalReconfiguration to set NR Cell 2 and NR Cell 4 as target candidate cells. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message on NR Cell 3 | --> | RRCReconfigurationComplete | - | - |
| 3 | Check: Does UE initiate a random access procedure by transmitting Preamble in NR Cell 2 within 10s? | --> | (PRACH Preamble) | 1 | F |
| 4 | SS adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 5 | Check: Does UE initiate a random access procedure by transmitting Preamble in NR Cell 2 within 10s. | --> | (PRACH Preamble) | 2 | P |
| 6 | The SS transmits Random Access Response. | <-- | Random Access Response | - | - |
| 7 | The UE transmits an RRCReconfigurationComplete message containing NR RRCReconfigurationComplete message on NR Cell 3 | --> | RRCReconfigurationComplete | - | - |
| 8 | Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on SCG DRB using NR radio path? | - | - | 2 | P |
| 9 | SS adjusts the cell-specific reference signal level according to row "T2". | - | - | - | - |
| 10 | Check: Does UE initiate a random access procedure by transmitting Preamble in NR Cell 4 within 10s? | --> | (PRACH Preamble) | 3 | F |

8.2.3.18.7.3.3	Specific message contents

Table 8.2.3.18.7.3.3-1: RRCReconfiguration (step 1, Table 8.2.3.18.7.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR-DC and CPC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |
| nr-SCG | RRCReconfiguration | OCTET STRING (CONTAINING RRCReconfiguration) Table 8.2.3.18.7.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| conditionalReconfiguration-r16 | ConditionalReconfiguration | Table 8.2.3.18.7.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.18.7.3.3-2: ConditionalReconfiguration (Table 8.2.3.18.5.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-25D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16::= SEQUENCE { |  |  |  |
| condReconfigToAddModList-r16 | CondReconfigToAddModList |  |  |
| } |  |  |  |

Table 8.2.3.18.7.3.3-3: CondReconfigToAddModList (Table 8.2.3.18.7.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table Table 4.6.3-25C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 { | 2 entries |  |  |
| CondReconfigToAddMod-r16[1] ::= SEQUENCE { |  | Entry 1, NR Cell 2 |  |
| condReconfigId-r16 | 1 |  |  |
| condRRCReconfig-r16 | RRCReconfiguration-CPC with condition CPC_NR_Cell_2 | Table 8.2.3.18.7.3.3-8 |  |
| condExecutionCondSCG-r17 SEQUENCE { |  |  |  |
| CondReconfigExecCondSCG-r17 ::= SEQUENCE (SIZE (1..2)) OF MeasId { | 1 entry |  |  |
| MeasId[1] | 1 | Identify a measurement configuration |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| CondReconfigToAddMod-r16[2] ::= SEQUENCE { |  | Entry 2, NR cell 4 |  |
| condReconfigId-r16 | 2 |  |  |
| condRRCReconfig-r16 | RRCReconfiguration-CPC with condition CPC_NR_Cell_4 | Table 8.2.3.18.7.3.3-8 |  |
| condExecutionCondSCG-r17 SEQUENCE { |  |  |  |
| CondReconfigExecCondSCG-r17 ::= SEQUENCE (SIZE (1..2)) OF MeasId { |  |  |  |
| MeasId[1] | 1 | Identify a measurement configuration |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.18.7.3.3-4: RRCReconfiguration (Table 8.2.3.18.7.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | MeasConfig | Table 8.2.3.18.7.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.18.7.3.3-5: MeasConfig (Table 8.2.3.18.7.3.3-4)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 8.2.3.18.7.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-condEventA3 | Table 8.2.3.18.7.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.18.7.3.3-6: MeasObjectNR (Table 8.2.3.18.7.3.3-5)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 | The SSB of NR Cell 1, NR Cell 2 and NR cell 4 have the same ARFCN value as specified in TS 38.508-1 [4] clause 6.2.3 |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.2.3.18.7.3.3-7: ReportConfigNR-condEventA3 (Table 8.2.3.18.7.3.3-5)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition CHO and EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| condTriggerConfig SEQUENCE { |  |  |  |
| condEventId CHOICE { |  |  |  |
| condEventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.18.7.3.3-8: RRCReconfiguration-CPC (Table 8.2.3.18.7.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.8.1-1AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |
| nr-SCG | RRCReconfiguration with condition CPC_NR_Cell_2 | Table 8.2.3.18.7.3.3-9 | CPC_NR_Cell_2 |
|  | RRCReconfiguration with condition CPC_NR_Cell_4 | Table 8.2.3.18.7.3.3-9 | CPC_NR_Cell_4 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.18.7.3.3-9: RRCReconfiguration (Table 8.2.3.18.7.3.3-8)

| Derivation Path: TS 38.508-1 [4] Table 4.8.1-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup SEQUENCE { | CellGroupConfig with condition PCI_NR_Cell_2 | OCTET STRING (CONTAINING CellGroupConfig) Table 8.2.3.18.7.3.3-10 | CPC_NR_Cell_2 |
|  | CellGroupConfig with condition PCI_NR_Cell_4 | OCTET STRING (CONTAINING CellGroupConfig) Table 8.2.3.18.7.3.3-10 | CPC_NR_Cell_4 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.18.7.3.3-10: CellGroupConfig (Table 8.2.3.18.7.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with Condition PSCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 2 |  | PCI_NR_Cell_2 |
|  | Physical Cell Identity of NR Cell 4 |  | PCI_NR_Cell_4 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.3.18.7.3.3-11: RRCReconfigurationComplete (step 7, Table 8.2.3.18.7.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-14 with Condition NR-DC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfigurationComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfigurationComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| selectedCondRRCReconfig-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CPC_NR_Cell_2 | When NR Cell 2 is selected as target PSCell |
| CPC_NR_Cell_4 | When NR Cell 4 is selected as target PSCell |

----- End of change ------