3GPP TSG-RAN5 Meeting #103	R5-243957

# Fukuoka, Japan, 20th May 2024 – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0641 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new PICS for MUSIM gap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TC 9.1.5.2.10 need UE support Dual Rx because it require UE to stay in RRC connection mode in NW A (NR Cell A) and monitor SSB in NW B (NR Cell G) simultaneously. TC 9.1.7.3 need UE support Dual Rx because it require UE to stay in RRC connection mode in NW A (NR Cell A) and monitor paging in NW B (NR Cell G) simultaneously. TC 8.1.2.1.6 intend to test UE use MUSIM gap to monitor paging in NW B while UE in RRC connection mode in NW A (NR Cell 1). So if UE support Dual Rx, MUSIM gap is not needed, it can receive paging without MUSIM gap.Therefore this TC is applicable to UE only support Single Rx. Singe Rx and Dual Rx are defined in TR 23.761 [TR 23.761 4.2] - Specific to Dual-USIM devices the study shall focus on Single Rx / Single Tx UEs and Dual Rx / Single Tx UEs. NOTE 2: Dual Rx allows the Dual-USIM device to simultaneously receive traffic from two networks. Single Rx allows the Dual-USIM device to receive traffic from one network at one time. Single Tx allows the Dual-USIM device to transmit traffic to one network at one time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add two pics: pc_Single_Rx pc_Dual_Rx |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 8.1.2.1.6 is incomplete. Single Rx UE may fail 9.1.5.2.10, 9.1.7.3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-243086r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Table A.4.3.13-1: Multi-SIM Capabilities

| Item | UE Sidelink Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support one or more Multi-SIM features include N1 NAS signalling connection release/Paging indication for voice services/Reject paging request/Paging restriction/IMSI offset and so on. | 24.501, 4.25 | Rel-17 | pc_5GC_MUSIM | No |  |  |
| 2 | Support of Multi-SIM N1 NAS signalling connection release | 24.501, 4.25 | Rel-17 | pc_5GC_MUSIM_NCR | No |  |  |
| 3 | Support of Multi-SIM Paging indication for voice services | 24.501, 4.25 | Rel-17 | pc_5GC_MUSIM_PIV | No |  |  |
| 4 | Support of Multi-SIM Reject paging request | 24.501, 4.25 | Rel-17 | pc_5GC_MUSIM_RPR | No |  |  |
| 5 | Support of Multi-SIM Paging restriction | 24.501, 4.25 | Rel-17 | pc_5GC_MUSIM_PR | No |  | A UE support Pging restriction shall support: - N1 NAS signalling connection release or - Reject paging request or - both of them |
| 6 | Support providing MUSIM assistance information with MUSIM gap preference and related MUSIM gap configuration | 38.306 4.2.2 | Rel-17 | pc_musim_GapPreference_r17 | No |  | UE supporting this feature supports 3 periodic gaps and 1 aperiodic gap. |
| 7 | Support providing MUSIM assistance information with indication of leaving RRC_CONNECTED state | 38.306 4.2.2 | Rel-17 | pc_musimLeaveConnected_r17 | No |  |  |
| 8 | Support of MUSIM test function SET MUSIM UAI | 38.509, 5.13 | Rel-17 | pc_Set_MUSIM_UAI_Info_NR | No |  |  |
| 9 | Support receiving traffic from one network at one time for Multi-USIM Device | 23.761 4.2 | Rel-17 | pc_Single_Rx_ | No |  |  |
| 10 | Support of simultaneously receiving traffic from two networks for Multi-USIM Device | 23.761 4.2 | Rel-17 | pc_Dual_Rx | No |  |  |

<End of modified section 1>