3GPP TSG-RAN5 Meeting #103	R5-243985

# Fukuoka, Japan,  20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1535 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New MT speech EVS test case 12.24a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At the RAN5#102 meeting the discussion paper R5-241310 explaining that the same requirements e.g. for voice and the EVS default configuration applies for IR.92 and NG.114. The conclusion was that RAN5 was willing to consider adding test coverage for EVS default configuration A.2 in IR.92 from Rel-15. This CR proposes to introduce a new Rel-15 test case for MT speech EVS call. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new test case, 12.24a MT MTSI speech call / EVS / Default configuration, was added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no test coverage for EVS default configuration A.2 in IR.92 from Rel-15. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.24a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 34.229-2 CR 0341 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final version of R5-243524. |  |  |  |  |  |  |  |  |

### 12.24.5	Test requirements

The UE shall send requests and responses as described in clause 12.24.4.

## 12.24a	MT MTSI speech call / EVS / Default configuration

### 12.24a.1	Definition

Test to verify that the UE correctly performs IMS mobile terminated voice call setup with EVS when using IMS Multimedia Telephony. This process is described in 3GPP TS 24.229 [10], clauses 5.1.3 and 6.1, TS 24.173 [65] and TS 26.114 [66].

### 12.24a.2	Conformance requirement

Same as TC12.24.

### 12.24a.3	Test purpose

1)	To verify that, when initiating MT MTSI speech call and SS needs to reserve resources, the UE performs correct exchange of SIP protocol signalling messages for setting up the session.

2)	To verify that within SIP signalling the UE performs the correct exchange of SIP header and parameter contents.

3)	To verify that within SIP signalling the UE performs the correct exchange of SDP contents.

4)	To verify that the UE is able to release the call.

### 12.24a.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with EN-DC support (TS 38.508-2 [158] A.4.1-3/2)

1-26)	The UE executes the procedures described in TS 36.508 [94] table 4.5A.20.3-1 steps 1 to 26.

NOTE:	No NR cell is required to be setup in the preamble

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | SS sends INVITE with the first SDP offer. |
| 2-3 |  |  | Steps 2-3 defined in annex C.45 | MTSI MT speech call. Referred from 36.508 [94] table 4.5A.20.3-1 for a UE with EN-DCsupport. |
| 4 |  |  | 183 Session Progress | The UE sends 183 response reliably with the SDP answer to the offer in INVITE |
| 5-15 |  |  | Steps 5-15 defined in annex C.45 | MTSI MT speech call. Referred from 36.508 [94] table 4.5A.20.3-1 for a UE with EN-DC support. |

Specific Message Content

INVITE (Step 1)

Use the default message “INVITE for MT Call” in annex A.2.9 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:65 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP 96 97 98 99 100 - b=AS:65 - b=RS:0 - b=RR:2000 Attributes for media: - a=rtpmap:96 EVS/16000/1 - a=fmtp:96 br=5.9-24.4; bw=nb-swb; max-red=220 - a=rtpmap:97 AMR-WB/16000/1 - a=fmtp:97 mode-change-capability=2; max-red=220 - a=rtpmap:98 telephone-event/16000 - a=fmtp:98 0-15 - a=rtpmap: 99 AMR/8000/1 - a=fmtp:99 mode-change-capability=2; max-red=220 - a=rtpmap: 100 telephone-event/8000 - a=fmtp: 100 0-15 - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv |

183 Session Progress (Step 4)

Use the default message "183 Session Progress" in annex A.2.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Status-Line |  |
| Reason-Phrase | Not checked |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=(user-name) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) EVS/16000 [Note 2] - a=fmtp: (format) br=5.9-24.4; bw=nb-swb; max-red=(att-field) [Note 2] Attributes for preconditions: - a=curr:qos local none or a=curr:qos local sendrecv - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv - a=conf:qos remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The value for fmt, payload type and format is not checked |

### 12.24a.5	Test requirements

The UE shall send requests and responses as described in clause 12.24a.4.