3GPP TSG-RAN5 Meeting #103	R5-242379r2

# Fukuoka, Japan, 20 – 24 May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2751 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of PC1 transmitter requirements for bands n7, n41 and n78 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_HPUE_FWVM_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing PC1 requirements for bands n7, n41 and n78 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of PC1 requirements for bands n7 n41 and n78 into: - Table 6.2.1.3-1: UE Power Class - Table 6.2.1.5-2a: Maximum Output Power test requirement for PC1 - Table 6.2.2.5-4c: MPR requirements for other than Band n14 for PC1 - Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR) - Clause 6.2.3.3.2: A-MPR for NS_04 - Clause 6.2.3.3.17: A-MPR for NS_46 - New Table 6.2.3.4.1-2g: Test Configuration table for NS_04 (PC1) - New Table 6.2.3.4.1-25a: Test Configuration table for NS_46 (PC1) - New Table 6.2.3.5-3a: UE PC1 test requirements (NS_04) for band n41 - New Table 6.2.3.5-30a: UE PC1 test requirements for NS_46 - Clause 6.5.3.3.3.25: Minimum conformance requirements (network signalling value "NS_46") - Clause 6.5.3.3.5.25 Test requirement (network signalling value “NS_46”) - 6.5.2.4.2.3 UTRA ACLR Minimum conformance requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC1 requirements for bands n7, n41 and n78 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.3, 6.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Band n41 MOP in R5-242466 (CR 2758), Band n41 MPR in R5-242467 (CR 2759), Band n41 UTRA ACLR in R5-242468 (CR 2760). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Modify Note in Table 6.2.3.3.1-1. Note numbering to be sorted out by MCC. r2: Remove bands from Table 6.2.2.5-4c header. Change note yy to Note 11 to align with 38.101-1. Add notes 9 and 10 as FFS. |  |  |  |  |  |  |  |  |

-------- Start of changes --------

## 6.2	Transmitter power

### 6.2.1	UE maximum output power

6.2.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.1.2	Test applicability

This test case applies to all types of NR Power Class 1 UE release 15 and forward.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and don't support Redcap.

6.2.1.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1.3-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n2 |  |  |  |  | 23 | ± 23 |
| n3 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n5 |  |  |  |  | 23 | ± 2 |
| n7 | 316 | +2/-3 |  |  | 23 | ± 23 |
| n8 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n12 |  |  |  |  | 23 | ± 23 |
| n13 |  |  |  |  | 23 | ±2 |
| n14 | 316 | +2/-3 |  |  | 23 | ± 2 |
| n20 |  |  |  |  | 23 | ± 23 |
| n24 |  |  |  |  | 23 | +2/-33 |
| n25 |  |  |  |  | 23 | ± 23 |
| n26 |  |  |  |  | 23 | ± 23 |
| n28 |  |  |  |  | 23 | +2/-2.5 |
| n30 |  |  |  |  | 23 | ± 2 |
| n34 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n38 |  |  |  |  | 23 | ± 2 |
| n39 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n40 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n41 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n48 |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  | 23 | ± 2 |
| n51 |  |  |  |  | 23 | ± 2 |
| n53 |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  | 23 | ±2 |
| n66 |  |  |  |  | 23 | ± 2 |
| n70 |  |  |  |  | 23 | ± 2 |
| n71 |  |  |  |  | 23 | +2/-2.5 |
| n74 |  |  |  |  | 23 | ± 2 |
| n77 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n78 | 316 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  | 23 | ± 2 |
| n81 |  |  |  |  | 23 | ± 2 |
| n82 |  |  |  |  | 23 | ± 2 |
| n83 |  |  |  |  | 23 | + 2/-2.5 |
| n84 |  |  |  |  | 23 | ± 2 |
| n86 |  |  |  |  | 23 | ± 2 |
| n91 |  |  |  |  | 23 | ±23, 4 |
| n92 |  |  |  |  | 23 | ±23, 4 |
| n93 |  |  |  |  | 23 | ±23, 4 |
| n94 |  |  |  |  | 23 | ±23, 4 |
| n95 |  |  |  |  | 23 | ±2 |
| n97 |  |  |  |  | 23 | ±2 |
| n99 |  |  |  |  | 23 | +2/-33 |
| n100 | 316 | +2/-3 |  |  | 23 | ±2 |
| n101 | 316 | +2/-3 |  |  | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: FFS NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

6.2.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1 for Power Class 3, Table 6.2.1.5-2 for Power Class 2, Table 6.2.1.5-2a for Power Class 1.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1.

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-2 for Power Class 2 as defined in TS 38.306.

Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ±(2+TT) |
| n2 |  |  |  |  | 23 | ±(23+TT) |
| n3 |  |  |  |  | 23 | ±(23+TT) |
| n5 |  |  |  |  | 23 | ±(2+TT) |
| n7 |  |  |  |  | 23 | ±(23+TT) |
| n8 |  |  |  |  | 23 | ±(23+TT) |
| n12 |  |  |  |  | 23 | ±(23+TT) |
| n13 |  |  |  |  | 23 | ±(2+TT) |
| n14 |  |  |  |  | 23 | ±(2+TT) |
| n20 |  |  |  |  | 23 | ±(23+TT) |
| n24 |  |  |  |  | 23 | +2+TT/-3.03-TT |
| n25 |  |  |  |  | 23 | ±(23+TT) |
| n26 |  |  |  |  | 23 | ±(23+TT) |
| n28 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n30 |  |  |  |  | 23 | ±(2+TT) |
| n34 |  |  |  |  | 23 | ±(2+TT) |
| n38 |  |  |  |  | 23 | ±(2+TT) |
| n39 |  |  |  |  | 23 | ±(2+TT) |
| n40 |  |  |  |  | 23 | ±(2+TT) |
| n41 |  |  |  |  | 23 | ±(23+TT) |
| n48 |  |  |  |  | 23 | +2+TT/-3-TT |
| n50 |  |  |  |  | 23 | ±(2+TT) |
| n51 |  |  |  |  | 23 | ±(2+TT) |
| n53 |  |  |  |  | 23 | ±(2+TT) |
| n65 |  |  |  |  | 23 | ±(2+TT) |
| n66 |  |  |  |  | 23 | ±(2+TT) |
| n70 |  |  |  |  | 23 | ±(2+TT) |
| n71 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n74 |  |  |  |  | 23 | ±(2+TT) |
| n77 |  |  |  |  | 23 | +2+TT/-3-TT |
| n78 |  |  |  |  | 23 | +2+TT/-3-TT |
| n79 |  |  |  |  | 23 | +2+TT/-3-TT |
| n91 |  |  |  |  | 23 | e |
| n92 |  |  |  |  | 23 | ±(23, 5+TT) |
| n93 |  |  |  |  | 23 | ±(23, 5+TT) |
| n94 |  |  |  |  | 23 | ±(23, 5+TT) |
| n100 |  |  |  |  | 23 | ±(2+TT) |
| n101 |  |  |  |  | 23 | ±(2+TT) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 NOTE 5: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. |  |  |  |  |  |  |

Table 6.2.1.5-2: Maximum Output Power test requirement for Power Class 2

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2+TT/-3-TT |  |  |
| n3 |  |  | 26 | +2+TT/-33-TT |  |  |
| n8 |  |  | 26 | +2+TT/-33-TT |  |  |
| n34 |  |  | 26 | +2+TT/-3-TT |  |  |
| n39 |  |  | 26 | +2+TT/-3-TT |  |  |
| n41 |  |  | 26 | +2+TT/-33-TT |  |  |
| n77 |  |  | 26 | +2+TT/-3-TT |  |  |
| n78 |  |  | 26 | +2+TT/-3-TT |  |  |
| n79 |  |  | 26 | +2+TT/-3-TT |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |

Table 6.2.1.5-2a: Maximum Output Power test requirement for Power Class 1

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n75 | 31 | +2+TT/-3-TT |  |  |  |  |
| n145 | 31 | +2+TT/-3-TT |  |  |  |  |
| n785 | 31 | +2+TT/-3-TT |  |  |  |  |
| n1005 | 31 | +2+TT/-3-TT |  |  |  |  |
| n1015 | 31 | +2+TT/-3-TT |  |  |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3. NOTE 5: Generally, PC1 UE for Band n14 is not targeted for smartphone form factor. The UE power class 1. Requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |

Table 6.2.1.5-2b: Void

Table 6.2.1.5-3: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

-------- Skipped unchanged clauses --------

Table 6.2.2.5-4c: UE Power Class test requirements for bands other than Band n14 for Power Class 1 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) |  | ΔTC,c (dB) | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | <50 MHz CBW | ≥50 MHz CBW |  | <50 MHz CBW | ≥50 MHz CBW |  |  |  | <50 MHz CBW | ≥50 MHz CBW |
| 1 | 31 | 0 | 0 | 0 | 0 | 31.0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 - TT | 28.0 - TT |
| 2 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 3 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 4 | 31 | 0 | 0.5 | 0.5 | 0 | 30.5 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 - TT | 27.5 - TT |
| 5 | 31 | 0 | 0 | 0 | 0 | 31.0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 - TT | 28.0 - TT |
| 6 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 7 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 8 | 31 | 0 | 1 | 1 | 0 | 30.0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 - TT | 27.0 - TT |
| 9 | 31 | 0 | 1 | 1 | 0 | 30.0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 - TT | 27.0 - TT |
| 10 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 11 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 12 | 31 | 0 | 2 | 2 | 0 | 29.0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 - TT | 26.0 - TT |
| 13 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 14 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 15 | 31 | 0 | 2.5 | 2.5 | 0 | 28.5 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 - TT | 25.5 - TT |
| 16 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 17 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 18 | 31 | 0 | 4.5 | 4.5 | 0 | 26.5 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 - TT | 23.5 - TT |
| 19 | 31 | 0 | 1.5 | 1.5 | 0 | 29.5 | 29.5 | 2.0 | 3 | 33.0 + TT | 26.5 - TT | 26.5 - TT |
| 20 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 21 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 22 | 31 | 0 | 3 | 3 | 0 | 28.0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 - TT | 25.0 - TT |
| 23 | 31 | 0 | 2 | 2 | 0 | 29.0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 - TT | 26.0 - TT |
| 24 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 25 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 26 | 31 | 0 | 3 | 3 | 0 | 28.0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 - TT | 25.0 - TT |
| 27 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 28 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 29 | 31 | 0 | 3.5 | 3.5 | 0 | 27.5 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 - TT | 24.5 - TT |
| 30 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 31 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 32 | 31 | 0 | 6.5 | 6.5 | 0 | 24.5 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 - TT | 21.5 - TT |
| 33 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 34 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 35 | 31 | 0 | 0 | 0 | 0 | 31.0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 - TT | 28.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

### 6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_40, NS_09 not complete.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_07, NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. Unless stated otherwise, Edge RB allocations get the same A-MPR as Outer RB allocations. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2.3.2	Test applicability

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, NS_06, NS_07, NS_27 and NS_35 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and NR Power Class 1 UE release 15 and forward.

The requirements of this test apply in test case 6.5.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 3 UE release 15 and forward that don’t support Tx diversity.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_07, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_24, NS_27, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49, NS_50 and NS_56 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

NOTE:	Test execution is not necessary if 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 are executed.

6.2.3.3	Minimum conformance requirements

6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where CEIL{x, 0.5} means x rounding upwards to closest 0.5dB, NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 Table 6.2.2.3-1.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 (NOTE 8) | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.3.7 |
| NS_03U | 6.5.2.3.3.3, 6.5.2.4.2.3 | n2, n25, n66, n86 (NOTE 1) |  |  | Clause 6.2.3.3.7 |
| NS_04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.3.2 |
| NS_05 | 6.5.3.3.3.4 | n1, n65, n84 (NOTE 1) | 5, 10, 15, 20 (Note 2) |  | Clause 6.2.3.3.4 (NOTE 7) |
| NS_05U | 6.5.3.3.3.4, 6.5.2.4.2.3 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.3.4 (NOTE 7) |
| NS_06 | 6.5.2.3.3.4 | n12 | 5, 10, 15 |  | N/A |
|  |  | n14 | 5, 10 |  |  |
| NS_07 | 6.5.2.3.3.4, 6.5.3.3.5.26 | n13 | 5,10 | Table 6.2.3.3.29-1 | Table 6.2.3.3.29-2 |
| NS_10 |  | n20 | 15, 20 | Table 6.2.3.3.3-1 | Table 6.2.3.3.3-1 |
| NS_12 | 6.5.3.3.17 | n26 | 5, 10 | Table 6.2.3.3.21-1 | Table 6.2.3.3.21-2 |
| NS_13 | 6.5.3.3.18 | n26 | 5 | Table 6.2.3.3.22-1 | Table 6.2.3.3.22-2 |
| NS_14 | 6.5.3.3.19 | n26 | 10, 15, 20 | Table 6.2.3.3.23-1 | Table 6.2.3.3.23-2 |
| NS_15 | 6.5.3.3.20 | n26 | 5, 10, 15, 20 | Table 6.2.3.3.24-1 | Table 6.2.3.3.24-2 |
| NS_17 | 6.5.3.3.3.2 | n28, n83 | 5, 10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.3.13-1, A2 |
|  |  |  | 30 |  | Table 6.2.3.3.13-1, A3, A4, A5 |
| NS_21 | 6.5.2.3.3.9 6.5.3.3.3.12 | n30 | 5,10 |  | Clause 6.2.3.3.14 |
| NS_24 | 6.5.3.3.3.13 | n65 (Note 4) | 5, 10, 15, 20 | Table 6.2.3.3.15-1 | Subclause 6.2.3.3.15 |
| NS_27 | 6.5.2.3.3.8 6.5.3.3.3.14 | n48 | 5, 10, 15, 20, 30, 40 | Table 6.2.3.3.16-1 | Table 6.2.3.3.16-2 |
| NS_35 | 6.5.2.3.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_37 | 6.5.3.3.3.6 | n74 (Note 3) | 10, 15 | Table 6.2.3.3.8-1 | Table 6.2.3.3.8-1 |
| NS_38 | 6.5.3.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.3.9-1 | Table 6.2.3.3.9-1 |
| NS_39 | 6.5.3.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.3.10-1 | Table 6.2.3.3.10-1 |
| NS_40 | 6.5.3.3.3.9 | n51 | 5 |  | Table 6.2.3.3.5-1 |
| NS_41 | 6.5.3.3.3.10 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.11-1 |
| NS_42 | 6.5.3.3.3.11 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.12-1 |
| NS_43 | 6.5.3.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_43U | 6.5.3.3.3.5, 6.5.2.4.2.3 | n8, n81 (NOTE 1) | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_44 | 6.5.3.3.24 | n38 | 25,30,40 | Table 6.2.3.3.20-1 | Table 6.2.3.3.20-1 |

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_45 | 6.5.3.3.3.21 | n53 | 5, 10 |  | Clause 6.2.3.3.25 |
| NS_46 | 6.5.3.3.3.25 | n7 | 1511, 25, 30, 40, 50 | Table 6.2.3.3.17-1 Table 6.2.3.3.17-311 | Table 6.2.3.3.17-2 Table 6.2.3.3.17-411 |
| NS_47 | 6.5.3.3.3.15 | n41 (Note 5) | 30 | Table 6.2.3.3.18-1 Table 6.2.3.3.18-3 | Table 6.2.3.3.18-2 Table 6.2.3.3.18-4 |
| NS_48 | 6.5.3.3.3.22 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.3.26-1, Table 6.2.3.3.26-3 | Table 6.2.3.3.26-2, Table 6.2.3.3.26-4 (NOTE 7) |
| NS_49 | 6.5.3.3.3.23 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.3.27-1 Table 6.2.3.3.27-3 | Table 6.2.3.3.27-2, Table 6.2.3.3.27-4 (NOTE 7) |
| NS_50 | 6.5.3.3.3.16 | n39 | 10, 15, 20, 25, 30, 40 |  | Clause 6.2.3.3.19 |
| NS_55 | NOTE 6 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NS_56 | 6.5.3.3.3.27 | n24, n99 | 5, 10 | Table 6.2.3.3.30 | Clause 6.2.3.3.30 |
| NS_100 | 6.5.2.4.2.3 | n1, n2, n3, n5, n8, n25, n26, n66, n80, n81, n84, n86 (Note 1) |  |  | Table 6.2.3.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWchannel where the lower channel edge is ≥1930 MHz,10 MHz BWchannel where the lower channel edge is ≥1950 MHz and 15 MHz BWchannel where the lower channel edge is ≥1955 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. PC1 operation is not allowed. NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. NOTE 7: The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 [TS 38.306], the additional relaxation of [2] dB is applicable. NOTE 8: The NS_01 label with the field additionalPmax [7] absent is default for all NR bands. NOTE 9: FFS NOTE 10: FFS NOTE 11: Applicable only for power class 1 operation. |  |  |  |  |  |

-------- Skipped unchanged clauses --------

6.2.3.3.2	A-MPR for NS_04

For NS_04, A-MPR is not added to MPR. Also, when NS_04 is signalled, MPR shall be set to zero in the PCMAX equations to avoid double-counting MPR.

For power Class 1.5, 2 and 3, allowed maximum power reduction is defined as A-MPR=max(MPR, A-MPR'),

Note that A-MPR'=0 dB means only MPR is applied,

where A-MPR' is defined as

if RBstart ≤ fstart,max,IMD3 / (12SCS) and LCRB ≤ AWmax,IMD3 / (12SCS) and FC - BWChannel/2 < FUL_low + offsetIMD3,
then

the A-MPR' is defined according to Table 6.2.3.3.2-2 PC3_A2 relative to 23 dBm for power class 3, PC2 A4 relative to 26 dBm for power class 2, and PC1.5_A6 relative to 29 dBm for power class 1.5,

else,

if RBstart ≤ LCRB/2 + start / (12SCS) and LCRB ≤ AWmax,regrowth / (12SCS) and FC - BWChannel/2 < FUL_low + offsetregrowth,
then

the A-MPR' is defined according to Table 6.2.3.3.2-2 PC3_A1 relative to 23 dBm for power class 3, PC2 A3 relative to 26 dBm for power class 2, and PC1.5_A5 relative to 29 dBm for power class 1.5,

else

A-MPR' = 0 dB and apply MPR.

With the parameters defined in Table 6.2.3.3.2-1.

Table 6.2.3.3.2-1: Parameters for region edges and frequency offsets (Power Class 1.5, 2 and 3)

| Parameter | Symbol | Value |  | Related condition |
| --- | --- | --- | --- | --- |
|  |  | CP-OFDM | DFT-s-OFDM |  |
| Max allocation start in IMD3 region | fstart,max,IMD3 | 0.33 BWChannel |  | RBstart ≤ fstart,max,IMD3 / (12SCS) |
| Max allocation BW in IMD3 region | AWmax,IMD3 | 4 MHz |  | LCRB ≤ AWmax,IMD3 / (12SCS) |
| Freq. offset required to avoid A-MPR in IMD3 region | offsetIMD3 | BWChannel – 6 MHz |  | FC - BWChannel/2 ≥ FUL_low + offsetIMD3 |
| Right edge of regrowth region | start | 0.08 BWChannel |  | RBstart ≤ LCRB/2 + start / (12SCS) |
| Max allocation BW in regrowth region | AWmax,regrowth | 100 MHz |  | LCRB ≤ Min(LCRB,Max, AWmax,regrowth / (12SCS)) |
| Freq. offset required to avoid A-MPR in regrowth region | offsetregrowth | Max (10 MHz, 0.25* BWChannel MHz) | Max (10 MHz, 0.45* BWChannel MHz) | FC - BWChannel/2 ≥ FUL_low + offsetregrowth |

Table 6.2.3.3.2-2: A-MPR' values Access (Power Class 1.5, 2 and 3)

| Modulation/Waveform |  | A-MPR' (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PC3_A1 | PC3_A2 | PC2_A3 | PC2_A4 | PC1.5_A51 | PC1.5_A61 |
| DFT-s-OFDM | Pi/2-BPSK | ≤ 3.5 | ≤ 3.5 | ≤ 3.5 | ≤ 5.5 | ≤ 5 | ≤ 7 |
|  | QPSK | ≤ 4 | ≤ 4 | ≤ 4.5 | ≤ 6 | ≤ 6 | ≤ 7.5 |
|  | 16-QAM | ≤ 4 | ≤ 4 | ≤ 5 | ≤ 6 | ≤ 6.5 | ≤ 7.5 |
|  | 64-QAM | ≤ 4 | ≤ 4.5 | ≤ 5 | ≤ 6.5 | ≤ 6.5 | ≤ 8 |
|  | 256-QAM | ≤ 4.5 | ≤ 6 | ≤ 6.5 | ≤ 8 | ≤ 8 | ≤ 9.5 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 16-QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 64-QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 256-QAM | ≤ 6.5 | ≤ 8 | ≤ 7.5 | ≤ 10 | ≤ 9 | ≤ 11.5 |
| NOTE 1: PC1.5 assumes dual Tx. |  |  |  |  |  |  |  |

For Power Class 1, NS_04 A-MPR is defined as 
A-MPR = max(MPR, A-MPRregrowth, A-MPRIMD3, A-MPRCIM3, A-MPRedge).

A-MPRregrowth is obtained from Table 6.2.3.3.2-3 in terms of total guard bandwidth (TGBW). The TGBW is defined as the frequency distance between the RB allocation and the additional spurious emission limit defined in Table 6.5.3.3.3.1-1, i.e.,

TGBW=falloc,low-2496 MHz,

where

falloc,low=FC-BWChannel2+BWGB+RBstart∙12 SCS

is the lower edge frequency of the RB allocation, FC is the channel centre frequency, BWChannel is the channel bandwidth, and BWGB is the minimum guard bandwidth defined in Table 5.3.3-1.

Table 6.2.3.3.2-3: A-MPRregrowth for NS_04 (Power Class 1)

| TGBW range | A-MPRregrowth *(dB) |
| --- | --- |
| G0dB(BWalloc) TGBW | 0 |
| G1dB(BWalloc) TGBW < G0dB(BWalloc) | 1 |
| G2dB(BWalloc) TGBW < G1dB(BWalloc) | 2 |
| G3dB(BWalloc) TGBW < G2dB(BWalloc) | 3 |
| G4dB(BWalloc) TGBW < G3dB(BWalloc) | 4 |
| G5dB(BWalloc) TGBW < G4dB(BWalloc) | 5 |
| TGBW < G5dB(BWalloc) | 6 |

Each function GA(BWalloc) defines the required minimum total guard bandwidth for A-MPR value A and is defined as

GA(BWalloc)=max⁡(0,  C2BWalloc100MHz2+C1BWalloc100MHz+C0),

where BWalloc=LCRB∙12 SCS is the allocation bandwidth, and C2, C1, and C0 are obtained from Table 6.2.3.3.2-4 for each combination of waveform, modulation, and back-off value A.

Table 6.2.3.3.2-4: Polynomial coefficients for determining the required 
total guard bandwidth for each value of A-MPRregrowth (Power Class 1)

| Waveform | Modulation | Back-off value A [dB] / Polynomial coefficients C2, C1, C0 [MHz] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| DFT-s-OFDM | Pi/2-BPSK | -3, 40, 10 | -91, 98, 0 | -248, 143, -9 | -2148, 604, -40 | 0,0,0 | N/A | N/A |
|  | QPSK | -27, 71, 1 | -49, 73, 0 | -76, 76, -3 | -108, 62, -3 | 0,0,0 | N/A | N/A |
|  | 16-QAM | -34, 81, -1 (NOTE 1) | -57, 92, -5 | -69, 82, -5 | -119, 79, -5 | 0,0,0 | N/A | N/A |
|  | 64-QAM | -23, 74, -1 (NOTE 1) | -61, 91, -4 (NOTE 1) | -84, 92, -5 | -116, 80, -5 | 0,0,0 | N/A | N/A |
|  | 256-QAM | 0, 0, 0 (NOTE 2) | N/A | N/A | N/A | N/A | N/A | N/A |
| CP-OFDM | QPSK | -32, 81, -2 (NOTE 1) | -36, 77, -3 | -40, 70, -3 | -52, 67, -3 | -67, 60, -3 | -101, 56, -4 | 0,0,0 |
|  | 16-QAM | -31, 80, -2 (NOTE 1) | -32, 72, -2 (NOTE 1) | -36, 66, -2 | -48, 62, -2 | -66, 58, -2 | -108, 60, -4 | 0,0,0 |
|  | 64-QAM | -28, 78, -2 (NOTE 1) | -33, 73, -2 (NOTE 1) | -36, 66, -2 (NOTE 1) | -48, 62, -2 | -65, 58, -2 | -124, 67, -5 | 0,0,0 |
|  | 256-QAM | 0, 0, 0 (NOTE 2) | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: Back-off value A is less than floor(MPR), thus this polynomial is irrelevant. NOTE 2: For 256-QAM, always A-MPRregrowth = 0. |  |  |  |  |  |  |  |  |

For both OFDM and DFT-S-OFDM, A-MPRIMD3 = max0,  min3,  4-2.77 BWalloc1 MHz dB 
if  3falloc,low-2FC  2490.5 MHz; otherwise, A-MPRIMD3 = 0 dB.

For OFDM, A-MPRCIM3 = max0,  min11,  12-2 BWalloc1 MHz dB if  4 FC-3 falloc,high   2490.5 MHz; 
otherwise, A-MPRCIM3 = 0.

For DFT-S-OFDM, A-MPRCIM3 = 3 dB if BWalloc 1.08 MHz and 4 FC-3 falloc,high   2490.5 MHz; 
otherwise, A-MPRCIM3 = 0.

Here, falloc,high=falloc,low+BWalloc is the upper edge frequency of the RB allocation.

For both OFDM and DFT-S-OFDM, if RBstart=0, LCRB=1,  and FC-BWChannel2-2496 MHz<360 kHz, A-MPRedge is defined in Table 6.2.3.3.2-5. Otherwise, A-MPRedge = 0 dB.

Table 6.2.3.3.2-5: A-MPRedge for NS_04  (Power Class 1)

| Waveform | BWChannel [MHz] | BWalloc [MHz] | A-MPR [dB] |
| --- | --- | --- | --- |
| DFT-s-OFDM | < 40 | N/A | 0 |
|  | 40…90 | -9.8CBW100MHz2+24.1CBW100MHz-6.3 | 3.5 |
|  | 100 |  | 11 |
| OFDM | < 90 | N/A | 0 |
|  | 90 | 6.5 | 3.5 |
|  | 100 | 8 | 11 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.2.

-------- Skipped unchanged clauses --------

6.2.3.3.17	A-MPR for NS_46

Table 6.2.3.3.17-1: A-MPR regions for NS_46 (Power class 3)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2534.5 ≤ FC ≤ 2557.5 |  | Note 1 | A3 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.14 | >0 | A4 |
|  |  | ≥4.14, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
| NOTE 1: > 9.72 MHz for DFT-s-OFDM, > 16.02 MHz for CP-OFDM. |  |  |  |  |

Table 6.2.3.3.17-2: A-MPR for NS_46 (Power class 3)

| Modulation/Waveform |  | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | QPSK | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 16 QAM | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 64 QAM | 4.5 | 5 |  | 3.5 | 6 | 10 |
|  | 256 QAM |  |  |  |  | 6 | 10 |
| CP-OFDM | QPSK | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 16 QAM | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 64 QAM | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 256 QAM | 6 |  |  |  | 7 | 11 |

Table 6.2.3.3.17-3: A-MPR regions for NS_46 (Power class 1)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 15 MHz | Fc > 2559.5 | ≥ 11.8 | ≥ -1.8*12*SCS*RBend + 32.5 | A1 |
|  |  | ≥ 13.3 | ≤ 1.62 | A2 |
| 20 MHz | Fc > 2547.5 | ≥ 15.3 | ≥ -1.5*12*SCS*RBend + 36.5 | A1 |
|  |  | ≥ 15 | ≤ 5.7 | A2 |
| 25 MHz | Fc > 2547.5 | ≥ 17 | ≥ -1.5*12*SCS*RBend + 43.5 | A3 |
|  |  | ≥ 18.5 | ≤ 6.7 | A2 |
|  | 2535.5 < Fc ≤ 2547.5 | ≥ 21.6 | ≥ 21.6 | A5 |
|  |  | ≥ 20.5 | ≤ 5.6 | A2 |
| 30 MHz | 2523 < Fc ≤ 2545 | ≥ 22.5 | ≥ -0.7*12*SCS*RBend + 41.0 | A5 |
|  |  | ≥ 23.4 | ≤ 7.2 | A2 |
|  | Fc > 2545 | ≥ 20.0 | ≥ -1.5*12*SCS*RBend + 50.5 | A8 |
|  |  | ≥ 21.6 | ≤ 7.2 | A9 |
|  |  | ≤ 9.6 | ≥ 1.1*12*SCS*RBend - 1.5 | A7 |
| 35 MHz | Fc ≤ 2532.5 | ≥ 28 | ≤ 7.2 | A2 |
|  | 2532.5 < Fc ≤ 2542.5 | ≥ 27.7 | ≥ -1.2*12*SCS*RBend + 59.4 | A4 |
|  |  | ≥ 24.5 | ≤ 7.2 | A2 |
|  | Fc > 2542.5 | ≤ 18.4 | ≥ 1.15*12*SCS*RBend – 3.4 | A7 |
|  |  | ≥ 21.6 | ≥ 9, ≥ -1.2*12*SCS*RBend + 47 | A8 |
|  |  | ≥ 23 | < 9 | A9 |
| 40 MHz | Fc > 2540 | ≤ 24 | ≥ 1.15*12*SCS*RBend - 6 | A7 |
|  |  | > 24, < 35 | ≥ -0.75*12*SCS*RBend + 40.5 | A9 |
|  |  | ≥ 34 | > 7.2 | A9 |
|  |  | ≥ 27 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 27 | ≤ 2.2 | A9 |
|  | 2530 < Fc ≤ 2540 | ≤ 10.8 | ≥ 1.2*12*SCS*RBend – 2 | A7 |
|  |  | ≥ 29 | ≤ 7.2 | A9 |
|  |  | ≥ 28 | ≥ -1*12*SCS*RBend + 56 | A10 |
|  | Fc ≤ 2530 | ≥ 30 | ≤ 7.2 | A2 |
| 50 MHz | Fc > 2535 | ≤ 19 | ≥ 1.17*12*SCS*RBend – 10 | A7 |
|  |  | > 19, ≤ 29.5 | ≥ 1.17*12*SCS*RBend – 10 | A11 |
|  |  | > 29.5 | > 12.8, ≥ -1.15*12*SCS*RBend + 60 | A9 |
|  |  | > 41 | > 7.2, ≤ 12.8 | A12 |
|  |  | ≥ 33 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 33 | ≤ 2.2 | A9 |
|  | 2525 < Fc ≤ 2535 | ≤ 14 | ≥ 1.17*12*SCS*RBend – 6 | A7 |
|  |  | > 14, ≤ 26 | ≥ 1.17*12*SCS*RBend – 6 | A11 |
|  |  | ≥ 34 | > 22.5, ≥ -1.25*12*SCS*RBend + 77.5 | A8 |
|  |  | ≥ 43 | > 7.2, ≤ 22.5 | A12 |
|  |  | ≥ 35 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 35 | ≤ 2.2 | A9 |
|  | Fc = 2525 | ≥ 47 | ≥ -1*12*SCS*RBend + 94 | A13 |
|  |  | ≤ 9 | ≥ 1.14*12*SCS*RBend – 2 | A7 |
|  |  | ≥ 37.4 | > 2.2, ≤ 7.6 | A8 |
|  |  | ≥ 37.4 | ≤ 2.2 | A9 |

Table 6.2.3.3.17-4: A-MPR regions for NS_46 (Power class 1)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | A10 | A11 | A12 | A13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 2 |  | 2 | 1 | 1 | 3 | 9.5 | 5 | 6 | 4 |  | 4 | 2.5 |
|  | QPSK | 3 | 4 | 3 | 2 | 2 | 6 | 11 | 6 | 6 | 5 |  | 4 | 4 |
|  | 16 QAM | 3 | 6.5 | 3 |  |  | 6 | 12 | 6 | 6.5 | 5 |  | 4 | 4 |
|  | 64 QAM | 3 | 7 | 3 |  |  | 6 | 12.5 | 6 | 7 | 5 |  | 4 | 4 |
|  | 256 QAM |  | 7 |  |  |  | 6 | 12.5 | 6 | 7 | 5 |  |  |  |
| CP-OFDM | QPSK | 4 | 12.5 | 6 | 4 |  | 6.5 | 15 | 9 | 12.5 | 5.5 | 10 | 5 | 4 |
|  | 16 QAM | 4 | 12.5 | 6 | 4 |  | 6.5 | 15 | 9 | 13 | 5.5 | 10 | 5 | 4 |
|  | 64 QAM | 4 | 13 | 6 |  |  | 6.5 | 15 | 9 | 13 | 5.5 | 10 | 5 | 4 |
|  | 256 QAM |  | 13 |  |  |  |  | 15 | 9 | 13 |  | 10 |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.17.

-------- Skipped unchanged clauses --------

Table 6.2.3.4.1-2: Test Configuration table for NS_04 (Power Class 1.5, 2 and 3)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_04 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 (Note 3, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 3 (Note 3) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 4 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 5 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 6 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 7 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 8 (Note 4, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 9 (Note 4) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 10 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 11 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 12 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 14 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 15 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 16 | Low |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 17 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 18 | High |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 19 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 21 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 22 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 23 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 24 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 25 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 26 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 27 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 28 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 29 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 30 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 31 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 32 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 33 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 34 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 35 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 36 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 37 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 38 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 39 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 40 | Low |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 41 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 42 | High |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 43 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 44 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 45 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 46 | Low |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 47 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 48 | High |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 49 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |

| A-MPR test parameters for NS_04 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 50 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 51 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 52 | Low |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 53 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 54 | High |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 55 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 56 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 57 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 58 | Low |  |  |  | CP-OFDM 256 QAM | Outer Full |
| 59 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 60 | High |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for Ues which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for band n41. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n41, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n41. NOTE 5: Only applicable for 10 MHz and 15 MHz channel bandwidth |  |  |  |  |  |  |

Table 6.2.3.4.1-2a: Additional test frequencies for NS_04 (SCS=15 kHz, ΔFRaster = 15 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 |  |  | 2505 | 501000 | 2500.32 | 500064 | 0 | 15 | 6255 | 500430 | 2 | 0 | 0 (0) | 0 |
| 15 | 79 |  |  | 2512.5 | 502500 | 2505.39 | 501078 | 0 | 15 | 6270 | 501630 | 4 | 1 | 2 (4) | 5 |

zz

Table 6.2.3.4.1-2b: Additional test frequencies for NS_04 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 |  |  | 2505 | 501000 | 2500.68 | 500136 | 0 | 30 | 6261 | 500910 | 18 | 0 | 0 (0) | 0 |
| 15 | 38 |  |  | 2512.5 | 502500 | 2505.66 | 501132 | 0 | 30 | 6273 | 501870 | 6 | 0 | 0 (0) | 0 |

Table 6.2.3.4.1-2c: Additional test frequencies for NS_04 (SCS=60 kHz, ΔFRaster = 15 kHz, without CORESET#0)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 |  |  | 2505 | 501000 | 2501.04 | 500208 | 0 | 15 | - | 500568 |
| 15 | 18 |  |  | 2512.5 | 502500 | 2506.02 | 501204 | 0 | 15 | - | 501564 |

Table 6.2.3.4.1-2d: Additional test frequencies for NS_04 (SCS=15 kHz, ΔFRaster = 15 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 |  |  | 2511 | 502200 | 2506.32 | 501264 | 0 | 15 | 6270 | 501630 | 2 | 0 | 0 (0) | 0 |
| 15 | 79 |  |  | 2513.505 | 502701 | 2506.395 | 501279 | 0 | 15 | 6273 | 501870 | 5 | 2 | 2 (4) | 6 |
| 20 | 106 |  |  | 2520 | 504000 | 2510.46 | 502092 | 0 | 15 | 6282 | 502590 | 10 | 1 | 1 (2) | 3 |
| 30 | 160 |  |  | 2535 | 507000 | 2520.6 | 504120 | 0 | 15 | 6306 | 504510 | 10 | 0 | 0 (0) | 0 |
| 40 | 216 |  |  | 2550 | 510000 | 2530.56 | 506112 | 0 | 15 | 6333 | 506670 | 6 | 1 | 2 (4) | 5 |
| 50 | 270 |  |  | 2565 | 513000 | 2540.7 | 508140 | 0 | 15 | 6357 | 508590 | 6 | 0 | 1 (2) | 2 |

Table 6.2.3.4.1-2e: Additional test frequencies for NS_04 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 |  |  | 2511 | 502200 | 2506.68 | 501336 | 0 | 30 | 6276 | 502110 | 18 | 0 | 0 (0) | 0 |
| 15 | 38 |  |  | 2513.52 | 502704 | 2506.68 | 501336 | 0 | 30 | 6276 | 502110 | 18 | 0 | 0 (0) | 0 |
| 20 | 51 |  |  | 2520 | 504000 | 2510.82 | 502164 | 0 | 30 | 6288 | 503070 | 14 | 0 | 2 (2) | 4 |
| 30 | 78 |  |  | 2535 | 507000 | 2520.96 | 504192 | 0 | 30 | 6312 | 504990 | 2 | 0 | 1 (1) | 2 |
| 40 | 106 |  |  | 2550 | 510000 | 2530.92 | 506184 | 0 | 30 | 6336 | 506910 | 2 | 0 | 0 (0) | 0 |
| 50 | 133 |  |  | 2565 | 513000 | 2541.06 | 508212 | 0 | 30 | 6363 | 509070 | 22 | 0 | 1 (1) | 2 |
| 60 | 162 |  |  | 2580 | 516000 | 2550.84 | 510168 | 0 | 30 | 6387 | 510990 | 10 | 0 | 1 (1) | 2 |
| 70 | 189 |  |  | 2595 | 519000 | 2560.98 | 512196 | 0 | 30 | 6414 | 513150 | 6 | 0 | 3 (3) | 6 |
| 80 | 217 |  |  | 2610 | 522000 | 2570.94 | 514188 | 0 | 30 | 6438 | 515070 | 6 | 0 | 2 (2) | 4 |
| 90 | 245 |  |  | 2625 | 525000 | 2580.9 | 516180 | 0 | 30 | 6462 | 516990 | 6 | 0 | 1 (1) | 2 |
| 100 | 273 |  |  | 2640 | 528000 | 2590.86 | 518172 | 0 | 30 | 6486 | 518910 | 6 | 0 | 0 (0) | 0 |

Table 6.2.3.4.1-2f: Additional test frequencies for NS_04 (SCS=60 kHz, ΔFRaster = 15 kHz, without CORESET#0)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 |  |  | 2511 | 502200 | 2507.04 | 501408 | 0 | 15 | - | 501768 |
| 15 | 18 |  |  | 2513.52 | 502704 | 2507.04 | 501408 | 0 | 15 | - | 501768 |
| 20 | 24 |  |  | 2520 | 504000 | 2511.36 | 502272 | 0 | 15 | - | 502632 |
| 30 | 38 |  |  | 2535 | 507000 | 2521.32 | 504264 | 0 | 15 | - | 504624 |
| 40 | 51 |  |  | 2550 | 510000 | 2531.64 | 506328 | 0 | 15 | - | 506688 |
| 50 | 65 |  |  | 2550 | 510000 | 2526.6 | 505320 | 0 | 15 | - | 505680 |
| 60 | 79 |  |  | 2580 | 516000 | 2551.56 | 510312 | 0 | 15 | - | 510672 |
| 70 | 93 |  |  | 2595 | 519000 | 2561.52 | 512304 | 0 | 15 | - | 512664 |
| 80 | 107 |  |  | 2610 | 522000 | 2571.48 | 514296 | 0 | 15 | - | 514656 |
| 90 | 121 |  |  | 2625 | 525000 | 2581.44 | 516288 | 0 | 15 | - | 516648 |
| 100 | 135 |  |  | 2640 | 528000 | 2591.4 | 518280 | 0 | 15 | - | 518640 |

Table 6.2.3.4.1-2g: Test Configuration table for NS_04 (Power Class 1)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Low, High |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_04 for PC1 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | Channel BW | SCS | N/A for A-MPR testing | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 (Note 3) | Low |  |  | Default | Default |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 (Note 3) | Low |  |  | Default | Default |  | DFT-s-OFDM PI/2 BPSK |  | Inner_1RB_Left |
| 3 (Note 3) | High |  |  | Default | Default |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 4 (Note 3) | High |  |  | Default | Default |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 5 (Note 3, 4) | Low |  |  | 10 MHz | Default |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 6 (Note 3, 4) | Low |  |  | 100 MHz | 30 kHz |  | DFT-s-OFDM PI/2 BPSK |  | 135@33 |
| 7 (Note 3, 4) | Low |  |  | 100 MHz | 60 kHz |  | DFT-s-OFDM PI/2 BPSK |  | 64@16 |
| 8 | Low |  |  | Default | Default |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 9 | Low |  |  | Default | Default |  | DFT-s-OFDM QPSK |  | Inner_1RB_Left |
| 10 | High |  |  | Default | Default |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 11 | High |  |  | Default | Default |  | DFT-s-OFDM QPSK |  | Outer Full |
| 12 (Note 4) | Low |  |  | 10 MHz | Default |  | DFT-s-OFDM QPSK |  | Outer Full |
| 13 (Note 4) | Low |  |  | 100 MHz | 30 kHz |  | DFT-s-OFDM QPSK |  | 135@33 |
| 14 (Note 4) | Low |  |  | 100 MHz | 60 kHz |  | DFT-s-OFDM QPSK |  | 64@16 |
| 15 | Low |  |  | Default | Default |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 16 | Low |  |  | Default | Default |  | DFT-s-OFDM 16 QAM |  | Inner_1RB_Left |
| 17 | High |  |  | Default | Default |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 18 | High |  |  | 100 MHz | 30 kHz |  | DFT-s-OFDM 16 QAM |  | 50@200 |
| 19 (Note 4) | Low |  |  | 10 MHz | Default |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 20 (Note 4) | Low |  |  | 100 MHz | 30 kHz |  | DFT-s-OFDM 16 QAM |  | 135@33 |
| 21 (Note 4) | Low |  |  | 100 MHz | 60 kHz |  | DFT-s-OFDM 16 QAM |  | 64@16 |
| 22 | Low |  |  | Default | Default |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 23 | Low |  |  | Default | Default |  | DFT-s-OFDM 64 QAM |  | Inner_1RB_Left |
| 24 | High |  |  | Default | Default |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 25 | High |  |  | 100 MHz | 30 kHz |  | DFT-s-OFDM 16 QAM |  | 50@200 |
| 26 (Note 4) | Low |  |  | 10 MHz | Default |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 27 (Note 4) | Low |  |  | 100 MHz | 30 kHz |  | DFT-s-OFDM 64 QAM |  | 135@33 |
| 28 (Note 4) | Low |  |  | 100 MHz | 60 kHz |  | DFT-s-OFDM 64 QAM |  | 64@16 |
| 29 | Low |  |  | Default | Default |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 30 | Low |  |  | Default | Default |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 31 | High |  |  | Default | Default |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 32 | High |  |  | Default | Default |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 33 (Note 4) | Low |  |  | 10 MHz | Default |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 34 (Note 4) | Low |  |  | 100 MHz | 30 kHz |  | DFT-s-OFDM 256 QAM |  | 135@33 |
| 35 (Note 4) | Low |  |  | 100 MHz | 60 kHz |  | DFT-s-OFDM 256 QAM |  | 64@16 |
| 36 | Low |  |  | Default | Default |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 37 | Low |  |  | Default | Default |  | CP-OFDM QPSK |  | Outer Full |
| 38 | High |  |  | 100 MHz | 30 kHz |  | CP-OFDM 16 QAM |  | 50@200 |
| 39 (Note 4) | Low |  |  | 10 MHz | Default |  | CP-OFDM QPSK |  | Outer Full |
| 40 (Note 4) | Low |  |  | 100 MHz | 30 kHz |  | CP-OFDM QPSK |  | 135@33 |
| 41 (Note 4) | Low |  |  | 100 MHz | 60 kHz |  | CP-OFDM QPSK |  | 64@16 |
| 42 | Low |  |  | Default | Default |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 43 | Low |  |  | Default | Default |  | CP-OFDM 16 QAM |  | Outer Full |
| 44 | High |  |  | 100 MHz | 30 kHz |  | CP-OFDM 16 QAM |  | 50@200 |
| 45 (Note 4) | Low |  |  | 10 MHz | Default |  | CP-OFDM 16 QAM |  | Outer Full |
| 46 (Note 4) | Low |  |  | 100 MHz | 30 kHz |  | CP-OFDM 16 QAM |  | 135@33 |
| 47 (Note 4) | Low |  |  | 100 MHz | 60 kHz |  | CP-OFDM 16 QAM |  | 64@16 |
| 48 | Low |  |  | Default | Default |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 49 | Low |  |  | Default | Default |  | CP-OFDM 64 QAM |  | Outer Full |
| 50 | High |  |  | 100 MHz | 30 kHz |  | CP-OFDM 16 QAM |  | 50@200 |
| 51 (Note 4) | Low |  |  | 10 MHz | Default |  | CP-OFDM 64 QAM |  | Outer Full |
| 52 (Note 4) | Low |  |  | 100 MHz | 30 kHz |  | CP-OFDM 64 QAM |  | 135@33 |
| 53 (Note 4) | Low |  |  | 100 MHz | 60 kHz |  | CP-OFDM 64 QAM |  | 64@16 |
| 54 | Low |  |  | Default | Default |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 55 | Low |  |  | Default | Default |  | CP-OFDM 256 QAM |  | Outer Full |
| 56 | High |  |  | Default | Default |  | CP-OFDM 256 QAM |  | 50@200 |
| 57 (Note 4) | Low |  |  | 10 MHz | Default |  | CP-OFDM 256 QAM |  | Outer Full |
| 58 (Note 4) | Low |  |  | 100 MHz | 30 kHz |  | CP-OFDM 256 QAM |  | 135@33 |
| 59 (Note 4) | Low |  |  | 100 MHz | 60 kHz |  | CP-OFDM 256 QAM |  | 64@16 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for band n41. NOTE 4: Applicable for requirement in regrowth region A-MPRregrowth |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

Table 6.2.3.4.1-25: Test Configuration table for NS_46 (Power class 3)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 25 MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_46 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 |  |  |  | N/A for A-MPR testing. | DFT-s-OFDM |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |
| 3 | Default | 25 | Default |  |  | 64QAM | 60@73 (A3) |  | 30@35 (A3) | 15@16 (A3) |
| 4 | Default | 50 | Default |  |  | 64QAM | Edge_1RB_Left (A4) |  |  |  |
| 5 | Default | 50 | Default |  |  | 16QAM | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 6 | Default | 50 | Default |  |  | 64QAM | 162@0 (A6) |  | 81@0 (A6) | 40@0 (A6) |
| 7 | Default | 50 | Default |  |  | 256QAM | 216@0 (A7) |  | 108@0 (A7) | 54@0 (A7) |
| 8 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A8) |  |  |  |
| 9 |  |  |  |  | CP-OFDM |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |
| 11 | Default | 25 | Default |  |  | QPSK | 90@43 (A3) |  | 45@20 (A3) | 23@8 (A3) |
| 12 | Default | 25 | Default |  |  | 256QAM | 90@43 (A3) |  | 45@20 (A3) | 23@8 (A3) |
| 13 | Default | 50 | Default |  |  | QPSK | Edge_1RB_Left (A4) |  |  |  |
| 14 | Default | 50 | Default |  |  | 64QAM | Edge_1RB_Left (A4) |  |  |  |
| 15 | Default | 50 | Default |  |  | QPSK | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 16 | Default | 50 | Default |  |  | 64QAM | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 17 | Default | 50 | Default |  |  | QPSK | 175@0 (A6) |  | 88@0 (A6) | 44@0 (A6) |
| 18 | Default | 50 | Default |  |  | 64QAM | 175@0 (A6) |  | 88@0 (A6) | 44@0 (A6) |
| 19 | Default | 50 | Default |  |  | QPSK | 220@0 (A7) |  | 110@0 (A7) | 55@0 (A7) |
| 20 | Default | 50 | Default |  |  | 256QAM | 220@0 (A7) |  | 110@0 (A7) | 55@0 (A7) |
| 21 | Default | 50 | Default |  |  | QPSK | Outer_Full (A8) |  |  |  |
| 22 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A8) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-25a: Test Configuration table for NS_46 (Power class 1)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 15 MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_46 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 15 | Default | N/A for A-MPR testing. | DFT-s-OFDM | 64QAM | Outer_Full (A1) |  |  |  |
| 2 | Default | 15 | Default |  |  | 256QAM | Edge_1RB_Right (A2) |  |  |  |
| 3 | Default | 50 | Default |  |  | 256QAM | 68@35 (A7) |  | 31@19 (A7) | 17@7(A7) |
| 4 | Default | 50 | Default |  |  | N/A | N/A (A11) |  |  |  |
| 5 | Default | 50 | Default |  |  | 256QAM | 146@20 (A9) |  | 74@10 (A9) | 38@5 (A9) |
| 6 | Default | 50 | Default |  |  | 64QAM | 10@260 (A12) |  | 10@123 (A12) | 10@55 (A12) |
| 7 | Default | 50 | Default |  |  | 256QAM | 28@160 (A8) |  | 14@80 (A8) | 7@40 (A8) |
| 8 | Default | 50 | Default |  |  | 256QAM | 12@40 (A9) |  | 6@20 (A9) | 3@10 (A9) |
| 9 | Default | 50 | Default |  |  | 256QAM | 57@20 (A7) |  | 28@10 (A7) | 14@5 (A7) |
| 10 | Default | 50 | Default |  |  | N/A | N/A (A11) |  |  |  |
| 11 | Default | 50 | Default |  |  | 256QAM | 195@0 (A8) |  | 97@0 (A8) | 48@0 (A8) |
| 12 | Default | 50 | Default |  |  | 64QAM | 122@117 (A12) |  | 61@59 (A12) | 30@30 (A12) |
| 13 | Default | 50 | Default |  |  | 256QAM | 16@179 (A8) |  | 8@90 (A8) | 4@45 (A8) |
| 14 | Default | 50 | Default |  |  | 256QAM | 11@195 (A9) |  | 5@98 (A9) | 2@49 (A9) |
| 15 | Default | 50 | Default |  |  | 64QAM | 229@34 (A13) |  | 115@16 (A13) | 57@9 (A13) |
| 16 | Default | 50 | Default |  |  | 256QAM | 46@0 (A7) |  | 23@0 (A7) | 11@0 (A7) |
| 17 | Default | 50 | Default |  |  | 256QAM | 17@195 (A8) |  | 9@97 (A8) | 5@48 (A8) |
| 18 | Default | 50 | Default |  |  | 256QAM | 11@201 (A9) |  | 5@101 (A9) | 2@51 (A9) |
| 19 | Default | 15 | Default |  | CP-OFDM | 64QAM | Outer_Full (A1) |  |  |  |
| 20 | Default | 15 | Default |  |  | 256QAM | Edge_1RB_Right (A2) |  |  |  |
| 21 | Default | 50 | Default |  |  | 256QAM | 68@35 (A7) |  | 31@19 (A7) | 17@7(A7) |
| 22 | Default | 50 | Default |  |  | 256QAM | 68@90(A11) |  | 31@45(A11) | 17@18(A11) |
| 23 | Default | 50 | Default |  |  | 256QAM | 146@20 (A9) |  | 74@10 (A9) | 38@5 (A9) |
| 24 | Default | 50 | Default |  |  | 64QAM | 10@260 (A12) |  | 10@123 (A12) | 10@55 (A12) |
| 25 | Default | 50 | Default |  |  | 256QAM | 28@160 (A8) |  | 14@80 (A8) | 7@40 (A8) |
| 26 | Default | 50 | Default |  |  | 256QAM | 12@40 (A9) |  | 6@20 (A9) | 3@10 (A9) |
| 27 | Default | 50 | Default |  |  | 256QAM | 57@20 (A7) |  | 28@10 (A7) | 14@5 (A7) |
| 28 | Default | 50 | Default |  |  | 256QAM | 57@87 (A11) |  | 28@54 (A7) | 14@22 (A7) |
| 29 | Default | 50 | Default |  |  | 256QAM | 195@0 (A8) |  | 97@0 (A8) | 48@0 (A8) |
| 30 | Default | 50 | Default |  |  | 64QAM | 122@117 (A12) |  | 61@59 (A12) | 30@30 (A12) |
| 31 | Default | 50 | Default |  |  | 256QAM | 16@179 (A8) |  | 8@90 (A8) | 4@45 (A8) |
| 32 | Default | 50 | Default |  |  | 256QAM | 11@195 (A9) |  | 5@98 (A9) | 2@49 (A9) |
| 33 | Default | 50 | Default |  |  | 64QAM | 229@34 (A13) |  | 115@16 (A13) | 57@9 (A13) |
| 34 | Default | 50 | Default |  |  | 256QAM | 46@0 (A7) |  | 23@0 (A7) | 11@0 (A7) |
| 35 | Default | 50 | Default |  |  | 256QAM | 17@195 (A8) |  | 9@97 (A8) | 5@48 (A8) |
| 36 | Default | 50 | Default |  |  | 256QAM | 11@201 (A9) |  | 5@101 (A9) | 2@51 (A9) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable Table from table 6.2.3.5-1 to Table 6.2.3.5-38. The allowed A-MPR values specified in Table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.1.3-1 apply.

Table 6.2.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

-------- Skipped unchanged clauses --------

Table 6.2.3.5-2: UE Power Class 2 test requirements (NS_04) for band n41 (Step 3)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 26 | 3.5 | 5.5 | 1.5 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 8 | 26 | 3.5 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 9 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 10 | 26 | 0.5 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 11 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 12 | 26 | 0.5 | 0 | 0 | 25.5 | 2.0 | 3 | 28+TT | 22.5-TT |
| 13 | 26 | 3.5 | 6 | 1.5 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 14 | 26 | 3.5 | 4.5 | 0 | 21 | 2.0 | 3 | 28+TT | 18.5-TT |
| 15 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 16 | 26 | 1 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 17 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 18 | 26 | 1 | 0 | 0 | 25 | 2.0 | 3 | 28+TT | 22-TT |
| 19 | 26 | 3.5 | 6 | 1.5 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 20 | 26 | 3.5 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 21 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 22 | 26 | 2 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 23 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 24 | 26 | 2 | 0 | 0 | 24 | 2.0 | 3 | 28+TT | 21-TT |
| 25 | 26 | 3.5 | 6.5 | 1.5 | 18 | 4.0 | 3 | 28+TT | 14-TT |
| 26 | 26 | 3.5 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 27 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 28 | 26 | 2.5 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 29 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 30 | 26 | 2.5 | 0 | 0 | 23.5 | 2.0 | 3 | 28+TT | 20.5-TT |
| 31 | 26 | 4.5 | 8 | 1.5 | 16.5 | 5.0 | 3 | 28+TT | 11.5-TT |
| 32 | 26 | 4.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 33 | 26 | 4.5 | 0 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 34 | 26 | 4.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 35 | 26 | 4.5 | 0 | 1.5 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 36 | 26 | 4.5 | 0 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 37 | 26 | 3.5 | 7.5 | 1.5 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 38 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 39 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 40 | 26 | 3 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 41 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 42 | 26 | 3 | 0 | 0 | 23 | 2.0 | 3 | 28+TT | 20-TT |
| 43 | 26 | 3.5 | 7.5 | 1.5 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 44 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 45 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 46 | 26 | 3 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 47 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 48 | 26 | 3 | 0 | 0 | 23 | 2.0 | 3 | 28+TT | 20-TT |
| 49 | 26 | 3.5 | 7.5 | 1.5 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 50 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 51 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 52 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 53 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 54 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 55 | 26 | 6.5 | 10 | 1.5 | 14.5 | 5.0 | 3 | 28+TT | 9.5-TT |
| 56 | 26 | 6.5 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 57 | 26 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 58 | 26 | 6.5 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 59 | 26 | 6.5 | 0 | 1.5 | 18 | 4.0 | 3 | 28+TT | 14-TT |
| 60 | 26 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Void NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-3: UE Power Class 3 test requirements (NS_04) for band n41 (Step 3) and UE Power Class 2 test requirements (NS_04) for band n41 (Step 4)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 3.5 | 3.5 | 1.5 | 21 | 2.0 | 2 | 28+TT | 19-TT |
| 2 | 23 | -3 | 3.5 | 3.5 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 3 | 23 | -3 | 3.5 | 0 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 4 | 23 | -3 | 1.2 | 3.5 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 5 | 23 | -3 | 3.5 | 0 | 1.5 | 21 | 2.0 | 2 | 28+TT | 19-TT |
| 6 | 23 | -3 | 1.2 | 0 | 0 | 24.8 | 2.0 | 2 | 28+TT | 22.8-TT |
| 7 | 23 | 0 | 0.5 | 3.5 | 1.5 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 8 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 9 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 2.0 | 2 | 25+TT | 20.5-TT |
| 10 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 11 | 23 | 0 | 0.5 | 0 | 1.5 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 12 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 2.0 | 2 | 25+TT | 20.5-TT |
| 13 | 23 | 0 | 1 | 4 | 1.5 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 14 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15 | 23 | 0 | 1 | 0 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 16 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 17 | 23 | 0 | 1 | 0 | 1.5 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 18 | 23 | 0 | 1 | 0 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 19 | 23 | 0 | 2 | 4 | 1.5 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 20 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 0 | 2 | 0 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 22 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | 0 | 2 | 0 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 24 | 23 | 0 | 2 | 0 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 25 | 23 | 0 | 2.5 | 4.5 | 1.5 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 26 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 27 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 28 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 29 | 23 | 0 | 2.5 | 0 | 1.5 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 30 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 31 | 23 | 0 | 4.5 | 6 | 1.5 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 32 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 33 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 34 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 35 | 23 | 0 | 4.5 | 0 | 1.5 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 36 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 37 | 23 | 0 | 3 | 5.5 | 1.5 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 38 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 40 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 41 | 23 | 0 | 3 | 0 | 1.5 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 42 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 43 | 23 | 0 | 3 | 5.5 | 1.5 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 44 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 45 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 46 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 47 | 23 | 0 | 3 | 0 | 1.5 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 48 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 49 | 23 | 0 | 3.5 | 5.5 | 1.5 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 50 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 51 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 52 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 53 | 23 | 0 | 3.5 | 0 | 1.5 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 55 | 23 | 0 | 6.5 | 8 | 1.5 | 13.5 | 5.0 | 2 | 25+TT | 8.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 57 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 58 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 59 | 23 | 0 | 6.5 | 0 | 1.5 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 60 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Void NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-3a: UE Power Class 1 test requirements (NS_04) for band n41 (Step 3)

| Test ID | PPowerClass (dBm) | MPR (dB) |  | A-MPR (dB) |  |  |  | ΔTC,c (dB) | PCMAX,c (dBm) |  | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | <50 MHz CBW | ≥50 MHz CBW | regrowth | IMD3 | CIM3 | edge |  | <50 MHz CBW | ≥50 MHz CBW |  |  |  | <50 MHz CBW | ≥50 MHz CBW |
| 1 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 11 | 0 | 20 | 20 | 2.5 | 3 | 33+TT | 17-TT | 17-TT |
| 2 | 31 | 0.5 | 0.5 | 0 | 2 | 0 | 0 | 0 | 29 | 29 | 2.5 | 3 | 33+TT | 26-TT | 26-TT |
| 3 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 4 | 31 | 0.5 | 0.5 | 0 | 0 | 0 | 0 | 0 | 32.5 | 32.5 | 2.5 | 3 | 33+TT | 29.5-TT | 29.5-TT |
| 5 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2 | 3 | 33+TT | 24-TT | 24-TT |
| 6 | 31 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 29 | 29 | 2.5 | 3 | 33+TT | 26-TT | 26-TT |
| 7 | 31 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 29 | 29 | 2.5 | 3 | 33+TT | 26-TT | 26-TT |
| 8 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 2 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 9 | 31 | 1 | 1 | 0 | 2 | 0 | 0 | 0 | 29 | 29 | 2.5 | 3 | 33+TT | 26-TT | 26-TT |
| 10 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21-4-TT | 19.5-TT |
| 11 | 31 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 30 | 30 | 2.5 | 3 | 33+TT | 27-TT | 27-TT |
| 12 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2 | 3 | 33+TT | 24-TT | 24-TT |
| 13 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 14 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 15 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 2.5 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 16 | 31 | 2 | 2 | 0 | 2.5 | 0 | 0 | 0 | 28.5 | 28.5 | 2.5 | 3 | 33+TT | 25.5-TT | 25.5-TT |
| 17 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 2 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 18 | 31 | 2 | 2 | 0 | 0 | 2 | 0 | 0 | 29 | 29 | 2.5 | 3 | 33+TT | 26-TT | 26-TT |
| 19 | 31 | 1 | 1 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2 | 3 | 33+TT | 24-TT | 24-TT |
| 20 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 21 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 22 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 3 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 23 | 31 | 2.5 | 2.5 | 0 | 3 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 24 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 3 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 25 | 31 | 2.5 | 2.5 | 0 | 0 | 3 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 26 | 31 | 2.5 | 2.5 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2 | 3 | 33+TT | 24-TT | 24-TT |
| 27 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 28 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 29 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 30 | 31 | 4.5 | 4.5 | 0 | 0 | 0 | 0 | 0 | 26.5 | 26.5 | 2.5 | 3 | 33+TT | 23.5-TT | 23.5-TT |
| 31 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 32 | 31 | 4.5 | 4.5 | 0 | 0 | 0 | 0 | 0 | 26.5 | 26.5 | 2.5 | 3 | 33+TT | 23.5-TT | 23.5-TT |
| 33 | 31 | 4.5 | 4.5 | 0 | 0 | 0 | 0 | 0 | 26.5 | 26.5 | 2 | 3 | 33+TT | 23.5-TT | 23.5-TT |
| 34 | 31 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 31 | 31 | 2 | 3 | 33+TT | 28-TT | 28-TT |
| 35 | 31 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 31 | 31 | 2 | 3 | 33+TT | 28-TT | 28-TT |
| 36 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 11 | 0 | 20 | 20 | 2.5 | 3 | 33+TT | 17-TT | 17-TT |
| 37 | 31 | 3 | 3 | 0 | 2.5 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 38 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 10.5 | 0 | 0 | 20.5 | 20.5 | 2.5 | 3 | 33+TT | 17.5-TT | 17.5-TT |
| 39 | 31 | 3 | 3 | 6 | 0 | 0 | 0 | 0 | 25 | 25 | 2 | 3 | 33+TT | 22-TT | 22-TT |
| 40 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 41 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 42 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 3 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 43 | 31 | 3 | 3 | 0 | 3 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 44 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 10.5 | 0 | 0 | 20.5 | 20.5 | 2.5 | 3 | 33+TT | 17.5-TT | 17.5-TT |
| 45 | 31 | 3 | 3 | 6 | 0 | 0 | 0 | 0 | 25 | 25 | 2 | 3 | 33+TT | 22-TT | 22-TT |
| 46 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 47 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 48 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 49 | 31 | 3.5 | 3.5 | 0 | 0 | 0 | 0 | 0 | 27.5 | 27.5 | 2.5 | 3 | 33+TT | 24.5-TT | 24.5-TT |
| 50 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 10.5 | 0 | 0 | 20.5 | 20.5 | 2.5 | 3 | 33+TT | 17.5-TT | 17.5-TT |
| 51 | 31 | 3.5 | 3.5 | 6 | 0 | 0 | 0 | 0 | 25 | 25 | 2 | 3 | 33+TT | 22-TT | 22-TT |
| 52 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 53 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 54 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 55 | 31 | 6.5 | 6.5 | 0 | 0 | 0 | 0 | 0 | 24.5 | 24.5 | 2.5 | 3 | 33+TT | 21.5-TT | 21.5-TT |
| 56 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 11 | 0 | 0 | 20 | 20 | 2.5 | 3 | 33+TT | 17-TT | 17-TT |
| 57 | 31 | 6.5 | 6.5 | 0 | 0 | 0 | 0 | 0 | 24.5 | 24.5 | 2 | 3 | 33+TT | 21.5-TT | 21.5-TT |
| 58 | 31 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 31 | 31 | 2 | 3 | 33+TT | 28-TT | 28-TT |
| 59 | 31 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 31 | 31 | 2 | 3 | 33+TT | 28-TT | 28-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

Table 6.2.3.5-30: UE Power Class 3 test requirements for NS_46 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 2 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 3 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 4 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 2 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | 23 | 0 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 7 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 9 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 10 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 11 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 12 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 13 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 14 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 16 | 23 | 0 | 3.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 17 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 18 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 19 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 20 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 21 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 22 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-30a: UE Power Class 1 test requirements for NS_46 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31 | 0 | 2.5 | 3 | 0 | 28 | 2 | 3 | 33+TT | 26-TT |
| 2 | 31 | 0 | 6.5 | 7 | 0 | 24 | 2 | 3 | 33+TT | 22-TT |
| 3 | 31 | 0 | 4.5 | 12.5 | 0 | 18.5 | 4 | 3 | 33+TT | 14.5-TT |
| 4 | 31 | 0 | 4.5 | 0 | 0 | 26.5 | 2 | 3 | 33+TT | 24.5-TT |
| 5 | 31 | 0 | 4.5 | 7 | 0 | 24 | 2 | 3 | 33+TT | 22-TT |
| 6 | 31 | 0 | 2.5 | 4 | 0 | 27 | 2 | 3 | 33+TT | 25-TT |
| 7 | 31 | 0 | 4.5 | 6 | 0 | 25 | 2 | 3 | 33+TT | 23-TT |
| 8 | 31 | 0 | 4.5 | 7 | 0 | 24 | 2 | 3 | 33+TT | 22-TT |
| 9 | 31 | 0 | 4.5 | 12.5 | 0 | 18.5 | 4 | 3 | 33+TT | 14.5-TT |
| 10 | 31 | 0 | 4.5 | 0 | 0 | 26.5 | 2 | 3 | 33+TT | 24.5-TT |
| 11 | 31 | 0 | 4.5 | 6 | 0 | 25 | 2 | 3 | 33+TT | 23-TT |
| 12 | 31 | 0 | 2.5 | 4 | 0 | 27 | 2 | 3 | 33+TT | 25-TT |
| 13 | 31 | 0 | 4.5 | 6 | 0 | 25 | 2 | 3 | 33+TT | 23-TT |
| 14 | 31 | 0 | 4.5 | 7 | 0 | 24 | 2 | 3 | 33+TT | 22-TT |
| 15 | 31 | 0 | 2.5 | 4 | 0 | 27 | 2 | 3 | 33+TT | 25-TT |
| 16 | 31 | 0 | 4.5 | 12.5 | 0 | 18.5 | 4 | 3 | 33+TT | 14.5-TT |
| 17 | 31 | 0 | 4.5 | 6 | 0 | 25 | 2 | 3 | 33+TT | 23-TT |
| 18 | 31 | 0 | 4.5 | 7 | 0 | 24 | 2 | 3 | 33+TT | 22-TT |
| 19 | 31 | 0 | 3.5 | 4 | 0 | 27 | 2 | 3 | 33+TT | 25-TT |
| 20 | 31 | 0 | 6.5 | 13 | 0 | 18 | 4 | 3 | 33+TT | 14-TT |
| 21 | 31 | 0 | 6.5 | 15 | 0 | 16 | 5 | 3 | 33+TT | 11-TT |
| 22 | 31 | 0 | 6.5 | 10 | 0 | 21 | 2 | 3 | 33+TT | 19-TT |
| 23 | 31 | 0 | 4.5 | 13 | 0 | 18 | 4 | 3 | 33+TT | 14-TT |
| 24 | 31 | 0 | 3.5 | 5 | 0 | 26 | 2 | 3 | 33+TT | 24-TT |
| 25 | 31 | 0 | 6.5 | 9 | 0 | 22 | 2 | 3 | 33+TT | 20-TT |
| 26 | 31 | 0 | 6.5 | 13 | 0 | 18 | 4 | 3 | 33+TT | 14-TT |
| 27 | 31 | 0 | 6.5 | 15 | 0 | 16 | 5 | 3 | 33+TT | 11-TT |
| 28 | 31 | 0 | 6.5 | 10 | 0 | 21 | 2 | 3 | 33+TT | 19-TT |
| 29 | 31 | 0 | 6.5 | 9 | 0 | 22 | 2 | 3 | 33+TT | 20-TT |
| 30 | 31 | 0 | 3.5 | 5 | 0 | 26 | 2 | 3 | 33+TT | 24-TT |
| 31 | 31 | 0 | 6.5 | 9 | 0 | 22 | 2 | 3 | 33+TT | 20-TT |
| 32 | 31 | 0 | 6.5 | 13 | 0 | 18 | 4 | 3 | 33+TT | 14-TT |
| 33 | 31 | 0 | 3.5 | 4 | 0 | 27 | 2 | 3 | 33+TT | 25-TT |
| 34 | 31 | 0 | 6.5 | 15 | 0 | 16 | 5 | 3 | 33+TT | 11-TT |
| 35 | 31 | 0 | 6.5 | 9 | 0 | 22 | 2 | 3 | 33+TT | 20-TT |
| 36 | 31 | 0 | 6.5 | 13 | 0 | 18 | 4 | 3 | 33+TT | 14-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

6.5.3.3.3.25	Minimum conformance requirements (network signalling value "NS_46")

When "NS_46" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.25-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.25-1: Additional requirements for “NS_46”

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1, 2 |
| Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1, 2 |
| Frequency range | 2595 | - | 2620 | -40 | 1 | 1 |
| NOTE 1: This requirement is applicable for power class 3 UE for all carriers confined in 2500-2570 MHz. Special restrictions apply for channel bandwidths up to 20MHz: For carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB with the minimum supported SCS of 15KHz. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5.3.3.25.

-------- Skipped unchanged clauses --------

6.5.3.3.5.25	Test requirement (network signalling value “NS_46”)

When "NS_46" is indicated in the cell, the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3.5-30. The power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.5.25-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5.3.3.25-1: Additional requirements for “NS_46”

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1, 2 |
| Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1, 2 |
| Frequency range | 2595 | - | 2620 | -40 | 1 | 1 |
| NOTE 1: This requirement is applicable for power class 3 UE for all carriers confined in 2500-2570 MHz. Sepcial restrictions apply for channel bandwidths up to 20MHz: For carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB with the minimum supported SCS of 15KHz. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

##### 6.5.2.4.2	UTRA ACLR

6.5.2.4.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5.2.4.2.2	Test applicability

This test case applies for network signalling values NS_03U, NS_05U, NS_43U, and NS_100 to all types of NR Power Class 3 UE release 15 and forward that don’t support Tx diversity.

6.5.2.4.2.3	Minimum conformance requirements

UTRA adjacent channel leakage power ratio (UTRAACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent(s) UTRA channel frequency.

UTRAACLR is specified for the first adjacent UTRA channel (UTRAACLR1) which centre frequency is ± 2.5 MHz from NR channel edge and for the 2nd adjacent UTRA channel (UTRAACLR2) which centre frequency is ± 7.5 MHz from NR channel edge.

The UTRA channel power is measured with an RRC filter with roll-off factor =0.22 and bandwidth of 3.84 MHz. The assigned NR channel power is measured with a rectangular filter with measurement bandwidth specified in Table 6.5.2.4.1.3-1.

If the measured adjacent channel power is greater than - 50dBm then the UTRAACLR1 and UTRAACLR2 shall be higher than the value specified in Table 6.5.2.4.2.3-1.

UTRAACLR is not applicable to the power class 3 UE operating in Band n12, n14, n17, and n30.

UTRAACLR is not applicable to the power class 1 UE operating in Band n7, n14, n78, n100 and n101.

Table 6.5.2.4.2.3-1: UTRA ACLR requirement

|  | Power class 3 |
| --- | --- |
| UTRAACLR1 | 33 dB |
| UTRAACLR2 | 36 dB |

UTRA ACLR requirement is applicable when signalled by the network with network signalling value indicated by the field additionalSpectrumEmission.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.2.4.2.

-------- End of changes --------