PP TSG-RAN5 Meeting # 103	R5-242546r2

# Fukuoka City, Fukuoka, JP,  20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3143 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to SIB19 and ephemeris information for NGSO and GSO condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Ireland, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SIB19 has missing information for GSO and NGSO and ephemeris information missing for NGSO test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to SIB19 and added ephemeris information NR-NTN test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NTN test cases will not be implementatble |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 4.6.3-56C, Table 4.6.3-84C, 6.3.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Resized bottom row of table 6.3.4.2.-1 to make note1 visible. Added MediaTek as co-source. Content of the Table 4.6.3-84C: NTN-Config r2: changed to use reference instead of using direct value |  |  |  |  |  |  |  |  |

<Beginning of changes>

<Unchanged sections omitted >

#### –	SIB19

Table 4.6.2-18C: SIB19

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 | NTN-Config with condition GSO |  | GSO |
|  | NTN-Config with condition NGSO |  | NGSO |
| t-Service-r17 | Not Present |  |  |
| referenceLocation-r17 | Not present |  |  |
| distanceThresh-r17 | Not present |  |  |
| ntn-NeighCellConfigList-r17 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

<Unchanged sections omitted >

#### –	EphemerisInfo

Table 4.6.3-56C: EphemerisInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  | GSO |
| positionX-r17 | Set to a value defined in Table 6.3.4.1-1. |  |  |
| positionY-r17 | Set to a value defined in Table 6.3.4.1-1. |  |  |
| positionZ-r17 | 0 |  |  |
| velocityVX-r17 | 0 |  |  |
| velocityVY-r17 | 0 |  |  |
| velocityVZ-r17 | 0 |  |  |
| } |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  | NGSO |
| positionX-r17 | Set to a value defined in Table 6.3.4.2-1. |  |  |
| positionY-r17 | Set to a value defined in Table 6.3.4.2-1. |  |  |
| positionZ-r17 | Set to a value defined in Table 6.3.4.2-1. |  |  |
| velocityVX-r17 | Set to a value defined in Table 6.3.4.2-1. |  |  |
| velocityVY-r17 | Set to a value defined in Table 6.3.4.2-1. |  |  |
| velocityVZ-r17 | Set to a value defined in Table 6.3.4.2-1. |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

<Unchanged sections omitted >

#### –	NTN-Config

Table 4.6.3-84C: NTN-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NTN-Config-r17 ::= SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ntn-UlSyncValidityDuration-r17 | s240 |  |  |
| cellSpecificKoffset-r17 | Set to a value defined in Table 6.3.4.1-1. |  | GSO |
|  | Set to a value defined in Table 6.3.4.2-1. |  | NGSO |
| kmac-r17 | Not present |  | GSO |
|  | Set to a value defined in Table 6.3.4.2-1. |  | NGSO |
| ta-Info-r17 SEQUENCE { |  |  |  |
| ta-Common-r17 | Set to a value defined in Table 6.3.4.1-1. |  | GSO |
|  | 0 |  | NGSO |
| ta-CommonDrift-r17 | 0 |  |  |
| ta-CommonDriftVariant-r17 | 0 |  |  |
| } |  |  |  |
| ntn-PolarizationDL-r17 | Not present |  |  |
| ntn-PolarizationUL-r17 | Not present |  |  |
| ephemerisInfo-r17 | EphemerisInfo |  |  |
| ta-Report-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

<Unchanged sections omitted >

### 6.3.4	Satellite ephemeris information for NTN

#### 6.3.4.1	GSO scenario

The satellite ephemeris information of GSO is defined in Table 4.6.3-56C for NTN with below assumption:

-	Inclination: 0

-	Eccentricity: 0

-	Latitude: 0

-	Longitude: 121.56076999

-	Altitude: 35786 km

-	Speed: 0 km/s

In case of multi-cell NTN GSO scenario, a maximum of 3 NTN cells can be used. The satellite ephemeris information is defined in Table 6.3.4.1-1 unless otherwise specified in a test case.

Table 6.3.4.1-1: Satellite ephemeris information in multi-cell NTN GSO scenario

| Cell type (Note 1) | positionX | positionY | positionZ | velocityVX/Y/Z | nta-Common | Koffset |
| --- | --- | --- | --- | --- | --- | --- |
| First | -16976014 | 27636499 | 0 | 0 | 65896378 | 550 |
| Second | -23550424 | 22301093 | 0 | 0 | 66485757 | 550 |
| Third | -14502736 | 29010891 | 0 | 0 | 66485757 | 550 |
| Note 1: First Cell is the cell defined as default serving cell in the test case at the end of the preamble. Second Cell and third Cell are used in multi-cell test cases with Second Cell being defined as the 1st neighbour cell and Third Cell being defined as the 2nd neighbour cell. For example, in TS 38.523-1[12] TC 9.4.1.1 running in GSO scenario, NGC Cell A is the serving cell in the preamble, so it uses the satellite ephemeris information of First Cell, while NGC Cell G uses the satellite ephemeris information of Second Cell |  |  |  |  |  |  |

#### 6.3.4.2	NGSO scenario

The satellite ephemeris information of NGSO is defined in Table 4.6.3-56C for NTN with below assumption:

-	Inclination: 90

-	Eccentricity: 0

-	Longitude: 121.56076999

-	Altitude: 600 km

-	Speed: 7.56 km/s

The UE position defined in clause 4.5.C is the projection position from one of the satellite information to the ground.

All NTN test cases defined in TS 38.523-1 [12], unless otherwise stated, shall use the first row of Table 6.3.4.2-1 as the ephemeris configuration of the serving cell, corresponding to zero Doppler and constant delay for NGSO scenario, keeping such information constant throughout the duration of each measurement in the test.

In case of multi-cell NTN NGSO scenario, a maximum of 3 NTN cells can be used. The satellite ephemeris information is defined in Table 6.3.4.2-1 unless otherwise specified in a test case, keeping such information constant throughout the duration of each measurement in the test.

Table 6.3.4.2-1: Satellite ephemeris information in multi-cell NTN NGSO scenario

| Cell type (Note 1) | positionX | positionY | positionZ | velocityVX | velocityVY | velocityVZ | k-Mac | k-Offset | Fd (Note 2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First | -2550081 | 4151097 | 2263034 | 21970 | -48164 | 114109 | 3 | 5 | 0.013 |
| Second | -2654549 | 4386991 | 1594205 | 14581 | -34487 | 120182 | 4 | 8 | 19.83 |
| Third | -2401021 | 3830217 | 2907041 | 28879 | -61410 | 105774 | 4 | 8 | -19.95 |
| Note 1: First Cell is the cell defined as default serving cell in the test case at the end of the preamble. Second Cell and Third Cell are used in multi-cell test cases with Second Cell being defined as the 1st neighbour cell and Third Cell being defined as the 2nd neighbour cell. For example, in TS 38.523-1[12] TC 9.4.1.1 running in NGSO scenario, NGC Cell A is the serving cell in the preamble, so it uses the satellite ephemeris information of First Cell, while NGC Cell G uses the satellite ephemeris information of Second Cell Note 2: Fd is the frequency Doppler in the service link, unit is ppm. The values are not broadcasted but resemble those that are calculated at the UE based on the corresponding cell satellite ephemeris information. They need to be considered for UL & DL transmission in SS side. |  |  |  |  |  |  |  |  |  |

<End of changes >