3GPP TSG-RAN5 Meeting # 103	R5-242550r2

# Fukuoka City, Fukuoka, Japan,  20th May 2024 – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4376 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new NR NTN conditional handover CondEventD1 test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Ireland |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 05-08-2024 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of new NR NTN conditional handover CondEventD1 test case |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case addition under clause 8.1.4.4.7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NTN workplan will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.4.7 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.508-2 CR 0632 TS/TR 38.523-2 CR 0470 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-242550r1 – “NTN” is added to the test case titile is added to the test case titile Table 8.1.4.4.7.3.2-1 is removed R5-242550r2 – NR cell ID changed to NR Cell 4 |  |  |  |  |  |  |  |  |

<Beginning of changes>

##### 8.1.4.4.7	Conditional handover / NTN / Location based / CondEventD1

8.1.4.4.7.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and supporting location based conditional handover and receiving an RRCReconfiguration message including ConditionalReconfiguration }

ensure that {

when { conditional handover execution condition is not satisfied }
    then { UE maintains connection with source gNB and starts evaluating the CHO execution conditions for the candidate cell(s) }

}

(2)

with { UE in NR RRC_CONNECTED state and supporting location based conditional handover }

ensure that {

when { UE receives an RRCReconfiguration message including ConditionalReconfiguration and condEvent D1 is configured as conditional handover trigger event }
    then { UE starts evaluating the conditional handover execution condition and perform conditional handover to the neighbour cell triggered in conditional configuration execution }

}

(3)

with { UE in NR RRC_CONNECTED state and supporting location based conditional handover }

ensure that {

when { UE perform conditional handover procedure successfully }
    then { UE releases stored CHO configurations }

}

8.1.4.4.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.300 9.2.3.4.1, and 9.2.3.4.2, and TS 38.331 clause 5.3.5.1, 5.3.5.13.1, 5.3.5.13.4. Unless otherwise stated these are Rel-17 requirements.

[TS 38.300, clause 9.2.3.4.1]

A Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met. The UE starts evaluating the execution condition(s) upon receiving the CHO configuration, and stops evaluating the execution condition(s) once a handover is executed.

The following principles apply to CHO:

-	The CHO configuration contains the configuration of CHO candidate cell(s) generated by the candidate gNB(s) and execution condition(s) generated by the source gNB.

-	An execution condition may consist of one or two trigger condition(s) (CHO events A3/A5, as defined in [12]). Only single RS type is supported and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously for the evalution of CHO execution condition of a single candidate cell.

-	Before any CHO execution condition is satisfied, upon reception of HO command (without CHO configuration), the UE executes the HO procedure as described in clause 9.2.3.2, regardless of any previously received CHO configuration.

-	While executing CHO, i.e. from the time when the UE starts synchronization with target cell, UE does not monitor source cell.

CHO is also supported for the IAB-MT in context of intra- and inter-donor IAB-node migration and BH RLF recovery.

CHO is not supported for NG-C based handover in this release of the specification.

[TS 38.331, clause 5.3.5.1]

Figure 5.3.5.1-1: RRC reconfiguration, successful

Figure 5.3.5.1-2: RRC reconfiguration, failure

The purpose of this procedure is to modify an RRC connection, e.g. to establish/modify/release RBs, to perform reconfiguration with sync, to setup/modify/release measurements, to add/modify/release SCells and cell groups, to add/modify/release conditional handover configuration, to add/modify/release conditional PSCell change configuration. As part of the procedure, NAS dedicated information may be transferred from the Network to the UE.

[TS 38.331, clause 5.3.5.13.1]

The network configures the UE with one or more candidate target SpCells in the conditional reconfiguration. The UE evaluates the condition of each configured candidate target SpCell. The UE applies the conditional reconfiguration associated with one of the target SpCells which fulfils associated execution condition. The network provides the configuration parameters for the target SpCell in the ConditionalReconfiguration IE.

The UE performs the following actions based on a received ConditionalReconfiguration IE:

1>	if the ConditionalReconfiguration contains the condReconfigToRemoveList:

2>	perform conditional reconfiguration removal procedure as specified in 5.3.5.13.2;

1>	if the ConditionalReconfiguration contains the condReconfigToAddModList:

2>	perform conditional reconfiguration addition/modification as specified in 5.3.5.13.3;

[TS 38.331, clause 5.3.5.13.1]

The network configures the UE with one or more candidate target SpCells in the conditional reconfiguration. The UE evaluates the condition of each configured candidate target SpCell. The UE applies the conditional reconfiguration associated with one of the target SpCells which fulfils associated execution condition. The network provides the configuration parameters for the target SpCell in the ConditionalReconfiguration IE.

The UE performs the following actions based on a received ConditionalReconfiguration IE:

1>	if the ConditionalReconfiguration contains the condReconfigToRemoveList:

2>	perform conditional reconfiguration removal procedure as specified in 5.3.5.13.2;

1>	if the ConditionalReconfiguration contains the condReconfigToAddModList:

2>	perform conditional reconfiguration addition/modification as specified in 5.3.5.13.3;

[TS 38.331, clause 5.3.5.13.4]

The UE shall:

1>	for each condReconfigId within the VarConditionalReconfig:

2>	if the RRCReconfiguration within condRRCReconfig includes the masterCellGroup including the reconfigurationWithSync:

3>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the masterCellGroup in the received condRRCReconfig to be applicable cell;

2>	else if the RRCReconfiguration within condRRCReconfig includes the secondaryCellGroup including the reconfigurationWithSync:

3>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the secondaryCellGroup within the received condRRCReconfig to be applicable cell;

2>	if condExecutionCondSCG is configured:

3>	in the remainder of the procedure, consider each measId indicated in the condExecutionCondSCG as a measId in the VarMeasConfig associated with the SCG measConfig;

2>	if condExecutionCond is configured:

3>	if it is configured via SRB3 or configured within nr-SCG or within nr-SecondaryCellGroupConfig (specified in TS 36.331[10]) via SRB1:

4>	in the remainder of the procedure, consider each measId indicated in the condExecutionCond as a measId in the VarMeasConfig associated with the SCG measConfig;

3>	else:

4>	in the remainder of the procedure, consider each measId indicated in the condExecutionCond as a measId in the VarMeasConfig associated with the MCG measConfig;

2>	for each measId included in the measIdList within VarMeasConfig indicated in the condExecutionCond or condExecutionCondSCG associated to condReconfigId:

3>	if the condEventId is associated with condEventT1, and if the entry condition applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell; or

3> if the condEventId is associated with condEventD1, and if the entry conditions applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig; or

3>	if the condEventId is associated with condEventA3, condEventA4 or condEventA5, and if the entry condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

4>	consider the event associated to that measId to be fulfilled;

3>	if the measId for this event associated with the condReconfigId has been modified; or

3>	if the condEventId is associated with condEventT1, and if the leaving condition applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell; or

3> if the condEventId is associated with condEventD1, and if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig; or

3>	if the condEventId is associated with condEventA3, condEventA4 or condEventA5, and if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

4>	consider the event associated to that measId to be not fulfilled;

2>	if event(s) associated to all measId(s) within condTriggerConfig for the applicable cell are fulfilled:

3>	consider the applicable cell, associated to that condReconfigId, as a triggered cell;

3>	initiate the conditional reconfiguration execution, as specified in 5.3.5.13.5;

NOTE 1:	Up to 2 MeasId can be configured for each condReconfigId. The conditional reconfiguration event of the 2 MeasId may have the same or different event conditions, triggering quantity, time to trigger, and triggering threshold.

NOTE 2:	Void.

[TS 38.331, Clause 5.5.4.15]

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition D1-1 and condition D1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition D1-3 or condition D1-4, i.e. at least one of the two, as specified below, are fulfilled;

Inequality D1-1 (Entering condition 1)

Ml1-Hys>Thresh1

Inequality D1-2 (Entering condition 2)

Ml2+Hys<Thresh2

Inequality D1-3 (Leaving condition 1)

Ml1+Hys<Thresh1

Inequality D1-4 (Leaving condition 2)

Ml2-Hys>Thresh2

The variables in the formula are defined as follows:

Ml1 is the distance between UE and a reference location for this event (i.e. referenceLocation1 as defined within reportConfigNR for this event), not taking into account any offsets.

Ml2 is the distance between UE and a reference location for this event (i.e. referenceLocation2 as defined within reportConfigNR for this event), not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresisLocation as defined within reportConfigNR for this event).

Thresh1 is the threshold for this event defined as a distance, configured with parameter distanceThreshFromReference1, from a reference location configured with parameter referenceLocation1 within reportConfigNR for this event.

Thresh2 is the threshold for this event defined as a distance, configured with parameter distanceThreshFromReference2, from a reference location configured with parameter referenceLocation2 within reportConfigNR for this event.

Ml1 is expressed in meters.

Ml2 is expressed in the same unit as Ml1.

Hys is expressed in the same unit as Ml1.

Thresh1 is expressed in the same unit as Ml1.

Thresh2 is expressed in the same unit as Ml1.

NOTE:	The definition of Event D1 also applies to CondEvent D1.

8.1.4.4.7.3	Test description

8.1.4.4.7.3.1	Pre-test conditions

System Simulator:

3 NR cells: NR Cell 1, NR Cell2, and NR Cell 4 as specified in TS 38.508-1 [4] Table 4.4.2.1.

System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

8.1.4.4.7.3.2	Test procedure sequence

Table 8.1.4.4.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 2 and NR Cell 4 as target candidate cell and configure condEvent D1 as trigger event on NR cell 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message in NR cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | Check: Does the UE initiate a random access procedure transmitting Preamble on NR Cell 2 within 10s? | --> | (PRACH Preamble) | 1 | F |
| 4 | The SS transmits Update UE location information message (Note 1) | <-- | UPDATE UE LOCATION INFORMATION | - | - |
| 5 | Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 2 within 10s? | --> | (PRACH Preamble) | 2 | P |
| 6 | The SS transmits Random Access Response. | <-- | Random Access Response | - | - |
| 7 | The UE transmits RRCReconfigurationComplete message in NR Cell 2. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 8 | Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 2? | - | - | 2 | - |
| 9 | The SS transmits Update UE location information message (Note 2). | <-- | UPDATE UE LOCATION INFORMATION | - | - |
| 10 | Check: Does the UE initiate a random access procedure transmitting Preamble in NR Cell 4 within 10s? | --> | (PRACH Preamble) | 3 | F |
| Note 1: UE location information is configured such that the conditional event D1 is satisfied for NR Cell 2 Note 2: UE location information is configured such that the conditional event D1 is satisfied for NR Cell 4 |  |  |  |  |  |

8.1.4.4.7.3.3	Specific message contents

Table 8.1.4.4.7.3.3-1: RRCReconfiguration (Step 1, Table 8.1.4.4.7.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR_MEAS and CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | MeasConfig | Table 8.1.4.4.7.3.3-6 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| conditionalReconfiguration-r16 | ConditionalReconfiguration | Table 8.1.4.4.7.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.7.3.3-2: ConditionalReconfiguration (Table 8.1.4.4.7.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-25D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16::= SEQUENCE { |  |  |  |
| condReconfigToAddModList-r16 | CondReconfigToAddModList | Table 8.1.4.4.7.3.3-3 |  |
| } |  |  |  |

Table 8.1.4.4.7.3.3-3: CondReconfigToAddModList (Table 8.1.4.4.7.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-25C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 { | 2 entries |  |  |
| CondReconfigToAddMod-r16[1] ::= SEQUENCE { |  | entry 1 |  |
| condReconfigId-r16 | 1 |  |  |
| condExecutionCond-r16::= SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| CondReconfigToAddMod-r16[1] ::= SEQUENCE { |  | entry 1 |  |
| condRRCReconfig-r16 | RRCReconfiguration-HO with condition HO_NR cell 2 | Table 8.1.4.4.7.3.3-4 |  |
| } |  |  |  |
| CondReconfigToAddMod-r16[2] ::= SEQUENCE { |  | entry 2 |  |
| condReconfigId-r16 | 2 |  |  |
| condExecutionCond-r16::= SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| condRRCReconfig-r16 | RRCReconfiguration-HO with condition HO_NR cell 4 | Table 8.1.4.4.7.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.7.3.3-4: RRCReconfiguration-HO (Table 8.1.4.4.7.3.3-3)

| Derivation Path: TS 38.508-1 [4] Table 4.8.1-1A with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig with condition PCI_NR cell 1 | Table 8.1.4.4.7.3.3-5 | HO_NR cell 1 |
|  | CellGroupConfig with condition PCI_NR cell 2 | Table 8.1.4.4.7.3.3-5 | HO_NR cell 2 |
|  | CellGroupConfig with condition PCI_NR cell 4 | Table 8.1.4.4.7.3.3-5 | HO_NR cell 4 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.1.4.4.7.3.3-5: CellGroupConfig (Table 8.1.4.4.7.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with Condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 1 |  | PCI_NR cell 1 |
|  | Physical Cell Identity of NR Cell 2 |  | PCI_NR Cell 2 |
|  | Physical Cell Identity of NR Cell 4 |  | PCI_NR Cell 4 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.7.3.3-6: MeasConfig (Table 8.1.4.4.7.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 8.1.4.4.7.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-condEventD1 | Table 8.1.4.4.7.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.7.3.3-7: MeasObjectNR (Table 8.1.4.4.7.3.3-6)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.1.4.4.7.3.3-8: ReportConfigNR-condEventD1 (Table 8.1.4.4.7.3.3-6) - FFS

<End of Changes >