3GPP TSG-RAN5 Meeting #103	R5-242573

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2776 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General updates of TS 38.521-1 clause 5 for R17 CA configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, Verizon, KDDI, WE Certification Oy, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some R17 NR CA combos which are already 100% completion in RAN4 and assigned to interested companies need to be added into operating bands and channel bandwidth of TS38.521-1 clause 5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of the following CA combos configurations in clause 5: - Intra-band CA_n5B - two bands: CA_n5B-n77A, CA_n5B-n77C, CA_n2A-n30A, CA_n5A-n30A, CA_n30A-n66A and CA_n30A-n77A - three bands: CA_n2A-n5A-n48B, CA_n2A-n5A-n48(2A), CA_n2A-n48B-n66A, CA_n2A-n48(2A)-n66A, CA_n2A-n48B-n77A, CA_n2A-n48(2A)-n77A, CA_n5A-n48B-n66A, CA_n5A-n48(2A)-n66A, CA_n5A-n48B-n77A, CA_n5A-n48(2A)-n77A, CA_n48B-n66A-n77A and CA_n48(2A)-n66A-n77A, - four bands: CA_n3A-n28A-n41A-n77A, CA_n2A-n5A-n48B-n66A, CA_n2A-n5A-n48B-n66A, CA_n2A-n5A-n48A-n77C, CA_n2A-n48A-n66A-n77C and CA_n5A-n48A-n66A-n77C - Update CA_n2A-n5A-n48A-n66A in Table 5.5A.3.3-1 to align the table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The activated NR CA configurations in clause 5 will be still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.1 5.2A.2.1 5.2A.2.3 5.5A.1 5.5A.3.1 5.5A.3.2 5.5A.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is still including BCS1 of CA_n5B under the WIC NR_CADC_NR_LTE_DC_R18-UEConTest. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Hide the bottom border of the cell for CA_n5B in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |

<< Start of changes >>

5.2A	Operating bands for CA

5.2A.1	Intra-band CA

NR intra-band contiguous carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.1-1 and Table 5.2A.1-2, where all operating bands are within FR1.

Table 5.2A.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n5 | n5 |
| CA_n40 | n40 |
| CA_n411 | n41 |
| CA_n461 | n46 |
| CA_n48 | n48 |
| CA_n66 | n66 |
| CA_n771 | n77 |
| CA_n781 | n78 |
| CA_n791 | n79 |
| NOTE 1: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. |  |

Table 5.2A.1-2: Intra-band non-contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n2(*) | n2 |
| CA_n48(*) | n48 |
| CA_n66(*) | n66 |
| CA_n71(*) | n71 |
| CA_n77(*) | n77 |
| CA_n78(*) | n78 |
| NOTE 1: The minimum requirements only apply for non simultaneous Tx/Rx between all carriers. for TDD combinations. NOTE 2: The notation CA_nX(*) in this table indicates intra-band non-contiguous CA for band nX. The configurations for each band are in 5.5A.2. |  |

5.2A.2	Inter-band CA

NR inter-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.2.1-1, 5.2A.2.2-1 and Table 5.2A.2.3-1, where all operating bands are within FR1.

If the mandatory simultaneous Rx/Tx capability applies for a lower order band combination, when the applicable lower order band combination is a band pair in a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for the band pair in the higher order band combination.

Table 5.2A.2-1: Void

Table 5.2A.2-2: Void

Table 5.2A.2-3: Void

5.2A.2.1	Inter-band CA (two bands)

Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n1-n3 | n1, n3 |  |
| CA_n1-n8 | n1, n8 |  |
| CA_n1-n28 | n1, n28 |  |
| CA_n1-n411 | n1, n41 |  |
| CA_n1-n771 | n1, n77 | No |
| CA_n1-n781 | n1, n78 | No |
| CA_n1-n791 | n1, n79 | No |
| CA_n2-n5 | n2, n5 |  |
| CA_n2-n14 | n2, n14 |  |
| CA_n2-n30 | n2, n30 |  |
| CA_n2-n48 | n2, n48 |  |
| CA_n2-n66 | n2, n66 |  |
| CA_n2-n77 | n2, n77 |  |
| CA_n3-n5 | n3, n5 |  |
| CA_n3-n8 | n3, n8 |  |
| CA_n3-n28 | n3, n28 |  |
| CA_n3-n411 | n3, n41 | No |
| CA_n3-n771 | n3, n77 | No |
| CA_n3-n781 | n3, n78 | No |
| CA_n5-n7 | n5, n7 |  |
| CA_n5-n30 | n5, n30 |  |
| CA_n5-n48 | n5, n48 |  |
| CA_n5-n66 | n5, n66 |  |
| CA_n5-n771 | n5, n77 |  |
| CA_n5-n781 | n5, n78 | No |
| CA_n7-n781 | n7, n78 |  |
| CA n8-n781 | n8, n78 | No |
| CA_n14-n30 | n14, n30 |  |
| CA_n14-n66 | n14, n66 |  |
| CA_n14-n77 | n14, n77 |  |
| CA_n20-n78 | n20, n78 |  |
| CA_n24-n41 | n24, n41 |  |
| CA_n24-n48 | n24, n48 |  |
| CA_n24-n77 | n24, n77 |  |
| CA_n25-n66 | n25, n66 |  |
| CA_n25-n77 | n25, n77 |  |
| CA_n25-n78 | n25, n78 |  |
| CA_n25-n466 | n25, n46 |  |
| CA_n26-n66 | n26, n66 |  |
| CA_n26-n70 | n26, n70 |  |
| CA_n28-n411 | n28, n41 |  |
| CA_n28-n771 | n28, n77 | No |
| CA_n28_n781 | n28, n78 | No |
| CA_n28-n791 | n28, n79 |  |
| CA_n29-n66 | n29, n66 |  |
| CA_n29-n70 | n29, n70 |  |
| CA_n29-n71 | n29, n71 |  |
| CA_n30-n66 | n30, n66 |  |
| CA_n30-n77 | n30, n77 |  |
| CA_n39-n41 | n39, n41 | No |
| CA_n41-n66 | n41, n66 |  |
| CA_n41-n711 | n41, n71 |  |
| CA_n41-n771 | n41, n77 |  |
| CA_n41-n791,3 | n41, n79 | No |
| CA_n46-n481,6 | n46, n48 |  |
| CA_n46-n666 | n46, n66 |  |
| CA_n48-n66 | n48, n66 |  |
| CA_n48-n70 | n48, n70 |  |
| CA_n48-n71 | n48, n71 |  |
| CA_n48-n7713,14,18 | n48, n77 |  |
| CA_n66-n70 | n66, n70 |  |
| CA_n66-n71 | n66, n71 |  |
| CA_n66-n77 | n66, n77 |  |
| CA_n66-n78 | n66, n78 |  |
| CA_n70-n71 | n70, n71 |  |
| CA_n71-n77 | n70, n71 |  |
| CA_n71-n78 | n71, n78 |  |
| CA_n77-n795,7 | n77, n79 |  |
| CA_n78-n795 | n78, n79 |  |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. NOTE 2: FFS. NOTE 3: The frequency range below 2506MHz for Band n41 is not used in this combination. NOTE 4: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 5: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 6: The PCell is allocated in the licensed band in this combination. NOTE 7: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 8: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. NOTE 9: FFS NOTE 10 FFS NOTE 11: FFS NOTE 12: FFS NOTE 13: Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n48 with an n77 implementation. NOTE 14: The band n48 and n77 will synchronize their uplink and downlink configurations and in commonly TDD network coordination NOTE 15: FFS NOTE 16: FFS NOTE 17: FFS NOTE 18: The minimum requirements for inter-band CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable CA configuration is a subset of a higher order CA configuration. |  |  |

5.2A.2.2	Inter-band CA (three bands)

Table 5.2A.2.2-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_n1-n3-n28 | n1, n3, n28 |  |
| CA_n1-n3-n77 | n1, n3, n77 |  |
| CA_n1-n3-n783 | n1, n3, n78 | No for CA_n1-n78, CA_n3-n78 |
| CA_n1-n8-n783 | n1, n8, n78 |  |
| CA_n1-n28-n773 | n1, n28, n77 |  |
| CA_n1-n28-n783 | n1, n28, n78 |  |
| CA_n1-n41-n773 | n1, n41, n77 |  |
| CA_n1-n78-n79 | n1, n78, n79 |  |
| CA_n2-n5-n48 | n2, n5, n48 |  |
| CA_n2-n5-n77 | n2, n5, n77 |  |
| CA_n2-n48-n66 | n2, n48, n66 |  |
| CA_n2-n48-n77 | n2, n48, n77 |  |
| CA_n2-n66-n77 | n2, n66, n77 |  |
| CA_n3-n28-n413 | n3, n28, n41 |  |
| CA_n3-n28-n773 | n3, n28, n77 |  |
| CA_n3-n28-n783 | n3, n28, n78 |  |
| CA_n3-n41-n773 | n3, n41, n77 |  |
| CA_n5-n48-n66 | n5, n48, n66 |  |
| CA_n5-n48-n77 | n5, n48, n77 |  |
| CA_n5-n66-n77 | n5, n66, n77 |  |
| CA_n25-n66-n77 | n25, n66, n77 |  |
| CA_n25-n66-n78 | n25, n66, n78 |  |
| CA_n26-n66-n70 | n26, n66, n70 |  |
| CA_n28-n41-n793 | n28, n41, n79 |  |
| CA_n29-n66-n70 | n29, n66, n70 |  |
| CA_n41-n66-n71 | n41, n66, n71 |  |
| CA_n48-n66-n70 | n48, n66, n70 |  |
| CA_n48-n66-n71 | n48, n66, n71 |  |
| CA_n48-n66-n77 | n48, n66, n77 |  |
| CA_n48-n70-n71 | n48, n70, n71 |  |
| CA_n66-n70-n71 | n66, n70, n71 |  |
| CA_n66-n71-n77 | n66, n71, n77 |  |
| CA_n66-n71-n78 | n66, n71, n78 |  |
| NOTE 1: FFS NOTE 2: FFS NOTE 3: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. NOTE 4: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

5.2A.2.3	Inter-band CA (four bands)

Table 5.2A.2.3-1: Inter-band CA operating bands involving FR1 (four bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1-n3-n28-n781 | n1, n3, n28, n78 |
| CA_n2-n5-n48-n66 | n2, n5, n48, n66 |
| CA_n2-n5-n48-n77 | n2, n5, n48, n77 |
| CA_n2-n5-n66-n77 | n2, n5, n66, n77 |
| CA_n2-n48-n66-n77 | n2, n48, n66, n77 |
| CA_n3-n28-n41-n77 | n3, n28, n41, n77 |
| CA_n5-n48-n66-n77 | n5, n48, n66, n77 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |

## <<Unchanged sections skipped>>

5.5A	Configurations for CA

5.5A.0	General

The configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said configuration for CA operating bands with Band n90 are not listed in the tables below but are covered by this specification.

Non-contiguous resource allocation and almost contiguous allocation are not applicable for each NR carrier of intra-band contiguous and non-contiguous CA configurations.

For a CA configuration with one or more operating band supporting asymmetric channel bandwidths as specified in sub-clause 5.3.6, requirements are defined for an asymmetric UL and DL channel bandwidth combination of a supported asymmetric channel bandwidth combination set for an operating band of the CA configuration when the said UL and DL channel bandwidths are also contained in a supported bandwidth combination set of the CA configuration.

5.5A.1	Configurations for intra-band contiguous CA

Power class 3 is supported for all uplinks. Power classes other than power class 3 are supported as indicated in Table 5.5A.1-1.

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier6 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n5B | - | 5, 10, 15 | 5, 10, 15 |  |  |  | 20 | 0 |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  | 25 | 1 |
| CA_n40B | - | 20 | 80 |  |  |  | 100 | 0 |
|  |  | 50 | 50 |  |  |  |  |  |
| CA_n41C4 | n414,5 CA_n41C | 40 | 80, 100 |  |  |  | 180 | 0 |
|  |  | 50, 60, 80 | 60, 80, 100 |  |  |  |  |  |
|  |  | 10 | 100 |  |  |  | 190 | 1 |
|  |  | 15, 20 | 90, 100 |  |  |  |  |  |
|  |  | 40 | 80, 90, 100 |  |  |  |  |  |
|  |  | 50, 60, 80, 90 | 60, 80, 90, 100 |  |  |  |  |  |
| CA_n46B | - | 20, 40, 60 | 20, 40 |  |  |  | 100 | 0 |
| CA_n46C | - | 60, 80 | 60, 80 |  |  |  | 160 | 0 |
| CA_n46D | - | 60, 80 | 80 | 80 |  |  | 240 | 0 |
| CA_n46M | - | 20, 40, 60 | 20, 40 | 20, 40 |  |  | 140 | 0 |
| CA_n46N | - | 20, 40, 80 | 20, 40 | 20, 40 | 20, 40 |  | 200 | 0 |
| CA_n46O | - | 20, 60 | 20, 40 | 20, 40 | 20, 40 | 20, 40 | 220 | 0 |
| CA_n48B | CA_n48B | 5 | 15, 20 |  |  |  | 40 | 0 |
|  |  | 10, 15, 20 | 10, 15, 20 |  |  |  |  |  |
|  |  | 15, 20 | 15, 20 |  |  |  |  |  |
|  | - | 10 | 50, 60, 80, 90 |  |  |  | 100 | 1 |
|  |  | 15, 20 | 40, 50, 60, 80 |  |  |  |  |  |
|  |  | 40 | 40, 50, 60 |  |  |  |  |  |
|  | - | 10, 15, 20, 30, 40 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90 |  |  |  | 100 | 2 |
| CA_n66B | - | 5 2 | 20, 40 |  |  |  | 50 | 0 |
|  |  | 10 | 15, 20, 40 |  |  |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |  |  |
| CA_n77C | CA_n77C | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | 10 | 100 |  |  |  | 200 | 1 |
|  |  | 15, 20 | 90, 100 |  |  |  |  |  |
|  |  | 25, 30 | 80, 90, 100 |  |  |  |  |  |
|  |  | 40 | 70, 80, 90, 100 |  |  |  |  |  |
|  |  | 50, 60,70, 80,90, 100 | 60, 70,80, 90, 100 |  |  |  |  |  |
| CA_n77D | - | 100 | 100 | 100 |  |  | 300 | 0 |
| CA_n78B | - | 20 | 50 |  |  |  | 70 | 0 |
| CA_n78C | CA_n78C4 | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | 10 | 100 |  |  |  | 200 | 1 |
|  |  | 15, 20 | 90, 100 |  |  |  |  |  |
|  |  | 25, 30 | 80, 90, 100 |  |  |  |  |  |
|  |  | 40 | 70, 80, 90, 100 |  |  |  |  |  |
|  |  | 50, 60, 70, 80, 90, 100 | 60, 70, 80, 90, 100 |  |  |  |  |  |
| CA_n78D | - | 100 | 100 | 100 |  |  | 300 | 0 |
| CA_n79C | CA_n79C | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
| CA_n79D | - | 100 | 100 | 100 |  |  | 300 | 0 |
| NOTE 1: Void. NOTE 2: 5 MHz is not applicable for 30/60 kHz SCS. NOTE 3: The aggregated bandwidth must be greater than or equal to the minimum for the bandwidth class defined in Table 5.3A.5-1, and smaller than or equal to the maximum aggregated bandwidth. NOTE 4: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 5: Power Class 1.5 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 6: Only single uplink carriers with power class other than PC3 are listed. |  |  |  |  |  |  |  |  |

Table 5.5A.1-2: Void

5.5A.2	Configurations for intra-band non-contiguous CA

Table 5.5A.2-1: NR CA configurations and bandwidth combination sets defined for intra-band non-contiguous CA

| NR CA Configuration | Uplink CA Configurations or single uplink carrier | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| CA_n48(2A) |  | 10, 15, 20, 40, 50, 60 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 1402 | 0 |
|  |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1402 | 1 |
| CA_n66(2A) | - | 5 (note), 10, 15, 20 | 5 (note), 10, 15, 20, 40 |  | 60 | 0 |
|  |  | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20, 25, 30, 40 |  | 80 | 1 |
|  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 |  | 80 | 2 |
| CA_n66(3A) | - | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 | 80 | 0 |
| CA_n71(2A) | - | 5,10, 15, 20 | 5,10,15, 20 |  | 30 | 0 |
| CA_n77(2A) | n773 CA_n77(2A) | 20, 40, 80, 100 | 20, 40, 80, 100 |  | 200 | 0 |
|  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 200 | 1 |
|  |  | See n77 channel bandwidths in Table 5.3.5-1 for each carrier |  |  | 200 | 4 and 5 |
| CA_n78(2A) |  | 10, 20, 40, 50, 60, 80, 90, 100 | 10, 20, 40, 50, 60, 80, 90, 100 |  | 200 | 0 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 200 | 1 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 200 | 2 |
| NOTE 1: 5 MHz is not applicable for 30/60 kHz SCS. NOTE 2: Parameter value accounts for both, the maximum frequency range of band n48 (150 MHz), and the minimum frequency gaps in between NR non-contiguous component carriers. NOTE 3: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |  |  |

5.5A.3	Configurations for inter-band CA

Table 5.5A.3-1: Void

Table 5.5A.3-2: Void

Table 5.5A.3-3: Void

5.5A.3.1	Configurations for inter-band CA (two bands)

Table 5.5A.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration |  | Uplink CA configuration or single uplink carrier5 |  |  | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  | CA_n1A-n3A |  |  | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  |  |  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 1 |
|  |  |  |  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
| CA_n1(2A)-n3A |  | - |  |  | n1 | CA_n1(2A)_BCS0 |  |  | 0 |
|  |  |  |  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
| CA_n1(2A)-n5A |  | - |  |  | n1 | CA_n1(2A)_BCS0 |  |  | 0 |
|  |  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |
| CA_n1A-n8A |  | CA_n1A-n8A |  |  | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  |  |  |  | n1 | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |
|  |  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |
| CA_n1(2A)-n8A |  | - |  |  | n1 | CA_n1(2A)_BCS0 |  |  | 0 |
|  |  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |
| CA_n1A-n28A |  | - |  |  | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |
| CA_n1A-n41A |  | CA_n1A-n41A |  |  | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 1 |
|  |  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
| CA_n1A-n77A |  | - |  |  | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
| CA_n1A-n78A |  | n784 CA_n1A-n78A |  |  | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n1 | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |
|  |  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n1 | 5, 10, 15, 20, 25, 30, 40 |  |  | 2 |
|  |  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n1 | 5, 10, 15, 20 |  |  | 3 |
|  |  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| CA_n1A-n78(2A) |  | CA_n1A-n78A |  |  | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n78 | CA_n78(2A)_BCS0 |  |  |  |
|  |  |  |  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 1 |
|  |  |  |  |  | n78 | CA_n78(2A)_BCS1 |  |  |  |
|  |  |  |  |  | n1 | 5, 10, 15, 20 |  |  | 2 |
|  |  |  |  |  | n78 | CA_n78(2A)_BCS2 |  |  |  |
| CA_n1A-n78C |  | CA_n1A-n78A |  |  | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n78 | CA_n78C_BCS0 |  |  |  |
|  |  |  |  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 1 |
|  |  |  |  |  | n78 | CA_n78C_BCS0 |  |  |  |
|  |  |  |  |  | n1 | 5, 10, 15, 20, 25, 30, 40 |  |  | 2 |
|  |  |  |  |  | n78 | CA_n78C_BCS0 |  |  |  |
| CA_n1(2A)-n78A |  | - |  |  | n1 | CA_n1(2A)_BCS0 |  |  | 0 |
|  |  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| CA_n1A-n79A |  | CA_n1A-n79A |  |  | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
| CA_n2A-n5A |  | CA_n2A-n5A |  |  | n2 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |
| CA_n2A-n14A |  | CA_n2A-n14A |  |  | n2 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n14 | 5, 10 |  |  |  |
| CA_n2A-n30A |  | CA_n2A-n30A |  | n2 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  | n30 | 5, 10 |  |  |  |
| CA_n2A-n48A |  | CA_n2A-n48A |  |  | n2 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n48 | 5, 10, 15, 20, 40, 507, 607, 807, 907, 1007 |  |  |  |
| CA_n2A-n48(2A) |  | CA_n2A-n48A |  |  | n2 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n48 | CA_n48(2A)_BCS0 |  |  |  |
|  |  |  |  |  | n2 | 5, 10, 15, 20 |  |  | 1 |
|  |  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |
| CA_n2A-n48B |  | CA_n48B CA_n2A-n48A |  |  | n2 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n48 | CA_n48B_BCS0 |  |  |  |
|  |  |  |  |  | n2 | 5, 10, 15, 20 |  |  | 1 |
|  |  |  |  |  | n48 | CA_n48B_BCS2 |  |  |  |
| CA_n2A-n66A |  | - |  |  | n2 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 40 |  |  |  |
|  |  | CA_n2A-n66A |  |  | n2 | 5, 10, 15, 20 |  |  | 1 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
| CA_n2(2A)-n66A |  | - |  |  | n2 | CA_n2(2A)_BCS0 |  |  | 0 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
| CA_n2A-n77A |  | n774 CA_n2A-n77A4 |  |  | n2 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| CA_n2A-n77C |  | n774 CA_n77C CA_n2A-n77A4 |  |  | n2 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n77 | CA_n77C_BCS0 |  |  |  |
|  |  |  |  |  | n2 | n2 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |
|  |  |  |  |  | n77 | CA_n77C_BCS 4 and 5 |  |  |  |
| CA_n3A-n5A |  | CA_n3A-n5A |  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 0 |
|  |  |  |  |  | n5 | 5, 10, 15, 20 |  |  |  |
| CA_n3(2A)-n5A |  | - |  |  | n3 | CA_n3(2A)_BCS0 |  |  | 0 |
|  |  |  |  |  | n5 | 5, 10, 15, |  |  |  |
| CA_n3A-n8A |  | CA_n3A-n8A |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  |  |  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  | 1 |
|  |  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |
| CA_n3(2A)-n8A |  | - |  |  | n3 | CA_n3(2A)_BCS0 |  |  | 0 |
|  |  |  |  |  | n8 | 5, 10, 15, 20 |  |  |  |
| CA_n3A-n28A |  | CA_n3A-n28A |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  |  |  |  | n28 | 5, 10, 15, 20 |  |  |  |
| CA_n3A-n41A |  | n414 CA_n3A-n41A4 |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  |  |  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
| CA_n3A-n77A |  | CA_n3A-n77A |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
| CA_n3A-n77(2A) |  | CA_n77(2A) CA_n3A-n77A |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  |  |  |  | n77 | CA_n77(2A)_BCS0 |  |  |  |
| CA_n3A-n78A |  | n784 CA_n3A-n78A4 |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n3 | 5, 10, 15, 20, 25, 30, 40, |  |  | 1 |
|  |  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| CA_n3A-n78(2A) |  | CA_n3A-n78A CA_n78(2A) |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  |  |  |  | n78 | CA_n78(2A)_BCS0 |  |  |  |
|  |  | CA_n3A-n78A |  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |
|  |  |  |  |  | n78 | CA_n78(2A)_BCS2 |  |  |  |
| CA_n3(2A)-n78A |  | - |  |  | n3 | CA_n3(2A)_BCS0 |  |  | 0 |
|  |  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| CA_n3A-n79A |  | CA_n3A-n79A |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
| CA_n5A-n7A |  | - |  |  | n5 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |
| CA_n5A-n30A |  | CA_n5A-n30A |  | n5 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  | n30 | 5, 10 |  |  |  |
| CA_n5A-n48A |  | CA_n5A-n48A |  |  | n5 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n48 | 5, 10, 15, 20, 40, 507, 607, 807, 907, 1007 |  |  |  |
| CA_n5A-n48(2A) |  |  |  | CA_n5A-n48A |  |  | n5 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  |  |  | n48 | CA_n48(2A)_BCS0 |  |  |  |
|  |  |  |  |  |  |  | n5 | 5, 10, 15, 20 |  |  | 1 |
|  |  |  |  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |  |
| CA_n5A-n48B |  | CA_n48B CA_n5A-n48A |  |  | n5 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n48 | CA_n48B_BCS0 |  |  |  |
|  |  |  |  |  | n5 | 5, 10, 15, 20 |  |  | 1 |
|  |  |  |  |  | n48 | CA_n48B_BCS2 |  |  |  |
| CA_n5A-n66A |  | CA_n5A-n66A |  |  | n5 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 40 |  |  |  |
|  |  |  |  |  | n5 | 5, 10, 15, 20 |  |  | 1 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
| CA_n5A-n77A |  | n774 CA_n5A-n77A4 |  |  | n5 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| CA_n5A-n77C |  | n774 CA_n5A-n77A4 CA_n77C |  |  | n5 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n77 | CA_n77C_BCS0 |  |  |  |
|  |  |  |  |  | n5 | 5, 10, 15, 20 |  |  | 1 |
|  |  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |
| CA_n5A-n78A |  | CA_n5A-n78A |  |  | n5 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n5 | 5, 10, 15, 20 |  |  | 1 |
|  |  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| CA_n5A-n78(2A) |  | CA_n5A-n78A |  |  | n5 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n78 | CA_n78(2A)_BCS2 |  |  |  |
| CA_n5B-n77A |  | CA_n5A-n77A |  | n5 | CA_n5B_BCS0 |  |  | 0 |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| CA_n5B-n77C |  | CA_n5A-n77A |  | n5 | CA_n5B_BCS0 |  |  | 0 |
|  |  |  |  | n77 | CA_n77C_BCS0 |  |  |  |
|  |  |  |  | n5 | CA_n5B_BCS0 |  |  | 1 |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |
| CA_n7A-n78A |  | CA_n7A-n78A |  |  | n7 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
| CA_n8A-n78A |  | CA_n8A-n78A |  |  | n8 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n8 | 5, 10, 15, 20 |  |  | 1 |
|  |  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
| CA_n8A-n78(2A) |  | CA_n8A-n78A |  |  | n8 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n78 | CA_n78(2A)_BCS1 |  |  |  |
| CA_n14A-n30A |  | CA_n14A-n30A |  |  | n14 | 5, 10 |  |  | 0 |
|  |  |  |  |  | n30 | 5, 10 |  |  |  |
| CA_n14A-n66A |  | CA_n14A-n66A |  |  | n14 | 5, 10 |  |  | 0 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
| CA_n14A-n77A |  | n774 CA_n14A-n77A4 |  |  | n14 | 5, 10 |  |  | 0 |
|  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| CA_n20A-n78A |  | CA_n20A-n78A |  |  | n20 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
| CA_n24A-n41A |  | CA_n24A-n41A |  |  | n24 | 5, 10 |  |  | 0 |
|  |  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
| CA_n24A-n41(2A) |  | CA_n24A-n41A |  |  | n24 | 5, 10 |  |  | 0 |
|  |  |  |  |  | n41 | CA_n41(2A)_BCS1 |  |  |  |
| CA_n24A-n48A |  | CA_n24A-n48A |  |  | n24 | 5, 10 |  |  | 0 |
|  |  |  |  |  | n48 | 5, 10, 15, 20, 40, 507, 607, 807, 907, 1007 |  |  |  |
| CA_n24A-n48B |  | CA_n24A-n48A |  |  | n24 | 5, 10 |  |  | 0 |
|  |  |  |  |  | n48 | CA_n48B_BCS1 |  |  |  |
| CA_n24A-n48(2A) |  | CA_n24A-n48A |  |  | n24 | 5, 10 |  |  | 0 |
|  |  |  |  |  | n48 | CA_n48(2A)_BCS0 |  |  |  |
| CA_n24A-n77A |  | CA_n24A-n77A |  |  | n24 | 5, 10 |  |  | 0 |
|  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| CA_n24A-n77C |  | CA_n24A-n77A |  |  | n24 | 5, 10 |  |  | 0 |
|  |  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |
| CA_n25A-n46A |  | - |  |  | n25 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n46 | 20, 40, 60, 80 |  |  |  |
| CA_n25A-n66A |  | CA_n25A-n66A |  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 30, 40 |  |  |  |
|  |  |  |  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
|  |  |  |  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |
|  |  |  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |  |  |
| CA_n25A-n77A |  | CA_n25A-n77A4 |  |  | n25 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |
|  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |
|  |  |  |  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |  |  |
| CA_n25A-n77(2A) |  | CA_n25A-n77A4 CA_n77(2A) |  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |
|  |  |  |  |  | n77 | CA_n77(2A)_BCS1 |  |  |  |
|  |  |  |  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |
|  |  |  |  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |  |  |
| CA_n25A-n78A |  | CA_n25A-n78A |  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |
|  |  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |
|  |  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| CA_n25A-n78(2A) |  | CA_n25A-n78A |  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |
|  |  |  |  |  | n78 | CA_n78(2A)_BCS0 |  |  |  |
|  |  |  |  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |
|  |  |  |  |  | n78 | CA_n78(2A)_BCS2 |  |  |  |
| CA_n26A-n66A |  | CA_n26A-n66A |  |  | n26 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
| CA_n26A-n66(2A) |  | CA_n26A-n66A |  |  | n26 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n66 | CA_n66(2A)_BCS0 |  |  |  |
| CA_n26A-n70A |  | CA_n26A-n70A |  |  | n26 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |
| CA_n28A-n41A |  | n414 CA_n28A-n41A4 |  |  | n28 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n28 | 5, 10, 15, 20,30 |  |  | 1 |
|  |  |  |  |  | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
| CA_n28A-n77A |  | CA_n28A-n77A |  |  | n28 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
| CA_n28A-n77(2A) |  | CA_n77(2A) CA_n28A-n77A |  |  | n28 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n77 | CA_n77(2A)_BCS0 |  |  |  |
| CA_n28A-n78A |  | CA_n28A-n78A |  |  | n28 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
| CA_n28A-n78(2A) |  | CA_n28A-n78A |  |  | n28 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n78 | CA_n78(2A)_BCS0 |  |  |  |
| CA_n28A-n79A |  | n794 CA_n28A-n79A4 |  |  | n28 | 5, 10, 15, 20, 30 |  |  | 0 |
|  |  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
| CA_n29A-n66A |  | - |  |  | n29 | 5, 10 |  |  | 0 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 40 |  |  |  |
| CA_n29A-n66B |  | - |  |  | n29 | 5, 10 |  |  | 0 |
|  |  |  |  |  | n66 | CA_n66B_BCS0 |  |  |  |
| CA_n29A-n66(2A) |  | - |  |  | n29 | 5, 10 |  |  | 0 |
|  |  |  |  |  | n66 | CA_n66(2A)_BCS0 |  |  |  |
| CA_n29A-n70A |  | - |  |  | n29 | 5, 10 |  |  | 0 |
|  |  |  |  |  | n70 | 5, 10, 15, 201,, 251 |  |  |  |
| CA_n29A-n71A |  | - |  |  | n29 | 5, 10 |  |  | 0 |
|  |  |  |  |  | n71 | 5, 10, 15, 20 |  |  |  |
| CA_n30A-n66A |  | CA_n30A-n66A |  | n30 | 5, 10 |  |  | 0 |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
| CA_n30A-n77A |  | CA_n30A-n77A |  | n30 | 5, 10 |  |  | 0 |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| CA_n39A-n41A |  | CA_n39A-n41A |  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |
|  |  |  |  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
| CA_n41A-n66A |  | - |  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 40 |  |  |  |
| CA_n41A-n71A |  | - |  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |
|  |  |  |  |  | n71 | 5, 10, 15, 20 |  |  |  |
| CA_n41A-n77A |  |  |  | CA_n41A-n77A |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 0 |
|  |  |  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  |  |  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  | 1 |
|  |  |  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| CA_n41A-n79A |  | n414, n794 CA_n41A-n79A4 |  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |
|  |  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
|  |  |  |  |  | n41 | 10, 15, 20, 40, 50, 60 |  |  | 1 |
|  |  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
|  |  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 2 |
|  |  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
| CA_n41C-n79A |  | CA_n41A-n79A CA_n41C |  |  | n41 | CA_n41C_BCS0 |  |  | 0 |
|  |  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
| CA_n46A-n48A |  | CA_n46A-n48A |  |  | n46 | 20, 40, 60, 80 |  |  | 0 |
|  |  |  |  |  | n48 | 20 |  |  |  |
| CA_n46B-n48A |  | CA_n46A-n48A |  |  | n46 | CA_n46B_BCS0 |  |  | 0 |
|  |  |  |  |  | n48 | 20 |  |  |  |
| CA_n46C-n48A |  | CA_n46A-n48A |  |  | n46 | CA_n46C_BCS0 |  |  | 0 |
|  |  |  |  |  | n48 | 20 |  |  |  |
| CA_n46D-n48A |  | CA_n46A-n48A |  |  | n46 | CA_n46D_BCS0 |  |  | 0 |
|  |  |  |  |  | n48 | 20 |  |  |  |
| CA_n46A-n66A |  | - |  |  | n46 | 20, 40, 60, 80 |  |  | 0 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
| CA_n48A-n66A |  | CA_n48A-n66A |  |  | n48 | 5, 10, 15, 20, 40, 507, 607, 807, 907, 1007 |  |  | 0 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 40 |  |  |  |
|  |  |  |  |  | n48 | 5, 10, 15, 20, 40, 507, 607, 807, 907, 1007 |  |  | 1 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
|  |  |  |  |  | n48 | 5, 10, 15, 20, 30, 40, 507, 607, 707, 807, 907, 1007 |  |  | 2 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40, |  |  |  |
| CA_n48A-n66(2A) |  | CA_n48A-n66A |  |  | n48 | 5, 10, 15, 20, 30, 40, 507, 607, 707, 807, 907, 1007 |  |  | 0 |
|  |  |  |  |  | n66 | CA_n66(2A)_BCS0 |  |  |  |
| CA_n48B-n66A |  | CA_n48A-n66A |  |  | n48 | CA_n48B_BCS0 |  |  | 0 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 40 |  |  |  |
|  |  |  |  |  | n48 | CA_n48B_BCS1 |  |  | 1 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
|  |  |  |  |  | n48 | CA_n48B_BCS2 |  |  | 2 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
| CA_n48(2A)-n66A |  | CA_n48A-n66A |  |  | n48 | CA_n48(2A)_BCS0 |  |  | 0 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
|  |  |  |  |  | n48 | CA_n48(2A)_BCS0 |  |  | 1 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
|  |  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  | 2 |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |  |
| CA_n48(2A)-n66(2A) |  | CA_n48A-n66A |  |  | n48 | CA_n48(2A)_BCS1 |  |  | 0 |
|  |  |  |  |  | n66 | CA_n66(2A)_BCS0 |  |  |  |
| CA_n48A-n70A6 |  | CA_n48A-n70A |  |  | n48 | 5, 10, 15, 20, 30, 40, 507, 607, 707, 807, 907, 1007 |  |  | 0 |
|  |  |  |  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |
| CA_n48(2A)-n70A |  | CA_n48A-n70A |  |  | n48 | CA_n48(2A)_BCS1 |  |  | 0 |
|  |  |  |  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |
| CA_n48B-n70A |  | CA_n48A-n70A |  |  | n48 | CA_n48B_BCS2 |  |  | 0 |
|  |  |  |  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |
| CA_n48A-n71A |  | CA_n48A-n71A |  |  | n48 | 5, 10, 15, 20, 30, 40, 507, 607, 707, 807, 907, 1007 |  |  | 0 |
|  |  |  |  |  | n71 | 5, 10, 15, 20 |  |  |  |
| CA_n48A-n71(2A) |  | CA_n48A-n71A |  |  | n48 | 5, 10, 15, 20, 30, 40, 507, 607, 707, 807, 907, 1007 |  |  | 0 |
|  |  |  |  |  | n71 | CA_n71(2A)_BCS0 |  |  |  |
| CA_n48(2A)-n71A |  | CA_n48A-n71A |  |  | n48 | CA_n48(2A)_BCS1 |  |  | 0 |
|  |  |  |  |  | n71 | 5, 10, 15, 20 |  |  |  |
| CA_n48(2A)-n71(2A) |  | CA_n48A-n71A |  |  | n48 | CA_n48(2A)_BCS1 |  |  | 0 |
|  |  |  |  |  | n71 | CA_n71(2A)_BCS0 |  |  |  |
| CA_n48B-n71A |  | CA_n48A-n71A |  |  | n48 | CA_n48B_BCS2 |  |  | 0 |
|  |  |  |  |  | n71 | 5, 10, 15, 20 |  |  |  |
| CA_n48A-n77A |  | n774 |  |  | n48 | 5, 10, 15, 20, 30, 40, 507, 607, 707, 807, 907, 1007 |  |  | 0 |
|  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| CA_n48A-n77C |  |  |  | CA_n77C |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  | 0 |
|  |  |  |  |  |  |  | n77 | CA_n77C_BCS0 |  |  |  |
|  |  |  |  |  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  | 1 |
|  |  |  |  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |
| CA_n48(2A)-n77A |  |  |  | - |  |  | n48 | CA_n48(2A)_BCS0 |  |  | 0 |
|  |  |  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  |  |  |  |  |  | n48 | CA_n48(2A)_BCS1 |  |  | 1 |
|  |  |  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  |  |  |  |  |  | n48 | CA_n48(2A)_BCS4 and 5 |  |  | 4 and 5 |
|  |  |  |  |  |  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |  |  |
| CA_n48B-n77A |  |  |  | CA_n48B |  |  | n48 | CA_n48B_BCS0 |  |  | 0 |
|  |  |  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  |  |  |  |  |  | n48 | CA_n48B_BCS1 |  |  | 1 |
|  |  |  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  |  |  |  |  |  | n48 | CA_n48B_BCS2 |  |  | 2 |
|  |  |  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| CA_n66A-n70A |  | - |  |  | n66 | 5, 10, 15, 20, 40 |  |  | 0 |
|  |  |  |  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |
| CA_n66B-n70A |  | - |  |  | n66 | CA_n66B_BCS0 |  |  | 0 |
|  |  |  |  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |
| CA_n66(2A)-n70A |  | - |  |  | n66 | CA_n66(2A)_BCS0 |  |  | 0 |
|  |  |  |  |  | n70 | 5, 10, 15, 201, 251 |  |  |  |
| CA_n66A-n71A |  | CA_n66A-n71A |  |  | n66 | 5, 10, 15, 20, 40 |  |  | 0 |
|  |  |  |  |  | n71 | 5, 10, 15, 20 |  |  |  |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |
|  |  |  |  |  | n71 | 5, 10, 15, 20 |  |  |  |
|  |  |  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |
|  |  |  |  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |  |  |
| CA_n66A-n71(2A) |  | - |  |  | n66 | 5, 10, 15, 20, 40 |  |  | 0 |
|  |  |  |  |  | n71 | CA_n71(2A)_BCS0 |  |  |  |
|  |  | CA_n66A-n71A |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |
|  |  |  |  |  | n71 | CA_n71(2A)_BCS0 |  |  |  |
| CA_n66(2A)-n71A |  | CA_n66A-n71A |  |  | n66 | CA_n66(2A)_BCS0 |  |  | 0 |
|  |  |  |  |  | n71 | 5, 10, 15, 20 |  |  |  |
| CA_n66(2A)-n71(2A) |  | CA_n66A-n71A |  |  | n66 | CA_n66(2A)_BCS1 |  |  | 0 |
|  |  |  |  |  | n71 | CA_n71(2A)_BCS0 |  |  |  |
| CA_n66B-n71A |  | CA_n66A-n71A |  |  | n66 | CA_n66B_BCS0 |  |  | 0 |
|  |  |  |  |  | n71 | 5, 10, 15, 20 |  |  |  |
| CA_n66A-n77A |  | n774 CA_n66A-n77A4 |  |  | n66 | 5, 10, 15, 20, 40 |  |  | 0 |
|  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |
|  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |
|  |  |  |  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |  |  |
| CA_n66A-n77(2A) |  | CA_n66A-n77A4 CA_n77(2A) |  |  | n66 | 5, 10, 15, 20, 40 |  |  | 0 |
|  |  |  |  |  | n77 | CA_n77(2A)_BCS0 |  |  |  |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |
|  |  |  |  |  | n77 | CA_n77(2A)_BCS1 |  |  |  |
|  |  |  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |
|  |  |  |  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |  |  |
| CA_n66A-n78A |  | CA_n66A-n78A |  |  | n66 | 5, 10, 15, 20, 40 |  |  | 0 |
|  |  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |
|  |  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| CA_n66A-n78(2A) |  | CA_n66A-n78A |  |  | n66 | 5, 10, 15, 20, 30, 40 |  |  | 0 |
|  |  |  |  |  | n78 | CA_n78(2A)_BCS1 |  |  |  |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |
|  |  |  |  |  | n78 | CA_n78(2A)_BCS2 |  |  |  |
| CA_n66A-n77C |  | n77 CA_n77C CA_n66A-n77A |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  | 0 |
|  |  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |
|  |  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  | 1 |
|  |  |  |  |  | n77 | CA_n77C_BCS1 |  |  |  |
| CA_n70A-n71A |  | CA_n70A-n71A |  |  | n70 | 5, 10, 15, 201, 251 |  |  | 0 |
|  |  |  |  |  | n71 | 5, 10, 15, 20 |  |  |  |
| CA_n70A-n71(2A) |  | CA_n70A-n71A |  |  | n70 | 5, 10, 15, 201 |  |  | 0 |
|  |  |  |  |  | n71 | CA_n71(2A)_BCS0 |  |  |  |
| CA_n71A-n77A |  | n774 CA_n71A-n77A4 |  |  | n71 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |
|  |  |  |  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |  |  |
| CA_n71A-n77(2A) |  |  |  | CA_n71A-n77A |  |  | n71 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  |  |  | n77 | CA_n77(2A)_BCS1 |  |  |  |
|  |  |  |  |  |  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |
|  |  |  |  |  |  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |  |  |
| CA_n71A-n78A |  |  |  | CA_n71A-n78A |  |  | n71 | 5, 10, 15, 20 |  |  | 0 |
|  |  |  |  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  |  |  |  |  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |
|  |  |  |  |  |  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |  |  |
| CA_n71A-n78(2A) |  |  |  | CA_n71A-n78A |  |  | n71 | 10, 15, 20 |  |  | 0 |
|  |  |  |  |  |  |  | n78 | CA_n78(2A)_BCS2 |  |  |  |
|  |  |  |  |  |  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |
|  |  |  |  |  |  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |  |  |
| CA_n77A-n79A |  | - |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |
|  |  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
| CA_n78A-n79A |  | - |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 0 |
|  |  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink. NOTE 2: The minimum requirements for intra-band contiguous or non-contiguous CA apply. NOTE 3: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 4: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination. NOTE 5: Only single uplink carriers with power class other than PC3 are listed. NOTE 6: The same configuration applies to corresponding NR-DC configuration in Table 5.5B.1-1. If UE supporting NR-DC configuration do not support the corresponding CA configuration, NR-DC configuration is used in CA test cases. NOTE 7: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |  |  |  |  |  |

5.5A.3.2	Configurations for inter-band CA (three bands)

Table 5.5A.3.2-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n28A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
| CA_n1A-n3A-n77A | CA_n1A-n3A CA_n1A-n77A CA_n3A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n78A | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |

| CA_n1A-n8A-n78A | CA_n1A-n8A CA_n1A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  | - | n1 | 5, 10, 15, 20 | 1 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |

| CA_n1A-n28A-n77A | CA_n1A-n28A CA_n1A-n77A CA_n28A-n77A | n1 | 5, 10, 15, 20 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n78A | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n41A-n77A | CA_n1A-n41A CA_n1A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n78A-n79A4 | CA_n1A-n78A CA_n1A-n79A CA_n78A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90 |  |
|  |  | n79 | 40, 50, 60, 80 |  |
| CA_n2A-n5A-n48A | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 |  |
| CA_n2A-n5A-n48B | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n2A-n5A-n48(2A) | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
| CA_n2A-n5A-n77A | n777, 9 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n77C | CA_n2A-n5A CA_n2A-n77A CA_n5A-n77A CA_n77C | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n48A-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48B-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48(2A)-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48A-n77A | n77 CA_n2A-n48A CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A-n77C | CA_n2A-n48A CA_n2A-n77A CA_n77C | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n48B-n77A | CA_n2A-n48A CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48(2A)-n77A | CA_n2A-n48A CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n77A | n777, 9 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n77C | CA_n2A-n66A CA_n2A-n77A CA_n66A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n3A-n28A-n41A | CA_n3A-n28A CA_n3A-n41A CA_n28A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n28A-n77A | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | CA_n41B_BCS0 |  |
| CA_n3A-n28A-n78A | - | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n41A-n77A | CA_n3A-n41A CA_n3A-n77A CA_n41A-n77A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48A-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48B-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48(2A)-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48A-n77A | n77 CA_n5A-n48A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48A-n77C | CA_n5A-n48A CA_n5A-n77A CA_n77C | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n48B-n77A | CA_n5A-n48A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48(2A)-n77A | CA_n5A-n48A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66A-n77A | n777, 9 CA_n5A-n66A CA_n66A-n77A7 CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66A-n77C | CA_n5A-n66A CA_n5A-n77A CA_n66A-n77A CA_n77C | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n25A-n66A-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66A-n77(2A) | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n66A-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66A-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n26A-n66A-n70A | CA_n26A-n66A CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n26A-n66(2A)-n70A | CA_n26A-n66A CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n28A-n41A-n79A | CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n29A-n66A-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66B-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66B_BCS0. |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66(2A)-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n41A-n66A-n71A |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66A-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 8010, 9010, 10010 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48A-n66(2A)-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 8010, 9010, 10010 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48(2A)-n66A-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48B-n66A-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48A-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 8010, 9010, 10010 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66(2A)-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 8010, 9010, 10010 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(2A)-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48B-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66A-n71(2A) | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 8010, 9010, 10010 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48A-n66A-n77A | n77 CA_n48A-n66A CA_n66A-n77A | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 7010, 8010, 9010, 10010 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n66A-n77C | CA_n48A-n66A CA_n66A-n77A CA_n77C | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48B-n66A-n77A | CA_n48A-n66A CA_n66A-n77A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 |  |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48(2A)-n66A-n77A | CA_n48A-n66A CA_n66A-n77A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n70A-n71A | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 8010, 9010, 10010 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(2A)-n70A-n71A | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n70A-n71(2A) | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 5010, 6010, 8010, 9010, 10010 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n66A-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66A-n70A-n71(2A) | CA_n66A-n71A CA_n70A-n71A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n66A-n71A-n77A | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n71A-n77(2A) | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n66A-n71A-n78A | CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n71A-n78(2A) | CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n66B-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | CA_n66B_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66(2A)-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink NOTE 2: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. NOTE 3: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 4: Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n78 with an n77 implementation. NOTE 5: FFS NOTE 6: FFS NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 8: FFS NOTE 9: Power Class 1.5 is allowed for single uplink carrier in this downlink/uplink combination. NOTE 10: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |

5.5A.3.3	Configurations for inter-band CA (four bands)

Table 5.5A.3.3-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (four bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier4 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n28A-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 901, 100 |  |
| CA_n2A-n5A-n48A-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |

| CA_n2A-n5A-n48B-n66A | - | n2 | 5, 10, 15, 20 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n48A-n77A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A CA_n5A-n48A CA_n5A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n48A-n77C | CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A CA_n5A-n48A CA_n5A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n5A-n66A-n77A | CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A-n66A-n77A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A CA_n48A-n66A CA_n66A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A-n66A-n77C | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  | CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A CA_n48A-n66A CA_n66A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n3A-n28A-n41A-n77A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48A-n66A-n77A | - | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n5A-n48A CA_n5A-n66A CA_n5A-n77A CA_n48A-n66A CA_n66A-n77A | n5 | 5, 10, 15, 20, 25 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48A-n66A-n77C | CA_n5A-n48A CA_n5A-n66A CA_n5A-n77A CA_n48A-n66A CA_n66A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. NOTE 2: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. NOTE 3: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 4: Only single uplink carriers with power class other than PC3 are listed. NOTE 5: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |

5.5B	Configurations for DC

For an NR-DC configuration specified in Table 5.5B-1, the bandwidth combination sets for the corresponding NR CA configuration in 5.5A.3, i.e.,dual uplink inter-band carrier aggregation with uplink assigned to two NR bands, are applicable to Dual Connectivity. If UE supporting NR-DC configuration do not support the corresponding CA configuration, NR-DC configuration is used in CA test cases.

Table 5.5B-1: Inter-band NR-DC configurations (two bands)

| NR-DC configuration | Uplink NR-DC configuration |
| --- | --- |
| DC_n48A-n70A | DC_n48A-n70A |

### <<End of changes>>