3GPP TSG-RAN5 Meeting # 103	R5-242760r1

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3171 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of system information combination for inter frequency NTN scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To cater the needs of inter frequency multi cell NTN test cases in NR NTN WP, new system information combination needs to be defined for scheduling of SIBs 1, 2, 4, 19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added system information combination for 3GPP NR FDD inter-frequency multi NTN cell scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR NTN WP will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.3.1.2, 4.4.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-242760r1: 1. XX in system information combination resolved to 32. |  |  |  |  |  |  |  |  |

<Start of modified section>

##### 4.4.3.1.2	Combinations of system information blocks for NR standalone and NE-DC

The combination of system information blocks required by a test case depends on the test case scenario. In this clause, several combinations of system information blocks are defined.

Regardless of the combination of system information blocks indicated as being used by a test case, the SS shall broadcast only the NR MIB on the NR Cell(s) configured on an SDL band and on NR SCell(s) configured on a band having no uplink frequency specified.

For NE-DC network scenario the SS shall in addition to broadcasting the NR system information blocks also broadcast the E-UTRA system information combination 1 on the E-UTRA cell(s) as specified in TS 36.508 [2].

Combination NR-1 is the default combination which applies to the following test case scenarios:

-	NR FDD single cell scenario except RRM test case scenarios

-	NR TDD single cell scenario except RRM test case scenarios

-	NR FDD inter-band DC component carriers cell scenario

-	NR TDD inter-band DC component carriers cell scenario

-	NR FDD and NR TDD inter-band DC component carriers cell scenario

Combination NR-2 applies to the following test case scenarios:

-	NR FDD intra-frequency multi cell scenario

-	NR TDD intra-frequency multi cell scenario

-	NR FDD and NR TDD dual mode multi cell roaming scenario

-	NR FDD single cell RRM test case scenario

-	NR TDD single cell RRM test case scenario

Combination NR-3 applies to the following test case scenarios:

-	NR FDD intra-frequency multi cell scenario with neighbouring cell related information

-	NR TDD intra-frequency multi cell scenario with neighbouring cell related information

Combination NR-4 applies to the following test case scenarios:

-	NR FDD inter-frequency multi cell scenario

-	NR TDD inter-frequency multi cell scenario

-	NR FDD inter-band multi cell scenario

-	NR TDD inter-band multi cell scenario

-	NR FDD and NR TDD dual mode multi cell non-roaming scenario

-	NR FDD intra-band carrier aggregation component carriers cell scenario

-	NR FDD inter-band carrier aggregation component carriers cell scenario

-	NR TDD intra-band carrier aggregation component carriers cell scenario

-	NR FDD and NR TDD inter-band carrier aggregation component carriers cell scenario

Combination NR-5 applies to the following test case scenarios:

-	NR FDD intra-band carrier aggregation component carriers cell scenario + NR FDD intra-frequency neighbour.

-	NR FDD inter-band carrier aggregation component carriers cell scenario+ NR FDD intra-frequency neighbour.

-	NR TDD intra-band carrier aggregation component carriers cell scenario+ NR FDD intra-frequency neighbour.

-	NR FDD and NR TDD inter-band carrier aggregation component carriers cell scenario+ NR FDD intra-frequency neighbour.

Combination NR-6 applies to the following test case scenarios:

-	3GPP inter-RAT NR FDD + E-UTRA FDD multi cell scenario

-	3GPP inter-RAT NR TDD + E-UTRA TDD multi cell scenario

-	3GPP inter-RAT NR TDD + E-UTRA FDD multi cell scenario

Combination NR-7 applies to the following test case scenarios:

-	NR FDD inter-frequency + 3GPP inter-RAT E-UTRA multi-cell scenario

-	NR TDD inter-frequency + 3GPP inter-RAT E-UTRA multi-cell scenario

Combination NR-8 applies to the following test case scenarios:

-	NR FDD ETWS single cell scenario

-	NR TDD ETWS single cell scenario

Combination NR-9 applies to the following test case scenarios:

-	3GPP NR FDD + CMAS single cell scenario

-	3GPP NR TDD + CMAS single cell scenario

Combination NR-10 applies to the following test case scenarios:

-	3GPP NR FDD + ETWS primary notification single cell scenario

-	3GPP NR TDD + ETWS primary notification single cell scenario

Combination NR-11 applies to the following test case scenarios:

-	3GPP NR FDD + ETWS secondary notification single cell scenario

-	3GPP NR TDD + ETWS secondary notification single cell scenario

Combination NR-12 applies to the following test case scenarios:

-	3GPP NR FDD + SNPN only single cell scenario

-	3GPP NR TDD + SNPN only single cell scenario

Combination NR-13 applies to the following test case scenarios:

-	3GPP NR FDD + CAG cell  multi cell scenario

-	3GPP NR TDD + CAG cell  multi cell scenario

Combination NR-14 applies to the following test case scenarios:

-	3GPP NR FDD single cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD single cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR FDD intra-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD intra-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR FDD inter-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD inter-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR FDD inter-band multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD inter-band multi cell scenario + NR sidelink communication with or without network scheduling.

Combination NR-15 applies to the following test case scenarios:

-	3GPP NR FDD single cell scenario + positioning test case which require posSIBs.

-	3GPP NR TDD single cell scenario + positioning test case which require posSIBs.

Combination NR-16 applies to the following test case scenarios:

-	3GPP NR FDD intra-frequency multi cell scenario + positioning test case which require posSIBs.

-	3GPP NR TDD intra-frequency multi cell scenario + positioning test case which require posSIBs.

Combination NR-17 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi cell scenario + positioning test case which require posSIBs.

-	3GPP NR TDD inter-frequency multi cell scenario + positioning test case which require posSIBs.

Combination NR-18 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi cell scenario + idle/inactive measurements.

-	3GPP NR TDD inter-frequency multi cell scenario + idle/inactive measurements.

Combination NR-19 applies to the following test case scenarios:

-	3GPP NR FDD + Extended field in SI scenario

-	3GPP NR TDD + Extended field in SI scenario

Combination NR-20 applies to the following test case scenarios:

-	3GPP NR FDD single cell scenario + MBS data.

-	3GPP NR TDD single cell scenario + MBS data.

-	3GPP NR FDD intra-frequency multi cell scenario + MBS data.

-	3GPP NR TDD intra-frequency multi cell scenario + MBS data.

Combination NR-21 applies to the following test case scenarios:

-	3GPP NR FDD intra-frequency multi cell scenario + MBS Service Continuity.

-	3GPP NR TDD intra-frequency multi cell scenario + MBS Service Continuity.

Combination NR-22 applies to the following test case scenarios:

-	3GPP NR FDD intra-frequency multi cell scenario + MBS data + MBS Service Continuity.

-	3GPP NR TDD intra-frequency multi cell scenario + MBS data + MBS Service Continuity.

Combination NR-23 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi cell scenario + MBS data

-	3GPP NR TDD inter-frequency multi cell scenario + MBS data.

Combination NR-24 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi cell scenario + MBS Service Continuity.

-	3GPP NR TDD inter-frequency multi cell scenario + MBS Service Continuity.

Combination NR-25 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi cell scenario + MBS data + MBS Service Continuity.

-	3GPP NR TDD inter-frequency multi cell scenario + MBS data + MBS Service Continuity.

Combination NR-26 applies to the following test case scenarios:

-	3GPP NR FDD + eNPN single cell scenario

-	3GPP NR TDD + eNPN single cell scenario

Combination NR-27 applies to the following test case scenarios:

-	3GPP NR FDD + eNPN multi cell scenario

-	3GPP NR TDD + eNPN multi cell scenario

Combination NR-28 applies to the following test case scenarios:

-	3GPP NR FDD single NTN cell scenario

Combination NR-29 applies to the following test case scenarios:

-	3GPP NR FDD intra-frequency multi NTN cell scenario

Combination NR-30 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi cell scenario + slice-based cell reselection in idle/inactive state

-	3GPP NR TDD inter-frequency multi cell scenario + slice-based cell reselection in idle/inactive state

Combination NR-32 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi NTN cell scenario

Table 4.4.3.1.2-1: Combinations of system information blocks

| System information block type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Combination No. | SIB1 | SIB2 | SIB3 | SIB4 | SIB5 | SIB6 | SIB7 | SIB8 | SIB9 | SIB10 | SIB11 | SIB12 | SIB13 | SIB14 | SIB16 | posSIBs | SIB18 | SIB19 | SIB20 | SIB 21 |
| NR-1 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-2 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-3 | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-4 | X | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-5 | X | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-6 | X | X |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-7 | X | X |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-8 | X |  |  |  |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-9 | X |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-10 | X |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-11 | X |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-12 | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| NR-13 | X | X |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| NR-14 | X | X |  | X |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| NR-15 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| NR-16 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| NR-17 | X | X |  | X |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| NR-18 | X |  |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| NR-19 | X | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| NR-20 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |
| NR-21 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |
| NR-22 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X |
| NR-23 | X | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |
| NR-24 | X | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |
| NR-25 | X | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X |
| NR-26 | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  | X |  |  |  |
| NR-27 | X | X |  |  |  |  |  |  |  | X |  |  |  |  |  |  | X |  |  |  |
| NR-28 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| NR-29 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| NR-30 | X | X |  | X |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| NR-32 | X | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |

##### 4.4.3.1.3	Scheduling of system information blocks

The scheduling configurations for combinations of system information blocks are defined in the following tables. There is no scheduling information for combination NR-1.

Table 4.4.3.1.3-1: Scheduling for combination NR-2

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |

Table 4.4.3.1.3-2: Scheduling for combination NR-3

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB3 |

Table 4.4.3.1.3-3: Scheduling for combination NR-4

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4 |

Table 4.4.3.1.3-4: Scheduling for combination NR-5

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB3 |
| 3 | 64 | SIB4 |

Table 4.4.3.1.3-5: Scheduling for combination NR-6

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB5 |

Table 4.4.3.1.3-6: Scheduling for combination NR-7

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4, SIB5 |

Table 4.4.3.1.3-7: Scheduling for combination NR-8

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB6 |
| 2 | 32 | SIB7 |

Table 4.4.3.1.3-8: Scheduling for combination NR-9

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB8 |

Table 4.4.3.1.3-9: Scheduling for combination NR-10

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
|  | 32 | SIB6 |

Table 4.4.3.1.3-10: Scheduling for combination NR-11

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB7 |

Table 4.4.3.1.3-11: Scheduling for combination NR-12

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 64 | SIB10 |

Table 4.4.3.1.3-12: Scheduling for combination NR-13

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB10 |

Table 4.4.3.1.3-13: Scheduling for combination NR-14

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4 |
| 3 | 64 | SIB12 |

Table 4.4.3.1.3-14: Scheduling for combination NR-15

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | posSibType1-1, posSibType1-2, posSibType1-3 (Note 1) |
|  | 32 | posSibType4-1 (Note 2) |
|  | 32 | posSibType5-1 (Note 3) |
|  | 32 | posSibType1-5, posSibType1-6, posSibType1-7 (Note 5) |
|  | 32 | posSibType1-8 (Note 6) |
| 2..N+1 | 32 | posSibType2-1, posSibType2-3, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9 (Note 4) |
|  | 32 | posSibType2-12, posSibType2-13 (Note 7) |
|  | 32 | posSibType2-17, posSibType2-18, posSibType2-19, posSibType2-20, posSibType2-21, posSibType2-22, posSibType2-23 (Note 8) |
| Note 1: Content of SI message 1 when UE supports GNSS as defined in TS 37.571-2 [20] Table 8.1-1: Sub-test 25. Note 2: Content of SI message 1 when UE supports Sensor as defined in TS 37.571-2 [20] Table 8.1-1: Sub-test 24. Note 3: Content of SI message 1 when UE supports MBS as defined in TS 37.571-2 [20] Table 8.1-1: Sub-test 23. Note 4: Content of SI messages 2..N, only present when UE supports GNSS as defined in TS 37.571-2 [20] Table 8.1-1: Sub-test 25. If UE supports N GNSS systems, N SIs are broadcasted, one SI per each GNSS system. Note 5: Content of SI message 1 when UE supports RTK corrections Note 6: Content of SI message 1 when UE supports SSR corrections Note 7: Content of SI messages 2..N, only present when UE supports RTK corrections. . If UE supports N GNSS systems, N SIs are broadcasted, one SI per each GNSS system. The posSibType2-13 must only be present when GLONASS is supported. Note 8: Content of SI messages 2..N, only present when UE supports SSR corrections. If UE supports N GNSS systems, N SIs are broadcasted, one SI per each GNSS system. |  |  |

Table 4.4.3.1.3-15: Scheduling for combination NR-16

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 32 | posSibType6-1 |

Table 4.4.3.1.3-16: Scheduling for combination NR-17

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4 |
| 3 | 32 | posSibType6-1 |

Table 4.4.3.1.3-17: Scheduling for combination NR-18

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 64 | SIB4 |
| 2 | 64 | SIB11 |

Table 4.4.3.1.3-18: Scheduling for combination NR-19

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2, SIB11 |

Table 4.4.3.1.3-19: Void

Table 4.4.3.1.3-20: Scheduling for combination NR-20

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB20 |

Table 4.4.3.1.3-21: Scheduling for combination NR-21

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB21 |

Table 4.4.3.1.3-22: Scheduling for combination NR-22

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB20 |
| 2 | 64 | SIB21 |

Table 4.4.3.1.3-23: Scheduling for combination NR-23

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4 |
| 3 | 64 | SIB20 |

Table 4.4.3.1.3-24: Scheduling for combination NR-24

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4 |
| 3 | 64 | SIB21 |

Table 4.4.3.1.3-25: Scheduling for combination NR-25

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4 |
| 3 | 64 | SIB20 |
| 4 | 64 | SIB21 |

Table 4.4.3.1.3-26: Scheduling for combination NR-26

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 64 | SIB10, SIB18 |

Table 4.4.3.1.3-27: Scheduling for combination NR-27

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB10, SIB18 |

Table 4.4.3.1.3-28: Scheduling for combination NR-28

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 64 | SIB19 |

Table 4.4.3.1.3-29: Scheduling for combination NR-29

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB19 |

Table 4.4.3.1.3-30: Scheduling for combination NR-30

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4 |
| 3 | 64 | SIB16 |

Table 4.4.3.1.3-32: Scheduling for combination NR-32

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4 |
| 3 | 64 | SIB19 |

<End of modified section>