3GPP TSG-RAN5 Meeting # 103	R5-242771r1

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3174 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Logical Channel Config Table 4.6.3-66 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | IE channelAccessPriority-r16 is missing in Logical Channel Config Table 4.6.3-66. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added IE channelAccessPriority-r16. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR unlicensed WP will remain incomplate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-242771r1: 1. bitRateMultiplier-r16 IE is added in Table 4.6.3-66. |  |  |  |  |  |  |  |  |

<Start of modified section>

#### –	LogicalChannelConfig

Table 4.6.3-66: LogicalChannelConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LogicalChannelConfig ::= SEQUENCE { |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |
| priority | 1 |  |  |
|  | 3 |  | SRB2, SRB4 |
| prioritisedBitRate | infinity |  |  |
| bucketSizeDuration | ms50 |  |  |
| allowedServingCells | Not present |  |  |
| allowedSCS-List | Not present |  |  |
| maxPUSCH-Duration | Not present |  |  |
| configuredGrantType1Allowed | Not present |  |  |
| logicalChannelGroup | 1 |  | HI |
|  | 2 |  | LO |
|  | 0 |  | SRB1, SRB2, SRB3, SRB4 |
| schedulingRequestID | SchedulingRequestId |  |  |
| logicalChannelSR-Mask | false |  |  |
| logicalChannelSR-DelayTimerApplied | false |  |  |
| bitRateQueryProhibitTimer | Not present |  |  |
| } |  |  |  |
| channelAccessPriority-r16 | Not present |  |  |
|  | 3 |  | SharedSpectrum AND (HI OR LO OR SRB2) |
| bitRateMultiplier-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| HI | Used for DRBs with high logical channel priority |
| LO | Used for DRBs with low logical channel priority |
| SRB1 | Establishment of SRB1 |
| SRB2 | Establishment of SRB2 |
| SRB3 | Establishment of SRB3 |
| SRB4 | Establishment of SRB4 |
| SharedSpectrum | Operation with shared spectrum channel access in FR1 |

<End of modified section>