3GPP TSG-RAN5 Meeting #103	R5-242818r3

# Fukuoka, Japan, 20th May 2024 – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4417 | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR 5GC Multilayer TC 11.4.12-Emergency Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Change 1: AP#102.02: “Resolve open issues captured in Editor’s note for TS 38.523-1 Emergency Call TCs 11.1.7 and 11.4.12” - TAU Request in step22a4 does not related to TP, the message content does not need to be checked either. - "active" flag in step4 may set to 1 in order to prolong the NAS signalling connection to establish PDU session. Change 2: Update TP2 2.1 UE network capability IE is not an fix length IE, if an UE does not support any of feature included in octet9-octet11, N1mode bit may not present. [TS 24.301 clause 9.9.3.34] The UE network capability is a type 4 information element with a minimum length of 4 octets and a maximum length of 15 octets. [TS 24.301 clause 5.5.3.2.2] For all cases except case b, if the UE supports N1 mode for 3GPP access, the UE shall set the N1mode bit to "N1 mode for 3GPP access supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message and shall include the UE additional security capability IE in the TRACKING AREA UPDATE REQUEST message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: - Delete the Editor’s Note 1.2 Align with the 38.508-1 Table 4.9.7.2.3-1, Not check the "active" flag value in both step4 and step22a4 Change 2: 2.1 Update TP2 to check “UE does not report ‘N1 mode supported’(i.e N1 mode bit = “N1 mode not supported” or not present)” in stead of “UE report ‘N1 mode not supported’” 2.2 Update N1mode bit from “0'B” to “0'B or Not present” in Table 11.4.12.3.3-4 of step4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail this test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Delete two space before “UE additional security capability” in Table 11.4.12.3.3-4 - Keep the value of N1 mode bit as ‘0’B in Table 11.4.12.3.3-4 for step4, and in conditions column add IF PRESENT.This would mean that IF this IE is PRESENT, it must be set to ‘0’B, otherwise this IE is not present. - Using another wording to update TP2 2nd revision: - TAU Request in step22a4 does not related to TP, the message content does not need to be checked either. 3rd revision: - Update coversheet |  |  |  |  |  |  |  |  |

<Start of modified section 1>

11.4.12	5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Disabling N1 mode / Emergency call establishment over EPS / Success

11.4.12.1	Test Purpose (TP)

(1)

with { UE supporting both S1 mode and N1 mode and operating in single-registration mode, and, the Network has indicated "interworking without N26 interface not supported", and, the Network has indicated "Emergency service Fallback not supported" regardless whether the UE supports emergency service fallback or not, and, the UE is in NR RRC_IDLE state }

ensure that {

when { User initiates an Emergency call and the UE completes Access control checking in 5GMM-IDLE mode }

then {  UE selects the E-UTRA cell, and, requests the establishment of an Emergency call by transmitting an RRCConnectionRequest message with establishmentCause set to 'emergency' }

}

(2)

with { UE establishes RRC connection in E-UTRA }

ensure that {

when { UE performs a TAU procedure caused by RAT change }

then { UE transmits TRACKING AREA UPDATE REQUEST with N1 mode capability set to ‘N1 mode not supported’ if included in the UE Network capability }

}

11.4.12.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501 [22], clauses 4.9.2, 5.6.1.4; TS 24.301 [21], subclauses 4.4.2.3, 5.5.3.2.2 and TS 24.229 [44] clause U.2.2.6.4. Unless otherwise stated these are Rel-15 requirements.

NOTE:	Conformance requirements in regard to establishing an emergency call in EPS are not provided. This can be found in IMS Emergency tests specified in TS 36.523-1 [13].

[TS 24.501, subclause 4.9.2]

The UE shall only disable the N1 mode capability for 3GPP access when in 5GMM-IDLE mode.

[TS 24.501, subclause 5.6.1.4]

For case h) in subclause 5.6.1.1, the UE shall treat the indication from the lower layers when the UE has changed to S1 mode or E-UTRA connected to 5GCN (see 3GPP TS 23.502 [9]) as successful completion of the procedure and stop timer T3517.

[TS 24.301, subclause 4.4.2.3]

Secure exchange of NAS messages via a NAS signalling connection is usually established by the MME during the attach procedure by initiating a security mode control procedure. After successful completion of the security mode control procedure, all NAS messages exchanged between the UE and the MME are sent integrity protected using the current EPS security algorithms, and except for the messages specified in subclause 4.4.5, all NAS messages exchanged between the UE and the MME are sent ciphered using the current EPS security algorithms.

...

During inter-system change from N1 mode to S1 mode in 5GMM-IDLE mode, if the UE is operating in the single-registration mode and:

1)	if the tracking area updating procedure is initiated as specified in 3GPP TS 24.501 [54], the UE shall transmit a TRACKING AREA UPDATE REQUEST message integrity protected with the current 5G NAS security context and the UE shall derive a mapped EPS security context (see subclause 8.6.1 of 3GPP TS 33.501 [56]). The UE shall include the eKSI indicating the 5G NAS security context value in the TRACKING AREA UPDATE REQUEST message.

After receiving the TRACKING AREA UPDATE REQUEST message including the eKSI, the MME forwards the TRACKING AREA UPDATE REQUEST message to the source AMF, if possible, to obtain the mapped EPS security context from the AMF as specified in 3GPP TS 33.501 [56]. The MME re-establishes the secure exchange of NAS messages by either:

-	replying with a TRACKING AREA UPDATE ACCEPT message that is integrity protected and ciphered using the mapped EPS NAS security context. From this time onward, all NAS messages exchanged between the UE and the MME are sent integrity protected and except for the messages specified in subclause 4.4.5, all NAS messages exchanged between the UE and the MME are sent ciphered; or

[TS 24.301, subclause 5.5.3.2.2]

The UE in state EMM-REGISTERED shall initiate the tracking area updating procedure by sending a TRACKING AREA UPDATE REQUEST message to the MME,

...

z)	when the UE performs inter-system change from N1 mode to S1 mode in EMM-IDLE mode, the UE operates in single-registration mode, and conditions specified in 3GPP TS 24.501 [54] apply;

...

zd)	when the UE performs inter-system change from N1 mode to S1 mode in EMM-CONNECTED mode.

For all cases except case b, the UE shall set the EPS update type IE in the TRACKING AREA UPDATE REQUEST message to "TA updating". For case b, the UE shall set the EPS update type IE to "periodic updating".

...

When initiating a tracking area updating procedure while in S1 mode, the UE shall use the current EPS NAS integrity key to integrity protect the TRACKING AREA UPDATE REQUEST message, unless the UE is performing inter-system change from N1 mode to S1 mode.

...

If a UE has established PDN connection(s) and uplink user data pending to be sent via user plane when it initiates the tracking area updating procedure, or uplink signalling not related to the tracking area updating procedure when the UE does not support control plane CIoT EPS optimization, it may also set an "active" flag in the TRACKING AREA UPDATE REQUEST message to indicate the request to establish the user plane to the network and to keep the NAS signalling connection after the completion of the tracking area updating procedure.

...

If the UE has a current EPS security context, the UE shall include the eKSI (either KSIASME or KSISGSN) in the NAS Key Set Identifier IE in the TRACKING AREA UPDATE REQUEST message. Otherwise, the UE shall set the NAS Key Set Identifier IE to the value "no key is available". If the UE has a current EPS security context, the UE shall integrity protect the TRACKING AREA UPDATE REQUEST message with the current EPS security context. Otherwise the UE shall not integrity protect the TRACKING AREA UPDATE REQUEST message.

...

For the case z and zd, the TRACKING AREA UPDATE REQUEST message shall be integrity protected using the 5GS security context available in the UE. The UE shall include a GUTI, mapped from 5G-GUTI (see 3GPP TS 23.501 [54] and 3GPP TS 23.003 [2]), in the Old GUTI IE in the TRACKING AREA UPDATE REQUEST message. In addition, the UE shall include Old GUTI type IE with GUTI set to "Native GUTI", and the UE shall include a UE status IE with a 5GMM registration status set to "UE is in 5GMM-REGISTERED state".

When the tracking area updating procedure is initiated in EMM-IDLE mode, the UE may also include an EPS bearer context status IE in the TRACKING AREA UPDATE REQUEST message, indicating which EPS bearer contexts are active in the UE. The UE shall include the EPS bearer context status IE in TRACKING AREA UPDATE REQUEST message:

-	...

-	for the case z; and

...

If the UE initiates the first tracking area updating procedure following an initial registration in N1 mode and the UE is operating in the single-registration mode, the UE shall include a UE radio capability information update needed IE in the TRACKING AREA UPDATE REQUEST message.

...

If the UE supports NB-S1 mode, Non-IP PDN type, or N1 mode, then the UE shall support the extended protocol configuration options IE.

For all cases except case b, if the UE supports the extended protocol configuration options IE, then the UE shall set the ePCO bit to "extended protocol configuration options supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

...

For all cases except case b, if the UE supports dual connectivity with NR, then the UE shall set the DCNR bit to "dual connectivity with NR supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message and shall include the UE additional security capability IE in the TRACKING AREA UPDATE REQUEST message.

...

For all cases except case b, if the UE supports N1 mode, the UE shall set the N1mode bit to "N1 mode supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message and shall include the UE additional security capability IE in the TRACKING AREA UPDATE REQUEST message.

[TS 24.229, subclause U.2.2.6.4]

NOTE:	This subclause covers only the case where the UE selects the IM CN subsystem in accordance with the conventions and rules specified in 3GPP TS 23.167 [4B] and describes the IP-CAN specific procedure. It does not preclude the use of CS domain. When a CS system based on 3GPP TS 24.008 [8] is to be used, clause B.5 applies.

When the UE operates in single-registration mode as described in 3GPP TS 24.501 [258] and the UE recognises that a call request is an emergency call, if:

1)	the IM CN subsystem is selected in accordance with the conventions and rules specified in 3GPP TS 23.167 [4B]; and

2)	the UE is currently registered to the 5GS services while the UE is in an NR cell connected to 5GCN;

then the following treatment is applied:

1)	if the EMC indicates "Emergency services not supported":

…

b)	if the UE supports emergency services fallback as specified in 3GPP TS 23.501 [257] and the emergency services fallback is not available (i.e., "ESFB is N" as described in 3GPP TS 23.167 [4B]) and if:

…

ii)	the EMF is set to "Emergency services fallback not supported" or the UE is not capable of accessing 5GCN via E-UTRA, the UE shall disable the N1 mode capability for 3GPP access as specified in 3GPP TS 24.501 [257] and attempt to select an E-UTRA cell connected to EPC. If the UE finds a suitable E-UTRA cell connected to EPC and the network provides the UE with the EMC BS set to "emergency bearer services in S1 mode supported" as described in 3GPP TS 24.301 [8J], the UE shall perform the procedures as described in subclause L.2.2.6 to establish a PDN connection for emergency bearer services; and

c)	if the UE does not support emergency services fallback as specified in 3GPP TS 23.501 [257], the UE shall disable the N1 mode capability for 3GPP access as specified in 3GPP TS 24.501 [257] and attempt to select an E-UTRA cell connected to EPC. If the UE finds a suitable E-UTRA cell connected to EPC and the network provides the UE with the EMC BS set to "emergency bearer services in S1 mode supported" as described in 3GPP TS 24.301 [8J], the UE shall perform the procedures as described in subclause L.2.2.6 to establish a PDN connection for emergency bearer services;

11.4.12.3	Test description

11.4.12.3.1	Pre-test conditions

System Simulator:

-	2 cells

-	NR Cell 1 as defined in TS 38.508-1 [4] Table 4.4.2-3. System information combination NR-6 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2.

-	E-UTRA Cell 1 as defined in TS 36.508 [7] Table 4.4.2-2. System information combination 31 as defined in TS 36.508 [7], subclause 4.4.3.1.1.

-	Power levels are constant and as defined in Tables 11.4.12.3.1-1/2.

Table 11.4.12.3.1-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter name | Unit | NR Cell 1 | E-UTRA Cell 1 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | - |  |
|  | RS EPRE | dBm/15kHz | - | -91 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | - |  |
|  | RS EPRE | dBm/15kHz | - | -91 |  |

Table 11.4.12.3.1-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter name | Unit | NR Cell 1 | E-UTRA Cell 1 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | - |  |
|  | RS EPRE | dBm/15kHz | - | -91 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | - |  |
|  | RS EPRE | dBm/15kHz | - | -91 |  |

UE:

None.

Preamble:

-	With E-UTRA Cell 1 "Serving cell" and NR Cell 1 "Non-suitable "Off" cell" in accordance with TS 38.508-1 [4], Table 6.2.2.1-3, the UE is brought to state RRC_IDLE using generic procedure parameters Connectivity (E-UTRA/EPC) and Unrestricted nr PDN (On) in accordance with the procedure described in TS 38.508-1 [4], clause 4.5.2. 4G GUTI and eKSI are assigned and security context established.

-	the UE is switched-off.

-	With E-UTRA Cell 1 "Non-suitable "Off" cell" and NR Cell 1 "Serving cell" in accordance with TS 38.508-1 [4], Table 6.2.2.1-3, the UE is brought to state 1N-A, NR RRC_IDLE Connectivity (NR) in accordance with the procedure described in TS 38.508-1 [4], Table 4.5.2.2-2. 5G-GUTI and ngKSI are assigned and security context established.

11.4.12.3.2	Test procedure sequence

Table 11.4.12.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Set the power levels according to “T0” as per Table 11.4.12.3.1-1/2. | - | - | - | - |
| 1 | Make the UE initiate an Emergency call. | - | - | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest message with 'establishmentCause' set to 'emergency' on E-UTRA cell 1? | --> | RRC: RRCConnectionRequest | 1 | P |
| 3 | SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. For some consequences related to the content of the TRACKING AREA UPDATE REQUEST message see the Notes in TS.36.508 [7], Table 4.5A.2.1-1. | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST | 2 | P |
| 5-8 | Steps 5-8 from the Generic Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. | - | - | - | - |
| 9 | SS responds with TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 10 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE | - | - |
| 11-17 | Steps 9-15 from the Generic Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. | - | - | - | - |
| 18 | The SS waits 1 second. | - | - | - | - |
| 18A | Set the power levels according to “T1” as per Table 11.4.12.3.1-1/2. | - | - | - | - |
| 19 | Release IMS Call as specified in the generic procedure in TS 34.229-1 [35] subclause C.32. | - | - | - | - |
| 20 | The SS releases the RRC connection. | <-- | E-UTRA RRC: RRCConnectionRelease | - | - |
| 21 | SS starts Timer T1=60s. | - | - | - | - |
| - | EXCEPTION: Steps 22a1 to 22b1 describe behaviour that depends on the UE implementation. |  |  |  |  |
| 22a1 | The UE transmits an RRCConnectionRequest. | --> | RRC: RRCConnectionRequest | - | - |
| 22a2 | Stop Timer T1 |  |  |  |  |
| 22a3-22a7 | Steps 2-6 of 36.508 Table 6.4.2.7-1 are executed. | - | - | - | - |
| 22b1 | Timer T1 expires. | - | - | - | - |

11.4.12.3.3	Specific message contents

Table 11.4.12.3.3-1: REGISTRATION REQUEST (Preamble; TS 38.508-1 [4], Table 4.5.2.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GMM capability |  |  |  |
| S1 mode (octet 3, bit 1) | '1'B | S1 mode supported |  |

Table 11.4.12.3.3-2: REGISTRATION ACCEPT (Preamble; TS 38.508-1 [4], Table 4.5.2.2-2)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS network feature support |  |  |  |
| Emergency service support indicator for 3GPP access (EMC) (octet 3, bit 3 and bit 4) | '00'B | Emergency services not supported |  |
| Emergency service fallback indicator for 3GPP access (EMF) (octet 3, bit 5 and bit 6) | '00'B | Emergency services fallback not supported |  |

Table 11.4.12.3.3-3: RRCConnectionRequest (step 2, Table 11.4.12.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | emergency |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.4.12.3.3-4: TRACKING AREA UPDATE REQUEST (step 4, Table 11.4.12.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.7.2-27 with condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| "Active" flag | Any allowed value |  |  |
| UE network capability |  |  |  |
| All octets with the exception of octet 9, bit 6 | Any allowed value |  |  |
| N1 mode supported (N1mode) (octet 9, bit 6) | '0'B (Note 1) | N1 mode not supported | IF PRESENT |
| UE additional security capability | Any allowed value |  | ENDC supported |
|  | Not present |  | ENDC not supported |
| Note 1: if an UE does not support any feature included in octet9-octet11 in UE network capability IE specified in TS 24.301 9.9.3.34, N1mode bit may not present. |  |  |  |

Table 11.4.12.3.3-4A: TRACKING AREA UPDATE REQUEST (step22a4 Table 11.4.12.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type | Any allowed value |  |  |
| NAS key set identifier | Any allowed value |  |  |
| UE network capability | Any allowed value |  |  |
| Additional update type | Not present or Any allowed value |  |  |
| UE additional security capability | Not present or Any allowed value |  |  |

Table 11.4.12.3.3-5: PDN CONNECTIVITY REQUEST (step 11, Table 11.4.12.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.7.3.20 with condition NOT(Interworking_with_5GS). |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Request type | '0100'B | emergency |  |
| Access point name | Not present |  |  |

Table 11.4.12.3.3-6: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 12, Table 11.4.12.3.2-1)

| Derivation path: TS 36.508 [7], Table 4.7.3-6 and table 4.6.1-8 with condition UM-DRB-ADD(2). |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | an additional EPS Bearer Id different from default EPS Bearer Id or/and any mapped EPS bearer |  |  |
| Access point name | sos | APN value as recommended by IR.88 clause 6.4 [39] |  |

<End of modified section 1>

<Start of modified section 2>

<End of modified section 2>