3GPP TSG-RAN5 Meeting #103	R5-243333r1

# Fukuoka, Japan, May 20th - May 24th 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0028 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update applicability for NB-NTN Demod TC 8.3.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Int |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test 2 is appliable to only UE supporting Non-Anchor mode of operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add applicability criteria for Test 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Applicability would be in-correct for Test 2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Remove WIC TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest and add correct WIC TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  |  |  |  |

<< Start of Changes >>

##### 8.3.1.1.1	Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone mode for category NB1 and NB2 under NTN fading conditions

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- Annex G.3.5 Minimum Test time is pending.

8.3.1.1.1.1	Test purpose

To verify the UE's ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for transmission on one antenna ports.

8.3.1.1.1.2	Test applicability

This test applies to all types of NB-IoT FDD UE release 17 and forward of category NB1 and NB2 that supports satellite access operation.

8.3.1.1.1.3	Minimum conformance requirements

The requirements are specified in Table 8.3.1.1.1.3-2, with the addition of the parameters in Table 8.3.1.1.1.3-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the performance.

Table 8.3.1.1.1.3-1: Test Parameters for NPDSCH under Standalone

| Parameter | Unit |  | Test 1, 2 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -99 (Note 2) |
| NPDCCH repetition number |  | subframe | 32 for Test 1; 128 for Test 2. |
| (npdcch-NumRepetitions-r13) |  | subframe | 64 for Test 1; 256 for Test 2. |
| (nPDCCH-startSF-USS-r13) |  |  | 1.5 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission. Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.3.1.1.1.3-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 70% | -4.7 | NB1, NB2 |
| 2 (Note 1) | 200kHz | Non-anchor | R.NB.2 FDD | 128 | NTN-TDLA100-10 | 1 | 1x1 | 70% | -10.6 | NB1, NB2 |
| Note 1: Applicable to UE supporting Non-Anchor mode of operation. |  |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.102[11] clause 8.3.1.1.1.1.

8.3.1.1.1.4	Test description

8.3.1.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [12] clause 4.1.

Frequencies to be tested: K.2.1.

Channel Bandwidths to be tested: As specified per test number in Table 8.3.1.1.1.3-2.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.9 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to Tables 8.3.1.1-1 and 8.3.1.1.1.3-1 as appropriate.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.0.1, H.1.1, H.2.1 and Annex H.4.2.

4.	Propagation conditions are set according to Annex B.0.

5.	UE location according to TS 36.508 [12] clause 8.2.6.1 is provided to the UE by any preconfigured means.

6.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 8.2.6.2.1-2 for NGSO (LEO-600). Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.2.6.3.1.

7.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

8.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 8.3.1.1.1.4.2.

8.3.1.1.1.4.2	Test procedure

1.	SS transmits NPDSCH via NPDCCH DCI format N1 for C_RNTI to transmit the DL RMC according to Tables 8.3.1.1.1.3-1 and 8.3.1.1.1.3-2.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table 8.3.1.1.1.5-1 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3.5 and G.3.6 in Annex G clause G.3.

4.	Repeat steps from 1 to 3 for each test interval in Tables 8.3.1.1.1.5-1 as appropriate.

8.3.1.1.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.5B and 8.1.6 with the following exceptions:

Table 8.3.1.1.1.4.3-1: Configure Non-anchor carrier in subtest 2

| Derivation Path: 36.508 Table 8.1.8.2.1.6-1 PhysicalConfigDedicated-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |  |
| carrierConfigDedicated-NB-r13 ::= SEQUENCE { |  |  | Non-anchor carrier |  |
| dl-CarrierConfig-r13::= SEQUENCE { |  |  |  |  |
| dl-CarrierFreq-r13 |  | Note 1 |  |  |
| downlinkBitmapNonAnchor-r13 |  | NULL |  |  |
| dl-GapNonAnchor-r13 |  | NULL |  |  |
| } |  |  |  |  |
| ul-CarrierConfig-r13 ::= SEQUENCE { |  |  |  |  |
| ul-CarrierFreq-r13 |  | Note 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| npdcch-ConfigDedicated-r13 |  | NPDCCH-ConfigDedicated-NB-DEFAULT | See subclause 8.1.6.3 in 36.508 |  |
| npusch-ConfigDedicated-r13 |  | NPUSCH-ConfigDedicated-NB-DEFAULT | See subclause 8.1.6.3 in 36.508 |  |
| uplinkPowerControlDedicated-r13 |  | UplinkPowerControlDedicated-NB-DEFAULT | See subclause 8.1.6.3 in 36.508 |  |
| } |  |  |  |  |
| Note 1: The frequency of Non-anchor carrier is located at 200KHz higher from the centre of the anchor carrier. |  |  |  |

Table 8.3.1.1.1.4.3-2: NPDCCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 36.508 Table 8.1.6.3-3 NPDCCH-ConfigDedicated-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| NPDCCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |  |
| npdcch-NumRepetitions-r13 |  | r64 for Test 1; r256 for Test 2. |  |  |
| npdcch-StartSF-USS-r13 |  | V1.5 |  |  |
| npdcch-Offset-USS-r13 |  | zero |  |  |
| } |  |  |  |  |

Table 8.3.1.1.1.4.3-3: NPUSCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 36.508 Table 8.1.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| NPUSCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |  |
| ack-NACK-NumRepetitions-r13 |  | R1 | Default |  |
| npusch-AllSymbols-r13 |  | TRUE | Default |  |
| groupHoppingDisabled-r13 |  | Not present | Default |  |
| } |  |  |  |  |

Table 8.3.1.1.1.4.3-4: NB-IoT Physical layer parameters for DCI format N1

| Derivation Path: 36.508 Table 8.1.3.6.1.2-2 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Repetition number | NRep = 32 |  | For Test Number 1 |
| Repetition number | NRep = 128 |  | For Test Number 2 |

8.3.1.1.1.5	Test requirement

Table 8.3.1.1.1.3-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in TS 36.102[11] Annex A clause A.1.1.1	for each throughput test shall meet or exceed the specified value in Table 8.3.1.1.1.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.3.1.1.1.5-1: Test requirements under standalone with 1 NRS ports

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 70% | --3.9 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.2 FDD | 128 | NTN-TDLA100-10 | 1 | 1x1 | 70% | --9.8 | NB1, NB2 |

<< End of Changes >>