3GPP TSG-RAN5 Meeting #103	R5-243445r2

# Fukuoka, Japan, 20th May 2024 – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4462 | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new RedCap HD-FDD Test Case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2024-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add new Test Case(TC) to cover: - If the UE supports half-duplex on one FDD band, the RedCap UE will bar the Cell which does not allow half-duplex RedCap UE in this FDD band. - If the UE supports half-duplex on one FDD band but supports full-duplex on the MFBI band,a RedCap UE can access to MFBI Cell which does not allow half-duplex RedCap UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new RedCap HD-FDD TC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirement for HD-FDD in the core spec would exist in any TC.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.29(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 38508-2 CR 0473 0636 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Test case is only for FDD so delete the TDD condition in the message content - In step 14a1 and 14b1,correct "px_NR_PrimaryBand=2 OR 5" to "px_NR_PrimaryBand=2 OR 25" . - Correct WIC in the coversheet. 2rd revision: 1. It is not appropriate to use px_NR_OverlappingSupportedBand_MFBI in this TC to indicate a band supported by UE, need a new PIXIT p x_NR_OverlappingSupportedBand_MFBI. 2. Rewording TP. For example move “the UE supporting half-duplex FDD operation on the band in the frequencyBandList.” in TP1 from “then” to “with”. 3. split the test case: 6.1.2.29 for TP1 tested on non-MFBI Cell and 6.1.2.29a for TP2/3 tested on MFBI Cell |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.1.2.29	Cell Selection / RedCap / halfDuplexRedCapAllowed

6.1.2.29.1	Test Purpose (TP)

(1)

with { UE supporting RedCap and supporting half-duplex FDD operation on the band in the frequencyBandList in NR RRC_IDLE state }

ensure that {

when { a cell fulfils the requirements for a suitable cell except halfDuplexRedCapAllowed is not present in SIB1 , and frequencyBandList only includes one band in SIB1 }

then { the UE considers the cell as barred and no camping on this cell can take place }

}

6.1.2.29.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.2.2.4.2.

[TS 38.331 clause 5.2.2.4.2]

Upon receiving the SIB1 the UE shall:

…

1>	if the UE is a RedCap UE and it is in RRC_IDLE or in RRC_INACTIVE, or if the RedCap UE is in RRC_CONNECTED while T311 is running:

2>	if intraFreqReselectionRedCap is not present in SIB1:

…

2> else:

…

3>	if the halfDuplexRedCapAllowed is not present in the acquired SIB1 and the UE supports only half-duplex FDD operation:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform barring based on intraFreqReselectionRedCap as specified in TS 38.304 [20], upon which the procedure ends;

…

1>	if in RRC_CONNECTED while T311 is not running:

…

1>	else:

…

2>	if the UE is not a RedCap UE, or for TDD if the UE is a RedCap UE, or for FDD if the UE is a RedCap UE and halfDuplexRedCapAllowed is present, or if the UE is a RedCap UE and the RedCap UE supports full-duplex FDD operation on this band:

3>	if neither trackingAreaCode nor trackingAreaList is provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];

3>	else if UE is IAB-MT and if iab-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list nor the selected SNPN nor the registered SNPN:

4>	consider the cell as barred in accordance with TS 38.304 [20];

3>	else:

…

4>	select the first frequency band in the frequencyBandList, for FDD from frequencyBandList for uplink, or for TDD from frequencyBandList for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present, and for RedCap UEs in FDD, if the halfDuplexRedCapAllowed is not present, for which the UE supports full-duplex FDD operation;

…

2>	else:

3>	consider the cell as barred in accordance with TS 38.304 [20]; and

3>	perform barring as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, is set to notAllowed;

6.1.2.29.3	Test description

6.1.2.29.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 11

-	TAI of NR Cell 1 is set to ''TAI-1'', TAI of NR Cell 11 is set to ''TAI-2''

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

None

Preamble:

-	The UE is in 1N-A state on NR Cell 1 using generic procedure parameter Connectivity (NR) and Test loop function(Off) according to TS 38.508-1 [4].

6.1.2.29.3.2	Test procedure sequence

Table 6.1.2.29.3.2-1 for FR1 and table 6.1.2.29.3.2-2 for FR2 illustrate the downlink power levels to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" are to be applied subsequently in the Main behaviour. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.1.2.29.3.2-1: Cell configuration changes over time for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -88 |  |

Table 6.1.2.29.3.2-2: Cell configuration changes over time for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | "Off" |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -82 |  |

Table 6.1.2.29.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS re-adjusts the SS/PBCH EPRE level of NR Cell 1 and NR Cell 11 according to row "T1" in table 6.1.2.29.3.2-1/2. | - | - | - | - |
| 2 | Check: Does the UE send an RRCSetupRequest on NR Cell 11 within the next 60 s? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 3 | SS re-adjusts the SS/PBCH EPRE level of NR Cell 1 and NR Cell 11 according to row "T0" in table 6.1.2.29.3.2-1/2. | - | - | - | - |
| 4 | Wait 60s to ensure UE select to NR Cell 1. | - | - | - | - |
| 5-13 | The generic test procedure in TS 38.508-1 [4] clause 4.9.4.2.2-1 on NR Cell 1. NOTE: The UE respond to paging and the RRC connection is released. | - | - | - | - |

6.1.2.29.3.3	Specific message contents

Table 6.1.2.29.3.3.3-1: SIB1 for NR Cell 11 (preamble and all steps, Table 6.1.2.29.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| RedCap-ConfigCommon-r17 SEQUENCE { |  |  |  |
| halfDuplexRedCapAllowed-r17 | Not present |  |  |
| cellBarredRedCap-r17 SEQUENCE { |  |  |  |
| cellBarredRedCap1Rx-r17 | Not Barred |  |  |
| cellBarredRedCap2Rx-r17 | Not Barred |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.1.2.29a	Cell Selection / RedCap / halfDuplexRedCapAllowed / MFBI

6.1.2.29a.1	Test Purpose (TP)

(1)

with { UE supporting RedCap and supporting half-duplex FDD operation on all the bands in the frequencyBandList and in NR RRC_IDLE state }

ensure that {

when { a MFBI cell fulfils the requirements for a suitable cell except halfDuplexRedCapAllowed is not present in SIB1 , and frequencyBandList includes two bands in SIB1. }

then { the UE considers the cell as barred and no camping on this cell can take place }

}

(2)

with { UE supporting RedCap and supporting only half-duplex FDD operation on the first band in the frequencyBandList and supporting full-duplex FDD operation on the second band in the frequencyBandList and in NR RRC_IDLE state }

ensure that {

when { a MFBI cell fulfils the requirements for a suitable cell except halfDuplexRedCapAllowed is not present in SIB1 , and frequencyBandList includes two bands in SIB1. }

then { the UE considers the cell as not barred and camps on it }

}

6.1.2.29a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.2.2.4.2.

[TS 38.331 clause 5.2.2.4.2]

Upon receiving the SIB1 the UE shall:

…

1>	if the UE is a RedCap UE and it is in RRC_IDLE or in RRC_INACTIVE, or if the RedCap UE is in RRC_CONNECTED while T311 is running:

2>	if intraFreqReselectionRedCap is not present in SIB1:

…

2> else:

…

3>	if the halfDuplexRedCapAllowed is not present in the acquired SIB1 and the UE supports only half-duplex FDD operation:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform barring based on intraFreqReselectionRedCap as specified in TS 38.304 [20], upon which the procedure ends;

…

1>	if in RRC_CONNECTED while T311 is not running:

…

1>	else:

…

2>	if the UE is not a RedCap UE, or for TDD if the UE is a RedCap UE, or for FDD if the UE is a RedCap UE and halfDuplexRedCapAllowed is present, or if the UE is a RedCap UE and the RedCap UE supports full-duplex FDD operation on this band:

3>	if neither trackingAreaCode nor trackingAreaList is provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];

3>	else if UE is IAB-MT and if iab-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list nor the selected SNPN nor the registered SNPN:

4>	consider the cell as barred in accordance with TS 38.304 [20];

3>	else:

…

4>	select the first frequency band in the frequencyBandList, for FDD from frequencyBandList for uplink, or for TDD from frequencyBandList for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present, and for RedCap UEs in FDD, if the halfDuplexRedCapAllowed is not present, for which the UE supports full-duplex FDD operation;

…

2>	else:

3>	consider the cell as barred in accordance with TS 38.304 [20]; and

3>	perform barring as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, is set to notAllowed;

6.1.2.29a.3	Test description

6.1.2.29a.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 11

-	TAI of NR Cell 1 is set to ''TAI-1'', TAI of NR Cell 11 is set to ''TAI-2''

-	NR Cell 11 is MFBI capable cell

-	NR Cell 11 belongs to the frequency which overlaps between bands controlled by IXITs px_NR_OverlappingSupportedBand_MFBI and px_NR_PrimaryBand.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	UE supports px_NR_OverlappingSupportedBand_MFBI

Preamble:

-	The UE is in 1N-A state on NR Cell 1 using generic procedure parameter Connectivity (NR) and Test loop function(Off) according to TS 38.508-1 [4].

6.1.2.29a.3.2	Test procedure sequence

Table 6.1.2.29a.3.2-1 for FR1 and table 6.1.2.29a.3.2-2 for FR2 illustrate the downlink power levels to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" are to be applied subsequently in the Main behaviour. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.1.2.29a.3.2-1: Cell configuration changes over time for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -88 |  |

Table 6.1.2.29a.3.2-2: Cell configuration changes over time for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | "Off" |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -82 |  |

Table 6.1.2.29a.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1 to 1b7 describe behaviour that depends on the band undertest and UE capabilities; the "lower case letter" identifies a step sequence that take place if a specific prior event takes place. | - | - | - | - |
| 1a1 | IF pc_halfDuplexFDD_TypeA_RedCap_MFBI= True, THEN SS re-adjusts the SS/PBCH EPRE level of NR Cell 1, NR Cell 11 according to row "T1" in table 6.1.2.29a.3.2-1/2. | - | - | - | - |
| 1a2 | Check: Does the UE send an RRCSetupRequest on NR Cell 11 within the next 60 s? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 1a3 | SS re-adjusts the SS/PBCH EPRE level of NR Cell 1 and NR Cell 11 according to row "T0" in table 6.1.2.29a.3.2-1/2. | - | - | - | - |
| 1a4 | Wait 60s to ensure UE select to NR Cell 1. | - | - | - | - |
| 1a5-1a13 | The generic test procedure in TS 38.508-1 [4] clause 4.9.4.2.2-1 on NR Cell 1. NOTE: The UE respond to paging and the RRC connection is released. | - | - | - | - |
| 1b1 | ELSE IF pc_halfDuplexFDD_TypeA_RedCap_MFBI= False THEN SS re-adjusts the SS/PBCH EPRE level of NR Cell 1, NR Cell 11 according to row "T1" in table 6.1.2.29a.3.2-1/2. | - | - | - | - |
| 1b2-1b7 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] clause 4.9.5 indicate that the UE is camped on NR Cell 11? NOTE: The RRC connection is released and UE is in RRC_IDLE mode. | - | - | 2 | - |

6.1.2.29a.3.3	Specific message contents

Table 6.1.2.29a.3.3.3-1: SIB1 for NR Cell 11 (preamble and all steps, Table 6.1.2.29a.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommonSIB | Table 6.1.2.29a.3.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| RedCap-ConfigCommon-r17 SEQUENCE { |  |  |  |
| halfDuplexRedCapAllowed-r17 | Not present |  |  |
| cellBarredRedCap-r17 SEQUENCE { |  |  |  |
| cellBarredRedCap1Rx-r17 | Not Barred |  |  |
| cellBarredRedCap2Rx-r17 | Not Barred |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.29a.3.3.3-2: ServingCellConfigCommonSIB (Table 6.1.2.29a.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 6.1.2.29a.3.3.3-3 |  |
| uplinkConfigCommon | UplinkConfigCommonSIB | Table 6.1.2.29a.3.3.3-5 |  |
| } |  |  |  |

Table 6.1.2.29a.3.3.3-3: DownlinkConfigCommonSIB (Table 6.1.2.29a.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| frequencyInfoDL | FrequencyInfoDL-SIB | Table 6.1.2.29a.3.3.3-4 |  |

Table 6.1.2.29a.3.3.3-4: FrequencyInfoDL-SIB (Table 6.1.2.29a.3.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-60 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoDL-SIB ::= SEQUENCE { |  |  |  |
| frequencyBandList SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-MultiBandInfo { | 2 entries |  |  |
| NR-MultiBandInfo[1] SEQUENCE { |  | entry 1 |  |
| freqBandIndicatorNR | An overlapping Band under test (px_NR_PrimaryBand). |  |  |
| nr-NS-PmaxList | NR-NS-PmaxList |  |  |
| } |  |  |  |
| NR-MultiBandInfo[2] SEQUENCE { |  | entry 2 |  |
| freqBandIndicatorNR | An overlapping frequency band MFBI under test (px_NR_OverlappingSupportedBand_MFBI). |  |  |
| nr-NS-PmaxList | NR-NS-PmaxList |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.29a.3.3.3-5: UplinkConfigCommonSIB (Table 6.1.2.29a.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-202 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkConfigCommonSIB SEQUENCE { |  |  |  |
| frequencyInfoUL | FrequencyInfoUL-SIB | Table 6.1.2.29a.3.3.3-6 |  |
| } |  |  |  |

Table 6.1.2.29a.3.3.3-6: FrequencyInfoUL-SIB (Table 6.1.2.29a.3.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-62 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB ::= SEQUENCE { |  |  |  |
| frequencyBandList SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-MultiBandInfo { | 2 entries |  |  |
| NR-MultiBandInfo[1] SEQUENCE { |  | entry 1 |  |
| freqBandIndicatorNR | An overlapping Band under test (px_NR_PrimaryBand). |  |  |
| nr-NS-PmaxList | NR-NS-PmaxList |  |  |
| } |  |  |  |
| NR-MultiBandInfo[2] SEQUENCE { |  | entry 2 |  |
| freqBandIndicatorNR | An overlapping frequency band MFBI under test (px_NR_OverlappingSupportedBand_MFBI). |  |  |
| nr-NS-PmaxList | NR-NS-PmaxList |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>